3GPP TSG RAN WG1 Meeting #97 	R1-1906233

Reno, US, May 13 - 17, 2019

Agenda Item:	7.2.11.4

Source:	NTT DOCOMO, INC.

Title:	Proposal for additional delay for absolute time arrival modeling in NLOS environment

Document for:	Discussion

Introduction

In email discussion on SI on Channel modeling for Indoor Industrial scenarios, way forward in which absolute time of arrival of multipath components is added as an additional modeling component was agreed [1, 2].  We proposed the model which is sum of delay time calculated by distance between BS and MS, and random variables of Gaussian distribution based on measurements in our lab [5]. On the other hand, [6] proposed the model which utilizes random variables of log normal distribution [6]. In this contribution, we study which model is more appropriate among the above two models.

Measurement

Delay profile measurements were performed in our lab and absolute time of arrival of first wave in NLOS environment is studied．Measurement environment is same as the environment which is described in our contribution regarding delay spread at RAN1#96 [3, 4]. Figure 1 shows measurement route and Table 1 shows measurement parameters. Figure 2 shows the distribution of measured delay spreads in NLOS environment respectively.

Fig. 2 Distribution of measured delay spread in NLOS environment

Fig. 3 Distribution of Δτ in NLOS environment

Fig. 4 Histogram of Δτ in NLOS environment

In this contribution, channel model for absolute additional delay in NLOS environment is studied. As a starting point, the model is assumed as

τ0= d3Dc+ Δτ		                                                                                                                              (1)

τ0 is absolute time of arrival of first wave (s), d3D is 3D distance between BS and MS (m), c is speed of light (m/s) and Δτ is additional delay (s). In [5], τ0 are obtained from measurement and the distribution of Δτ is studied. Figure 3 and Fig. 4 shows CDF and histogram of Δτ respectively. From results, the distribution can be approximated as Gaussian distribution. And median is 55 (ns) and standard deviation is 28 (ns).

Fig. 5 Distribution of lgΔτ in NLOS environment

Fig. 6 Histogram of lgΔτ in NLOS environment

On the other hand, [6] reports log of  Δτ is Gaussian distribution based on ray-tracing simulation. Therefore, we apply same analysis to our measurement results in this contribution. In following analysis,

lgΔτ =log10(Δτ/1s) 																	  (2)

is defined. Figure 5 shows CDF of lgΔτ and Fig. 6 shows histogram of lgΔτ. From results, lgΔτ is more close to Gaussian distribution. Median of lgΔτ is -7.3 and standard deviation of lgΔτ is 0.22. From these analysis results the following proposal can be made.

Proposal: The following two models can be considered for absolute delay time in NLOS environment. It can be considered to down select from these two models based on further studies on measurement.

Model 1

Absolute time of arrival in NLOS environment is given by

τ0=max⁡(d3Dc, τ0')                                                                                                                                               (3a)

τ0'= d3Dc+ Δτ                                                                                                                                        (3b)

where d3D is 3D distance between BS and MS (m), c is speed of light (m/s) and Δτ  (s) is additional delay which is Δτ ~N(Δτmed, σΔπ). Value of Δτmed and σΔπ is FFS.

Model 2

Absolute time of arrival in NLOS environment is given by

τ0=max⁡(d3Dc, τ0')                                                                                                                                               (4a)

τ0'= d3Dc+ Δτ                                                                                                                                        (4b)

lgΔτ =log10(Δτ/1s)     																(4c)

where d3D is 3D distance between BS and MS (m), c is speed of light (m/s) and Δτ  (s) is additional delay which is lgΔτ ~N (lgΔτmed, σlgΔτ). Value of lgΔτmed and σlgΔτ is FFS.

Conclusion

In this contribution, channel model for absolute additional delay in NLOS environment is studied. The followings are our proposals based on the discussion.

Proposal: The following two models can be considered for absolute delay time in NLOS environment. It can be considered to down select from these two models based on further studies on measurement.

Model 1

Absolute time of arrival in NLOS environment is given by

τ0=max⁡(d3Dc, τ0')                                                                                                                                               (3a)

τ0'= d3Dc+ Δτ                                                                                                                                        (3b)

where d3D is 3D distance between BS and MS (m), c is speed of light (m/s) and Δτ  (s) is additional delay which is Δτ ~N(Δτmed, σΔπ). Value of Δτmed and σΔπ is FFS.

Model 2

Absolute time of arrival in NLOS environment is given by

τ0=max⁡(d3Dc, τ0')                                                                                                                                               (4a)

τ0'= d3Dc+ Δτ                                                                                                                                        (4b)

lgΔτ =log10(Δτ/1s)     																(4c)

where d3D is 3D distance between BS and MS (m), c is speed of light (m/s) and Δτ  (s) is additional delay which is lgΔτ ~N (lgΔτmed, σlgΔτ). Value of lgΔτmed and σlgΔτ is FFS.

- References

RP-182138, SID on Channel Modeling for Indoor Industrial Scenarios, Ericsson, 3GPP TSG-RAN Meeting #81, Gold Coast, Australia, September 10th – 13th 2018.

R1-1903124, Email discussion on measurements and observations, Ericsson, 3GPP TSG-RAN Meeting #96, Athens, Greece, February 25th – March 1st, 2019

R1-1813337, On accuracy of InH path loss model of 38.901 for factory scenario, NTT DOCOMO, INC., Spokane, WA, USA, November 12th – 16th, 2018.

R1-1902817, Discussion on delay spread for factory scenario, NTT DOCOMO, INC., 3GPP TSG-RAN Meeting #96, Athens, Greece, February 25th – March 1st, 2019.

R1-1902817, Proposal for additional delay for absolute time arrival modeling in NLOS environment, NTT DOCOMO, INC., Fraunhofer HHI and Rohde & Schwarz, 3GPP TSG-RAN Meeting #97 bis, Xi'an, China, Apr. 8th – 12th, 2019.

R1-1904118, Absolute time of arrival model in IIOT scenario, ZTE, Sanechips, BJTU, Tongji University, 3GPP TSG-RAN Meeting #97 bis, Xi'an, China, Apr. 8th – 12th, 2019.