3GPP TSG RAN WG1 Meeting #97 R1-1906262 Reno, USA, 13th – 17th May, 2019 CR-Form-v12.0. CHANGE REQUEST. 38.211; CR; DRAFT; rev; -; Current version:; 15.5.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on frequency domain shift of SRS. Source to WG:; ZTE. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-5-3. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In the section of 6.4.1.4.3 in TS38.211, the reference point for k0(pi)=0 was defined based on the value of SRS frequency domain shift in RAN#83, but the reference point of is not updated accordingly in the following paragraph.. Summary of change:; The common resource block grid is replaced by the reference point.. Consequences if not approved:; The reference point of is not correct.. Clauses affected:; 6.4.1.4.3. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR just fixes the misalignment between two paragraphs, there is no impact on implementation.. This CR's revision history:; This is the first version for this CR.. <Unchanged part omitted> 6.4.1.4.3 Mapping to physical resources When SRS is transmitted on a given SRS resource, the sequence rpi(n,l') for each OFDM symbol and for each of the antenna ports of the SRS resource shall be multiplied with the amplitude scaling factor in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with to resource elements in a slot for each of the antenna ports according to The length of the sounding reference signal sequence is given by where is given by a selected row of Table 6.4.1.4.3-1 with where is given by the field b-SRS contained in the higher-layer parameter freqHopping. The row of the table is selected according to the index given by the field c-SRS contained in the higher-layer parameter freqHopping. The frequency-domain starting position k0(pi) is defined by where If NBWPstart≤nshift the reference point for k0(pi)=0 is subcarrier 0 in common resource block 0, otherwise the reference point is the lowest subcarrier of the BWP. The frequency domain shift value adjusts the SRS allocation with respect to the reference point and is contained in the higher-layer parameter freqDomainShift in the SRS-Config IE. The transmission comb offset is contained in the higher-layer parameter transmissionComb in the SRS-Config IE and is a frequency position index. Frequency hopping of the sounding reference signal is configured by the parameter , given by the field b-hop contained in the higher-layer parameter freqHopping. If , frequency hopping is disabled and the frequency position index remains constant (unless re-configured) and is defined by for all OFDM symbols of the SRS resource. The quantity is given by the higher-layer parameter freqDomainPosition and the values of and for are given by the selected row of Table 6.4.1.4.3-1 corresponding to the configured value of . If , frequency hopping is enabled and the frequency position indices are defined by where Nb is given by Table 6.4.1.4.3-1, and where regardless of the value of . The quantity counts the number of SRS transmissions. For the case of an SRS resource configured as aperiodic by the higher-layer parameter resourceType, it is given by within the slot in which the symbol SRS resource is transmitted. The quantity is the repetition factor given by the field repetitionFactor contained in the higher-layer parameter resourceMapping. For the case of an SRS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, the SRS counter is given by for slots that satisfy . The periodicity in slots and slot offset are given in clause 6.4.1.4.4.

<Unchanged part omitted>