3GPP TSG RAN WG1 #97		                                                      R1-1906266

Reno, USA, May 13th – 17th, 2019

Source:	vivo, ZTE

Title:	Link budget evaluation for IMT-2020 self-evaluation

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

- Introduction

According to Report ITU-R M.2411 [1], it is required to provide the link budget template to ITU-R. The final submission time is in June 2019. To support final submission, we additionally provide link budget results on frame structure configuration ‘DDDDD DDSUU’ in this contribution, as the frame structure will be deployed with high probability in future commercial network.

- Discussion

The evaluation results on frame structure configuration ‘DDDDD DDSUU’ for Indoor Hotspot- eMBB, Dense Urban-eMBB and Rural-eMBB scenarios can be found in the accompanying attachment. To align with future commercial use case, updating the additional link budget results in TR 37.919 can be considered.

Attachment:

- DDDDD DDSUU-Link budget template - Channel Model A

- DDDDD DDSUU-Link budget template - Channel Model B

Proposal 1:

- Support to add the additional link budget results in TR 37.919 for final IMT-2020 submission.

The detailed evaluation assumptions on frame structure configuration ‘DDDDD DDSUU’ are listed in below Table 1.

Table 1 Evaluation assumptions for link budget

| Configuration | Values |
| --- | --- |
| Test environment | Indoor Hotspot-eMBB Dense Urban-eMBB Rural-eMBB |
| System configuration | NR 4GHz with 30kHz SCS for Indoor Hotspot-eMBB and Dense Urban-eMBB NR 700MHz with 30kHz SCS for Rural-eMBB |
| Subframe configuration | DDDDD DDSUU |
| Physical channel | DL: PDCCH, PDSCH UL: PUCCH, PUSCH |
| Transmission bit data for date channel | 3 times of the IMT-Advance’s evaluation assumption |
| Spectral efficiency | Calculated based on the transmission bit rate and bandwidth. |
| Cable loss for BS | 1dB or 3dB |
| DL occupied channel bandwidth | For TDD：20MHz |
| UL occupied channel bandwidth | 1 PRB for control channel and 4 PRB for data channel (the minimum granularity for scheduling in frequency domain is 4PRB) |
| Channel state | NLOS O2I NLOS ( For Dense Urban-eMBB and Rural-eMBB) |

- Conclusion

In this contribution, link budget results on frame structure configuration ‘DDDDD DDSUU’ are provided with the following proposal.

Proposal 1:

- Support to add the additional link budget results in TR 37.919 for final IMT-2020 submission.

- References

- Report ITU-R M.2411, “Requirements, evaluation criteria and submission templates for the development of IMT-2020”, Nov. 2017