3GPP TSG RAN WG1 #97		R1-1906269

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.4.2.2

Source:	Lenovo, Motorola Mobility

Title:	Discussion on resource allocation for NR sidelink Mode 2

Document for:	Discussion

Introduction

In RAN#83 Plenary, a new WID on 5G V2X with NR sidelink was approved with the following resource allocation Mode 2 related objectives [1]:

- Mode 2 - Sensing and resource selection procedures based on sidelink pre-configuration and configuration by NR Uu and LTE Uu as per the study outcome

During the RAN1#96bis and RAN1#95, regarding resource allocation, the related agreements were made as follows:

Agreements:

- NR V2X supports an initial transmission of a TB without reservation, based on sensing and resource selection procedure

- NR V2X supports reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB, based on sensing and resource selection procedure

  - This functionality can be enabled/disabled by (pre-)configuration

- FFS Standalone PSCCH transmissions for resource reservations are supported in NR V2X

In this contribution, we provide our views on sensing and resource selection/reservation mechanism in Mode 2 for NR sidelink.

Discussion

Sensing and resource selection for periodic traffic and aperiodic traffic

In LTE V2X, in UE autonomous resource selection mode (i.e. mode 4), a long duration sensing-based resource selection mechanism was introduced to support semi-persistent transmission for periodic traffic. In NR V2X, it is noted that not all 5G V2X services are periodic. Some event-triggered, aperiodic traffic like pre-crash warning message are more important than periodic transmission. Event-triggered traffic is not predictable. For example, in case of emergency, the brake message is generated. It is critical to transmit it fast and reliably. Therefore, for UE determines sidelink transmission resource(s) mode (i.e., Mode 2), the legacy long-term sensing mechanism can be used for periodic traffic, meanwhile a fast sensing mechanism is needed for aperiodic traffic so that the generated message can be delivered timely. Further considering the periodic and aperiodic traffics are transmitted in shared pool. We can consider to combine long-term sensing and fast sensing mechanism. For example, the fast sensing can be used to select resource based on long-term sensing result.

Proposal 1:

- For periodic traffic, legacy long-term sensing mechanism can be used.

- For aperiodic traffic, fast sensing mechanism is needed.

- Consider to combine long-term sensing and fast sensing mechanism for periodic and aperiodic transmitted in shared pool.

Sensing mechanism

In LTE V2X for long term sensing mechanism, Tx UE can identify occupied/reserved sidelink resources by decoding SCI in unicast, groupcast and broadcast case. It can be reused in NR V2X to assign resource for periodic traffic with fixed packet size. Potential enhancement should focus on the periodic traffic with various packet size.

For fast sensing mechanism, there are two options can be considered as followed:

- Option 1: Tx UE performs one shot sensing (i.e., Category 2: LBT without random back-off) on a predefined resource, e.g., the last symbol of timeslot, for detecting a pre-emption indicator or a reservation indicator.

- Option 2: Tx UE performs LBT with random back-off (i.e., Category 3: LBT with random back-off with fixed size of contention window, or Category 4: LBT with random back-off with variable size of contention window). In this way, besides control and data region, define a sensing region is needed. Different traffic types can have different sensing region sizes corresponding to their QoS requirements. It is beneficial to avoid lower priority traffic to select the resource used by higher priority traffic. Comparing to option 1, LBT mechanism with random back-off can further help to avoid resource selection collision between traffics with same QoS requirement (e.g., multiple aperiodic traffics). The overhead of sensing region need to be considered with considering the resource utilization efficiency.

Proposal 2:

- Reuse LTE-V2X long term sensing mechanism for periodic traffic in NR V2X.

  - FFS: enhancements for periodic traffic with various packet size

- Following fast sensing mechanisms should be considered for NR V2X:

  - One shot sensing

  - LBT with random back-off

Resource selection mechanism

Based on above discussed sensing mechanism, the UE with periodic traffic performs long term sensing to select and reserve resource for SPS transmission. For the UE with aperiodic traffic, traffic generation is uncertain. If the UE with aperiodic traffic is not allowed to pre-empte the in use or reserved resource for periodic traffic, it can perform a long term sensing to avoid collision with periodic traffic firstly, and then perform a fast sensing to further avoid collision with aperiodic traffic. But when the traffic load is high in a shared resource pool, the UE with aperiodic traffic may not select an available resource for sidelink transmission, even if the aperiodic traffic has lower latency and higher reliability requirement. In contrast, if the resource selection mechanism based on some priority rule, the UE with aperiodic traffic may always have higher transmission priority to meet the lower latency and higher reliability requirement. It can perform a long term sensing and fast sensing to select an available resource for sidelink transmission. If there is no suitable resource for aperiodic traffic transmission due to large packet size or heavy load in shared pool. The UE with aperiodic traffic can pre-empt the in use or reserved resource for periodic traffic based on priority/QoS comparison. In this way, besides designing pre-emption indication, it also needs to consider the collision avoidance among multiple pre-emption UEs and resource reselection mechanism for the pre-empted UE.

Based on above discussion, when periodic traffic and aperiodic traffic are transmitted in shared resource pool, for example, pre-emption mechanism and resource reselection mechanism should be considered.

Proposal 3: Consider pre-emption mechanism and resource reselection mechanism, when periodic traffic and aperiodic traffic are transmitted in shared resource pool.

Resource reservation for sidelink transmission

In previous meeting, RAN1 has agreed that NR V2X supports reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB. It can ensure Tx UE occupies a set of sidelink resources for a certain time duration. The other UE who performs sensing based resource selection can avoid to select the occupied resources. This mechanism is same as the sensing based resource selection and reservation in LTE V2X. It is beneficial to periodic SPS traffic.

In addition, in LTE V2X, the resource reservation mechanism also includes the reservation for initial transmission and retransmission of one TB. The different time offset between initial transmission of a TB and its associated blind re-transmission of the same TB can further help to avoid collisions among the transmissions from multiple UEs. For example, if two UEs select a set of resources for SPS transmissions, only initial transmission of a TB or retransmission of the TB could have collision, UEs can detect this potential collision based on receiving the other transmission of the TB.

In NR V2X, besides above resource reservation mechanism for SPS transmissions, we also need to discuss whether design resource reservation mechanism can avoid the collision between dynamic transmission and dynamic/periodic transmission, i.e., standalone reservation transmissions for resource reservations.

Reservation of a resource for an initial transmission

Reservation of a resource for dynamic/initial transmission is used to indicate the UEs who perform sensing which resource has been already selected. In this way, the UEs will not select the reserved resource, i.e., the UEs don’t use the resource reserved by the other UEs. As shown in Figure 1, UE1 send a reservation transmission to reserve its control/data transmission. UE2 and UE3 detect the reservation transmission and avoid to select reserved resource by UE1.

Figure 1, UE2 and UE3 avoid to select the reserved resource by UE1

Observation 1: Reservation indication can avoid the collision in some extent.

But if multiple UEs send the reservation indications at same timeslot or cannot detect the reservation indications each other, there may be control/data transmission collisions on full or partial overlapped resource. As shown in Figure 2, the reservation indication has a predefined time offset with its associated reserved resource for control/data transmission. UE1, UE2 and UE3 send the reservation indications at same timeslot with/without overlapped resource in frequency domain, and their reserved resource for control/data transmissions will have collision. Even if predefined time offset between reservation indication and its reserved resource is associated with QoS/priority level of control/data transmission, it still cannot avoid the control/data transmission collision among multiple UEs with same QoS/priority level.

Figure 2, Control/data transmissions fromUE1, UE2 and UE3 have collisions

Observation 2: Control/data transmission collision cannot be avoided when their associated reservation indications have collision.

As mentioned in section 2.2, if a sensing region is defined for reservation indication transmission, UE can send the reservation indication with random back-off in sensing region. It can avoid the transmission collision of reservation indications among multiple UEs and further avoid the collision of control/data transmissions which are reserved by the reservation indications. As shown in Figure 3, UE1, UE2 and UE3 send the reservation indications in defined sensing region, and they can detect the reservation indications each other. Based on some priority rule, UE1 may get the reserved resource for its control/data transmission without collision. In this way, the fast sensing mechanism is used to avoid the collision of reservation indication transmission. The overhead of sensing region need to be considered with considering control/data transmission delay and the resource utilization efficiency.

Figure 3, UE 1, UE2 and UE3 send the reservation indications in sensing region

Proposal 4: Fast sensing mechanism is considered to avoid the collision of reservation indication transmission with considerations on control/data transmission delay and resource utilization efficiency

Reservation of resources for feedback-based HARQ retransmission(s)

Reservation of resource for feedback-based HARQ transmission(s) can provide retransmission resource for Tx UE timely when NACK is received from Rx UE in unicast/groupcast case. If there is no reserved resource for retransmission, Tx UE could request a retransmission resource from network or autonomously select a resource for retransmission. Both of them may increase the delay of retransmission. Reserving the retransmission resource can reduce this delay, but it has issue on resource utilization efficiency. In other words, the probability for initial transmission is at least 90% probability, so the reserved resource only for retransmission will not be used in most instances.

If the delay of request or autonomously select re-transmission resource is unacceptable, reservation of resources for retransmission is needed. In addition, resource utilization efficiency for resource reservation should be taken into account, and conditional resource reservation could also be considered (e.g. based on channel status, CBR etc). How to utilize the unused re-transmission resource can be considered from the following aspects:

Option 1: Tx UE uses the re-transmission resource for new TB transmission

Option 2: Rx UE uses the re-transmission resource

Option 3: The others UEs, except Tx UE and Rx UE, use the re-transmission resource

Option 4: Conditionally reserve re-transmission resource (e.g. based on channel status, CBR etc)

Proposal 5: If resource reservation for HARQ retransmission is supported, how to utilize the unused re-transmission resource and feasibility of conditional resource reservation should be considered.

Conclusion

In this contribution, we focus on sensing and resource selection mechanism in NR V2X Mode 2 and present our views. We have the following proposals:

Proposal 1:

- For periodic traffic, legacy long-term sensing mechanism can be used.

- For aperiodic traffic, fast sensing mechanism is needed.

- Consider to combine long-term sensing and fast sensing mechanism for periodic and aperiodic transmitted in shared pool.

Proposal 2:

- Reuse LTE-V2X long term sensing mechanism for periodic traffic in NR V2X.

  - FFS: enhancements for periodic traffic with various packet size

- Following fast sensing mechanisms should be considered for NR V2X:

  - One shot sensing

  - LBT with random back-off

Proposal 3: Consider pre-emption mechanism and resource reselection mechanism, when periodic traffic and aperiodic traffic are transmitted in shared resource pool.

Observation 1: Reservation indication can avoid the collision in some extent.

Observation 2: Control/data transmission collision cannot be avoided when their associated reservation indications have collision.

Proposal 4: Fast sensing mechanism is considered to avoid the collision of reservation indication transmission with considerations on control/data transmission delay and resource utilization efficiency

Proposal 5: If resource reservation for HARQ retransmission is supported, how to utilize the unused re-transmission resource and feasibility of conditional resource reservation should be considered.

- References

- RP-190766, New WID on 5G V2X with NR sidelink