3GPP TSG RAN WG1 Meeting #97                                                                      	     R1-1906280

Reno, USA, 13rd – 17th May, 2019

Agenda Item:	6.2.2.2

Source:	Lenovo, Motorola Mobility

Title:	UL transmission in preconfigured resources for NBIoT

Document for:	Discussion and decision

Introduction

The contribution is the update from R1-1904569.

In the RAN#80 plenary meeting, a new work item for Rel.16 NBIoT is approved. One of the objectives is to specify the uplink transmission based on preconfigured resource for NBIoT UEs.

Improved UL transmission efficiency and/or UE power consumption:

- Specify support for transmission in preconfigured resources in idle and/or connected mode based on SC-FDMA waveform for UEs with a valid timing advance [RAN1, RAN2, RAN4]

  - Both shared resources and dedicated resources can be discussed

Note: This is limited to orthogonal (multi) access schemes

In the last RAN1 meeting, the following agreements are achieved for the design of uplink transmission based on preconfigured resource for UE power saving:

Agreement

In idle mode, a UE can be configured such that TA is always valid within a given cell.

- Up to RAN2 how to implement

  - e.g. PUR Time Alignment Timer or NRSRP Threshold = infinity

Agreement

The value(s) of NRSRP threshold(s) is UE specific

Agreement

The UE monitors the NPDCCH for at least a time period after a PUR transmission.

- FFS: Details of the time period

- FFS: UE behaviour if nothing is received in that time period.

- FFS: If and how often UE monitors NPDCCH after a PUR allocation in which it has not transmitted

Agreement

Reuse existing field(s) of DCI format N0 to convey the dedicated PUR ACK

Agreement

After data transmission on PUR, upon unsuccessful decoding by eNB, the UE can expect an UL grant for retransmission on NPDCCH. Other behaviors are FFS.

- Working Assumption#1

- In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

- FFS: Which PUR configurations and PUR parameters will be signaled via L1

- FFS: Definition of PUR configurations and PUR parameters

The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted.

- Working Assumption#2

For dedicated PUR

- During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE

  - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived

- FFS if the UE monitors any additional RNTI which may be shared with other UEs.

- Note: The same RNTI may be used over non-overlapping time and/or frequency resources

Send an LS to RAN2 to include two above working assumptions. Ask whether the first bullet in working assumption #2 is feasible. If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed. (LS is endorsed in eMTC agenda item)

Agreement

For dedicated PUR in idle mode, the PUR configuration is configured by UE-specific RRC signaling.

In this contribution, detailed considerations of transmissions in pre-configured UL resources, especially TA issues and HARQ related issue based on dedicated PUR in IDLE mode for Rel.16 NBIoT are presented.

Discussion

Dedicated PUR Issues

HARQ

- In the RAN1 #95 meeting, it was agreed that if multi-TB grant is not enabled, a dedicated PUR allocation is associated to only a single TB and a single HARQ process. However, the detail of the scheduling multiple TBs in Rel.16 is not finalized at this stage, especially the HARQ, DCI design, which is even another story for transmission in IDLE mode. So we propose to move supporting multiple HARQ process number with multi-TB grant on PUR transmission to later release.

- Proposal 1: For uplink transmission based dedicated PUR, only single TB and single HARQ process are supported in Rel.16.

- When UE is being assigned with PUR resource, e.g., time/frequency resource, it is also necessary to assign a dedicate RNTI for UE to monitor HARQ feedback. After uplink transmission based PUR, UE start to monitor the (N)PDCCH with the dedicate RNTI expecting an explicit ACK upon successful decoding by eNB or explicit NACK/an UL retransmission GRANT upon unsuccessful decoding by eNB.

- Proposal 2: After transmission based PUR, UE start to monitor the (N)PDCCH with the dedicate RNTI expecting an explicit ACK upon successful decoding by eNB or an explicit NACK/an UL retransmission GRANT upon unsuccessful decoding by eNB.

- In response to the ACK, the UE could continue new transmission in the next PUR resource or request for the release of PUR. In response to NACK (e.g., explicit NACK, or the UL retransmission GRANT), the UE could retransmit in the configured new grant resource or wait till the next PUR transmission to send a retransmission or fallback to legacy RACH/EDT.

- Proposal 3: In response to NACK, the UE could then retransmit in configured new grant resource or wait till the next PUR transmission or fallback to legacy RACH/EDT.

PUR Search Space

For connected mode UEs, UE-specific NPDCCH can be used for monitoring HARQ feedback after uplink transmission. However, for IDLE mode UEs, there are at least two types of search space, and the necessity of introduce new UE-specific search space for dedicated PUR is widely discussed for the search space for PUR.

Solution 1:  Modified common search space

Considering the eNB resource efficiency and UE power saving, in IDLE mode, it is natural to use NPDCCH common search space as starting point, e.g., NPDCCH type 1 common search space for paging and type 2 NPDCCH common search space for RAR. Due to the similarity of uplink transmission based PUR and RACH procedure, it is preferable to use type 2 common search space as baseline. In order to further reduce blind detection for different UE (e.g., UE with different coverage/potential repetitions), dedicated (reduced) search space based type 2 common search space can be newly designed for different UEs.

Solution 2: New UE-specific search space

Considering the scheduling flexibility and HARQ feedback delay, A modified type 2-like common search space can be a kind of new UE-specific search space for UE to monitor after uplink transmission based on dedicated PUR. The new search space can be periodic and start with an absolute time offset for dedicates UE, and a PUR search window is configured. In order to reduce power consumption, only search space after the PUR transmission until the end of the short PUR search window will be monitored by UE and UE is not expected to monitor the PUR search space in case skipping uplink transmission based on PUR.

- The PUR search space is associated with a particular PUR resource. In order to align the search space to different PUR resources for different UEs; the starting position of PUR search space can be configured as an absolute subframe similar as type 2 NPDCCH common search space. If the PUR resource itself is configured as an absolute subframe, the starting position of PUR search space can also be relative to the end of the PUR resource (PUR transmission). For the PUR search space configuration, it further includes maximal repetition Rmax, period of PUR NPDDCH G, and absolute offset αT.

- The PUR search window starts at the subframe that contains the end of the PUR transmission plus N (e.g. 3 or [40]) subframes and has length of M for the corresponding coverage level. In order to offload the NPDCCH and reduce the PUR NPDCCH blind detection, M and N can be configured in PUR configuration and M should be configured much shorter than RAR search window with unit of NPDCCH period. Within PUR search window, if there is no response received by UE, UE will transmit the original data in the next PUR resource. If the attempt of transmission exceeds a predefined attempt value, UE will fall back to legacy RACH or EDT.

Figure 1: UE-specific search space for PUR

Proposal 4: Considering the scheduling flexibility and HARQ feedback delay, modified common search space can be a kind of new UE-specific search space for UE to monitor only after uplink transmission based on dedicated PUR.

- The starting position of PUR search space can be absolute subframe similar configured as type 2 MPDCCH common search space.

- The PUR search window starts at the subframe that contains the end of the PUR transmission plus N (e.g. 3) subframes and has length of M for the corresponding coverage level.

If Type1 common search space for paging and PUR search space are configured in different narrowbands and PUR search space in time domain overlaps with PTW, the UE should determine which narrowband to monitor: 1) It is up to UE implementation; 2) explicit indicated by eNB; 3) implicitly determined by eNB-UE (eNB and UE are aware the collision/overlap occurring).

Proposal 5: If PUR search space overlaps with Type 1 common search space in different narrowbands, the UE should determine which narrowband to monitor: 1) It is up to UE implementation; 2) explicit indicated by eNB; 3) implicitly determined by eNB-UE.

Fallback

For transmission in PUR, fallback mechanism to RACH/EDT procedure was agreed in the last meeting, but the details is still FFS. The fallback mechanism can be categorized to two cases.  Firstly, the fallback to RACH/EDT is triggered by UE and no new signalling is required. For example, TA validation failure; transmission in PUR unsuccessfully decoded, e.g., ACK is not received during a preconfigured window or count to a preconfigured number.  In this case, UE can transmit preamble in the next RACH occasion if fallback decision made.

Secondly, the fallback to RACH/EDT is triggered by eNB and an indication signalling needs to be introduced, e.g., multiple PUR transmission failure, system load balance requirement, PUR resource unavailable, etc. Fallback indicator can be sent via dedicated RRC signalling or DCI signalling when eNB finds UE’s condition is no longer applicable for PUR.

Proposal 6: Fallback to RACH/EDT can be triggered by eNB or UE for different cases.

Configuration

Dedicated PUR is defined as PUSCH resource used by a single UE. For periodic resource configuration, the resource can be configured as periodic starting subframe and resource duration for each resource occasion as shown in Figure 2. For the periodic starting subframe of resource occasion, resource configuration of LTE SPS and NR type 1 configured grant uplink transmission can be the starting point as shown in Figure 3. For aperiodic resource configuration, multiple PUR resources can be configured with different periods to one particular UE.

The resource occasion number of PUR can be configured as a predefined value, the predefined value can be 1 to deal with traffic model of one shot uplink transmission and can also be zeros or infinite to indicate the PUR resource is always valid or reserved for UE.

Proposal 7:  PUR resource can be configured as periodic starting subframe and resource duration for each PUR resource occasion.

Figure 2 Periodic PUR resource configuration illustration

Figure 3 LTE SPS resource configuration illustration

Release PUR

Similar to LTE SPS release mechanism, it is agreed that release of the dedicated preconfigured resources are supported, details for NW triggered and UE triggered are FFS.  There are several potential PUR release mechanisms: 1) PUR resource released after the transmission until to preconfigured number. 2) Implicit PUR release mechanism after consecutive multiple missed transmissions to avoid the waste of allocated PUR. 3) eNB trigger re-configuration/release of PUR configured to a UE by dedicated RRC or DCI.

- Proposal 8: PUR resource is implicitly released by 1) transmission until to the preconfigured number; 2) consecutive multiple missed transmissions.

- Proposal 9: eNB may trigger re-configuration/release of PUR configured to a UE by dedicated RRC or DCI.

Shared PUR Issues

In RAN1 #94bis, it was agreed that “In idle mode, dedicated PUR is supported”, while the “Support for CFS PUR” and the “Support for CBS PUR” remained FFS.

For CFS PUR, multiple UEs share the same configured time-frequency resource with orthogonal multiple access, e.g., MU-MIMO and CDM. For MU-MIMO, it can be transparently to configured multiple UE same time-frequency resource with UE-specific DMRS via dedicated PUR. Therefore, CFS PUR using MU-MIMO with UE specific DMRS pattern can be supported without any specification work.

For CBS PUR, contention based PUR is required to ensure the reliability of uplink transmission and solve collision problems. Furthermore, CBS PUR may cause more specification effort, waste more UE power consumption, and even bring more challenges for eNB receiver, so we propose that CBS PUR is not supported in Rel.16 NBIoT.

- Proposal 10: CFS PUR using MU-MIMO with UE specific DMRS pattern can be supported without any specification work.

- Proposal 11: CBS PUR is not supported in Rel.16 NBIoT.

Conclusions

In this contribution, considerations of multiple transport block transmission with DCI are provided. The following proposals are given.

- Proposal 1: For uplink transmission based dedicated PUR, only single TB and single HARQ process are supported in Rel.16.

- Proposal 2: After transmission based PUR, UE start to monitor the (N)PDCCH with the dedicate RNTI expecting an explicit ACK upon successful decoding by eNB or an explicit NACK/an UL retransmission GRANT upon unsuccessful decoding by eNB or fallback to legacy RACH/EDT.

- Proposal 3: In response to NACK, the UE could then retransmit in configured new grant resource or wait till the next PUR transmission.

Proposal 4: Considering the scheduling flexibility and HARQ feedback delay, modified common search space can be a kind of new UE-specific search space for UE to monitor only after uplink transmission based on dedicated PUR.

- The starting position of PUR search space can be absolute subframe similar configured as type 2 MPDCCH common search space.

- The PUR search window starts at the subframe that contains the end of the PUR transmission plus N (e.g. 3) subframes and has length of  M for the corresponding coverage level

Proposal 5: If PUR search space overlaps with Type 1 common search space in different narrowbands, the UE should determine which narrowband to monitor: 1) It is up to UE implementation; 2) explicit indicated by eNB; 3) implicitly determined by eNB-UE.

- Proposal 6: Fallback to RACH/EDT can be triggered by eNB or UE.

- Proposal 7:  PUR resource can be configured as periodic starting subframe and resource duration for each PUR resource occasion.

- Proposal 8: PUR resource is implicitly released by 1) transmission until to the preconfigured number; 2) consecutive multiple missed transmissions.

- Proposal 9: eNB may trigger re-configuration/release of PUR configured to a UE by dedicated RRC or DCI.

- Proposal 10: CFS PUR using MU-MIMO with UE specific DMRS pattern can be supported without any specification work.

- Proposal 11: CBS PUR is not supported in Rel.16 NBIoT.

- References

RP-181421, “New WID on Rel-16 NBIoT enhancements for LTE”, Huawei, RAN #80

R1-1905538, “Feature lead summary of Support for transmission in preconfigured UL resources”, Huawei, HiSilicon, RAN1#96bis

R1-1903906, “NB-IOT Pre-configured UL Resources Design Considerations”, Sierra Wireless, RAN1#96bis

R1-1903892, “Support for transmission in preconfigured UL resources in NB-IoT”, Ericsson, RAN1#96bis

R1-1903910, “Transmission in preconfigured UL resources”, Huawei, HiSilicon, RAN1#96bis

R1-1904382, “Discussion on transmission in preconfigured UL resources for NB-IoT”, Samsung, RAN1#96bis

R1-1904569, “UL transmission in preconfigured resources for NBIoT”, Lenovo, MotoMobility, RAN1#96bis