3GPP TSG-RAN WG1 Meeting # 97	R1-1906295

# Reno, USA, 13th – 17th May, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Timing for secondary cell activation/deactivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When a UE receives an activation command in a PDSCH for an SCell in slot n, it is not clear in which slot UE applies the actions of CSI reporting and sCellDeactivationTimer associated with the secondary cell if the numerology of the cell on which CSI reporting for the activated cell is reported or activated SCell is different from the numerology of the cell on which PUCCH is transmitted with HARQ-ACK information for the PDSCH reception providing the activation command. Similar issue exists for SCell deactivation. More details can be found in R1-1906296. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace slot n+k by the exact timing and remove the definition of k, k1 and . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear UE behaviour on timing for secondary cell activation/deactivation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR clarifies the timing for Scell activation and deactivation. If network is not implemented according to the CR but UE is, or if UE is not implemented according to this CR but network is, there may be different understandings between gNB and UE on the timing for Scell activation and deactivation in case the numerology of the cell on which the CSI reporting for the activated SCell is reported/the activated SCell is different from the numerology of the cell on which PUCCH with HARQ-ACK information for the PDSCH reception providing the activation/deactivation command. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.3	Timing for secondary cell activation / deactivation

When a UE receives an activation command [11, TS 38.321] in a PDSCH for a secondary cell in slot n, the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133] and no earlier than the earliest slot 3 msec after the end of the slot for a PUCCH transmission with HARQ-ACK information for the PDSCH reception providing the activation command, except for the following:

-	the actions related to CSI reporting on a serving cell that is active that the UE applies in the earliest slot 3 msec after the end of the slot for a PUCCH transmission with HARQ-ACK information for the PDSCH reception providing the activation command

-	the actions related to the sCellDeactivationTimer associated with the secondary cell [11, TS 38.321] that the UE applies from 3 msec after the end of the slot for a PUCCH transmission with HARQ-ACK information for the PDSCH reception providing the activation command

-	the actions related to CSI reporting on a serving cell which is not active at 3 msec after the end of the slot for a PUCCH transmission with HARQ-ACK information for the PDSCH reception providing the activation command that the UE applies in the earliest slot 3 msec after the end of the slot for a PUCCH transmission with HARQ-ACK information  in which the serving cell is active.

If a UE receives a deactivation command [11, TS 38.321] for a secondary cell or the sCellDeactivationTimer associated with the secondary cell expires in slot , the UE applies the corresponding actions in [11, TS 38.321] no later than the minimum requirement defined in [10, TS 38.133], except for the actions related to CSI reporting on a serving cell which is active which the UE applies in the earliest slot 3 msec after the end of the slot for a PUCCH transmission with HARQ-ACK information for the PDSCH reception providing the deactivation command or in the earliest slot 3 msec after the sCellDeactivationTimer associated with the secondary cell expires.