3GPP TSG RAN WG1 Meeting#97																R1-1906308

Reno, USA, May 13th – 17th, 2019

Source:	CATT

Title:	Physical-layer procedures to support UE/gNB measurements

Agenda Item:	7.2.10.4

Document for:	Discussion and Decision

# Introduction

The following agreements were made on the Physical-layer procedures to support UE/gNB measurements for NR positioning in RAN1#96bis [1]:

Agreement: The potential physical layer procedures to support UE/gNB measurements for NR positioning may include: 1. UE procedures for receiving DL PRS, including a. DL PRS timing aspects b. Bandwidth part operation c. Beam management aspects d. Measurement/reporting related aspects 2. UE procedures for transmitting UL PRS, including a. Transmission timing and timing adjustments b. Power control c. Beam management aspects 3. gNB procedures to support NR positioning measurements, including a. DL PRS configuration aspects b. UL PRS configuration aspects c. Beam management aspects d. Measurement/reporting related aspects Agreement: The DL PRS configuration provided to the UE is independent of the configured DL BWPs of the UE Agreement: For intra-frequency measurements: - The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. - Select from one of the following options for the measurement bandwidth - Option 1: The UE measurement is within the DL BWP configuration - Option 2: The UE can measure outside the DL BWP configuration - FFS: Scenarios when measurements gaps would need to be configured. - When not configured with a measurement gap, the UE is only required to measure DL PRS within the active DL BWP and with the same numerology as the active DL BWP. Agreement: For positioning purposes, to assist UE to perform Rx beamforming, consider one or more of the following options: - Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell. - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS). - Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. - Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. - FFS: Whether this option can be achieved by Option 1 - FFS on specification impacts. Agreement: For positioning purposes, with regard to the UL SRS transmission timing, at least the following options have been identified: - Option 1: No adjustment is made to the configured TA. That is, support only a TA configuration which is based only on the serving cell (i.e., the TA value applied to the corresponding UL symbol is the same as the latest TA for regular UL symbols). - Option 2: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value can be provided by the UE to the network upon request. - Note: The adjustment value is not needed to be known at the measuring cell for the purpose of UL-AoA measurement - Option 3: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value is provided by the network to the UE. Agreement: For positioning purposes, with regards to the UL SRS transmission power, at least the following options have been identified: - Option 1: The UL SRS Tx power is constant (i.e., no power control is supported). - FFS: The value of the constant Tx power and how it is determined - Option 2: The UL SRS Tx power is based on the existing power control procedure. - Option 3: The UL SRS Tx power is determined by modifying the existing power control procedure, for example: - Support configuring a DL reference signal of a neighboring cell to be used for SRS path loss estimation. - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS). - FFS: which power control modes are supported (e.g., open loop only, or open loop and closed loop). Agreement: For positioning purposes, with regard to UL Beam management/alignment towards serving and neighbouring cells, consider one or more of at least the following options: - Option 1: Support configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS. - FFS: Which Reference DL RS can be used (e.g., SSB/CSI-RS/DL-PRS). - Option 2: Support UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources. - Option 3: The Tx beam is fixed for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2. - FFS: How the fixed beam is selected

In this contribution, we further discuss the physical-layer procedures to support UE/gNB measurements for NR positioning.

# UE procedures for receiving DL PRS

## DL PRS timing aspects

When NR DL PRSs are transmitted for supporting DL TDOA, it is expected that the cells involved in DL TDOA positioning are precisely time synchronized. Thus, the UE can base the timing of the neighbouring cells on the timing of the serving cell.

Proposal 1: When a UE is configured to measure DL PRS for supporting DL TDOA positioning, the UE can assume the timing of the neighbouring cells and the timing of the serving cell are synchronized.

For RTT-based positioning, the cells involved in the UE positioning may not necessarily be precisely time synchronized. In this case, the neighbouring cells and serving cell may not be precisely synchronized and the UE may use the DL reference signals from a neighbouring cell or the serving cell for UE Rx-Tx time difference measurements. The UE may need to have a basic assumption on the time offsets between the cells within a certain range to support the UE Rx-Tx time difference measurements from the neighbouring cells.

For NR TDD network, the time offset between the cells is required to be with 3 s as defined in TS 38.133, which should be good enough for supporting the UE Rx-Tx time difference measurements from the neighbouring cells.

For NR FDD network, there is currently no requirement for the time offset between the cells. Thus, there is a need to define the maximum time offset of the neighbouring cells for supporting UE Rx-Tx time difference measurements. Taking CSI-RS measurements for supporting UE mobility as an example, where it is assumed that the absolute value of the time difference of a radio frame between two cells is less than half-frame. In order to support RTT-based positioning, it should be reasonable to have the same assumption.

Proposal 2: When a UE is configured to measure DL PRS for supporting NR RTT positioning, the UE can assume the timing offset between the neighbouring cell and the timing of the serving cell is within 3 μs, if both cells are TDD cells and within half-frame if any of the cells is not TDD cell.

## Bandwidth part operation

It was agreed in RAN1#96bis that the DL PRS configuration provided to the UE is independent of the configured DL BWPs of the UE. If a measurement gap is not configured, the UE is only required to measure DL PRS with the same numerology as the active DL BWP within the active DL BWP. If a measurement gap is configured, the UE is expected to measure DL PRS with the different numerology within the active DL BWP. In addition, it was agreed that if a measurement gap is configured, the UE is expected to measure DL PRS (with the same or different numerology as the active DL BWP) outside the active DL BWP.

However, it is not decided whether the UE is required to measure DL BWP only within the DL BWP configuration or can be outside the DL BWP configuration. In our view, once a measurement gap is configured, the UE should be able to measure any assigned DL PRS within the same carrier frequency, since when the measurement gap is configured, the UE should even be able to measure the inter-frequency DL PRS. Thus, the UE should be able to measure DL BWP outside the DL BWP configuration. On the other hand, if the UE is only required to measure DL BWP within the DL BWP configuration, it may force the network to configure the UE with large DL BWP as much as possible, which could have adverse impact on the DL BWP configuration for supporting data scheduling and UE power consumption, given that time spent on data communication is in general much more higher than positioning.

Proposal 3: If the measurement gap is configured, a UE is expected to measure DL PRS outside the DL WBP configuration for intra-frequency measurement (i.e., Option 2).

## DL PRS beam management aspects

The following options were identified for UE Rx beamforming for positioning

- Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell.

  - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS).

- Option 2: The UE may perform an Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter.

- Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter.

  - FFS: Whether this option can be achieved by Option 1

- FFS on specification impacts.

For the fast detection of the DL PRS, it may also be helpful for supporting the association of the DL PRS with the SSB, i.e., whether the DL PRS is QCLed with the associated SSB. Although it might be useful to provide the whether the DL PRS is QCLed with the CSI-RS or other DL-PRS, we don’t see the strong use cases. For positioning purpose, if CSI-RS QCL information is provided, the motivation is to help the UE to search for the DL PRS from neighbouring cells. The UE, however, in general, are not assigned with CSI-RS for neighbouring cells. If the UE is assigned with CSI-RS for neighbouring cells, the associated SSB is in general provided, since the UE is not required to measure CSI-RS without first detect the associated SSB. Thus, SSB information is always available, but CSI-RS is not. For the use case of using QCL information from another DL-PRS implied two DL-PRS are in the same beam direction and one of them is already detected. It implies there is duplicated DL PRS transmission, and the UE needs to detect one of them before it can take advantage of the QCL information. Thus, the benefit of configuring a DL PRS with a CSI-RS and/or another DL PRS is unclear.

Proposal 4: DL PRS resource can be configured with the associated SSB. Also, the QCL relationship between the DL PRS resource and the associated SSB can be indicated in DL PRS configuration. But, the benefit of configuring a DL PRS to be QCLed with a CSI-RS and/or another DL PRS is unclear.

When there is no DL beam correspondence for DL PRS, which may often be the case for the reception on the DL PRS from the neighbouring cells, the UE may have to perform an Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. It means the design of the DL PRS should support the repetition of the DL PRS transmission with the same downlink spatial domain transmission filter. When the repetition of the DL PRS transmission with the same downlink spatial domain transmission filter is supported, it will be up to UE’s implementation on whether and when to use the beam sweeping for the reception of the DL PRS.

Observation 1: The design of the DL PRS should support the repetition of the DL PRS transmission with the same downlink spatial domain transmission filter. However, it will be up to UE’s implementation on whether and when to use the beams weeping for the reception of the DL PRS. There is no need to force the UE to do one way or the other in the specs.

When DL PRS resources are transmitted with different downlink spatial domain transmission filter, the UE may or may not use fixed Rx beam to receive the DL PRS. It should be up to UE’s implementation.

Observation 2: It should be up to UE on whether to use the fixed or non-fixed beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. There is no need to force the UE to do one way or the other in the specs.

## DRX Impact on the reception of the DL PRS

For NR positioning, if the UE is configured with DRX, it needs to be discussed whether the UE is required to perform the measurements of the DL PRS during the non-active time. The reception of the DL PRS (e.g., duration, periodicity, etc.) and that for the DRX will be configured independently. The DRX ‘ON’ and the time for the cell transmits the DL PRS may not be matched.  In order not missing the window for the detection of the DL PRS, the UE may need to be activated during DRX ‘OFF’ to measure the DL PRS. Thus, there is a need to define whether and when the UE is required to perform measurement of DL PRS resources for UE positioning during DRX ‘OFF’. The requirements may depend on the DL PRS design, DL PRS configuration and also the DRX configuration with the consideration of the balance between the UE power consumption and the positioning performance.

Proposal 5: For a UE configured with DRX, it needs to consider on whether the UE is required to be activated to measure DL PRS resources for UE positioning when the UE is in DRX ‘OFF’ status.

# UE procedures for transmitting UL PRS

## UL SRS transmission power

The following options were discussed on the UL SRS transmission power for positioning:

- Option 1: The UL SRS Tx power is constant (i.e., no power control is supported).

  - FFS: The value of the constant Tx power and how it is determined

- Option 2: The UL SRS Tx power is based on the existing power control procedure.

- Option 3: The UL SRS Tx power is determined by modifying the existing power control procedure, for example:

  - Support configuring a DL reference signal of a neighboring cell to be used for SRS path loss estimation.

    - FFS: Which reference signals can be used (e.g. a CSI-RS, or an SSB, or DL PRS).

  - FFS: which power control modes are supported (e.g., the open loop only, or open loop and closed loop).

- UL power control is commonly associated with the serving cell, i.e., allowing the serving cell to receive the UE signals with target BLER while minimizing the power consumption and interference. In order for a neighboring cell to receive the SRS from a UE properly, one possible approach is to introduce new SRS transmission power approach, such as adjusting the SRS transmission power based on the PL from the target cell to the UE. The main issue for the adjustment of SRS transmission power is that the SRS signals may not only be received by the target neighboring cell but multiple cells, including the serving cell. Thus, changing the SRS Tx power control based on the PL of the neighboring cells should avoid introducing large interference for the reception of UL signals for other cells. One way to avoid such an adverse impact on the reception of UL signals for other cells is that the network to have the control on whether the UE is allowed to adjust the SRS transmission power based on the

Proposal 6: For UL positioning, the network should have the control on whether the UL SRS Tx power should be based on the existing SRS power control procedure (Option 2) or other new SRS  power control procedure if introduced in Rel-16 for NR positioning.

- Technically, setting cell-specific power control is beneficial for neighboring cell SRS hearing. But there are a number of issues to be considered.

- Issue 1: How to configure the power adjustment factor for the target cell

- The SRS transmission power control may be linked to the signal strength of the DL signal, or equivalently the estimated path-loss, of the target neighboring cell. The problem is that the measurement of the signal strength of DL signal of target cell might not be accurate when beam correspondence doesn’t hold, or when UE hasn’t conduct path-loss measurement of the target neighboring cell.

- For FR2, a simpler solution could be introducing an additional resource-specific power offset on top of the current SRS power procedure. The power offset can be configured by the gNB via LMF or inter-gNB coordination based on history information, or previous SRS training. With this power offset, gNB can configure transmission power of SRS on demand. This is especially useful in aperiodic SRS or semi-persistent SRS configuration. Currently, P0 and path-loss reference RS are resourceset specific with RRC configuration, which is not flexible enough. Hence, configuring a resource specific or resource set specific power offset based on legacy SRS power control with dynamic signaling would provide more flexibility.

Proposal 7:  For FR2, new SRS power control procedure for NR positioning may be supported, where the UL SRS Tx power is determined by modifying the existing power control procedure by adding a power adjustment factor, e.g., a power offset, on an SRS resource configuration. Different power adjustment factors may be configured for different SRS resources targeting different cells (Option 3)

- Issue 2: Interference to other neighboring cells

For FR1, adjusting the SRS power control based on the received signal strength of DL signal of a target neighboring may problematic because of the width of the UL Tx beam. It will be difficult to control the SRS transmission beam precisely points to the target cell but not cause significant interference to the serving and/or the neighbouring cells.

Proposal 8:  For FR1, the UL SRS Tx power should be based on the existing power control procedure (Option 2)

## UL SRS transmission timing

For UL SRS transmission timing, the following options were discussed:

- Option 1: No adjustment is made to the configured TA. That is, support only a TA configuration which is based only on the serving cell (i.e., the TA value applied to the corresponding UL symbol is the same as the latest TA for regular UL symbols).

- Option 2:  The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value can be provided by the UE to the network upon request.

  - Note: The adjustment value is not needed to be known at the measuring cell for the purpose of UL-AoA measurement

- Option 3: The configured TA can be adjusted for the purpose of transmitting UL SRS intended to a neighboring cell. The adjustment value is provided by the network to the UE.

- SRS from a UE may be received from multiple cells/gNBs for supporting NR positioning. Due to the distance differences from the UE to these cells/gNBs, the time of arrival from the UE to these cells/gNBs are different, and there can be potentially mutual interference between the SRS signals and other UL signals. To reduce the potential mutual interference due to the different of the TOAs, which will mainly be the interference from other UL signals for the SRS signals which are transmitted from the UE far away from the cell when TA is large, there were proposals to let the UE to use different TAs for transmitting the SRSs to different cells.

- There are, however, a number of practical issues for the approach. First, the UE needs to know the TAs for difference cells. The information is not available to the UE unless the UE knew its own positions and the positions of the cells/gNBs. Second, changing the TAs to fit the remote gNB may cause interference to nearby cells/gNBs. The interference may be significant due to the UE is closer to these cells/gNBs, and the UL SRS transmission beam is in general wide beam and thus in general unable to pinpoint to the remote cells without impact nearby cells. On the other hand, due to the UL power limitation, we do not expect the cells very far away from a UE will/should be involved in the UL positioning for a UE anyway. Thus, we do not see the need and benefits, but potential harm, to let the UE to adjust TAs for the transmission of the SRS for remove cells/gNBs.

Proposal 9: For UL positioning, there should be no adjustment is made to the configured TA for UL SRS transmission (Option 1).

## UL SRS beam alignment and beam sweeping procedures

For positioning purposes, with regard to UL Beam management/alignment towards serving and neighbouring cells, it was agreed to consider one or more of at least the following options:

- Option 1:  Support configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS.

  - FFS: Which Reference DL RS can be used (e.g., SSB/CSI-RS/DL-PRS).

- Option 2: Support UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources.

- Option 3: The Tx beam is fixed for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2.

  - FFS: How the fixed beam is selected

For the transmission of the UL PRS, it may also be helpful for supporting the QCL association of the UL SRS with a DL RS. Although it might be useful to provide the information of the UL SRS is QCLed with the CSI-RS or a DL-PRS, we don’t see the strong use cases. For positioning purpose, if CSI-RS information is provided, the motivation is to help the UE to search for the DL PRS from neighbouring cells. The UE, however, in general, are not assigned with CSI-RS for neighbouring cells. If the UE is assigned with CSI-RS for neighbouring cells, the associated SSB is in general provided, since the UE is not required to measure CSI-RS without first detect the associated SSB. Thus, the SSB information is always available, but CSI-RS is not. For the use case of using QCL information from another DL-PRS implied two DL-PRS are in the same beam direction and one of them is already detected. It implies there is duplicated DL PRS transmission, and the UE needs to detect one of them before it can take advantage of the QCL information. Thus, the benefit of configuring a DL PRS with a CSI-RS and/or another DL PRS is unclear.

Proposal 10: DL PRS resource can be configured with the associated SSB. Also, the QCL relationship between the DL PRS resource and the associated SSB can be indicated in DL PRS configuration. But, the benefit of configuring a DL PRS to be QCLed with a CSI-RS and/or another DL PRS is unclear.

When there is no DL beam correspondence for DL PRS, which may often be the case for the reception on the DL PRS from the neighbouring cells, the UE may have to perform an Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter. It means the design of the DL PRS should support the repetition of the DL PRS transmission with the same downlink spatial domain transmission filter. When the repetition of the DL PRS transmission with the same downlink spatial domain transmission filter is supported, it will be up to UE’s implementation on whether and when to use the beamsweeping for the reception of the DL PRS.

Observation 3: The design of the DL PRS should support the repetition of the DL PRS transmission with the same downlink spatial domain transmission filter. However, it will be up to UE’s implementation on whether and when to use the beam sweeping for the reception of the DL PRS. There is no need to force the UE to do one way or the other in the specs.

When DL PRS resources are transmitted with different downlink spatial domain transmission filter, the UE may or may not use fixed Rx beam to receive the DL PRS. It should be up to UE’s implementation.

Observation 4: It should be up to UE on whether to use the fixed or non-fixed beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. There is no need to force the UE to do one way or the other in the specs.

- NR supports the transmission and the reception of multiple beams for all NR operation frequency bands. The rel-15 beam management mechanism is established mainly for the case with one single cell. The SRS signal transmitted from UE in an SRS resource may be in an omni-direction or in a beam direction. For supporting NR positioning, it requires multiple cells to receive the SRS signals from the UE.

- One option to support multiple cells to receive the SRS signals from the UE is based on the principle of the reciprocity, In this case, a UE will first measure DL reference signals (e.g., SSBs) of multiple cells and then transmit the SRS in the corresponding UL beam directions based on received DL Rx beams. For this procedure, UE is required to be able to detect multiple cell DL beams.

- Another option is not based on the principle of the reciprocity but based on SRS beam sweeping. It does not require the UE to first detect the DL reference signals from multiple cells. In this option, the gNB can configure multiple SRS resources for a UE for supporting the UL beam sweeping. That is, the UE transmits the UL SRS to different UL beam directions according to the SRS configuration. gNBs will attempt to receive the SRS at the configured the SRS transmission duration without knowing exactly the UL Tx beam direction, i.e., the gNB Rx beams do not match the UE’s Tx beams. Configuring multiple resource or resource set with beam sweeping way can help different gNB to get expected UL beamformed SRS.

Regardless of which option is used, the information related to the beam transmission direction and beamwidth of the DL and UL reference signals can be used to effectively improve the accuracy and performance of the UTDOA positioning system.

Proposal 11: The beam information of the DL reference signals (e.g., SSB, CSI-RS tec.) and UL reference signals (e.g., SRS) should be included in the messages exchanged between UE, gNBs, and LMF for supporting UL positioning based on SRS signals.

# Conclusion

In this contribution, we discussed the NR DL positioning technologies, and have the following observations

Observation 1: The design of the DL PRS should support the repetition of the DL PRS transmission with the same downlink spatial domain transmission filter. However, it will be up to UE’s implementation on whether and when to use the beams weeping for the reception of the DL PRS. There is no need to force the UE to do one way or the other in the specs.

Observation 2: It should be up to UE on whether to use the fixed or non-fixed beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. There is no need to force the UE to do one way or the other in the specs.

Observation 3: The design of the DL PRS should support the repetition of the DL PRS transmission with the same downlink spatial domain transmission filter. However, it will be up to UE’s implementation on whether and when to use the beam sweeping for the reception of the DL PRS. There is no need to force the UE to do one way or the other in the specs.

Observation 4: It should be up to UE on whether to use the fixed or non-fixed beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter. There is no need to force the UE to do one way or the other in the specs.

Based on the discussion, we made the following proposals:

Proposal 1: When a UE is configured to measure DL PRS for supporting DL TDOA positioning, the UE can assume the timing of the neighbouring cells and the timing of the serving cell are synchronized.

Proposal 2: When a UE is configured to measure DL PRS for supporting NR RTT positioning, the UE can assume the timing offset between the neighbouring cell and the timing of the serving cell is within 3 μs, if both cells are TDD cells and within half-frame if any of the cells is not TDD cell.

Proposal 3: If the measurement gap is configured, a UE is expected to measure DL PRS outside the DL WBP configuration for intra-frequency measurement (i.e., Option 2).

Proposal 4: DL PRS resource can be configured with the associated SSB. Also, the QCL relationship between the DL PRS resource and the associated SSB can be indicated in DL PRS configuration. But, the benefit of configuring a DL PRS to be QCLed with a CSI-RS and/or another DL PRS is unclear.

Proposal 5: For a UE configured with DRX, it needs to consider on whether the UE is required to be activated to measure DL PRS resources for UE positioning when the UE is in DRX ‘OFF’ status.

Proposal 6: For UL positioning, the network should have the control on whether the UL SRS Tx power should be based on the existing SRS power control procedure (Option 2) or other new SRS  power control procedure if introduced in Rel-16 for NR positioning.

Proposal 7:  For FR2, new SRS power control procedure for NR positioning may be supported, where the UL SRS Tx power is determined by modifying the existing power control procedure by adding a power adjustment factor, e.g., a power offset, on an SRS resource configuration. Different power adjustment factors may be configured for different SRS resources targeting different cells (Option 3)

Proposal 8:  For FR1, the UL SRS Tx power should be based on the existing power control procedure (Option 2)

Proposal 9: For UL positioning, there should be no adjustment is made to the configured TA for UL SRS transmission (Option 1).

Proposal 10: DL PRS resource can be configured with the associated SSB. Also, the QCL relationship between the DL PRS resource and the associated SSB can be indicated in DL PRS configuration. But, the benefit of configuring a DL PRS to be QCLed with a CSI-RS and/or another DL PRS is unclear.

Proposal 11: The beam information of the DL reference signals (e.g., SSB, CSI-RS tec.) and UL reference signals (e.g., SRS) should be included in the messages exchanged between UE, gNBs, and LMF for supporting UL positioning based on SRS signals.

# Reference

- Chairman’s Note 3GPP TSG RAN WG1#96bis

- 3GPP TR 38.855, “Study on NR positioning support”

- R1-1905346, “DL and UL Reference Signals for NR Positioning”, CATT

- R1-1905348, “UE and gNB measurements for NR Positioning”, CATT

- R1-1905349, “Physical-layer procedures to support UE/gNB measurements”, CATT