3GPP TSG RAN WG1 Meeting #97                                               R1-1906323

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.5.1

Source:	CATT

Title:	Simulation assumption for NTN evaluation

Document for:	Discussion

Introduction

In RAN1 #96bis meeting [1] [2], there were some discussions on simulation assumption for calibration purpose and performance evaluation purpose. However, still there are many parameters to be determined for evaluation.

In this contribution, we provided the purposed parameters setting and related technical analysis.

Discussion

Regarding the simulation scenarios, scenario A(GEO – transparent), C2 (LEO – transparent – moving beams) and D2 (regenerative – moving beams) were prioritized. Among the identified scenarios, there are some common parameters, which are suitable for all scenarios; furthermore, there are some scenario specific parameters, which could be adapted to different scenario. Another issue is how to differentiate calibration simulation and evaluation simulation. In our view, calibration simulation is highly relevant with performance evaluation, where the most of parameters setting should be same.

System level simulation assumption

For system level simulation, we can divide the parameters setting into two tables，in which one table is basic calibration set, and another is additional set for performance evaluation. Moreover, some common setting should be assumed for each scenario.

Regarding the common parameter setting, the following table can be considered.

Table 1: common parameter for calibration simulation

| Frequency band | S-band / Ka- Band |
| --- | --- |
| Maximum Bandwidth per beam for DL | S-band : 20Mhz Ka band : 400Mhz or 200Mhz |
| Maximum Bandwidth per beam for UL | S-band : 20Mhz Ka band : 400Mhz or 200Mhz |
| Satellite polarization configuration | Single polarization |
| Antenna port | 1 TX, 1RX |
| Number of beams | 8 |
| Frequency re-use factor | 4 |
| Deployment scenarios | Baseline : Rural |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UEs coverage distribution | 10 UEs per beam with uniform distribution |
| Metrics for calibration | Baseline: Coupling loss, Geometry |

Table 2: common parameter for performance simulation

| UE attachment | Geometry based |
| --- | --- |
| Receiver | MMSE |
| CSI reference signals configuration | 1 port CSI-RS, period=20ms |
| Scheduler | PF |
| Traffic model | FTP model 1 |
| Metrics for performance evaluation | Baseline: UE throughput (5%, 50%, 95%) |

Proposal 1: Adopt table 1 and table 2 for NTN evaluation.

Besides above tables, there are some controversial parameters to be discussed.

- Beam layout

One issue in beam layout is how to define the beam pattern and the inter-beam distance. If fixing the inter-beam distance, beam pattern for each beam will be different for different bore-sight. If using the fixed beam pattern in different bore-sight, actual beam coverage will be different in the ground. Before we make the decision to use fixed inter-beam distance or using fixed bore-sight for each beam, this relationship between beam pattern and cell size should be clarified.

Regarding the beam shape in ground, the hexagonal cell layout is used in the terrestrial network. Naturally, the hexagonal beam layout can be applied in non-terrestrial network. However, in non-terrestrial network, in order to reduce the Doppler shift impact due to beam moving, it is often proposed that the cell shape in each beam is flat, where the distance along the beam moving direction is shorter than the distance crossing the moving direction. In real deployment, the impact of Doppler layout should be minimized. Then we propose the hexagonal cell layout and rectangle beam layout can be assumed as both candidates for performance evaluation.

Figure 1: Rectangle based beam layout

Proposal 2: Support hexagonal beam layout and rectangle beam layout for performance evaluation.

- Satellite parameters setting

For satellite parameter setting, one big issue is how to plan the beam size. It is related to link budget, performance requirement and deployment cost. Smaller beam size, higher operation cost. Hence, we need reasonable analysis for possible system configuration.

In the table 3-4, we provide the antenna parameters of satellite. Due to more bandwidth resources in Ka band, the performance evaluation in Ka band is preferred.

Table 3: Example of satellite parameters for downlink transmissions

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Equivalent satellite antenna aperture |  | 5 m | 0.5 m | 0.5 m |
| Satellite altitude |  | 35’786 km | 1’200 km | 600 km |
| Satellite EIRP |  | 40 dBW/MHz | 12 dBW/MHz | 6 dBW/MHz |
| Satellite beam diameter | Hexagonal beam | 400 km | 100 km | 60 km |
|  | Rectangle beam | 1600*400 km | 400*100 km | 240*60 km |

Table 4: Example of satellite parameters for uplink reception

| Parameters/Scenarios | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Equivalent satellite antenna aperture | 5 m | 0.5 m | 0.5 m |
| G/T | 21 dB K-1 | 12 dB K-1 | 12 dB K-1 |

Proposal 3:  Adopt table 3 and table 4 as satellite parameter set for performance evaluation.

- UE parameters setting

For UE parameter setting, it depends on UE type, carrier band and antenna size.

Table 5: Example of UE transmission parameters

| Equivalent satellite antenna aperture | Ka-band | 0.5 m VSAT |  |  |
| --- | --- | --- | --- | --- |
|  |  | GEO | LEO-1200 | LEO-600 |
| UE EIRP density |  | 28 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |

Table 6: Example of UE receiver parameters

| Equivalent satellite antenna aperture (Note1) | Ka-band | 0.5 m VSAT |
| --- | --- | --- |
| G/T |  | 10 dB K-1 |

Proposal 4:  Adopt table 5 and table 6 as UE parameter set for performance evaluation.

- Channel model

Since the coverage in NTN network is very critical, we don’t think NLOS case is a main scenario due to limited link budget, even in low frequency band. In this sense, LOS based channel model should be prioritized.

Proposal 5:  Use the LOS based channel model for performance evaluation.

Conclusions

In this contribution, we analyzed the deployment scenario and provided our views on simulation assumptions.

Proposal 1: Adopt table 1 and table 2 for NTN evaluation.

Proposal 2: Support hexagonal beam layout and rectangle beam layout for performance evaluation.

Proposal 3: Adopt table 3 and table 4 as satellite parameter set for performance evaluation.

Proposal 4: Adopt table 5 and table 6 as UE parameter set for performance evaluation.

Proposal 5: Use the LOS based channel model for performance evaluation.

References

- RAN1 #96bis Chairman notes

- R1-1905794, NTN evaluation assumptions summary	, Thales