3GPP TSG RAN WG1 Meeting #97                                                                       R1-1906337

Reno, USA, May 13th – 17th, 2019

Source:	CATT

Title:	Self-evaluation on LTE Spectrum Efficiency in Indoor Hotspot-eMBB

Agenda Item:	7.2.7.2

Document for:	Discussion and Decision

Introduction

At RAN#75 meeting, Study Item on Self Evaluation towards IMT-2020 submission was approved [1], the self evaluation will provide the performances towards all the ITU-R IMT-2020 requirements. At RAN#81 meeting, the preliminary self evaluation results of 5G are provided [2] and the evaluation results are captured in TR 37.910[3]. For the spectrum efficiency evaluation in Indoor Hotspot-eMBB, only the NR spectrum efficiency evaluation results are provided. Remaining open issues include [4]:

The evaluation results for eMBB spectral efficiency with ideal channel estimation need to be updated with realistic channel estimation. The LTE part evaluation is to be provided in the test environments not yet evaluated (i.e. other than Rural – eMBB and Urban Macro – mMTC). Further clarification of evaluation assumptions e.g. interference error modeling is needed.

In this document, the evaluation results of channel model B on the spectral efficiencies in Indoor Hotspot-eMBB based on Rel-15 LTE are supplemented based on our contribution [5] of meeting #96bis.

The key minimum technical performance requirements are provided in Report ITU-R M.2410 [6] for the test environment of Indoor Hotspot-eMBB.

TABLE 1

5th percentile user spectral efficiency

| Test environment | Downlink (bit/s/Hz) | Uplink (bit/s/Hz) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 0.3 | 0.21 |

TABLE 2

Average spectral efficiency

| Test environment | Downlink (bit/s/Hz/TRxP) | Uplink (bit/s/Hz/TRxP) |
| --- | --- | --- |
| Indoor Hotspot – eMBB | 9 | 6.75 |

Evaluation methodology and assumption

System-level simulation with proper link to system interface based on link-level simulation results could be used to evaluate average spectral efficiency and 5th percentile user spectral efficiency.

In this section, we provide our considerations of LTE technical parameters and overhead for evaluation.

Technical parameters

In [3] and [7], the NR technical parameters for evaluation are provided. In this section, we provide our considerations of LTE technical parameters for evaluation for both DL and UL. And more detailed evaluation assumptions can be found in Table A-1.

The considerations of technical parameters for DL FDD are shown in Table 1.

Table 1 LTE parameters for DL FDD evaluation

| DL FDD – FR1 |  | LTE |
| --- | --- | --- |
| Waveform |  | OFDM based |
| Multiple access |  | OFDMA |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | Turbo, Max code-block size=6144 bit |
| Numerology | subframe | 14 OFDM symbol per subframe |
|  | SCS | 15kHz |
| Number of RBs |  | 50 |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | All DL 6 MBSFN subframes are assumed |
| MIMO | Transmission mode | Closed loop SU/MU-MIMO adaptation |
|  | Codebook | LTE advanced CSI codebook Detailed configuration to be reported |
|  | PRB bundling | 3PRB (It impacts DMRS channel estimation, but does not limit the scheduling unit.) |
|  | MU dimension | Up to 8 layer (considering up to 8 orthogonal DMRS ports) |
|  | SU dimension | Rank adaptation up to 8 layers (depending on UE antenna elements) |
|  | Codeword (CW)-to-layer mapping | For 1 layers, CW1; For 2 layers or more, two CWs |
|  | DMRS transmission for overhead | DMRS pattern according to TS36.211 |
|  | CSI-RS transmission | Non-precoded according to CSI measurement and interference measurement |
|  | CSI feedback | Subband based (PUSCH 3-2) |
|  | Interference measurement | SU-CQI |
| HARQ | ACK/NACK delay | According to TS36.213 |
|  | Re-transmission delay | 8ms |

The considerations of technical parameters for DL TDD are shown in Table 2. It is noted that the parameters that share the same value as in Table 1 (LTE FDD table) is not shown.

Table 2 LTE parameters for DL TDD evaluation

| DL TDD – FR1 |  | LTE |
| --- | --- | --- |
| Number of RBs |  | 100 |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | DSUUD, S is 11DL:1GP:2UL MBSFN subframes are assumed. |
| MIMO | PRB bundling | 2PRB - (It impacts DMRS channel estimation, but does not limit the scheduling unit.) |
|  | Downlink CSI measurement | - SRS and CSI-RS based - Option A: Precoded CSI-RS based - Option B: Non-precoded CSI-RS based |
|  | SRS transmission | - Non-precoded SRS - 1/2/4 SRS ports SRS estimation error model shown in R1-145389 can be used as starting point. |
| HARQ | Re-transmission delay | The re-transmission delay is according to the UL/DL configurations, the minimum RTT is 8ms. |

The considerations of technical parameters for UL FDD are shown in Table 3.

Table 3 LTE parameters for UL FDD evaluation

| UL FDD – FR1 |  | LTE |
| --- | --- | --- |
| Waveform |  | DFT-s-FDMA based |
| Multiple access |  | SC-FDMA |
| Modulation |  | Up to 256QAM |
| Coding on PUSCH |  | Turbo, Max code-block size=6144 bit |
| Numerology | subframe | 14 OFDM symbol per subframe |
|  | SCS | 15kHz |
| Number of RBs |  | 50 |
| Simulation bandwidth |  | 10 MHz |
| Frame structure |  | All UL |
| MIMO | Transmission scheme | UL codebook based SU-MIMO |
|  | UL Codebook | LTE 2Tx/4Tx codebook |
|  | SU dimension | Rank adaptation up to 4 layers depending on UE antenna |
|  | Codeword (CW)-to-layer mapping | For 1 layers, CW1; For 2 layers or more, two CWs |
|  | DMRS transmission for overhead | DMRS pattern according to TS36.211 |
|  | SRS transmission | Non-precoded |
| HARQ | Delay of DCI and PUSCH | According to TS36.213 |
|  | Re-transmission delay | 8ms |
| Power control schemes |  | Open loop power control according to TS36.213 |

The considerations of technical parameters for UL TDD are shown in Table 4. It is noted that the parameters that share the same value as in Table 3(LTE FDD table) is not shown.

Table 4 LTE parameters for UL TDD evaluation

| UL TDD – FR1 |  | LTE |
| --- | --- | --- |
| RB number |  | 100 |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | DSUUD, S is 11DL:1GP:2UL |
| HARQ | Re-transmission delay | The re-transmission delay is according to the UL/DL configurations, the minimum RTT is 8ms. |

Overhead

Downlink overhead

The calculation of overhead for downlink spectral efficiency evaluation should take into account the configuration of the PSS/SSS, PBCH, PDCCH, DM-RS, CSI-RS and CRS. In additional, the MBSFN subframes can be considered to decrease the overhead of CRS. Some reasonable options of downlink overhead parameters are listed in Table 5 and 6.

Table 5 Recommended options of DL overhead assumption parameter values for LTE

|  | FR1 |
| --- | --- |
| PSS/SSS | 288 REs/10ms |
| PBCH | 288 REs/10ms |
| PDCCH | 2 OFDM symbols per subframe |
| DM-RS | 12 REs/PRB, ports {7,8,11,13} |
| CSI-RS for channel measurement | Non-precoded CSI-RS: 8,16,32 CSI-RS ports with periodicity of 10 subframe Precoded CSI-RS: 4 CSI-RS ports per UE with periodicity of 10 subframe |
| CRS | 4 ports |
| MBSFN configurations | MBSFN subframes - FDD: #1, #2, #3, #6, #7, and #8 subframe can be configured to be MBSFN subframe, 6 MBSFN subframes in 10ms - TDD: #3, #4, #7, #8, and #9 subframe can be configured to be MBSFN subframe, for the DSUUD frame configurations, 2 MBSFN subframes in 10ms. The non-MBSFN regin in the MBSFN subframe is 2 OFDM symbols |
| GP | 1 OS for DL,1 OS for UL |

Table 6 Detailed DL overhead assumption parameters values for FR1 for LTE

| Assumption parameters |  |  |  |  |  | OH value (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMRS | CSI-RS | CRS | PSS/SSS | PBCH | PDCCH | FDD | TDD |
| 12REs/ subframe | 8ports,10ms | - 16REs/ subframes for FDD and TDD normal subframes - 12REs/ subframe for DwPTS - 0REs in MBSFN subframes | 288 REs/10ms | 288 REs/10ms | 2OS | 26.400 | 31.409 |
|  | 16ports,10ms |  |  |  |  | 26.876 | 32.253 |
|  | 32ports,10ms |  |  |  |  | 27.829 | 33.941 |
|  | 40ports,10ms |  |  |  |  | / | 34.785 |

Uplink overhead

The calculation of overhead for uplink spectral efficiency evaluation should take into account the PUCCH, SRS and DM-RS. The reasonable options of uplink overhead parameters are listed in Table 7 and 8.

Table 7 Recommended options of UL overhead assumption parameter values for LTE

|  | FR1 |
| --- | --- |
| PUCCH | FDD: 2PRB pairs with 9 subframes and 4 PRB pair with 1 subframe TDD: 2PRB pairs with 3 subframes and 4 PRB pair with 1 subframe |
| SRS | FDD: 1 complete symbols with 5ms period TDD: 2 complete symbols in UpPTS with 5ms period |
| DM-RS | 24REs/ subframe |
| GP | 1 OS for DL,1 OS for UL |

Table 8 Detailed UL overhead assumption parameter values for FR1 for LTE

| Assumption parameters |  |  | OH value (%) |
| --- | --- | --- | --- |
| DM-RS | SRS | PUCCH |  |
| FDD (10MHz, 15KHz SCS, 50RB) |  |  |  |
| 24REs/subframe | 12*(50-2)*2 REs/10ms | (12*14*2*9+12*14*4) REs | 20.057 |
| TDD (20MHz, 15KHz SCS, 100RB) |  |  |  |
| 24REs/subframe | 12*2*100*2 REs/10ms | (12*14*2*3+12*14*4) REs | 23.607 |

Evaluation results

The preliminary evaluation results for average spectral efficiency and 5th percentile spectral efficiency in Indoor Hotspot-eMBB based on Rel-15 LTE are provided. The detailed evaluation assumptions can be found in Annex.

Downlink evaluation results

The preliminary evaluation results for DL average spectral efficiency and DL 5th percentile spectral efficiency on LTE in Indoor Hotspot -eMBB are provided in Table 9 and Table 10.

Table 9 Initial DL average spectral efficiency evaluation for 12 TRxPs

| Test environment | Duplex scheme | DL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment | SEavg | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (bit/s/Hz/TRxP) |  |  |
| 12 TRxPs Config. A, 4GHz InH-A | FDD | Non-precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) | 9 | 9.81 | 9.00% |
|  | TDD DSUUD | SRS and Non-precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) |  | 11.81 | 31.22% |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) |  | 11.76 | 30.67% |
| 12 TRxPs Config. A, 4GHz InH-B | FDD | Non-precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) |  | 9.86 | 9.56% |
|  | TDD DSUUD | SRS and Non-precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) |  | 11.88 | 32.00% |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) |  | 11.78 | 30.89% |

Table 10 Initial DL 5th percentile spectral efficiency evaluation for 12 TRxPs

| Test environment | Duplex scheme | DL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment | SEuser | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (bit/s/Hz) |  |  |
| 12 TRxPs Config. A, 4GHz InH-A | FDD | Non-precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) | 0.3 | 0.35 | 16.67% |
|  | TDD DSUUD | SRS and Non-precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) |  | 0.42 | 40.00% |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) |  | 0.40 | 33.33% |
| 12 TRxPs Config. A, 4GHz InH-B | FDD | Non-precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) |  | 0.35 | 16.67% |
|  | TDD DSUUD | SRS and Non-precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) |  | 0.41 | 36.67% |
|  |  | SRS and Precoded CSI-RS based | 32T/4R, (4,4,2,1,1;4,4) |  | 0.39 | 30.00% |

Observation 1: For 12 TRxPs topology with sub 6 GHz, LTE can fulfill the DL spectral efficiency requirements for the Indoor Hotspot-eMBB test environment in both TDD and FDD mode.

Uplink evaluation results

The preliminary evaluation results for UL average spectral efficiency and UL 5th percentile spectral efficiency on LTE in Indoor Hotspot -eMBB are provided in Table 11 and Table 12.

Table 11 Initial UL average spectral efficiency evaluation for 12 TRxPs

| Test environment | Duplex scheme | UL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment | SEavg | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (bit/s/Hz/TRxP) |  |  |
| 12 TRxPs Config. A, 4GHz InH-A | FDD | Codebook-based | 4T/32R, (4,4,2,1,1;4,4) | 6.75 | 8.47 | 25.53% |
|  | TDD DSUUD | Codebook-based | 4T/32R, (4,4,2,1,1;4,4) |  | 8.05 | 19.25% |
| 12 TRxPs Config. A, 4GHz InH-B | FDD | Codebook-based | 4T/32R, (4,4,2,1,1;4,4) |  | 7.96 | 17.93% |
|  | TDD DSUUD | Codebook-based | 4T/32R, (4,4,2,1,1;4,4) |  | 7.74 | 14.67% |

Table 12 Initial UL 5th percentile spectral efficiency evaluation for 12 TRxPs

| Test environment | Duplex scheme | UL CSI measurement | TRxP and UE antenna Config. (M,N,P,Mg,Ng;Mp,Np) at TRxPs | Require- ment | SEuser | Gain (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (bit/s/Hz) |  |  |
| 12 TRxPs Config. A, 4GHz InH-A | FDD | Codebook-based | 4T/32R, (4,4,2,1,1;4,4) | 0.21 | 0.51 | 144.51% |
|  | TDD DSUUD | Codebook-based | 4T/32R, (4,4,2,1,1;4,4) |  | 0.54 | 154.85% |
| 12 TRxPs Config. A, 4GHz InH-B | FDD | Codebook-based | 4T/32R, (4,4,2,1,1;4,4) |  | 0.49 | 133.33% |
|  | TDD DSUUD | Codebook-based | 4T/32R, (4,4,2,1,1;4,4) |  | 0.49 | 133.33% |

Observation 2: For 12 TRxPs topology with sub 6 GHz, LTE can fulfill the UL spectral efficiency requirements for the Indoor Hotspot-eMBB test environment in both TDD and FDD mode.

Conclusion

In this contribution, we provide our considerations of LTE technical parameters and overhead for evaluation. Also the preliminary LTE evaluation results of Indoor hotspot-eMBB are provided. We have the following observations:

Observation 1: For 12 TRxPs topology with sub 6 GHz, LTE can fulfill the DL spectral efficiency requirements for the Indoor Hotspot-eMBB test environment in both TDD and FDD mode.

Observation 2: For 12 TRxPs topology with sub 6 GHz, LTE can fulfill the UL spectral efficiency requirements for the Indoor Hotspot-eMBB test environment in both TDD and FDD mode.

- References

  - RP-170832, “New Study Item on Self Evaluation towards IMT-2020 submission”, Huawei, Ericsson，Dubrovnik, Croatia, March 6 - 9, 2017.

  - RP-182097, “Draft LTI on 3GPP 5G Preliminary Description Template and Self-Evaluation”, 3GPP TSG RAN，Gold Coast, Australia, September 10 - 13, 2018.

  - 3GPP TR 37.910, “Study on Self Evaluation towards IMT-2020 Submission”, V1.0.0, 2018-09.

  - RP-182539，“Status Report to TSG”, Huawei, Sorrento, Italy, December 10-13, 2018.

  - R1-1904553, “Self-evaluation on LTE Spectrum Efficiency in Indoor Hotspot-eMBB”, CATT, Xi’an, China, 8th – 12th April, 2019.

  - Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017.

  - R1-1803386, “Summary of offline discussion for IMT-2020 self evaluation”, Huawei, Ericsson, Athens, Greece, 26th February – 2nd March 2018.

- Annex: Evaluation assumptions

In this contribution, the evaluation assumptions for Indoor hotspot-eMBB are provided in the following table.

Table A-1 Evaluation assumptions for DL and UL

| Indoor Hotspot - eMBB | Config. A |
| --- | --- |
| Carrier frequency for evaluation | 4 GHz |
| Simulation bandwidth | FDD: 10M+10M TDD: 20M |
| Slots structure | FDD: paired spectrum, DL + UL TDD: unpaired spectrum, DSUUD |
| Subcarrier spacing | 15KHz |
| Symbols number per slot | 14 |
| BS antenna height | 3 m |
| Total transmit power per TRxP | FDD: 21 dBm TDD: 24 dBm |
| UE power class | 23 dBm |
| Inter-site distance | 20m |
| TRxP number per site | 1 |
| Mechanic tilt | 180° in GCS |
| Electronic tilt | 90° in LCS |
| Antenna configuration at TRxP | (M,N,P,Mg,Ng; Mp,Np) = (4,4,2,1,1; 4,4) (dH,dV) = (0.5, 0.5)λ +45°, -45° polarization |
| Antenna configuration at UE | (M,N,P,Mg,Ng; Mp,Np) = (1,2,2,1,1; 1,2), (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| UE speeds of interest | 3 km/h |
| BS noise figure | 5 dB |
| UE noise figure | 7 dB |
| BS antenna element gain | 5dBi |
| BS antenna element pattern | See Table 10 in Report ITU-R M.2412 |
| UE antenna element gain | 0 dBi |
| UE antenna element pattern | Omni-directional |
| Thermal noise level | -174 dBm/Hz |
| Traffic model | Full buffer |
| UE density | 10 UEs per TRxP |
| UE antenna height | 1.5m |
| Channel model variant | Channel model A and B |
| UT attachment | Based on RSRP from port 0 |
| Wrapping around method | No wrapping around |
| Polarized antenna model | Model-2 in TR36.873 |
| TX mode | SU/MU-MIMO |
| scheduler | PF |
| Receiver | MMSE-IRC |
| DL CSI feedback | 10ms period with delay |
| DL CSI method | FDD: LTE advanced CSI codebook based TDD: SRS and non-precoded CSI-RS / precoded CSI-RS based channel reciprocity |
| LTE Type II codebook parameters | 2 beam basis, WB+SB and 4PSK quantification |
| HARQ ACK delay | Refer to TS 36.213 |
| HARQ retransmission delay | The re-transmission delay is according to the UL/DL configurations, the minimum RTT is 8ms. |
| Channel estimation | Non-ideal |
| Interference estimation | Non-ideal |
| UE precoder scheme | Codebook based for FDD and TDD |
| UL CSI derivation | SRS based with delay |
| UL waveform | DFT-s-FDMA |
| IoT | <10dB |