3GPP TSG RAN WG1 Meeting #97                                R1-1906342

Reno, USA, 13th – 17th May, 2019

Agenda item:		7.2.7.2

Source:	CATT

Title:	Further Self-Evaluation on Link Budget for NR

Document for:   Discussion and Decision

1	Introduction

According to Report ITU-R M.2411, the proponent of IMT-2020 should provide link budget template to ITU-R as one of the submission templates. Result collection for link budget in NLOS case has been conducted in RAN #81 meeting. Remaing open issues include[1]:

Link budget evaluation should be complemented by potential additional evaluation results, e.g. link budget for LOS and for Urban Macro-mMTC in NR, and link budget for Indoor Hotspot-eMBB, Dense Urban-eMBB and Urban-Macro URLLC in LTE.

In this contribution we provide our supplement on link budget evaluation for NR based on previous preliminary evaluation [2]. Compared to the results provided in [3] at last meeting, results in NLOS and LOS case for Urban Macro-URLLC are additionally provided. This is the resubmission of R1-1904558.

2.		Considerations on evaluation assumptions

In this contribtion, we are fucus on LOS case of three eMBB test environments, Indoor Hotspot-eMBB, Dense Urban-eMBB and Rural-eMBB and NLOS/LOS case for Urban Macro-URLLC. The evaluation assumptions refer to those used for NLOS link budget evaluation in RAN #81 meeting [2].

Table 1 Evalution configurations applied for link budget

|  | Configuration | Note |
| --- | --- | --- |
| Test environment | Indoor Hotspot-eMBB: LOS case Dense Urban-eMBB: LOS case Rural-eMBB: LOS case Urban Macro-URLLC: LOS and NLOS case | Suplement evaluation towards preliminary results in RAN #81 meeting [2]. |
| System configuration | NR 4GHz with 30KHz SCS for Indoor Hotspot-eMBB and Dense Urban-eMBB. NR 700MHz with 30KHz SCS for Rural-eMBB and URLLC | Fucus on the FR1 and 30KHz SCS for preliminary evaluations |
| Subframe configuration | TDD, DSUUD for three eMBB test environments with S slot = (11DL:1GP:2UL); FDD for Urban Macro-URLLC | This is the same as that in 4G evaluation. |
| Physical channel | DL: PDCCH, PDSCH UL: PUCCH, PUSCH DL and UL for three eMBB test environments; DL for Urban Macro-URLLC |  |
| Transmission bit data for date channel | 3 times of the 4G TDD evaluation assumption | This is aligned with the offline discussion during RAN#94 |
| Spectral efficiency | Calculated based on the transmission bit rate and bandwidth. | Considering the effctive bandwidth based on the subframe configuraion. For guard period (GP), 50% of GP symbols are considered as downlink overhead, and 50% of GP symbols are considered as uplink overhead. It is noted that the required SE is different with different subframe configurations. |
| DL occupied channel bandwidth | 20MHz | It is noted that, for InH environment, the required SE is almost 6 times of that in the 4G evaluation since the evaluated bandwidth is half of that in 4G evaluation, while for other scenarios, the required SE is almost 3 times of that in the 4G evaluation. |
| UL occupied channel bandwidth | 1 PRB for control channel and 4 PRB for data channel (the minimum granularity for scheduling in frequency domain is 4PRB) | It is noted that, for InH environment, the required SE is almost 3 times of that in the 4G evaluation since the actual bandwidth is similar while for DU and Rural enviorment, the evaluated bandwidth is about twice of that in 4G evaluation, i.e. SE (using 30KHz) would be about 1.5 times of 4G in the same scenario. |
| Channel state | LOS (for Indoor-eMBB, Dense Urban-eMBB, Rural-eMBB, Urban Macro-URLLC) O2I LOS ( For Dense Urban-eMBB, Rural-eMBB, Urban Macro-URLLC) NLOS (for Urban Macro-URLLC) O2I NLOS ( For Urban Macro-URLLC) | Suplement evaluation for initial results in RAN #81 meeting [2]. |
| Cable loss for BS | 0dB or 3dB | Considering different BS antenna types |
| Penetration loss + shadow fading margin | The penetraion margin is the expectation of the penetraion. The impact of the penetration standard deviation (SD) is considered into the calculation of shadow fading margin. The shadow fading margin is calculated by path loss model slope, the integrated SD of shadow fading SD and penetration SD, and target area coverage reliability (ACR). | The same evaluation methodologies as those in 4G evaluation are applied [4][5] The detailed shadow fading margin calcuation in LOS cases, considering path loss slope, breakpoint and etc., can be referred to [4]. |

3	Evaluation results

In this contribution, we discussed evaluation assumptions of link budget under some scenarios which were not evaluated and the attached preliminary evaluation resuls are provided.

For the cable loss of BS, the values of (8) and (12) in the attached tables can be selected between 0 and 3.

Attachment:

1. link budget template for NR - Channel Model A

2. link budget template for NR - Channel Model B

References

- RP-182539，“Status Report to TSG”, Huawei, Sorrento, Italy, December 10-13, 2018

- RP-181764,	“Self-Evaluation on Link Budget for eMBB”, CATT, September, RAN #81

- R1-1902017, “Further Self-Evaluation on Link Budget for NR”, CATT, January, RAN1 #96

- R1-1904552, “Discussions on calculation of shadow fading margin for link budget”, CATT, April, RAN1 #96b

- R1-1808416, “Discussions on penetration margin and SD for link budget”, CATT., August, RAN1 #94