3GPP TSG RAN WG1 Meeting #97						R1-1906352

Reno, USA, May 13th – 17th, 2019

Source:            CATT

Title:                 Power saving schemes for UE adaptation to the BWP/SCell operation

Agenda Item:   7.2.9.3

Document for: Discussion and Decision

Introduction

In last RAN#83, power saving WID is agreed [1].This contribution is shown the details and the procedures of the UE power schemes with UE adaptation to the BWP/Scell, and evaluation results on power saving gain. This contribution is resubmitted of RAN1 96bis contribution [2].

Power Saving Schemes with UE adaptation to BWP

From the results of Rel-16 study of power saving in NR[3], the power saving schemes with UE adaptation to BWP operation show 8.5% - 45% power saving gain.  The power saving gain is contributed by UE adaptation to the BWP operation based on the different traffic and system load.   The dynamic switching of the BWP is achieved by the L1 signalling included in the scheduling DCI. The UE adaptation to the BWP operation is to allow UE to reduce the power consumption during the BWP switching and achieve the link adaptation gain for the PDSCH reception.

UE Adaptation to BWP switching in NR R15

The BWP switching in Rel-15 is based on the BWP switching indication included in the scheduling DCI format.  Rel-15 BWP adaptation can achieve the UE power saving by monitoring PDCCH on narrow band BWP and switching to wideband BWP when data transmission is arrival.  The CSI feedback of the wideband BWP is needed in order to achieve link adaptation.  If UE did not provide CSI information for target BWP in time, the link adaptation gain could not be achieved and the performance of PDSCH reception is degraded. In order to achieve the link adaptation after BWP switching to the wideband BWP, the aperiodic CSI-RS needs to be triggered and transmitted along with the BWP switching.  This will allow UE to perform the CSI measurements on the target wideband BWP and feed back the aperiodic CSI subsequently after BWP switching. The time offset between DCI transmission and CSI-RS transmission is configured by RRC signalling whose value configuration is {0, 1, 2, 3, 4, 16, 24} slots。.  The CSI reporting information is triggered by UL DCI and carried by PUSCH.  When the aperiodic CSI is fed back to the gNB, the gNB can schedule the PDSCH transmission and achieve the link adaptation gain. The procedure is shown as following figure.

Figure 1  BWP siwthching with legacy R15 scheme

Rel-15 BWP switching would have unnecessary UE power consumption on PDCCH monitoring without any PDSCH scheduling after switching to wideband BWP. Furthermore, the latency of CSI acquisition is dependent on BWP transition time, time offset between DCI for the request of CSI-RS transmission, latency between CSI reporting slot and DCI UL scheduling, and UL/DL transmission direction configuration.  Considering smallest latency of CSI acquisition time, the example of the timeline is shown in Figure 2, where time slot between DCI with CSI request is assumed as 0 slot and CSI reporting time offset is assumed 1slot from DCI with CSI request and scheduling DCI.

Figure 2  BWP siwthching with legacy R15 scheme and smalleset CSI acquistion lantency

Proposal 1:  NR Rel-15 DCI trigger dynamic BWP switching could achieve UE power saving. Further enhancement of Rel-15 BWP adaptation with power saving scheme could reduce the UE power consumption.

UE Adaptation to BWP switching with enhancement scheme

Rel-15 BWP adaptation could have further power consumption reduction if UE does not perform any unnecessary processing after the BWP switching to the wideband BWP.  The key of reducing the UE power consumption is to reduce the PDCCH monitoring before the CSI acquisition and report feedback to the gNB for PDSCH scheduling transmission.

Rel-15 BWP switching with fast CSI acquisition and reduced PDCCH monitoring could further reduce the UE power consumption as shown in Figure 3. UE monitor PDCCH on narrow band BWP1. When data arrives, DCI triggers BWP switching and A-CSI RS transmission on target wideband BWP2.  UE would not monitor the DL search space on the PDCCH for PDSCH reception on BWP2 during the period.  UE would only monitor UL search space after a configured period and feed back the updated CSI to the gNB for scheduled DL data transmission. When data transmission is finished, DCI for DL scheduling could be used to indicate BWP switching to BWP1 to continue PDCCH monitoring.

Figure 3  BWP siwthching and CSI acquisition with power saving signal

Proposal 2: Rel-15 DCI based BWP switching could be enhanced with suspending PDCCH monitoring and fast CSI acquisition to achieve further UE power saving.

Evaluation results for UE Adaptation to BWP switching with enhancement scheme

The simulation results are evaluated based on above analysis. There are two cases modelled. Case1 is based on legacy R15 BWP switching scheme, which is baseline.  DCI DL format could be used to trigger BWP switching. Then DCI UL format could trigger CSI request.  Case2 is based on the reduced PDCCH monitoring and fast CSI acquisition.

Table 1  Evaluation cases of power saving schemes for UE adaptation to BWP switching

| Cases | Item | Note |
| --- | --- | --- |
| Case1 | Legacy R15 BWP switching | - UE perform PDCCH monitoring in narrow BWP1; - CSI-RS reception in wideband BWP2 after BWP switching ; - PDSCH reception in wideband BWP2; - A-CSI RS could be triggered by DCI. |
| Case2 | Reduced PDCCH monitoring and fast CSI acquisition | - UE performs PDCCH monitoring in narrow BWP1; - DCI trigger BWP switching and A-CSI RS transmission; - Suspend DL search space monitoring during fast CSI acquisition - PDSCH reception in wideband BWP2. |

The procedure of the power saving schemes with UE adaptation to the BWP switching with additional PDCCH monitoring reduction and fast CSI acquisition are as follows,

- Step1: UE is configured with BWP operations based on Rel-15 DCI triggered BWP switching

  - A narrow band BWP is configured for the power saving purpose. UE performs PDCCH monitoring on the narrow band BWP

  - Up to 4 wideband BWPs are configured for the data reception with different data size

- Step2: UE performs PDCCH monitoring on narrow band BWP

- Step3:When UE detects the DCI, if DCI trigger BWP switching,

  - Aperiodic CSI-RS is transmitted

  - UE performs CSI measurements on the target wideband BWP

    - UE suspend DL search space monitoring during the CSI measurements in a configured period of time.

- Step4: UE feeds back the CSI to gNB and starts monitoring the DL search space.

- Step 5: UE is scheduled to receive DL data from wideband BWP with link adaptation gain

- Step6: After complete reception of data from wideband BWP, UE is triggered by DCI to switch to narrow band BWP.

The simulation parameters and the detailed results based on the evaluation cases are shown in Annex. The list of UE power saving gains is shown in below table.

Table 2 The power saving gain of the enhancement of BWP adaptation

| Traffic type | FTP3 |  | IM |  |
| --- | --- | --- | --- | --- |
|  | case1 | case2 | case1 | case2 |
| Slot-averaged power consumption(unit/slot) | 15.15 | 13.40 | 3.75 | 3.52 |
| Power saving gain | 0.00% | 11.56% | 0.00% | 5.93% |

Based on simulation results shown in above table, it is observed that:

- Rel-15 DCI based BWP switching could be enhanced with suspending PDCCH monitoring and fast CSI acquisition evaluated as the Case 2 can provide power saving gain about 5.9%~11.6% comparing with the benchmark of R15 BWP operation.

Proposal 3:  The power saving scheme with enhancement of Rel-15 BWP adaptation could provide up to 11.6% power saving gain.

Conclusion

In this contribution, the enhancement of UE BWP adaptation is discussed and analyzed.  We have the following proposals,

Proposal 1:  NR Rel-15 DCI trigger dynamic BWP switching could achieve UE power saving. Further enhancement of Rel-15 BWP adaptation with power saving scheme could reduce the UE power consumption.

Proposal 2: Rel-15 DCI based BWP switching could be enhanced with suspending PDCCH monitoring and fast CSI acquisition to achieve further UE power saving.

Proposal 3:  The power saving scheme with enhancement of Rel-15 BWP adaptation could provide up to 11.6% power saving gain.

References

- RP-190727, “New WID: UE Power Saving in NR”, CATT, CAICT

- R1-1905370, Power saving schemes for UE adaptation to the BWP/SCell operation, CATT

- TR 38.840, V1.0.0 (2019-03)

- TS 38.214 V15.5.0 (2019-03)

Annex

Simulation assumptions for power saving consumption are shown as following table.

Table 3 Simulation assumption for frequency domain adaptation

| Simulation case | Assumptions |  |
| --- | --- | --- |
| Frequency domain adaptation | DRX cycle | 320ms,160ms 320ms for IM;160ms for FTP3 |
|  | On-duration-timer | 10ms,8ms |
|  | Inactivity timer | 100ms for 160ms DRX-cycle,80ms for 320ms DRX-cycle |
|  | MAC-CE function | Enable. Inactivity timer could be terminated by MAC-CE. |
|  | CSI acquisition | aperiodic CSI-RS triggering by DCI |
|  | TDD configuration | DSUUD, SCS=30KHz |
|  | Bandwidth | 100MHz |
|  | UL transmission | Disable, UL slot is assumed as micro-sleep and ideal feedback for ACK, CSI. |
|  | Transmission rate | Transmission rate is based on DU scenario SE with 256 transmission antenna, gNB: (M,N,P,Mg,Ng; Mp,Np) = (16,8,2,1,1;2,8).10UE is dropped within one TRP. |
|  | Traffic model | FTP3 with 0.5Mbytes packet size and 200ms inter-arrival time; IM with 0.1Mbytes packet size and 2sec inter-arrival time |