3GPP TSG RAN WG1 #97		R1-1906356

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.2.1.3

Source:	Spreadtrum Communications

Title:	Discussion on UL Signals and Channels in NR-U

Document for:	Discussion and decision

Introduction

NR-based Access to Unlicensed spectrum was approved to be studied and specified as the WI in RAN#82. The achieved agreements for UL signals and channels in last meeting are listed below [2].

Agreement: For a 20 MHz carrier bandwidth, if enhancements to PF0 and PF1 are supported, a mapping to physical resources of at least one full interlace is supported - FFS: Whether or not to support enhancements to PF0/1. - Companies are encouraged to provide user multiplexing capacity and UCI payload analysis for enabling the decision for relevant use cases Agreement: Decisions on which additional PUCCH formats (enhanced or combination of legacy and enhanced) are supported should be at least based on the following. - Which PUCCH format(s) are to be used at least for the following use cases: - HARQ ACK prior to dedicated PUCCH resource configuration - HARQ ACK, SR, CSI and combinations thereof after dedicated PUCCH resource configuration - Specification impact, e.g., UE procedures in 38.213 and 38.212, for all proposed PUCCH formats to be supported - User multiplexing capacity and UCI payload analysis for all proposed PUCCH formats to be supported - In-band and out-of-band emissions Agreement: Support RRC configuration of an SRS resource to start at any OFDM symbol within a slot by extending the RRC parameter startPosition of resourceMapping of SRS-Config for Rel-16 to have a value range 0..13. Agreement: Only further discuss/consider Option 3 (PDSCH mapping type B with durations other than 2/4/7 symbols) as potential enhancements to PDSCH transmissions.

This contribution gives some considerations for PUCCH/PUSCH/SRS in NR-U operation, including:

- Interlaced structure

  - PRB based interlace for 60kHz for 20MHz

  - Interlace design for a carrier with wide bandwidth

- PUSCH

  - PUSCH transmission in the partial slot

  - PUSCH resource allocation in the frequency domain

- PUCCH

  - NR PUCCH format 2 and 3

  - Interlace based PUCCH formats

Discussion

In this section, we discuss the potential design principles of PUSCH/PUCCH/SRS in NR-U operation.

Interlaced structure

PRB based interlace for 60 kHz for 20MHz

In RAN1 AH 1901 meeting, PRB-based interlace transmission of PUSCH and PUCCH in 20 MHz carrier bandwidth was identified. For 15 kHz SCS, M = 10 interlaces with N = 10 or 11 PRBs / interlace; and for 30 kHz SCS, M = 5 interlaces with N = 10 or 11 PRBs / interlace. However the interlaces number M and PRB number per interlace N are not decided yet. Since the PRB number in 20MHz with 60 kHz SCS should be decided in RAN4, there can be 24 PRBs or 26 PRBs contained in 20MHz bandwidth. The three candidates M and N are provided before and listed below. The factor of OCB should be satisfied, so that we give every OCB accordingly. It is clearly, M = 2 should be adopted for 60 kHz if it is supported in NR-U.

The following table gives calculated OCB for 15 kHz and 30 kHz according to these given candidates. The calculation of OCB use the equation of:

OCB=N-1*M+1*12*∆f20*103

Table 1: OCB for interlaces within 20MHz bandwidth

| SCS ∆f | Candidates |  | OCB |
| --- | --- | --- | --- |
| 60 kHz 24 PRBs | M = 4 | N = 6 | 75.6% |
|  | M = 3 | N = 8 | 79.2% |
|  | M = 2 | N = 12 | 82.8% |
| 60 kHz 26 PRBs | M = 4 | N = 6 | 75.6% |
|  |  | N = 7 | 90.0% |
|  | M = 2 | N = 13 | 90.0% |
|  | M = 3 | N = 8 | 79.2% |
|  |  | N = 9 | 90.0% |

- Within a 20 MHz carrier bandwidth, PRB-based interlace with M =2, N = 12 (24 PRBs) or 13(26 PRBs) should be considered for 60 kHz SCS if it is supported in NR-U.

Interlace design for a carrier with wide bandwidth for PUCCH

There are two interlace designs for a carrier with wide bandwidth:

- Alt-1: Same interlace spacing for all interlaces regardless of carrier BW.

  - This alternative uses Point A as a reference for the interlace definition

- Alt-2: Interlacing defined on a sub-band (20 MHz) basis. (Note: Possible interlace spacing discontinuity at edges of sub-band).

In 3GPP RAN1 AH1901 meeting, same spacing (M) between consecutive PRBs in an interlace for all interlaces was agreed for PUSCH on the carriers with wide bandwidth. But it is still FFS for PUCCH.

Firstly, if same interlace design is used for PUCCH and PUSCH, it is more convenient for PUCCH and PUSCH multiplexing transmission. Secondly, there are some unnecessary guard band for Alt 2 in any two adjacent subbands, since interlaces are only based on the fixed number of PRBs in 20MHz. For example, it is 106 PRBs in 20MHz carrier BW when SCS is 15 kHz. If we use this PRB number for interlacing definition, there is almost 1MHz guard band between two subbands which can’t be used.

From this point of view, we prefer Alt 1 for PUCCH interlacing structure

- Same interlace spacing for all interlaces regardless of carrier BW should be used for both PUCCH and PUSCH.

PUSCH

PUSCH transmission in the partial slot

In RAN1#95 meeting, following options have been identified as possible candidates for PUSCH transmission in the partial slot at least for the first PUSCH(s) transmitted in the UL transmission burst.

- Option 1: PUSCH(s) as in Rel-15 NR

- Option 2: Multiple starting positions in one or multiple slot(s) are allowed for PUSCH(s) scheduled by a single UL grant (i.e., not a configured grant) and one of the multiple PUSCH starting positions can be decided depending on LBT outcome.

In our understanding, flexible starting positions for PUSCH is already supported in Rel-15 NR by means of type-B resource mapping, where type-B is for non-slot based transmission. For type-B, we have flexible starting positions and flexible lengths for PUSCH. This section provides specification impacts aspects of each option as given below:

- Option 1 is baseline and it could be applied with no modification in NR-U. For this option, since the starting position and transmission length is determined by the prior received UL grant, they cannot adapt to the outcome of LBT. Therefore, this option would result in less channel access opportunities compared to option 2. Performance impact may be significant due to lack of channel access opportunities.

- Option 2 would provide more channel access opportunities than option 1. However, some enhancements in RE mapping rules and DMRS locations are needed. Compared to option 1, performance improvement may be obvious.

- Option 2 should be supported for PUSCH transmission in the partial slot

PUSCH resource allocation in the frequency domain

For PUSCH resource allocation in the frequency domain, LAA interlaces indication should be used as a starting point. Rel-15 NR support type 0 and type 1 resource allocation. Type 1 resource allocation can be easily used for interlace resources, which start position and duration can be re-interpretation with start interlace and interlace duration. Meanwhile Type 0 resource allocation is the method of bitmap, which can provide more schedule flexible but with larger bit width. So at least type 1 resource allocation should be used for frequency domain resource allocation.

- At least type 1 resource allocation should be used for frequency domain resource allocation.

PUCCH

5 PUCCH formats are supported in Rel-15 NR. PUCCH formats 0 and 2 are short PUCCH durations, supporting 1 or 2 symbols. PUCCH formats 1, 3 and 4 are long durations with equal or more than 4 symbols.

It was agreed to support two legacy PUCCH format in NR-U. Considering PUCCH time domain duration, both short and long PUCCH as PUCCH format 2 and 3 were agreed to be supported, one can be used in the end of DL transmission and the other can be used in UL centered slot.

In addition, some enhancements for PUCCH format(s) to support PRB-based frequency block-interlaced transmission is needed.

NR PUCCH format 2 and 3

In NR, gNB can choose a PUCCH resource for UE, contains PUCCH start symbol, duration and max PRB number in frequency domain. UE will determine the actually PRBs for transmission based on code rate and UCI bits. There are some cases that UCI mapping to contiguous allocations can’t meet the minimum temporal allowance of 2 MHz, such as less than 12/6/3 PRBs for 15/30/60 kHz SCS. And there are two alternatives for this problems.

Alternative 1: these cases are not allowed.

Alternative 2: PUCCH number of OFDM symbols may be selected by UE according to the payload size and the total configured PUCCH frequency resources

- Consider the cases where contiguous allocation is less than minimum temporal allowance of 2MHz.

Interlace based PUCCH formats

Another method to meet the regulatory restrictions on PSD and EIRP as well as on transmission occupied channel BW, PUCCH can use similarly interlaced waveform as PUSCH. In RAN1 96 meeting, it was agreed that support short and long PUCCH durations based on enhancements of at least Rel-15 PUCCH formats PF2 and PF3. However, two open issues including user multiplexing and how to support small payloads are still not identified.

UE multiplexing for interlace based PUCCH can provide more PUCCH capacity at the same opportunity, and can improve interlaced PUCCH utilization efficiency. Since at least one interlace PRBs are configured for one PUCCH, but its UCI only needs one half of its total PRBs. Some methods such as repetitions can be implied but decreasing the resource efficiency. Hence, if suitable UE multiplexing factor is configured semi-statically based on its maximum payload size, a more efficient UCI transmission can be achieved.

For enhanced PUCCH format 2 [3], a possible method is FD-OCC can used for UCI and DMRS in two adjacent REs in a PRB when duration of PUCCH format 2 is one. If its duration is 2, both FD-OCC and TD-OCC can be used for two adjacent REs in two symbols. For enhanced PUCCH format 3, a similar pre-DFT method as PUCCH format 4 was also proposed in [3].

For small payload, we propose to support enhanced of legacy PUCCH formats 0/1(single PRB) by repetition of the coded bit. PUCCH format 0 and 1 can be used for less than 2 bits UCI. There are many scenarios that UE needs only 1 or 2 bits feedback or request, such as Msg. 4 HARQ-ACK, SR request, and semi-persistent PDSCH HARQ-ACK feedback etc. If padding operation applied to all these cases, which would be less resource efficiency.

The following table give an analysis of user multiplexing capacity for enhanced PUCCH format 0/1/2/3. PUCCH format 0 and format 1 use repetition across all the PRBs in one interlace. PUCCH format 2 and format 3 use the methods proposed in [3]. 2 UE multiplexing using frequency domain CDM for enhanced PH2. The duration of PUCCH format 2 and 0 is 1 symbol. And the duration of PUCCH format 1 and 3 is 14 symbols and without frequency hopping.

Table 2: User multiplexing capacity for PUCCH format 0/1/2/3

|  | User multiplexing capacity for 1 bit feedback |  |  | User multiplexing capacity for 2 bits feedback |  |
| --- | --- | --- | --- | --- | --- |
|  | SR | HARQ-ACK | HARQ-ACK/SR | HARQ-ACK | HARQ-ACK/SR |
| PUCCH format 0 | 12 | 6 | 3 | 3 | 1 |
| PUCCH format 1 | 84 [note 1] | 84[note 1] | 42[note2] | 84 | 42[note2] |
| PUCCH format 2 | 2 | 2 | 2 | 2 | 2 |
| PUCCH format 3 | 4 | 4 | 4 | 4 | 4 |

Note 1: A PF1 has 12 circle shift and 7 OCCs, so 84 users can be multiplexed.

Note 2: when HARQ-ACK with or without SR feedback, there are two PF1 resources. So 42 users can be multiplexed.

From Table 2, it shows enhanced PF 0 format can provide more multiplexing capacity compared with enhanced PF 2 when there is 1 bit feedback or 2 bits HARQ-ACK only feedback. Enhanced PF 1 has significant user multiplexing capacity compared with enhanced PF 3. From this side, we propose to support enhanced PF 0 and PF 1 using interlace structure.

- Format 0/1 should be enhanced for interlaced PUCCH structure with small payload UCI.

Conclusion

In the contribution, we discuss the PUSCH/PUCCH/SRS in NR-U operation, and raise the proposals as follows.

Proposal 1.	Within a 20 MHz carrier bandwidth, PRB-based interlace with M =2, N = 12 (24 PRBs) or 13(26 PRBs) should be considered for 60 kHz SCS if it is supported in NR-U.

Proposal 2.	Same interlace spacing for all interlaces regardless of carrier BW should be used for both PUCCH and PUSCH.

Proposal 3.	Option 2 should be supported for PUSCH transmission in the partial slot

Proposal 4.	At least type 1 resource allocation should be used for frequency domain resource allocation.

Proposal 5.	Consider the cases where contiguous allocation is less than minimum temporal allowance of 2MHz.

Proposal 6.	Format 0/1 should be enhanced for interlaced PUCCH structure with small payload UCI.

Reference

- RP-182878	New WID on NR-based Access to Unlicensed Spectrum

- Chairman's Notes RAN1#96bis final

- R1-1904998	UL signals and channels for NR-U	Qualcomm Incorporated