3GPP TSG RAN WG1 #97			R1-1906357

Reno, USA, May 13th – 17th, 2019

Agenda item:	7.2.6.1

Source: 	Spreadtrum Communications

Title: 	Discussion on PDCCH enhancements for URLLC

Document for:	Discussion and decision

Introduction

Work item of “Physical layer enhancements for NR ultra-reliable and low latency communication (URLLC)” was approved to be studied and specified as the WI in RAN#83. Some PDCCH enhancements need to be supported in this work item [1], and the following agreements were achieved during the last meeting [2].

Agreements:

Support configurable number of bits for the following fields for DL DCI format scheduling Rel-16 URLLC.

- Carrier indicator (0 bit or at least one non-zero bit)

- PRB bundling size indicator (0 or 1 bit)

- Rate matching indicator (0, 1 or 2 bits)

- ZP CSI-RS trigger (0, 1 or 2 bits)

Agreements:

The following fields from Rel-15 DCI format 1_1 are not included (in case new DCI format) or can be configured to be absent (0 bit) as in Rel-15 (in case reusing the existing format) in the DL DCI format scheduling Rel-16 URLLC.

- Modulation and coding scheme for TB 2

- New data indicator for TB 2

- Redundancy version for TB 2

- CBG transmission information

- CBG flushing information

Agreements:

Keep the following two fields without any change from Rel-15 DCI in DCI format scheduling Rel-16 URLLC:

- Identifier for DCI formats (1 bit) (when applicable)

- New data indicator (1 bit)

Agreements:

The following field from Rel-15 DCI format 0_1 are not included (in case new DCI format) or can be configured to be absent (0 bit) as in Rel-15 (in case reusing the existing format) in the UL DCI format scheduling Rel-16 URLLC:

- CBG transmission information

Agreements:

For both option 4 and 6, frequency hopping is supported

- FFS details

In this contribution, we provide our considerations on PDCCH enhancements for URLLC, including DCI formats and the increased PDCCH monitoring capability.

Discussion

DCI formats for URLLC

DCI format 1_1 and DCI format 0_1 can be a starting point for DCI formats scheduling Rel-16 URLLC. Some bit fields that were agreed to be included are marked with green. And the bit fields that were agreed to be not included are marked with grey. For the other bit fields in DCI formats, we give some proposals as shown in Table 1 and Table 2.

DL DCI format for URLLC

For the DL DCI format scheduling Rel-16 NR URLLC, the reduced bit width for each field listed in Table 1 can be considered for further study.

Time domain resource allocation can use a reduced bit length. For URLLC mini-slot scheduling, the starting symbol should be defined with respect to the responding DCI to reduce the size of DCI payload. Configurable number of bits (0 or 1 bit) for “VRB-to-PRB mapping” is more suitable, since when the packed size of URLLC traffic is small, interleaved resource allocation can provide additional frequency diversity gain. Therefore, this field can be configurable by higher layer signalling.

Table 1: DL DCI format scheduling Rel-16 NR URLLC

| DCI Bit field | Size (bits) | Status or proposals |
| --- | --- | --- |
| Identifier for DCI formats | 1 | When applicable |
| Carrier Indicator | 0 bit or at least one non-zero bit | Configurable |
| Bandwidth part indicator | 0 | Not support this field. Since BWP switching by DCI introduces additional delay, this field is unnecessary for URLLC traffic. |
| Frequency domain resource assignment | BWP & RA type dependent | Larger RBG can be used to reduce its bit width |
| Time domain resource assignment | 0, 1, 2; | Configurable |
| VRB-to-PRB mapping | 0 or 1; | Configurable. |
| PRB bundling size indicator | 0 or 1; | Configurable |
| Rate matching indicator | 0, 1, or 2; | Configurable |
| ZP CSI-RS trigger | 0, 1, or 2; | Configurable |
| Modulation and coding scheme for TB1 | 5 | Fixed, use the MCS table defined in Rel-15 URLLC |
| New data indicator for TB1 | 1 | If NDI toggled, it is initial transmission |
| Redundancy version for TB1 | 0-1 | Configurable. Considering the low data rate of URLLC, IR gain will not be evident |
| Modulation and coding scheme for TB2 | 5; | not included (in case new DCI format) or can be configured to be absent |
| New data indicator for TB2 | 1 |  |
| Redundancy version for TB2 | 2 |  |
| HARQ process number | 1-2 | Considering the latency boundary of URLLC traffic, there will not be too many URLLC HARQ processes at the same time, 2 or 4 processes will be enough for URLLC traffic. |
| Downlink assignment index | 0, 2, 4; | Configurable |
| TPC command for scheduled PUCCH | 2; | Fixed |
| PUCCH resource indicator | 0-2; | Configurable. When a UL sub-slot is used instead of slot for HARQ-ACK feedback, a reduced number of PUCCH resources can be used. |
| PDSCH-to-HARQ feedback timing indicator | 0-2 | Configurable. Considering the quick HARQ-ACK response for URLLC traffic, fewer k1 values can be used in the HARQ-ACK multiplexing window. |
| Antenna port(s) | 0-2; | Configurable |
| Transmission configuration indication | 0 or 3; | Configurable |
| SRS request | 0-1; | Configurable |
| CBG transmission information (CBGTI) | 0, 2, 4, 6, or 8; | not included (in case new DCI format) or can be configured to be absent |
| CBG flushing out information (CBGFI) | 0, or 1; |  |
| DMRS sequence initialization | 0; | Not support this field |

UL DCI format for URLLC

For the UL DCI format scheduling Rel-16 NR URLLC, the reduced bit width for each field listed in Table 2 can be considered for further study.

Table 2: UL DCI format scheduling Rel-16 NR URLLC

| DCI Bitfield | Size (bits) | Comments |
| --- | --- | --- |
| Identifier for DCI formats | 1 | When applicable |
| Carrier Indicator | 0 bit or at least one non-zero bit | Configurable. Similar as DL DCI scheduling Rel-16 NR URLLC |
| UL/SUL indicator | 0 or 1 | Configurable |
| Bandwidth part indicator | 0 | Not support this field. Since BWP switching by DCI introduces additional delay, this field is unnecessary for URLLC traffic. |
| Frequency domain resource assignment | BWP & RA type dependent | Larger RBG can be used to reduce its bit width |
| Time domain resource assignment |  | It should be considered together with PUSCH transmission schemes. |
| Frequency hopping flag | 0, or 1; | Configurable. |
| Modulation and coding scheme | 5 | Fixed, use the MCS table defined in Rel-15 URLLC |
| New data indicator | 1 |  |
| Redundancy version | 0-1 | Configurable. Considering the low data rate of URLLC, IR gain will not be evident |
| HARQ process number | 1-2 | Considering the latency boundary of URLLC traffic, there will not be too many URLLC HARQ processes at the same time, 2 or 4 processes will be enough for URLLC traffic. |
| 1st downlink assignment index | 1, 2; | 1 for semi-static HARQ-ACK codebook, 2 for dynamic HARQ-ACK codebook, |
| 2nd downlink assignment index | 0 | Not needed. |
| TPC command for scheduled PUSCH | 2; | Fixed |
| SRS resource indicator | 0-2 | Configurable |
| Precoding information and number of layers | 0-6; | Configurable |
| Antenna ports | 0-2 | Configurable |
| SRS request | 0-1 | Configurable |
| CSI request | 0-1 | Configurable |
| CBG transmission information (CBGTI) |  |  |
| PTRS-DMRS association | 0 | Not needed |
| beta_offset indicator | 0 | Not needed |
| DMRS sequence initialization | 1; | Not needed |
| UL-SCH indicator | 1; | Configurable |
| Repetition factor | FFS |  |

It is suitable to introduce some fields in DCI formats scheduling Rel-16 NR URLLC in order to support MIMO transmission. Since PDSCH scheduled by DCI 1_0 can only transmit using one DMRS port, and the QCL assumption is identical to the CORESET of PDCCH. It can neither adjust PDSCH’s QCL source nor DMRS ports according to the current channel states. On the other hand, for PUSCH scheduled by DCI format 0_0 in a cell, the UE shall transmit PUSCH according to the spatial relation corresponding to the PUCCH resource with the lowest ID within the active UL BWP of the cell. Similarly, PUSCH’s spatial relation is fixed when scheduled by DCI format 0_0. Thus, it may reduce the reliability of URLLC traffic, decrease its scheduling flexibility, and lower its performance. So it is beneficial to support multi-antenna transmission for URLLC traffic.

For new features introduced in URLLC, such as priority indication [0~3 bits] should be studied with UCI multiplexing. And repetition factor of PUSCH transmission was agreed for option 4 in the last meeting [2].

- For time domain resource allocation indication for PDSCH for Rel-16 URLLC, using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV is configurable.

- Support configurable number of bits (0, 1, 2 bits) depending on the configured TDRA table for “Time domain resource assignment” for the DL DCI format scheduling Rel-16 URLLC.

- Support configurable number of bits (0 or 1 bit) for “VRB-to-PRB mapping” in DL DCI format scheduling Rel-16 URLLC.

- The fields to support MIMO transmission should be contained in DCI format(s) scheduling Rel-16 NR URLLC.

- Table 1 and Table 2 should be considered for DCI format(s) scheduling Rel-16 NR URLLC to reduce the bit width of some fields in DCI 0_1 and DCI 1_1.

Increased PDCCH monitoring capability

In Rel-15 NR, PDCCH monitoring always satisfies the CCEs/BDs limits. If the configured numbers are more than these limits, UE would follow the search space dropping rules until meeting CCEs/BDs limits. It is clear that the limit of the number of CCEs/BDs does have significant impact on UE complexity. The maximum numbers of BD/CCE limits with different numerologies are defined in 38.213 [2].

It was agreed to study the increased PDCCH monitoring capability on at least the maximum number of non-overlapped CCEs per slot for channel estimation for at least one SCS subject to restrictions including, but not necessary limited to, those identified in TR 38.824 as well as enhancements for PDCCH monitoring capability on the maximum number of monitored PDCCH candidates per slot (with potential restrictions) can be further considered. [1].

CCEs limits

There was a good discussion on non-overlapped CCEs limits during the last meeting but without any achievements due to non-sufficient discussion. Here is the offline proposal [4], which can be the starting point.

Proposal 4-4: Take the following frame work as the potential working assumption for increased UE capability on the maximum number of non-overlapping CCEs for channel estimation:

- PDCCH monitoring span follows the definition in UE feature 3-5b

- Support increased PDCCH monitoring capability with limitation on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span.

  - FFS whether to also define the limitation per OFDM symbol

- The limitation on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span is reported by UE together with X and Y as a capability.

  - One or more cases in the table below can be reported by UE:

    - M is the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span for the corresponding case

      - FFS the value of M

        - FFS the impact from UE capability on processing time (i.e. Capability 1 and capability 2)

        - Companies are encouraged to report the potential aspects that have impact on the value of M

|  | X | Y | M |
| --- | --- | --- | --- |
| Case 1 | 1 | 1 |  |
| Case 2 | 2 | 1 |  |
| Case 3 | 2 | 2 |  |
| Case 4 | 4 | 1 |  |
| Case 5 | 4 | 2 |  |
| Case 6 | 4 | 3 |  |
| Case 7 | 7 | 1 |  |
| Case 8 | 7 | 2 |  |
| Case 9 | 7 | 3 |  |
| Note: Other cases are not precluded |  |  |  |

- FFS the PDCCH monitoring capability on the maximum number of non-overlapped CCEs per slot for channel estimation for handling of the mandatory PDCCH monitoring capability

- FFS whether the PDCCH monitoring capability defined here is applied to only pure URLLC case or also applied to the case of multiplexing eMBB and URLLC

- The PDCCH monitoring capability is defined per component carrier

PDCCH monitoring span

X is the minimum time separation of symbols (including the cross-slot boundary case) between the start of two spans. Y is maximums consecutive OFDM symbols of a slot in a span. And the span pattern repeats in every slot. The same (X, Y) limit must be satisfied by all spans.  It is clear that a UE monitors at most Y OFDM symbols in a span, and a span can consist of several SSs. For example, Y=2, a span can include one monitoring occasion or two monitoring occasions depends on the CORESET durations of these monitored search spaces. REGs within a CORESET are numbered in increasing order in a time-first manner. When CORESET duration is larger than 1, REGs always contain more than 1 OFDM symbols. From this side, non-overlapping CCEs limits per OFDM symbol might be unnecessary when some CORESET durations are larger than 1. Thus the limitation on the maximum number of non-overlapping CCEs per PDCCH monitoring span is more reasonable compared with limitation per OFDM symbol.

- Support increased PDCCH monitoring capability with limitation on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span, and do not define the limitation per OFDM symbol.

Reported combinations of (X, Y)

In UE feature 3-5b, it gives some candidate values set for (X, Y), including:

- (X, Y) = (7, 3)

- (X, Y) = {(4, 3) and (7, 3)}

- (X, Y) = {(2, 2) and (4, 3) and (7, 3)}

There are at most 3 values of X to be reported, 2, 4 and 7 in UE feature 3-5b. But during the offline discussion, (X, Y) = (1, 1) was also proposed to be supported. We do not think this is a typical PDCCH monitoring span configuration. X equals 2 will be enough for the URLLC traffic. For example, when X equals 2, there can be 7 PDCCH monitoring spans in a slot. In addition, if every span contains 2 monitoring occasions, there can be up to 14 monitoring occasions in a slot. There is no need to introduce additional specification work for defining the case of (X, Y) = (1, 1).

- Not support (X, Y) = (1, 1).

M value

The limitations on the maximum number of non-overlapping CCEs for channel estimation per slot are defined per SCS configuration. Considering this, CCEs limits per PDCCH monitoring span should be defined in terms of SCS configuration as well. So we propose:

- The value of CCEs limits per PDCCH monitoring span reported by UE together with (X, Y) and SCS numerology as a capability.

M is the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span for the corresponding case (X, Y). So there are two options for (X, Y, M):

Option 1: There can be several values of M for each (X, Y). UE can choose one value from the set of M values in addition to (X, Y) capability signalling.

Option 2: There is only one value of M for each (X, Y). UE only report (X, Y) as a UE capability. If UE reports (X, Y) can be supported, and then gNB and UE can directly get the fixed value of M for each (X, Y) with different SCSs. The following equation for determining M is proposed.

where = 1.5 for capability 1 and  = 2 for capability 2.

- Study the two options for (X, Y, M):

- Option 1: There can be several values of M for each (X, Y). UE also needs to report M value for (X, Y) as a UE capability.

- Option 2: There is only one value of M for each (X, Y). UE only report (X, Y) as a UE capability.

BDs limits

With the stringent latency and reliability requirements for URLLC, scheduling multiple PDSCHs/PUSCHs in a slot is necessary. As a sequence, it is necessary to configure multiple PDCCH monitoring occasions within a slot, such as 7 monitoring occasions per slot.

For blind decoding, we do not prefer to use larger BD limits, other implementation methods such as fewer PDCCH candidates can be configured for every monitoring occasion. As shown in Table 2, more occasions are configured, the current blind decoding number can at least provide 2 PDCCH candidates for every PDCCH monitoring occasion. So we prefer not to increase the PDCCH blind decoding number for Rel-16 URLLC.

Table 3: Max no. of PDCCH BDs per monitoring occasion

| SCS | Max no. of PDCCH BDs per slot | Max no. of PDCCH BDs per monitoring occasion |  |
| --- | --- | --- | --- |
|  |  | 7 monitoring occasions per slot | 4 monitoring occasions per slot |
| 15 kHz | 44 | 6 | 11 |
| 30 kHz | 36 | 4 | 9 |
| 60 kHz | 22 | 3 | 5 |
| 120 kHz | 20 | 2 | 5 |

- The number of PDCCH blind decoding does not increase in Rel-16 NR URLLC.

Conclusion

In this contribution, we made the following observations and proposals.

Proposal 1.	For time domain resource allocation indication for PDSCH for Rel-16 URLLC, using the starting symbol of the PDCCH monitoring occasion in which the DL assignment is detected as the reference of the SLIV is configurable.

Proposal 2.	Support configurable number of bits (0, 1, 2 bits) depending on the configured TDRA table for “Time domain resource assignment” for the DL DCI format scheduling Rel-16 URLLC.

Proposal 3.	Support configurable number of bits (0 or 1 bit) for “VRB-to-PRB mapping” in DL DCI format scheduling Rel-16 URLLC.

Proposal 4.	The fields to support MIMO transmission should be contained in DCI format(s) scheduling Rel-16 NR URLLC.

Proposal 5.	Table 1 and Table 2 should be considered for DCI format(s) scheduling Rel-16 NR URLLC to reduce the bit width of some fields in DCI 0_1 and DCI 1_1.

Proposal 6.	Support increased PDCCH monitoring capability with limitation on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span, and do not define the limitation per OFDM symbol.

Proposal 7.	Not support (X, Y) = (1, 1).

Proposal 8.	The value of CCEs limits per PDCCH monitoring span reported by UE together with (X, Y) and SCS numerology as a capability.

Proposal 9.	Study the two options for (X, Y, M):

•	Option 1: There can be several values of M for each (X, Y). UE also needs to report M value for (X, Y) as a UE capability.

•	Option 2: There is only one value of M for each (X, Y). UE only report (X, Y) as a UE capability.

Proposal 10.	The number of PDCCH blind decoding does not increase in Rel-16 NR URLLC.

References

RP-190726	 Physical layer enhancements for NR ultra-reliable and low latency communication (URLLC)	Huawei, HiSilicon

3GPP RAN1, “Chairman's Notes RAN1 96bis final”

3GPP TS 38.213: "NR; Physical layer procedures for control"

R1-1905876 Summary of 7.2.6.1 PDCCH enhancements Huawei, HiSilicon

Appendix

BDs/CCEs limits per slot in Rel-15 NR

Table 10.1-2: Maximum number  of monitored PDCCH candidates per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of monitored PDCCH candidates per slot and per serving cell |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 10.1-3: Maximum number  of non-overlapped CCEs per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of non-overlapped CCEs per slot and per serving cell |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

UE feature 3-5b

|  | 3-5b | All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap | PDCCH monitoring occasions of FG-3-1, plus additional PDCCH monitoring occasion(s) can be any OFDM symbol(s) of a slot for Case 2, and for any two PDCCH monitoring occasions belonging to different spans, where at least one of them is not the monitoring occasions of FG-3-1, in same or different search spaces, there is a minimum time separation of X OFDM symbols (including the cross-slot boundary case) between the start of two spans, where each span is of length up to Y consecutive OFDM symbols of a slot. Spans do not overlap. Every span is contained in a single slot. The same span pattern repeats in every slot. The separation between consecutive spans within and across slots may be unequal but the same (X, Y) limit must be satisfied by all spans. Every monitoring occasion is fully contained in one span. In order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. The span duration is max{maximum value of all CORESET durations, minimum value of Y in the UE reported candidate value} except possibly the last span in a slot which can be of shorter duration. A particular PDCCH monitoring configuration meets the UE capability limitation if the span arrangement satisfies the gap separation for at least one (X, Y) in the UE reported candidate value set in every slot, including cross slot boundary. For the set of monitoring occasions which are within the same span: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD The number of different start symbol indices of spans for all PDCCH monitoring occasions per slot, including PDCCH monitoring occasions of FG-3-1, is no more than floor(14/X) (X is minimum among values reported by UE). The number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions of FG-3-1, is no more than 7. The number of different start symbol indices of PDCCH monitoring occasions per half-slot including PDCCH monitoring occasions of FG-3-1 is no more than 4 in SCell. |  | Yes |  | Type 3 | N.A. | N.A. |  | This capability is necessary for each SCS. |  | Optional with capability signaling Candidate value set for (X, Y): {(7, 3), (4, 3) and (7, 3), (2, 2) and (4, 3) and (7, 3)} | Optional with capability signaling Candidate value set for (X, Y): {(7, 3), (4, 3) and (7, 3), (2, 2) and (4, 3) and (7, 3)} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |