3GPP TSG RAN WG1 #97	R1-1906390

Reno, USA, 13th – 17th May, 2019

Agenda Item:	7.2.4.1

Source: 	NEC

Title:	Physical layer structure for NR sidelink

Document for:	Discussion/Decision

Introduction

The work item of 5G V2X with NR sidelink was approved in RAN#83[1], in RAN1#96bis, physical layer structure was discussed with following agreements and working assumption [2]:

Agreements:

- Polar code adopted for Rel-15 NR DCI is applied to PSCCH.

- LDPC codes used for Rel-15 NR DL-SCH is applied to a transport block delivered by PSSCH.

Agreements:

- The starting symbol and the number of symbols for a PSCCH are assumed to be known to the receiving UE before decoding the PSCCH.

Agreements:

- For the purpose of evaluation of PSCCH design, RAN1 assumes 60 bits, 90 bits, 120 bits as the total SCI sizes including 24 bits CRC.

  - Other sizes are not precluded.

Agreements:

  - QPSK is used for PSCCH.

Working assumption:

  - Transmission of 1 TB with up to 2 layers in a PSSCH is supported.

Agreements:

- At least for transmission perspective of a UE in a carrier, at least TDM between PSCCH/PSSCH and PSFCH is allowed for a PSFCH format for sidelink in a slot.

  - FFS the details of the corresponding PSFCH format

  - FFS whether it is also applicable from system/resource pool perspective or not

  - i.e., in this case, there is no simultaneous transmission of PSCCH and PSFCH and there is no simultaneous transmission of PSSCH and PSFCH.

  - FFS FDM between PSCCH/PSSCH and a PSFCH format which uses last symbol(s) available for sidelink in a slot

  - FFS TDM/FDM between PSCCH/PSSCH and other PSFCH format(s), if supported, which is/are different from the PSFCH format which uses last symbol(s) available for sidelink in a slot

In this contribution, we will discuss and give our views on resource pool, control and data multiplexing, DMRS, AGC settling, and PSFCH multiplexing.

Discussion

Resource pool

The structure of resource pool for PSSCH was discussed in RAN1#96 meeting [3]. It was agreed that the time domain granularity of a PSSCH resource pool can be either all symbols in a slot or a subset of consecutive symbols in a slot. On the Uu carrier, slot format may be dynamically changed, however, typically resource pool is configured semi-statically. Hence a reference slot format for resource pool configuration should be semi-statically configured by gNB/eNB. This reference slot format should also be propagated via PSBCH to out of coverage, so as to ensure the communication in partial coverage scenario. Furthermore, as CP-OFDM is supported for NR sidelink, frequency domain resources for a resource pool are not necessarily to be contiguous any more, and this can increase the flexibility on resource pool configuration and also increase the frequency diversity of PSSCH. Since the resource pool is configured via RRC layer signaling, the impact of configuring non-continuous frequency resource as a resource pool is trivial.

Proposal 1:

- A reference slot format for resource pool configuration should be semi-statically configured by base station;

- PSSCH resource pool consisting of non-contiguous resources in frequency domain should be supported in NR sidelink.

Whether separate or shared resource pool for unicast/groupcast/broadcast was discussed through email last meeting. In our view, at least separate resource pool should be used for broadcast due to following reasons:

- The contents of SCI for broadcast may be much less than unicast/groupcast, methods to align SCI size are needed if shared resource pool is used;

- There is no need to configured PSFCH resource in the resource pool for broadcast, which can increase the resource efficiency;

- Unicast/groupcast may be subject to SL pathloss based power control, which implies that the transmission power of unicast/groupcast can be much lower than broadcast, the power difference between them may complicate the sensing behavior in mode 2, and also worsen the near-far problem;

Proposal 2:

- Separate resource pool should be used for broadcast;

Control and data multiplexing

If RAN4 confirmed later that transient period is not needed for Option 3, there are two FFS issues on control and data multiplexing. The first one is how to determine the starting symbol of PSCCH and associated PSSCH in Option 3. In our view, PSCCH should start from symbol#0. As some UEs may not need to settle AGC in a slot or it can settle AGC fast in the first symbol of a slot, and these UEs are able to receive PSCCH transmitted in the first symbol, which can increase the decoding ratio of PSCCH. On the contrary, if PSSCH is transmitted in symbol#0, depending on the DMRS design of PSCCH and PSSCH, the PSSCH transmitted in this symbol may not be used for decoding of the whole PSSCH.

Proposal 3:

- Some redundancy of PSCCH should be transmitted in symbol#0 of a slot.

The other FFS issue is if additional multiplexing options need to be supported to increase PSCCH coverage. In our view, the number of symbols that can be used for one PSCCH can be configurable in Option 3. In other words, if larger coverage is needed then more symbols can be configured for PSCCH, and vice versa. Hence, there is no need to support other options just for the purpose of PSCCH coverage enhancement.

Proposal 4:

- Only Option 3 with configurable number of PSCCH symbols should be supported for PSCCH and PSSCH multiplexing.

DMRS for PSCCH and PSSCH

For the CP-OFDM waveform, DMRS for PDCCH in NR Uu can be reused for PSCCH, the DMRS is inserted continuously in time domain and have ¼ density in frequency domain, which should be sufficient to guarantee the performance of PSCCH.

Proposal 5:

- NR PSCCH reuses DMRS design for NR PDCCH.

Physical data channel in NR Uu supports two DMRS configurations, and for each DMRS configuration, DMRS pattern can be either single-symbol or double-symbol. For single-symbol DMRS configuration 1 or 2, up to 4 or 6 DMRS ports are supported, and the number of supported DMRS ports are double for double-symbol DMRS pattern. Considering the requirement on number of layers in NR sidelink and also the DMRS overhead, single-symbol DMRS pattern should be used for PSSCH. And for CP-OFDM waveform, DMRS configuration 2 is preferable than DMRS configuration 1, since the former has lower frequency domain density and could further reduce the DMRS overhead given DMRS and data can be multiplexed in the same symbol.

Proposal 6:

- Configuration 2 DMRS with single-symbol defined in NR Uu should be used for NR sidelink.

As agreed in the last meeting, multiple DMRS patterns are supported in time domain. To completely exploit the benefit of multiple DMRS patterns, a Tx UE should be allowed to select the appropriate DMRS pattern dynamically according to instant channel status. For this purpose, multiple DMRS patterns should be supported in one resource pool, and the DMRS pattern used by the transmitter should be indicated in the associated PSCCH to enable PSSCH decoding and sensing at the receiver side.

Proposal 7:

- Multiple DMRS patterns should be supported in one resource pool, DMRS pattern used by transmitter should be indicated in the associated PSCCH.

Time domain location of data channel DMRS in NR Uu is related to the mapping type of data channel, i.e. for mapping type A, the first DMRS is placed in the 2nd or 3rd OFDM symbol within the slot, and for mapping type B, the first DMRS is placed in the first data channel symbol. If Option 3 is used for PSCCH and PSSCH multiplexing, both mapping type A and mapping type B may occur for PSSCH in one slot, to guarantee the accuracy of channel estimation for each part of PSSCH, it is preferable to insert DMRS separately for each PSSCH part.

Figure 1 DMRS for PSCCH and different mapping types of PSSCH in Option 3

Proposal 8:

- In Option 3, DMRS for PSSCH part with mapping type A and PSSCH part mapping type B should be inserted separately.

AGC settling and Gap

In LTE V2X the first whole symbol of a subframe is used for AGC settling, and the last whole symbol of a subframe is reserved for Tx/Rx switching (Gap). According to the agreements in RAN1#94[3], the time needed for AGC settling and Tx/Rx switching may be (up to RAN4 confirmation) shorter than half symbol for 15 kHz and 30 kHz SCS. Hence, for 15 kHz and 30 kHz SCS, there is possibility to further reduce the overhead of AGC and gap in NR V2X.

In case of the duration of one symbol is longer than the time for AGC settling plus the time for Tx/Rx switching, one example for AGC settling and Tx/Rx switching is given in Figure 2. As Tx/Rx switching is only necessary for the UEs switching Tx/Rx behavior in next slot, there is no need to configure gap for Tx/Rx switching from system point of view. For a UE is transmitting in the current slot and going to receiving in the next slot, it can perform Tx to Rx switching at the beginning of the next slot, and then use the remaining part of the first symbol for AGC settling. On the contrary, if the UE is receiving at the current slot and going to transmitting in the next slot, if can perform the Rx to Tx switching at the end of the current slot. For a UE receiving continuously, it only needs to settle AGC at the beginning of the first symbol. In this way, all symbols in a slot can be used for transmission.

Proposal 9:

- In case of the duration of one symbol is longer than the time for AGC plus the time for Tx/Rx switching, system gap for Tx/Rx switching is not configured.

Figure 2 AGC and/or Tx/Rx switching time for a UE switching from Tx to Rx, for a switching from Rx to Tx, and for a UE receiving continuously

In case of the duration of one symbol cannot accommodate both AGC and Tx/Rx switching, LTE sidelink design can be reused, i.e. reserving one symbol for AGC and Tx/Rx switching respectively.

Furthermore, as not all UEs need to settle AGC at the beginning of a slot, in another word, some UE is capable to receive and decoding the first symbol, it is beneficial to transmit data or control information rather than an AGC signal in the first symbol.

Proposal 10:

- PSCCH and PSSCH are transmitted in the first symbol for AGC settling.

PSFCH multiplexing

As agreed in the last meeting, at least from transmitting UE perspective, PSFCH is TDMed with PSCCH and PSSCH. To avoid additional AGC time, from system perspective, the starting time and duration of all PSFCH on a carrier should be the same.

Proposal 11:

- The starting time and duration of all PSFCH on a carrier should be the same;

Moreover, in case of unicast, at most one (frequency, code) PSFCH resource is needed for one sub-channel. Recalling the sub-channel configuration in LTE V2X, where the whole bandwidth can be configured as one sub-channel, in this case, it is too wasteful to configure all PRBs in a symbol as PSFCH resource. Hence, from system point of view, FDM between PSFCH and PSCCH/PSSCH should be supported.

Proposal 12:

- From system point of view, FDM between PSFCH and PSCCH/PSSCH should be supported;

As agreed in the last meeting, each PSFCH is mapped to a time, frequency, and code resource. For unicast, the time, frequency and code resource for a PSFCH can be determined according to the location of associated PSCCH/PSSCH resources. For groupcast, 2 Options for HARQ feedback in groupcast were agreed in last meeting. As power from multiple UEs may accumulated on one PSFCH resource in Option 1, a dedicated PRB (i.e. no CDM multiplexing among different UEs) should be used to avoid interference to other multiplexed users. Additionally, in Option 2, more than one PSFCH resources are needed for one PSCCH/PSSCH, hence, one-to-many mapping from PSCCH/PSSCH to PSFCH should be supported. In case of unicast and groupcast sharing a single resource pool, one slot may be configured with both PSFCH resources for unicast and groupcast,  to avoid the PSFCH collision between unicast and groupcast, separate PSFCH resource sets should be (pre-)configured for unicast and groupcast.

Proposal 13:

- Time, frequency and code resource for unicast is determined according to the location of associated PSCCH/PSSCH resources;

- In case of unicast and groupcast sharing a single resource pool, separate PSFCH resource sets should be (pre-)configured for unicast and groupcast;

Conclusion

In this contribution, we discussed the issues related to physical layer structure in NR-V2X, we have following proposals:

Proposal 1:

- A reference slot format for resource pool configuration should be semi-statically configured by base station;

- PSSCH resource pool consisting of non-contiguous resources in frequency domain should be supported in NR sidelink.

Proposal 2:

- Separate resource pool should be used for broadcast;

Proposal 3:

- Some redundancy of PSCCH should be transmitted in symbol#0 of a slot.

Proposal 4:

- Only Option 3 with configurable number of PSCCH symbols should be supported for PSCCH and PSSCH multiplexing.

Proposal 5:

- NR PSCCH reuses DMRS design for NR PDCCH.

Proposal 6:

- Configuration 2 DMRS with single-symbol defined in NR Uu should be used for NR sidelink.

Proposal 7:

- Multiple DMRS patterns should be supported in one resource pool, DMRS pattern used by transmitter should be indicated in the associated PSCCH.

Proposal 8:

- In Option 3, DMRS for PSSCH part with mapping type A and PSSCH part mapping type B should be inserted separately.

Proposal 9:

- In case of the duration of one symbol is longer than the time for AGC plus the time for Tx/Rx switching, system gap for Tx/Rx switching is not configured.

Proposal 10:

- PSCCH and PSSCH are transmitted in the first symbol for AGC settling.

Proposal 11:

- The starting time and duration of all PSFCH on a carrier should be the same;

Proposal 12:

- From system point of view, FDM between PSFCH and PSCCH/PSSCH should be supported;

Proposal 13:

- Time, frequency and code resource for unicast is determined according to the location of associated PSCCH/PSSCH resources;

- In case of unicast and groupcast sharing a single resource pool, separate PSFCH resource sets should be (pre-)configured for unicast and groupcast;

References

- RP-190766, New WID on 5G V2X with NR sidelink, LG Electronics, Huawei;

- Chairman's Notes RAN1#96bis;

- Chairman's Notes RAN1#96;

- Chairman's Notes RAN1#94;