3GPP TSG RAN WG1 #97	R1-1906449

Reno, USA, May 13th – 17th, 2019

Source:	OPPO

Title:	PUSCH enhancement for URLLC

Agenda Item:	7.2.6.3

Document for:	Discussion and Decision

Introduction

In RAN 96bis, some agreements on PUSCH were achieved:

Agreements:

- Option 5 is not considered further as part of PUSCH enhancements.

Agreements:

For option 4, dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH is supported for PUSCH enhancements. The dynamic indication can be enabled or disabled by the gNB.

- FFS the exact signaling method

- FFS the exact DCI format(s)

- FFS the exact mechanism to enable or disable

- FFS the DCI activating type 2 configured grant PUSCH

Agreements:

For option 6,

- For dynamic PUSCH

  - For semi-static DL symbol(s), to down-select

    - Option 1: it is not expected that the resource allocation has conflict with semi-static DL symbol(s).

    - Option 2: if the resource allocation has conflict with semi-static DL symbol(s), the repetition is not transmitted.

  - For dynamically indicated DL symbol(s) (via format 2_0), it is not expected at the UE that the resource allocation has conflict with dynamically indicated DL symbol(s).

    - Note: this is the same as Rel-15 behavior.

- For configured grant PUSCH,

  - For type 1 configured grant PUSCH, and PUSCH other than the first PUSCH (including all repetitions) associated with the type 2 configured grant activation,

    - If a repetition conflicts with semi-static DL symbol(s), the repetition is not transmitted.

    - FFS: If a repetition conflicts with dynamically indicated DL symbol(s) (via format 2_0), the repetition is not transmitted.

  - FFS For the first PUSCH (including all repetitions) associated with the type 2 configured grant activation, follow the same handling as dynamic PUSCH.

Agreements:

- For option 6, at least for dynamic grants, it is not expected that one repetition (i.e., one SLIV) spans across slot boundary.

Agreements:

For both option 4 and 6, frequency hopping is supported

- FFS details

In this contribution, we shall focus on details for option 4.

Potential issues for one grant to schedule PUSCH repetition

The following issues are open for option 4:

- Dynamic indication

Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH is supported for PUSCH enhancements. The dynamic indication can be enabled or disabled by the gNB. One solution for dynamic indication is to include the parameter “dynamic indication” into time domain resource allocation. Usually, for cell center or middle UE, cross-slot boundary transmission is very low probability and latency due to cross-slot boundary is very short due to transmission duration is short, however, flexible starting symbol and length is required to support various TB size. Therefore, for cell center or middle UE, various starting symbol and length and only one repetition number (=1) is included in time domain resource table, shown in Table 1. For cell edge UE, limited transmission duration, e.g. only long transmission duration, is needed and unused fields can be used for repetition number >1. Table 2 shows an example of time domain resource allocation for cell edge UE.

Table 1 An example of time domain resource allocation (Cell center or middle UE)

| Row index | PUSCH mapping type | K2 | S | L | Repetition number |
| --- | --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14 | 1 |
| 2 | Type A | j | 0 | 10 | 1 |
| 3 | Type A | j | 0 | 4 | 1 |
| 4 | Type B | j | 2 | 4 | 1 |
| 5 | Type B | j | 4 | 4 | 1 |
| 6 | Type B | j | 6 | 4 | 1 |
| 7 | Type B | j | 8 | 4 | 1 |
| 8 | Type A | j+1 | 0 | 14 | 1 |
| 9 | Type A | j+1 | 0 | 10 | 1 |
| 10 | Type A | j+1 | 0 | 4 | 1 |
| 11 | Type A | j+2 | 0 | 14 | 1 |
| 12 | Type A | j+2 | 0 | 10 | 1 |
| 13 | Type A | j+2 | 0 | 10 | 1 |
| 14 | Type B | j | 10 | 4 | 1 |
| 15 | Type A | j+3 | 0 | 14 | 1 |
| 16 | Type A | j+3 | 0 | 10 | 1 |

Table 2 An example of time domain resource allocation (Cell edge UE)

| Row index | PUSCH mapping type | K2 | S | L | Repetition number |
| --- | --- | --- | --- | --- | --- |
| 1 | Type B | j | 0 | 14 | 1 |
| 2 | Type B | j | 2 | 14 | 1 |
| 3 | Type B | j | 4 | 14 | 1 |
| 4 | Type B | j | 6 | 14 | 1 |
| 5 | Type B | j | 8 | 14 | 1 |
| 6 | Type B | j | 10 | 14 | 1 |
| 7 | Type B | j | 12 | 14 | 1 |
| 8 | Type B | j+1 | 0 | 14 | 1 |
| 9 | Type B | j+1 | 2 | 14 | 1 |
| 10 | Type B | j+1 | 4 | 14 | 1 |
| 11 | Type B | j+1 | 6 | 14 | 1 |
| 12 | Type B | j | 4 | 7 | 4 |
| 13 | Type B | j | 6 | 7 | 4 |
| 14 | Type B | j | 8 | 7 | 4 |
| 15 | Type B | j | 10 | 7 | 4 |
| 16 | Type B | j | 12 | 7 | 4 |

Whether to enable or disable repetition can be indicated by higher layer signaling pusch-TimeDomainAllocationList implicitly. DCI activating type 2 configured grants PUSCH can applies the same method and pusch-TimeDomainAllocationList.

Proposal 1: Repetition number is included in pusch-TimeDomainAllocationList

- Frequency hopping

For dynamic transmission, Frequency hopping provides frequency diversity. However, URLLC transmission is usually wide transmission band and short time duration to reduce latency. For wideband transmission, frequency hopping only provides limited frequency diversity. Therefore, simple frequency hopping is enough for dynamic transmission.

For configured grant, collision among UE in one grant free can not be controlled by gNB and hopping is only way to resolve collision. To recognize UE as early as possible, hopping with small time granularity, e.g. symbol-level, should be considered for grant free.

In addition, Current hopping pattern only depends on two parameters, and, available hopping pattern is very limited and randomness of collision among UE is very limited, even leads persistent collision, which is fatal for URLLC. Non-orthogonal resource allocation, especially for data, is usual for URLLC to support more grant free UE in limited resource. If non-orthogonal resource for DMRS is applied, UE cannot be identified and data cannot be demodulated. So we suggest more flexible hopping pattern can be considered in terms of hopping sequence and hopping resource.

Proposal 2: Larger than 2 hopping and UE-specific hopping pattern is benefit for interference randomization.

- DMRS allocation

When one repetition is segmented, then DMRS is required in multiple segmented repetition parts. And DMRS resource allocation, including front-loaded and additional DMRS, in each segmented repetition part is determined by segmented repetition part duration and dmrs-AdditionalPosition.

Proposal 3: DMRS resource allocation, including front-loaded and additional DMRS, in each segmented repetition part is determined by segmented repetition part duration and dmrs-AdditionalPosition.

For Type A PUSCH mapping, DMRS resource allocation scheme needs additional specification effort for subsequent PUSCH repetition (repetition index>1) due to DMRS resource reference is slot boundary and DMRS time domain resource is usually different for multiple PUSCH repetitions. And Type B provides flexible scheduling and simple DMRS resource allocation scheme. So only Type B PUSCH mapping is supported for repetition number>1.

Proposal 4: Only Type B PUSCH mapping is supported for repetition number>1.

- Interaction with the DL/UL direction

For slot repetition, if transmission direction in one PUSCH repetition is not all uplink or flexible, then the PUSCH repetition is discarded. For mini-slot repetition, the similar rule can be a starting point.

Proposal 5: If transmission direction in one PUSCH repetition is not all uplink or flexible, then the PUSCH repetition is discarded.

- TBS determination

For slot repetition, TBS is determined by first repetition. Similarly, for mini-slot repetition and multi-segments, TBS is determined by first actual repetition or segment.

Proposal 6: TBS is determined by first actual repetition or segment.

Conclusions

In this contribution, we show our views on PUSCH enhancement for URLLC with following proposals:

Proposal 1: Repetition number is included in pusch-TimeDomainAllocationList.

Proposal 2: Larger than 2 hopping and UE-specific hopping pattern is benefit for interference randomization.

Proposal 3: DMRS resource allocation, including front-loaded and additional DMRS, in each segmented repetition part is determined by segmented repetition part duration and dmrs-AdditionalPosition.

Proposal 4: Only Type B PUSCH mapping is supported for repetition number>1.

Proposal 5: If transmission direction in one PUSCH repetition is not all uplink or flexible, then the PUSCH repetition is discarded.

Proposal 6: TBS is determined by first actual repetition or segment.

References

- RAN1 96bis chairman notes