3GPP TSG RAN WG1 #97	R1-1906530

Reno, USA, May 13th – 17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS38.214 on CSI reference resource for SSB-based L1-RSRP reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-5-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following description is referred to determine the duration of CPU occupation for SSB-based L1-RSRP: “A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the PUSCH/PUCCH carrying the report.” However, the definition of CSI reference resource corresponding to SSB is missing in 38.214. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the reference resource definition assoicated with SSB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The rule determining CPU occupation for SSB-based L1-RSRP is broken. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis This CR is to correct an error which causes undefined CSI reference resource for SSB-based L1-RSRP report. No impact on current NW/UE behaviour as the correction is common understanding. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.2.2.5	CSI reference resource definition

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref,

-	where  and and  are the subcarrier spacing configurations for DL and UL, respectively

-	where for periodic and semi-persistent CSI reporting

-	if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or

-	if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot.

-	where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Subclause 5.4.

-	when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting.

< Unchanged parts are omitted >