3GPP TSG RAN WG1 Meeting #97	R1-1906533

# Reno, USA, May 13th – 17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PUCCH/PUSCH multiplexing considering cancellation of grant-free PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-5-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PUCCH/PUSCH multiplexing timeline conditions defined in 38.213 section 9.2.5 do not restrict the monitoring occasions when the UE is expected to receive a PDCCH that effectively cancels a grant-free PUSCH that is overlapping with a PUCCH. Consequently, the UE is either unable to evaluate the overlap between PUCCH and grant-free PUSCH before the PUCCH start boundary, or its time budget to enable/cancel transmission of PUCCH is tighten. This change is necessary to enable the UE to evalute PUCCH and grant-free PUSCH overlap condition in time for the the configuration or cancelation of the PUCCH transmission. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add text to existing paragraphs in section 9.2.5 of 38.213 to: - Clarify that existing PUCCH/PUSCH multiplexing timeline conditions also apply to the reception of PDCCH including a DCI format that indicates transmission of a dynamic PUSCH that cancels a grant-free PUSCH overlapping with a PUCCH, even when the dynamic PUSCH does not overlap with the PUCCH. - Ensure that the PUCCH/PUSCH multiplexing timeline conditions also apply to the reception of PDCCH including a DCI format that indicates to restart the same configuredGrantTimer as the one used by a grant-free PUSCH that overlaps with a PUCCH when the restart of such configuredGrantTimer causes the cancellation the grant-free PUSCH. Note the dynamic PUSCH scheduled by the PDCCH that restarts the configuredGrantTimer may or may not overlap with PUCCH(s) and/or PUSCH(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If a grant-free PUSCH overlapping with a PUCCH is cancelled due to the reception of a PDCCH including a DCI format that indicates transmission of a dynamic PUSCH that overlaps with the grant-free PUSCH, the UE will not transmit PUCCH and UCI will not be transmitted on the dynamic PUSCH that superseds the grant-free PUSCH. If a grant-free PUSCH overlapping with a PUCCH may be cancelled due to the UE detecting a PDCCH including a DCI format that indicates to restart the same configuredGrantTimer as the one used by the grant-free PUSCH, the time budget the UE has to either enable or cancel PUCCH transmission is reduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impact Analysis: Impacted functionality: multiplexing of PUCCH(s) and PUSCH(s) If the UE is implemented according to the change but the NW is not, - For some scheduling, NW may think it valid, but UE will regard it as error case. If the NW is implemented according to the change but the UE is not, - NW will avoid some scheduling since it is specified as error case. UE may take extra implementation effort to support error case. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------- Start of Text Proposal ---------------------------------------------

9.2.5	UE procedure for reporting multiple UCI types

< Unchanged parts are omitted >

If a UE would transmit multiple overlapping PUCCHs in a slot or overlapping PUCCH(s) and PUSCH(s) in a slot and, when applicable as described in Subclauses 9.2.5.1 and 9.2.5.2, the UE is configured to multiplex different UCI types in one PUCCH, and at least one of the multiple overlapping PUCCHs or PUSCHs is in response to a DCI format detection by the UE, the UE multiplexes all corresponding UCI types if the following conditions are met. The overlapping PUSCH(s) include PUSCH scheduled by DCI format 0_0 or DCI format 0_1 and/or PUSCH with configured grant, as well as those PUSCH(s) that do not directly overlap with a PUCCH , but overlap with other PUSCH(s) that overlap with the PUCCH.  If one of the PUCCH transmissions or PUSCH transmissions is in response to a DCI format detection by the UE, the UE expects that the first symbol  of the earliest PUCCH or PUSCH, among a group overlapping PUCCHs and PUSCHs in the slot, satisfies the following timeline conditions

-	 is not before a symbol with CP starting after  after a last symbol of any corresponding PDSCH, where  corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH scheduling the PDSCH, the SCS configuration of the PDSCH, and the smallest SCS configuration for the group of overlapping PUCCHs and PUSCHs where the UE transmits HARQ-ACK information in response to the reception of the PDSCH

-	 is not before a symbol with CP starting after  after a last symbol of any corresponding SPS PDSCH release, where  is described in Subclause 10.2 and  corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH providing the SPS PDSCH release and the smallest SCS configuration for the group of overlapping PUCCHs or overlapping PUCCHs and PUSCHs where the UE transmits HARQ-ACK information in response to the detection of the SPS PDSCH release

-	if there is no aperiodic CSI report multiplexed in a PUSCH in the group of overlapping PUCCHs and PUSCHs,  is not before a symbol with CP starting after  after a last symbol of

-	a PDCCH with the DCI format scheduling the PUSCH, and

-	any PDCCH scheduling a PDSCH or SPS PDSCH release with corresponding HARQ-ACK information in an overlapping PUCCH in the slot, and

-	a PDCCH scheduling a PUSCH with the same HARQ process on the same serving cell as the PUSCH with configured grant in the group of overlapping PUCCHs and PUSCHs and the restarting configuredGrantTimer cancels the PUSCH with configured grant.

where  corresponds to the smallest SCS configuration among the SCS configuration of the PDCCHs and the smallest SCS of the overlapping PUCCHs and PUSCHs, and  if there is no overlapping PUSCH

-	if there is an aperiodic CSI report multiplexed in a PUSCH in the group of overlapping PUCCHs and PUSCHs,  is not before a symbol with CP starting after  after a last symbol of

-	a PDCCH with the DCI format scheduling the PUSCH, and

-	any PDCCH scheduling a PDSCH or SPS PDSCH release with corresponding HARQ-ACK information in an overlapping PUCCH in the slot, and

-	a PDCCH scheduling a PUSCH with the same HARQ process on the same serving cell as the PUSCH with configured grant in the group of overlapping PUCCHs and PUSCHs and the restarting configuredGrantTimer cancels the PUSCH with configured grant.

where  corresponds to the smallest SCS configuration among the SCS configuration of the PDCCHs, the smallest SCS configuration for the group of the overlapping PUCCHs and PUSCHs, and the smallest SCS configuration of aperiodic CSI-RS associated with the DCI format scheduling the PUSCH, and  for ,  for  and  for

-	, , , , , and  are defined in [6, TS 38.214], and  and  are defined in [4, TS 38.211].

< Unchanged parts are omitted >

----------------------------------------------- End of Text Proposal ---------------------------------------------