3GPP TSG RAN WG1 Meeting #97	R1-1906599

Reno, USA, May 13-17, 2019

Agenda Item:	7.2.4.2.3

Source:	Huawei, HiSilicon

Title:	Details of LTE Uu configuration of NR sidelink

Document for:	Discussion and decision

# Introduction

A new WID on NR-V2X was approved at RAN#83 [1], including objectives for cross-RAT control of sidelink resource allocation as follows:

- Resource allocation [RAN1, RAN2]

  - Mode 1

    - NR sidelink scheduling by NR Uu and LTE Uu as per the study outcome

  - Mode 2

    - Sensing and resource selection procedures based on sidelink pre-configuration and configuration by NR Uu and LTE Uu as per the study outcome

The agreements achieved in the study item phase for LTE Uu configuration of NR sidelink include:

Agreements:

It is supported that LTE Uu provides at least necessary semi-static configuration for NR mode-2 SL communications

FFS details

Further study impact and benefits of LTE Uu managing NR mode-1 SL communications

Agreements:

- LTE Uu to schedule NR sidelink mode 1 is supported:

  - The support is done based on type 1 configured grant with configuration restricted to time/frequency resources & periodicity, with the condition that no additional function/procedure is to be introduced for LTE Uu

  - Both DCI based scheduling and type 2 configured grant scheduling are not supported for scheduling NR sidelink mode 1

- When NR sidelink mode-1 is controlled by gNB, dynamic scheduling, and configured grants of type 1 and type 2 are supported. When controlled by eNB, neither dynamic scheduling nor type 2 configured grant are supported, and the support of NR sidelink mode-1 is based on type 1 configured grants with some simplifications.

- This contribution provides details on the Mode 1-Type 1 configured grant configuration by LTE and how eNB configures the resources for NR mode 2.

# Configurations for NR mode-1 SL

## NR Uu configured grant configuration

The design of NR sidelink structure and procedure is continuing to be discussed in the work item, and more agreements are still needed to make how NR sidelink mode-1 works clear. In the meantime, NR Uu configured grant configuration is a suitable a starting point to discuss the details of configuration for NR sidelink mode-1 type 1 by LTE Uu.

The configuration for the configured grant is as follows [2]:

#### –	ConfiguredGrantConfig

The IE ConfiguredGrantConfig is used to configure uplink transmission without dynamic grant according to two possible schemes. The actual uplink grant may either be configured via RRC (type1) or provided via the PDCCH (addressed to CS-RNTI) (type2).

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                		   OPTIONAL,   -- Need S,

cg-DMRS-Configuration               DMRS-UplinkConfig,

mcs-Table                           ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

mcs-TableTransformPrecoder          ENUMERATED {qam256, qam64LowSE}                                         OPTIONAL,   -- Need S

uci-OnPUSCH                         SetupRelease { CG-UCI-OnPUSCH }                                         OPTIONAL,   -- Need M

resourceAllocation                  ENUMERATED { resourceAllocationType0, resourceAllocationType1, dynamicSwitch },

rbg-Size                            ENUMERATED {config2}                                                    OPTIONAL,   -- Need S

powerControlLoopToUse               ENUMERATED {n0, n1},

p0-PUSCH-Alpha                      P0-PUSCH-AlphaSetId,

transformPrecoder                   ENUMERATED {enabled, disabled}                                          OPTIONAL,   -- Need S

nrofHARQ-Processes                  INTEGER(1..16),

repK                                ENUMERATED {n1, n2, n4, n8},

repK-RV                             ENUMERATED {s1-0231, s2-0303, s3-0000}                                  OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer                    INTEGER (1..64)                                                     OPTIONAL,   -- Need R

rrc-ConfiguredUplinkGrant               SEQUENCE {

timeDomainOffset                        INTEGER (0..5119),

timeDomainAllocation                    INTEGER  (0..15),

frequencyDomainAllocation               BIT STRING (SIZE(18)),

antennaPort                             INTEGER (0..31),

dmrs-SeqInitialization                  INTEGER (0..1)                                                  OPTIONAL,   -- Need R

precodingAndNumberOfLayers              INTEGER (0..63),

srs-ResourceIndicator                   INTEGER (0..15)                                                 OPTIONAL,   -- Need R

mcsAndTBS                               INTEGER (0..31),

frequencyHoppingOffset                  INTEGER (1.. maxNrofPhysicalResourceBlocks-1)                   OPTIONAL,   -- Need R

pathlossReferenceIndex                  INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1),

...

}                                                                                                           OPTIONAL,       -- Need R

...

}

CG-UCI-OnPUSCH ::= CHOICE {

dynamic                                 SEQUENCE (SIZE (1..4)) OF BetaOffsets,

semiStatic                              BetaOffsets

}

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

According to the agreement, the configuration from LTE Uu for NR sidelink mode-1 type 1 is restricted to time/frequency resources and periodicity. Hence, the green highlighted parameters of the NR Uu configured grant are needed.

Proposal 1: The following parameters of NR Uu configured grant are needed for LTE Uu configuration of NR sidelink mode-1 type 1:

- resourceAllocation

- periodicity

- configuredGrantTimer

- timeDomainOffset

- timeDomainAllocation

- frequencyDomainAllocation

- frequencyHoppingOffset

## More necessary configuration needed for NR mode-1 SL

The configuration restricted to time/frequency resources and periodicity aims to simplify/facilitate network management and/or UE implementation, based on an assumption that it is challenging for an eNB to obtain dynamic information about the NR sidelink without introducing new feedback information over LTE Uu, and changes in eNB schedulers. Thus the agreement assumes the LTE network provides time and frequency domain resources, and leaves the selection and adaption of transmission schemes and parameters to sidelink transmission UEs, e.g., MCS/TBS, etc, in a way similar to how the LTE sidelink works, even in mode 3.

However, if some parameters as cyan highlighted from NR Uu configured grant configuration, e.g., rbg-Size, dmrs-Seqinitialization, etc, are up to sidelink transmission UE choice, such parameters need to be indicated dynamically on the NR sidelink, which would unnecessarily increase the size of control information, whether carried by SCI or DMRS. Therefore, such parameters would be better to be configured by LTE Uu configuration as well.

Proposal 2: In addition to the parameters related to time/frequency resources & periodicity, at least the following parameters of NR Uu configured grant are needed for LTE Uu configuration of NR sidelink mode-1 type 1:

- rbg-Size

- dmrs-SeqInitialization

In addition, RAN2 has agreed at RAN2#105bis [3] that multiple active configured sidelink grants are supported in NR sidelink mode 1. It is natural that NR sidelink mode 1 operation supports the same  number of type 1 configured grants when under LTE Uu control.

Proposal 3: NR SL mode 1, when controlled by LTE Uu, supports the same number of Type 1 configured sidelink grants as when controlled by NR Uu.

For some advanced V2X service scenarios (e.g., platooning) temporary blockage of the LTE Uu link can be addressed in a similar way as in case of NR Uu controlling NR SL mode-1 communications in [4]. That is, during a temporary connection interruption, the NR UE can start or continue using a NR SL type-1 configured grant previously provided by the eNB.

Proposal 4: When it experiences RRC connection interruption, the NR UE can start or continue using a NR SL type-1 configured grant which was provided by the eNB, following procedures defined for the NR Uu interface.

# Configurations for NR mode-2 SL

Regarding NR mode-2 SL, it was agreed that mode 2 supports the sensing and resource (re)-selection procedures and FFS resource granularity for sensing and resource (re)-selection, e.g., PRB(s), slots, resources patterns, etc. Therefore, the resources for NR mode-2 SL sensing and resource (re)-selection procedures will be configured by gNB.

RAN2 has agreed at RAN2#105bis [3] that:

- LTE Uu supports configuration for NR mode-2 operation through RRC signaling and through SIB signaling. However, on demand or per valid area mode-2 configuration via SIB signaling is not supported.

- New system information block type should be designed to support NR sidelink Tx and Rx resource pool configuration via LTE Uu. New system information will be defined as container (OCTET STRING) and actual information will follow what is defined in NR RRC.

- eNB should be able to configure the NR V2X mode 2 sidelink resource pool or type 1 configured grant for NR V2X sidelink communication via dedicated signalling.

More agreements in RAN1 are needed for NR sidelink mode-2 operation regarding sensing and resource (re)-selection working in the control of NR Uu. According to the RAN2 agreement, that new system information block will be designed to support NR sidelink Tx and Rx resource pool configuration and it will be defined as a container, and follow what is defined in NR RRC. Hence, for NR V2X mode 2 sidelink resource pool configuration via dedicated signaling, the signaling design should follow what is defined in NR dedicated signaling.

Proposal 5: For NR V2X mode 2 sidelink resource pool configuration via dedicated RRC signaling from LTE Uu, information contained in the dedicated signaling will follow what is defined in NR RRC dedicated signaling.

# Conclusions

This contribution focuses on the configurations for NR mode-1 and mode-2 SL in the control of LTE Uu, which leads to the following proposals:

Proposal 1: The following parameters of NR Uu configured grant are needed for LTE Uu configuration of NR sidelink mode-1 type 1:

- resourceAllocation

- periodicity

- configuredGrantTimer

- timeDomainOffset

- timeDomainAllocation

- frequencyDomainAllocation

- frequencyHoppingOffset

Proposal 2: In addition to the parameters related to time/frequency resources & periodicity, at least the following parameters of NR Uu configured grant are needed for LTE Uu configuration of NR sidelink mode-1 type 1:

- rbg-Size

- dmrs-SeqInitialization

Proposal 3: NR SL mode 1, when controlled by LTE Uu, supports the same number of Type 1 configured sidelink grants as when controlled by NR Uu.

Proposal 4: When it experiences RRC connection interruption, the NR UE can start or continue using a NR SL type-1 configured grant which was provided by the eNB, following procedures defined for the NR Uu interface.

Proposal 5: For NR V2X mode 2 sidelink resource pool configuration via dedicated RRC signaling from LTE Uu, information contained in the dedicated signaling will follow what is defined in NR RRC dedicated signaling.

# References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, RAN#83, Shenzhen, China, March 2019.

TS 38.331, “NR; Radio Resource Control (RRC) protocol specification (Release 15)”.

RAN2 Chair notes, RAN2#105bis.

R1-1906010, “Sidelink resource allocation mode 1”, Huawei, HiSilicon, RAN1#97, Reno, USA, May 2019.