3GPP TSG RAN WG1 Meeting #97	 R1-1906602

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.8.5

Source:	Huawei, HiSilicon

Title:	Evaluation results of DFT-based compression codebook for rank 1 and 2

Document for:	Discussion and Decision

Introduction

It has been agreed at RAN1 #95 [1] that:

Agreement

For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002

- Note: The same DFT-based compression scheme is extended for Type II port selection codebook

- Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme

  - FFS: detailed signaling mechanism

- Note: Additional compression scheme(s) are not precluded

It has been agreed at RAN1 AH1901 [2] that:

Agreement

Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R≤19,

Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R>19, downselect among the following alternatives in RAN1#96

- Alt1: N3N3 is smallest multiple of 2, 3, or 5 which is  ≥NSB×R

- Alt2: N3N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

It has been agreed at RAN1 96bis [3] that:

Agreement

Support L=6 for the following combinations of p and beta

- p value equals to 1/4, beta value equals to {1/4, 1/2, 3/4}

- p value equals to 1/2, beta value equals to 1/4

Above applies only for the case of 32 ports, rank 1 or 2, R=1

Note that the payload size for L=6 should not exceed that of Rel-15 type-2 codebook

The above feature is UE optional

FFS: Further specification support to relax UE processing complexity

In this contribution, some additional evaluation results are provided as the supplement of the main contribution [4].

Overhead and performance of DFT-based codebook

For rank 1 case, the overhead of codebook is as follows.

- Spatial domain basis selection

  - Orthogonal group: log2O1O2 bits

  - L beams selection: log2N1N2L bits, where L∈{2,4,6}

  - For candidate values of O1, O2, N1 and N2, Table 5.2.2.2.1-2 in 38.214 can be re-used.

- Frequency domain basis selection

  - M basis selection: log2N3M bits, where M=p*N3, p∈{14,12}

- Size-K0 subset

  - Subset indication bitmap: 2LM

- Coefficients quantization for pl,m and φl,m

  - Index of the leading coefficient: log2KNZ bits, where KNZ≤2LM*β, β∈{14,12,34}

  - Reference amplitude of the second polarization: 4 bits

  - Quantization: A+P*(KNZ-1) bits, where A=3, P∈{3, 4}

For the sake of discussion, the overhead of all the supported parameter combinations for 32 ports, 16PSK and 13 subbands case is listed in the following Table I. Despite the potential designs in UCI, only the overhead for codebook design is included.

Table I. Codebook overhead of all the supported parameters for 32 ports, 16PSK and 13 subbands case.

| L | p | β | O1O2 | SD basis selection | FD basis selection | Bitmap | Leading coefficient | WB amp | Coefficients | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 1/4 | 1/4 | 4 | 7 | 10 | 16 | 2 | 4 | 21 | 64 |
| 2 | 1/4 | 1/2 | 4 | 7 | 10 | 16 | 3 | 4 | 49 | 93 |
| 2 | 1/4 | 3/4 | 4 | 7 | 10 | 16 | 4 | 4 | 77 | 122 |
| 2 | 1/2 | 1/4 | 4 | 7 | 11 | 28 | 3 | 4 | 42 | 99 |
| 2 | 1/2 | 1/2 | 4 | 7 | 11 | 28 | 4 | 4 | 91 | 149 |
| 2 | 1/2 | 3/4 | 4 | 7 | 11 | 28 | 5 | 4 | 140 | 199 |
| 4 | 1/4 | 1/4 | 4 | 11 | 10 | 32 | 3 | 4 | 49 | 113 |
| 4 | 1/4 | 1/2 | 4 | 11 | 10 | 32 | 4 | 4 | 105 | 170 |
| 4 | 1/4 | 3/4 | 4 | 11 | 10 | 32 | 5 | 4 | 161 | 227 |
| 4 | 1/2 | 1/4 | 4 | 11 | 11 | 56 | 4 | 4 | 91 | 181 |
| 4 | 1/2 | 1/2 | 4 | 11 | 11 | 56 | 5 | 4 | 189 | 280 |
| 4 | 1/2 | 3/4 | 4 | 11 | 11 | 56 | 6 | 4 | 287 | 379 |
| 6 | 1/4 | 1/4 | 4 | 13 | 10 | 48 | 4 | 4 | 77 | 160 |
| 6 | 1/4 | 1/2 | 4 | 13 | 10 | 48 | 5 | 4 | 161 | 245 |
| 6 | 1/4 | 3/4 | 4 | 13 | 10 | 48 | 6 | 4 | 245 | 330 |
| 6 | 1/2 | 1/4 | 4 | 13 | 11 | 84 | 5 | 4 | 140 | 261 |

As a reference, the maximum overhead of Rel-15 Type II is 351 bits for the 32 ports and 13 subbands case. For all the parameter combinations listed in Table I, only the overhead for (L, p, β) = (4, 1/2, 3/4) exceeds the Rel-15 Type II overhead. However, this situation happens for odd number of subbands (e.g. 13) due to the ceiling functions. For 12 subbands case, the overhead for (L, p, β) = (4, 1/2, 3/4) is 328 bits, which is almost same as the Rel-15 overhead which is 327 bits for this case. Besides, for 8PSK phase, the overhead of all the parameter combinations will be below that of Rel-15 Type II.

The performance and overhead for all the supported parameter combinations for 32 ports, 16PSK and 13 subbands case are shown in Figure 1. For the points with small parameters, the Rel-16 codebook has similar performance with Rel-15 Type II with around half overhead. For the points with large parameters, the Rel-16 codebook has similar overhead with Rel-15 Type II with around 10% performance gain.

Figure 1. The performance and overhead of all supported parameter combinations for 32 ports case.

Observation 1: For 32 ports case, the Rel-16 codebook with (L, p, β) = (6, 1/4, 3/4) has the best performance, with around 10% performance gain and less overhead compared with Rel-15 Type II codebook.

Values of N3

When , the following alternatives are listed in the agreements of RAN1 AH1901, where NSB is the number of CQI subbands.

- Alt1: N3 is smallest multiple of 2, 3, or 5 which is ≥NSB×R

- Alt2: N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

For Alt1, since the maximum candidate of NSB is 19 for R=1, the UE implementation may be different for 13 subbands and 14 subband cases. Alt 1 has certain drawback for segmented UE implementation and slight performance uncertainty.

The padding scheme is the UE implementation, however it may have influence on the performance. Supposing m columns are needed to pad in total, three typical padding schemes are listed and compared as follows.

- Padding Scheme 1: Padding m columns of zeros to the end.

- Padding Scheme 2: Padding m columns copied from the last column to the end.

- Padding Scheme 3: Padding ceil(m/2) columns copied from the first column to the beginning, and m-ceil(m/2) columns copied from the last column to the end.

For the determination of N3 when NSB×R>13, the optional value of (NSB×R) are summarized in Table II. To minimize the range of optional N3 values for Alt2, (NSB×R) FD units are divided into two segments as equally as possible, and N3 for each segment is selected as the smallest multiple of 2/3/5 which is equal or larger than NSB×R2.

Table II. Optional values of NSB×R and N3

| NSB×R | R | Alt 1: Optional value of N3 (multiple of 2, 3, 5) | Alt 2: Optional value of N3 for each segment |
| --- | --- | --- | --- |
| 14 | 1, 2 | 15 | 8 |
| 15 | 1 | 15 | 8 |
| 16 | 1, 2 | 16 | 8 |
| 17 | 1 | 18 | 9 |
| 18 | 1, 2 | 18 | 9 |
| 19 | 1 | 20 | 10 |
| 20 | 2 | 20 | 10 |
| 22 | 2 | 24 | 12 |
| 24 | 2 | 24 | 12 |
| 26 | 2 | 27 | 15 |
| 28 | 2 | 30 | 15 |
| 30 | 2 | 30 | 15 |
| 32 | 2 | 32 | 16 |
| 34 | 2 | 36 | 18 |
| 36 | 2 | 36 | 18 |
| 38 | 2 | 40 | 20 |

From Table I, it can be observed that the candidate number of N3 values for Alt2 is smaller than that for Alt1, because the maximum number of FD units for each segment can be limited within 20. Less candidate number of N3 means lower implementation complexity. However, the segmentation introduces additional specification impact. For example, although it is claimed that no padding is needed to align the DFT size with the multiple of 2, 3, or 5, the segmentation rules still need to be defined to avoid the misalignment between reported PMI and actual FD units. Moreover, the UCI design for NSB×R>13 is different from that for NSB×R≤13, because different FD basis subset and non-zero coefficients should be indicated for different segments.

The performance-overhead trade-off curves for different padding and segment schemes are illustrated in Fig. 2. It can be observed that different padding schemes as well as the scheme without padding have almost same performance. However, the segment scheme performs worse.

Figure 2. The performance-overhead curves for different padding and segment schemes.

Another system level simulation result is shown in Fig. 3. In our simulation, BW=50MHz, SCS=30kHz and NSB=17. For Alt1, N3 is selected as 18 and length-17 actual FD units are allocated in FD units with index 0~16. No channel estimation samples is assumed for FD unit 17. As for Alt2, two segments has the same number of FD units. Segment 1 contains subband 0~8, and segment 2 contains subband 8~16. To ensure similar overhead, the number of FD basis for Alt1 and Alt2 are M=6 and M=3, respectively. β=3/4 is assumed for Size-K0 subset. As shown in Fig. 3, Alt1 and Alt2 almost have the same performance. Based on the above analysis and simulation results, no obvious benefit is exhibited through segmentation.

Figure 3. The performance of Alt 1 and Alt 2 for N3 value determination, BW=50 MHz, SCS=30 kHz, NSB=17, R=1.

Observation 2: Although the difference among padding schemes is not significant, the variance of performance gain can be up to 2%. And segmentation shows performance loss.

Proposal 1: For the value of N3 when NSB*R > 13, Alt2 with two segments is not preferred. Also Alt 1 has certain drawback for segmented UE implementation and slight performance uncertainty.

Conclusions

The contribution discusses the codebook design or enhancement for Rel-16, based on which the following observations and proposals are made.

Observation 1: For 32 ports case, the Rel-16 codebook with (L, p, β) = (6, 1/4, 3/4) has the best performance, with around 10% performance gain and less overhead compared with Rel-15 Type II codebook.

Observation 2: Although the difference among padding schemes is not significant, the variance of performance gain can be up to 2%. And segmentation shows performance loss.

Proposal 1: For the value of N3 when NSB*R > 13, Alt2 with two segments is not preferred. Also Alt 1 has certain drawback for segmented UE implementation and slight performance uncertainty.

- References

“RAN1 Chairman’s Notes”, Spokane, USA, November 12-16, 2018.

“RAN1 Chairman’s Notes”, Taipei, January 21-25, 2019.

“RAN1 Chairman’s Notes”, Xi’an, China, April 8-12, 2019.

R1-1906028, Huawei, HiSilicon, “Discussion on CSI enhancement”, Reno, USA, May 13th-17th, 2019.

- Appendix

| Parameters | Dense Urban (Macro layer only) |
| --- | --- |
| Duplex mode | FDD |
| Carrier frequency | 4GHz |
| Subcarrier spacing | 15kHz for 10MHz |
| System bandwidth | 10MHz (13 subbands, 4 PRBs for each subband) |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites |
| Channel model | SCM-3D-UMa |
| Inter-BS distance | 200m |
| Minimum distance | 35m |
| BS antenna height | 25m |
| BS Tx power | 41dBm |
| BS antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH, dV) = (0.5, 0.8) λ |
| UE antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1); the polarization angles are 0 and 90 |
| UE distribution | 80% indoor, 3km/h; 20% outdoor, 30km/h |
| UE receiver | MMSE-IRC |
| MIMO scheme | SU/MU-MIMO switch for overhead reduction; |
| Scheduler | PF |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |