3GPP TSG RAN WG1 Meeting #97                                   R1-1906648

Reno, USA, 13th – 17th May, 2019

Agenda item:		7.2.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On Enhancements to Initial Access Procedures for NR-U

Document for:		Discussion and Decision

# 1. Introduction

During RAN Plenary #82 meeting the TR related to the Study on NR-Based Access to Unlicensed Spectrum [1] has been approved. In the same meeting a new WID related to NR-Based Access to Unlicensed Spectrum has been approved (latest update [2]), starting the Specifications phase for Release 16.

In this contribution, we discuss details of initial access and mobility related physical layer procedures for NR-U. We consider the following procedures:

- DRS Transmission

- Random Access

- RRM and RLM

- Paging

# 2. DRS Transmission

The NR-U WID [2] lists the following within the physical layer procedure(s) to be specified:

Initial access: specify required NR modifications to increase the maximum number of candidate SS/PBCH block positions within the DRS transmission window; to handle reduced SS/PBCH block and RMSI transmission opportunities due to LBT failure; to determine frame timing and QCL assumptions from the detected SS/PBCH block; single SS/PBCH block numerology assumed per band for Pcells in unlicensed spectrum. (RAN1)

## 2.1 On Coverage of DRS in Initial Access

Beamforming for broadcast signaling is one of the most important new features in NR design. However, in unlicensed spectrum the total RF output power is normally restricted by EIRP limitation. For example, the RF output power EIRP limitation is 23 dBm and 24 dBm in 5 GHz unlicensed band, as regulated in ETSI 301.839 and FCC 15.247, respectively. This limitation captures both transmit power and antenna gain (beamforming gain + antenna element gain). This means directional transmission with high beamforming gain cannot lead to any additional coverage in unlicensed spectrum, if the device is able to reach maximum output power with omni-direction. Hence, broadcast signaling (such as SSS/PSS/PBCH/RMSI) benefits from a single beam operation in low frequency unlicensed spectrum (e.g., <7 GHz), leading to much higher efficiency compared to beamforming with beam sweeping operation. Beam sweeping for broadcast signals means repeating the same information (MIB, SIB1, OSI, Paging) to multiple directions requiring correspondingly multiple time domain resources thus increasing the control overhead. In addition, since the beams would need to be transmitted into all directions to provide coverage anyway, no reduction of interference can be achieved.

However, note that restricting the NR-U operation at sub 7 GHz to single SSB does not impose restrictions on using beamforming for PDSCH; on the contrary, gNB can make use of all its antennas with beamforming based on PMI feedback, beamformed CSI-RS, or SRS.

Due to PSD regulations, synch signals (PSS and SSS) in an SS/PBCH block would be transmitted with 12.8 or 15.8 dBm transmit power assuming 15 or 30 kHz SCS, respectively, and PSD limit of 10dBm/Mhz. That corresponds to 10.2 or 7.2 dB lower transmission power than full carrier / channel (20 MHz) allocation would allow for other downlink signals and channels.

Observation 1: NR-PSS and NR-SSS are transmitted with 10.2 or 7.2 dB lower transmission power, when using 15 or 30 kHz SCS, than downlink signal that is spread across the 20 MHz sub-band.

To look at further the potential coverage issue of SS/PBCH block we analysed (im)balance between downlink and uplink maximum coupling losses (MCLs) by performing a link budget analysis in the Appendix for NR-PSS, NR-SSS, NR-PBCH and PRACH. Based on the link budget analysis the Table 1 summarizes the observed MCL values. It’s to be noted that PRACH format A1 is targeted to small cells and it is constructed from a CP and two symbols. Increasing the SCS provides some gain due to wider bandwidth thanks to increased frequency diversity but similar gain is also visible for both DL and UL signals.

Table 1 MCLs [dB] for the joint NR-PSS/NR-SSS detection, NR-PBCH detection and PRACH formats A1 and A2. In parenthesis the difference in MCL compared to PRACH format A1 is shown.

|  | 15 kHz SCS |  | 30 kHz SCS |  |
| --- | --- | --- | --- | --- |
|  | MCL [dB] | Supported distance [m] (UMiNLoS) | MCL [dB] | Supported distance [m] (UMiNLoS) |
| NR-PSS/NR-SSS | 115 dB (-9.0) | 105 | Result N/A | Result N/A |
| NR-PBCH | 116 dB (-8.0) | 111 | 117 dB (-8.6) | 119 |
| NR PRACH format A1 | 124 dB(0.0) | 184 | 125.6 dB(0.0) | 204 |
| NR PRACH format A2 | 126.8 dB (+2.8) | 219 | 127.6 dB (+2.0) | 231 |

Based on above summary it can be noticed that NR-PSS/NR-SSS and NR-PBCH in NR-U are having roughly 8-9 dB and 11-12 dB worse maximum coupling loss than PRACH format A1 and A2, respectively. Even more important to note, typically for inter-cell detection and measurement the coverage for SSBs should be far beyond the cell radius. Thus, the supported cell radius e.g. in UMiNLoS environment would be way below 100 m.

Observation 2: Single SSB detection performance in NR-U assuming Rel15 structure is relatively poor, supporting cell radius way below 100 m.

As discussed above, beamforming for the SSB (i.e. multi-beam SSB) cannot improve coverage compared to single-beam SSB. As a result, even with multi-beam configuration, the coverage of DRS is quite limited.

Repetition of SSBs within a DRS transmission from a single beam would be a straightforward solution allowing to use existing SSB locations. However, problematic is that the number of SSBs QCLed together should be known prior to initial search. That would basically require fixing the number of SSBs belonging to DRS transmission from a single beam in all standalone (SA) cases to be the same which would be challenging to define. In non-standalone (NSA) deployment, UE can be informed via licensed carrier the number of QCLed SSBs assumed in certain unlicensed carrier for the cell search. In other words, in non-standalone case the number of QCLed SSBs applied in unlicensed carrier can be made configurable to support coverage extension for the DRS.

Thus, related to

- “The feasibility and benefits of beam repetition for soft combining reception of SSBs within the same DRS transmission may be further considered.”

we make the following observation and proposal.

Observation 3: It would be beneficial to improve detection performance of the DRS both in single-beam and multi-beam configuration (i.e. DRS transmitted using one beam).

Proposal 1: Support configurable number of QCLed consecutive SSBs in NR-U for configured DRS transmissions.

## 2.2 On Transmission Opportunities for DRS

In RAN1#94bis the following agreement was made that is the starting point for the subsequent discussion related to transmission opportunities for DRS:

Agreement: For SSB transmissions as part of DRS: - It is considered beneficial to expand the maximum number of candidate SSB positions within DRS transmission window to [Y], for e.g., Y = [64] - FFS: How to derive frame timing from detected SS/PBCH block - Transmitted SSBs do not overlap - FFS: Shift granularity between candidate SSBs positions/candidate groups of SSBs - Maximum number of transmitted SSBs is [X] within DRS transmission window. X <= 8 - FFS: Duration of DRS transmission window - FFS: Duration of the transmitted DRS within the window, including SSBs and other multiplexed signals/channels - FFS: relationship between transmitted SSB index and QCL assumption at UE - FFS: If and how to support beam repetition for soft combining of SSBs within the same DRS transmission

In RAN1#96bis the following agreement was made related to the duration of DRS transmission window:

Agreement: The maximum DRS transmission window duration is 5 ms. - The maximum number of candidate SSB positions within a DRS transmission window, Y, is selected as Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS. - Note: The number of starting points for DRS transmissions with the 5 ms window that can use a Cat. 2 LBT is to be discussed further as part of channel access discussions. - FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

Related to the DRS transmission window configuration:

- FFS: If the DRS transmission window is configurable, and if yes, how to configure and indicate the window, including the range of configurable values.

We think that the duration of the DRS transmission window should be fixed and repeated once at least every 20ms.

Proposal 2: The duration of the DRS transmission window is fixed, periodicity is ≥ 20ms.

The potential impacts of the ETSI harmonized standards (COT duration ≤ 1ms when CAT2 LBT is used, duty cycle currently ≤ 5%) are discussed in our companion contribution [4].

Given now the higher number of candidates SSB positions within a half-frame than the maximum number of beams supported the following items need different solution than specified in NR Rel-15:

- FFS: How to derive frame timing from detected SS/PBCH block

- FFS: relationship between transmitted SSB index and QCL assumption at UE

We consider that as a design principle and target for the above two open items is to retain the NR Rel-15 functionality regarding to RRM measurements, i.e. UE should be able to perform RRM measurements of the neighbour cells without reading the PBCH. That is to prevent increasing UE complexity and power consumption.

Proposal 3: Support UE performing RRM measurements of the neighbour cells without need to read the PBCH of the measured cells.

Solutions were discussed in RAN1 #96bis and the following proposals have been agreed:

Agreement: For a given cell, the UE may assume that the PBCH DMRS sequence index is the same for SS/PBCH blocks that are transmitted at the same candidate positions across DRS transmission windows. Agreement: UE determines serving cell timing from the detected SSB candidate position, where the SSB candidate positions within the DRS transmission window are indexed from 0,…,Y-1 (Y = 10 for 15 kHz SCS and Y = 20 for 30 kHz SCS).

The first agreed proposal says that the same SSB candidate position (i.e. SSB index) implies both the same beam and the same DMRS sequence index, meaning that SS/PBCH blocks in the same SSB candidate position are QCLed across DRS transmission windows; the second agreed proposal says that there are Y SSB candidate positions within a DRS transmission window, a given detected SSB position being used by the UE to determine serving cell timing. On the other hand, these two agreed proposals trigger further questions to be addressed. Firstly, if the number of DMRS sequences used is increased to cover 10/20 candidate positions, this has a potential impact on performance as compared to baseline NR Rel-15 as well as a significant specification effort. Based on the above discussion, we have the following proposal:

Proposal 4: For a given cell, the number of DMRS sequences used is kept as for NR Rel-15, i.e. up to 8 different DMRS sequences.
Note: The first candidate position of the DRS transmission window is located at the first half slot of a half frame.

If this proposal is agreed, then with respect to the second FFS:

- FFS: QCL assumption for SSBs in different candidate positions within a DRS transmission window and across DRS transmission windows

If number of transmitted beams Q is different than number of transmitted DMRS sequences Stx, then it may be difficult to determine, without payload in PBCH, for which DMRS sequences a UE may assume QCL to be the same. In fact, it was previously proposed online that if number of transmitted DMRS sequences Stx is reduced to be equal to the number of transmitted beams Q, then the proposal would work out of box. However, as a consequence of Q=Stx the PBCH payload might be unnecessary increased. This is illustrated on an example in Table 2, a typical scenario with Q = 1 is considered, and Stx=1 DMRS sequence s=0  is transmitted. To recover timing t, index c in PBCH of 5bits [ceil(log2(20))] will be needed.

Table 2 Increased PBCH overhead in case the same DMRS sequence index implies the same beam

|  | 5 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| c | 00000 | 00001 | 00010 | 00011 | 00100 | 00101 | 00110 | 00111 | 01000 | 01001 | 01010 | 01011 | 01100 | 01101 | 01110 | 01111 | 10000 | 10001 | 10010 | 10011 |
| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| s | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

We think that 5bits for c is unnecessary. If number of transmitted sequences Stx is an integer multiple of Q, then:

- the beam index can be uniquely identified by mod(s,Q) without reading the PBCH

- with current set of DMRS sequences Stot=8  maximum value ceilY=20/Stx=5-1=3 is needed for c, requiring 2 bits.

The number of transmitted sequences Stx given Q could be based on Table 3.

Table 3 Number of transmitted sequences Stx assuming Stot=8

| Q | Stx | Maximum c for Y=20 |
| --- | --- | --- |
| 1,2,4,8 | 8 | 2 |
| 3,6 | 6 | 3 |
| 5 | 5 | 3 |
| 7 | 7 | 2 |

With this proposal the Table 2 would be updated as follows, showing the benefit related to PBCH overhead:

Table 4 Reduced PBCH overhead

|  | 5 ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| c | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 01 | 01 | 01 | 01 | 01 | 01 | 01 | 01 | 10 | 10 | 10 | 10 |
| t | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| s | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 |

Based on above discussion, we have the following proposals:

Proposal 5: To recover the beam index of the SSB, the UE performs an operation mod(s,Q) where Q denotes the number of transmitted beams and s denotes the DMRS sequence index.

Proposal 6: The UE determines the timing t=Stx*c+s , where c is the cycle index indicated in the PBCH, Stx is the number of cycled/transmitted DMRS sequences from the total number of sequences Stot and given by the number of transmitted beams Q as the maximum integer multiple of the number of transmitted beams per cell Q*i, subject to Q*i≤S_tot, and s denotes the DMRS sequence index.

In case SSB repetition is supported (see section 2.1), R being the number of repeated SSBs (R > 1) a similar scheme applies:

- the beam index can be uniquely identified by mod(floor(s/R),Q) without reading the PBCH

- the number of cycled/transmitted DMRS sequences (Stx) is an integer multiple of (R * Q), with Stx ≤ Stot, as above

We have then the following proposals, which are equivalent to the proposals 5 and 6 if R = 1:

Proposal 7: To recover the beam index of the SSB, the UE performs an operation mod(floor(s/R),Q) where Q denotes the number of transmitted beams, s denotes the DMRS sequence index and R is the number of repeated SSBs.

Proposal 8: The UE determines the timing t=Stx*c+s , where c is the cycle index indicated in the PBCH, Stx is the number of cycled/transmitted DMRS sequences from the total number of sequences Stot and given by the number of transmitted beams Q as the maximum integer multiple of the number of transmitted beams per cell multiplied by the repetition factor R*Q*i, subject to R*Q*i≤S_tot, and s denotes the DMRS sequence index.

As per the above proposals the cycle index c is provided to the UE via the PBCH and requires 2 bits. To use the PBCH MIB to transmit the cycle index would have negative impacts upon PBCH soft-combining for QCLed SSBs inside a DRS transmission window, as depending from the DRS burst starting position the cycle index may have different values across QCLed SSBs.
It is therefore proposed to use the 2 reserved bits of the PBCH transport block ([8], section 7.1) for this purpose.

Proposal 9: The cycle index c is carried using the 2 reserved bits of the PBCH transport block.

Based upon comments made during the RAN1 #96bis meeting we think it may be useful to further describe how both initial cell search and neighbour cells RRM measurements should be performed according to our proposal.

Initial cell search

It has been commented that as Stx depends from Q this increases the UE complexity related to PBCH-DMRS detection because of the increase of PBCH-DMRS combinations corresponding to two PSS/SSS peaks separated by 8 slots. Although we acknowledge the impact of our proposal upon the number of PBCH-DMRS combinations its actual impact upon the UE complexity remains to be clarified in our view; we would have the same feedback for similar comments related to PBCH soft-combining.

Observation 4: Actual impact of our proposal upon UE complexity remains to be clarified in our view.

On the other hand, in our proposal the knowledge of Q is necessary to allow the UE to determine the timing. As it has been commented that the knowledge of the timing is beneficial to acquire the RMSI we propose to have Q carried through the PBCH MIB; with this change the PBCH MIB payload will remain unchanged over a 160ms TTI, as for NR Rel-15.

In the case SSBs repetition is supported the same applies for R, the number of repeated SSBs.

Observation 5: the PBCH MIB payload will remain unchanged over a 160ms TTI if Q and R (if supported) are added.

Proposal 10: For initial cell search Q and R (if supported) are provided to the UE via the PBCH MIB.

Neighbour cells RRM measurements

As a consequence of the proposal 3 the PBCH MIB cannot be used to provide the UE with Q for neighbour cells RRM measurements; in the case SSBs repetition is supported the same applies for R, the number of repeated SSBs. Based upon NR Rel-15 Specifications ([9]) we propose to provide the UEs with Q and R (if supported) according to the following:

Proposal 11: For neighbour cells RRM measurements Q and R (if supported) are provided to the UE via the following signalling:
- For UEs in idle/inactive mode, SIB3/IntraFreqNeighCellInfo IE (for intra-frequency measurements) and SIB4/ InterFreqNeighCellInfo IE (for inter-frequency measurements).
- For UEs in connected mode, dedicated RRC signalling (MeasObjectNR IE).

Another topic of interest is the support of beam-level RRM measurements reporting. Such feature is already supported by NR Rel-15 and should be useful e.g. for the two following use cases, both related to the handover procedure:

- It could help the network to prepare proper random access resources by allocating the UE to a specific SSB beam during the random access procedure.

- It could help the network to take the proper decision for which target cell the handover should actually take place. For instance for two given target cells with similar cell-level measurements results but with different beam-level results the network may prefer to select a target cell for which the beam-level results distribution is the most homogeneous one, or for which the number of strongest beams is the highest.

Additionally, it could be also used by the network for network optimizations; e.g. it might be useful for the network to be aware that UEs that are currently served by beam #X actually measure beam #Y as the strongest one.

In our view all these use cases are relevant for NR-U as well, meaning that beam-level neighbour cells RRM measurements should be supported for NR-U.

Observation 6: beam-level neighbour cells RRM measurements should be supported for NR-U.

As already commented by some Companies for synchronized networks it may be possible for the gNB to derive the neighbour cells timing – hence the beam index – from the knowledge of the serving cell timing; however, such a way forward is not applicable for unsynchronized networks.

As per our proposal 7 beam index = mod(floor(s/R),Q) our proposal is compliant with the above observation, i.e. the UE may retrieve the beam index corresponding to QCLed SSBs without reading the PBCH payload for both synchronized and unsynchronized networks.

Observation 7: our proposal allows the support of beam-level neighbour cells RRM measurements for both synchronized and unsynchronized networks.

During the RAN1 #96 and following offline email discussions a list of design targets for the DRS transmission has been circulated and commented. Although such list has not been discussed online – and therefore not agreed – we fully support such an initiative, as we believe it would be very useful to understand and compare the various proposals.

Based upon the latest version of this list we are aware of, the way our proposal fulfils these design targets is provided below:

Table 5 Compliance of our proposal with RAN1 #96 design targets

| Design target | Our proposal | Comments |
| --- | --- | --- |
| No [(or minimum)] change of PBCH DMRS sequence design from Rel-15 NR | No change | Precise meaning of “minimum change” unclear for us |
| Full flexibility on number of actual transmitted SSBs within a DRS burst i.e., from 1 to Stx | Yes | Number of actual transmitted SSBs depending upon LBT success |
| Full flexibility on starting position of DRS burst within a DRS transmission window i.e., from candidate position #0 to #Y-1 | Yes | The DRS burst starts where the LBT is successful |
| Support transmitting one SSB per slot or two SSBs per slot using the two SSB positions | Yes | Related to the support of one SSB per slot the proposal 8 has to be updated as follows (the proposal 7 being left unchanged): t=2*(Stx*c+s) |
| Support asynchronous neighbor cell measurement | Yes | SSB beam index known via the PBCH DMRS sequence without reading the PBCH payload |
| Same (or less number of) PBCH DMRS sequence used for QCLed SSBs across DRS bursts | Same number in case Stx sequences are transmitted, less number otherwise |  |
| PBCH bits for timing related information as common as possible within a DRS burst and/or across DRS bursts to facilitate PBCH soft-combining within the burst and/or across bursts | - Within a DRS burst: yes for QCLed SSBs when repetitions are used (R > 1). Our proposal 9 also facilitates PBCH soft-combining within a burst. - Across DRS bursts: not mandatory in our view, but our proposal 9 also facilitates PBCH soft-combining across bursts. | PBCH payload = 2 bits |
| Ability to support the transmission repetition of same beam in SSB candidate positions within a DRS transmission window | Yes | See proposals 7 and 8 |

Observation 8: Our proposal is compliant with all above design targets.

## 2.3 Wideband operation and delivery of RMSI/OSI/Paging

Wideband operation in Rel-15 is illustrated on an example in Figure 1 where there are multiple SSBs (both cell defining CD-SSBs and non-cell defining NCD-SSBs) in wideband network carrier. In the figure there are two cell-defining SSBs which identify two serving cells that may have overlapping BWPs in frequency domain. From the UE perspective, each serving cell is associated to at most a single SSB. Further, there are two NCD-SBs also on the same carrier. These NCD-SSBs indicate (for initial access UE) where the UE may find the cell defining SSB. RRM measurements based on both CD and NCD-SSBs can be configured, i.e. on all SSB1, SSB2, SSB3 and SSB4.

Figure 1 Rel-15 wideband operation with multiple SSBs (both cell defining and non-cell defining SSBs).

In NR-U, multiple SSBs with RMSI CORESET in wideband carrier could be used to increase robustness (in terms of sub-band LBT failure) for RMSI, OSI and paging delivery by configuring UE with a CORESET#0 and search space set#0 on multiple sub-bands. Thus, a UE can monitor RMSI, OSI and paging and perform RACH procedure for the same serving cell on any of those sub-bands. In other words, gNB would operate a single-cell with multiple CD-SSBs.

The signaling could be provided in RMSI so that UE may perform RACH procedure on any sub-bands belonging to the same cell (UE would carry out the procedure on the sub-band it transmits PRACH preamble). This is shown in Figure 2 where UE# detects SSB4 and associated RMSI provides configuration for RMSI CORESET on other sub-bands on the wideband carrier.

From idle UE point of view, there should not be much difference between operating wideband scenarios using single cell or multiple cell, because UE can freely choose where it performs initial access. However, for RRC-Connected UE, single-cell scenario would avoid need for HO, and such make the system more efficient and agile.

Observation 9: Robustness for RMSI/OSI/paging delivery can be increased by providing UE with configuration of CORESETs and associated search space sets in multiple sub-bands for SI and paging monitoring as well as for RACH procedure. Whether this is achieved with single cell or multiple cells is FFS.

Figure 2 Providing CORESET for RMSI CORESET on multiple sub-bands for RMSI/OSI/Paging delivery.

# 3. Random Access

The NR-U WID [2] lists the following within the physical layer procedure(s) to be specified:

Random access: specify required NR modifications to enhance RACH procedure in line with the agreements during the study phase, including 4-step RACH modifications to handle reduced Msg 1/2/3/4 transmission opportunities due to LBT failure (RAN1/RAN2); LBT for 2-step RACH and application of PRACH and PUSCH format improvements for NR-U to 2-step RACH. (RAN1)

In this contribution we discuss two possible ways to alleviate the impact of LBT failures upon the Random Access procedure:
- Increase of Msg1/MsgA transmission opportunities
- Reduction of contention failures

3.1 Increase of Msg1/MsgA transmission opportunities

Following the approval during RAN Plenary #82 of a new WID related to 2-step RACH in NR (latest update [3]) and except for the topics mentioned on top of section 3, the 2-step RACH design is out of the scope of the NR-U WID. Hence, the proposals/observations in this section assume 4-step RACH as a baseline but are in principle also applicable to 2-step RACH as far as MsgA transmission is concerned.

To increase Msg1/MsgA transmission opportunities, several options were identified in [1], as listed within subclause 7.2.1.3.2. In the subsequent we discuss further the considered options.

1) Option 1.a. - Multiple PRACH resources across multiple LBT sub-bands/carriers for both contention-free and contention-based RA:

Multiple PRACH resources in frequency domain spread across multiple 20 MHz sub-bands can provide robustness against LBT failures. Therefore, an efficient way to increase the transmission opportunities of Msg1/MsgA is to allow the configuration of PRACH resources on multiple 20 MHz sub-bands. The UE can select the PRACH resources to be used for transmission of Msg1/MsgA also based on the outcome of separate Listen-Before-Talk (LBT) procedures performed on different 20 MHz sub-bands. Such diversity in the carrier/frequency domain can improve the robustness of the random-access procedure in unlicensed spectrum.

It is assumed here that a UE is able to transmit on a single BWP, hence “sub-band” stands here for “20 MHz part of a BWP”.

Configured PRACH resources

Used PRACH resources

LBT failure

Successful LBT

20 MHz

or

Initial /
Active
BWP (up to 4 sub-bands)

Figure 3: Frequency/carrier diversity for transmission Msg1/MsgA in NR-U RA procedure

Proposal 12: Support multiple PRACH resources in frequency domain spread across multiple 20 MHz sub-bands.

This proposal is applicable when the BWP corresponds to at least two sub-bands, otherwise a single LBT procedure is performed. On the other hand this proposal is also applicable for UEs which do not support a bandwidth > 20 MHz; in such a case the UE would not be able to perform simultaneous LBT procedures but it may select any of the available sub-bands and perform LBT on the selected sub-band.

Observation 10: Such proposal is applicable when the BWP corresponds to at least two sub-bands and also for UEs which do not support a bandwidth > 20 MHz. In such a case the UE would not able to perform simultaneous LBT on multiple sub-bands but it may select any of the available sub-bands and perform LBT on the selected sub-band.

The gNB listens for the preamble transmission on all configured resources. Once it detects a RACH preamble on a given sub-band, it sends Msg2 on the same sub-band; it is assumed as a baseline that the UE performs the complete random access procedure in one sub-band at a time, i.e. the sub-band actually used for Msg1/MsgA transmission.

Proposal 13: After the UE has selected one sub-band for transmission of Msg1/MsgA, the RA procedure is completed on the same sub-band.

2) Option 2.a. - For connected mode UE, scheduling of PRACH resources via DCI:

      - Triggered PRACH within TXOP can use a new resource

Option 2.a. is an efficient mechanism to provide robustness against LBT failures. That can be supported e.g. by allowing PRACH resources being multiplexed in UL portion of a shared COT acquired by the gNB as discussed in [5]. A group-common PDCCH can be used to schedule PRACH resources within the COT, similar to the PDCCH ordered PRACH feature for NR Rel-15. It may be seen beneficial to enable this option for idle mode UEs as well but the negative impacts upon the UE power consumption related to GC-PDCCH monitoring may be seen as an issue.

Proposal 14: Support PRACH resources being multiplexed in a flexible manner in UL portion of a shared COT acquired by the gNB for connected mode UEs.

3) Option 2.b. - For idle mode UE, scheduling of PRACH resources via paging

i. 	Note: potential inefficiency in network resource due to paging across multiple cells

It’s unclear what would be benefit from option 2.b. compared to regular RMSI based RACH resource allocation in idle mode. Furthermore, as paging is transmitted across multiple cells the overhead would be potentially significant. In particular the following should be clarified:
- Is it the intention to schedule PRACH resources via every paging message ? 
Or is there any additional indication needed ?
- For a paging message via which PRACH resources need to be scheduled, is it the intention to schedule PRACH resources immediately after the PDSCH carrying the paging message ?

Observation 11: Benefits from 2.b. in idle mode are not clear and should be anyway balanced with the drawbacks indicated in the above note.

  - Option 2.c. - Additional, new RACH resources are used immediately following detection of DRS transmission

DRS is expected to have CORESET for PDCCH and thus PDCCH (e.g. GC-PDCCH) can be used to schedule PRACH resources after DRS which is actually covered by option 2.a.

Observation 12: Option 2.c. is covered in option 2.a.

Proposal 15: Support option 2.c. as a particular case of option 2.a.

  - Option 2.d. - Multiple PRACH transmissions before Msg2 reception in RAR window for initial access

- Number of allowed transmissions is pre-defined or indicated, e.g., in RMSI

- FFS: How to handle potential multiple RARs to same UE

Allowing UE to transmit multiple Msg1 before end of the RAR window was discussed already quite extensively during Rel-15 NR SI/WI phases. Motivation was mainly a latency reduction for a UE without beam correspondence but was not agreed because of added interference (uncontrolled interference) and load, and that UE beam correspondence capability is transparent in random access procedure. In NR-U we don’t see motivation to introduce possibility to transmit multiple Msg1 before end of RAR window. What is needed is to provide UE with multiple Msg1 transmission opportunities, which provides robustness against LBT, i.e. option 2.d. is not NR-U specific issue. Furthermore, for RAR robustness against LBT, extended RAR window is considered to be enough.

Proposal 16: Do not support multiple Msg1 transmissions from a UE before end of RAR window.

  - Option 2.e. - Group wise SSB-to-RO mapping by frequency first-time second manner, where grouping is in time domain

Rel-15 NR baseline supports multiple ROs per SSB in both frequency and time domains. Furthermore, an association period, starting from frame 0, for mapping SSBs to PRACH occasions is the smallest value in the set determined by the PRACH configuration period according Table 8.1-1 in [6] such that SSBs are mapped at least once to the ROs within the association period.

Related agreement from RAN1#AH1801:

- For the cyclic mapping of association between ROs and all the actually transmitted SS/PBCH blocks, if there are leftover ROs after an integer number of cycles within the defined period, no SS/PBCH blocks are mapped to these leftover ROs

Within an association period there are multiple full cycles for mapping SSBs to ROs. Effectively, with the existing standard the same effect that option 2.e. targets can be achieved by mapping multiple SSBs to 1 RO. The standard allows up to 16 SSBs to be mapped to one RO. For instance, let’s assume that there are 32 actually transmitted SSBs, with a PRACH configuration that has 8 ROs within one PRACH configuration period (as an example). In this case,

ROs 0, 2, 4 and 6 are associated with SSB 0 to 15

ROs 1, 3, 5 and 7 are associated with SSB 16 to 31

Proposal 17: Remove option 2.e. as it can be supported already by Rel-15 NR.

3.2 Reduction of contention failures

Following the approval during RAN Plenary #82 of a new WID related to 2-step RACH in NR [3] and except for the topics mentioned on top of section 3, the 2-step RACH design is out of the scope of the NR-U WID. Hence, the proposals/observations in this section are applicable to 4-step RACH only.

As illustrated in Figure 4, (a) when two or more UEs select the same PRACH preamble in the same PRACH opportunity; (b) they get assigned the same PUSCH resources for their Msg3 transmission; and therefore (c) their Msg3 collide.

Figure 4: Msg 3 collision due to UE1 and UE2 selecting the same PRACH preamble in the same PRACH opportunity.

Observation 13: When two or more UEs select the same PRACH preamble in the same PRACH opportunity, they will be assigned the same PUSCH resources for their Msg3 transmission, resulting in a collision.

In the current NR 4-step procedure, the UEs select always the same DMRS port when transmitting their Msg3 over the PUSCH. If each UE selects one DMRS port out of the 8 different DMRS available in a PUSCH resource (as depicted in Figure 5), then the gNB when using advanced receivers (i.e. receivers capable of multi-packet detection and decoding) is more likely to recover from the colliding transmissions. This approach does not require a physical change in the current Msg2 MAC payload, i.e. since the gNB does not know at this stage how many UEs selected the same PRACH preamble, then it does not need to indicate in the RAR payload which DMRS port each UE should use. Instead, since no coordination is possible between the UEs or by the gNB, the DMRS port selection should be performed by the UE in a pseudo-random way.

Figure 5: Proposal where multiple DMRS ports are available to be selected from at each PUSCH resource element.

Observation 14: PUSCH collisions can be overcome if, when transmitting Msg3, the UE may pseudo-randomly select one of the available DMRS ports in their assigned PUSCH resource.

When receiving their Random Access Response, all the UEs that selected the same PRACH preamble in the same PRACH opportunity get indicated the same timing advance, regardless of each UE actual distance to the gNB antenna. The indicated timing advance will be computed based on the time shift of the highest correlation peak of the Zadoff-Chu sequence associated with the PRACH preamble. In large cells, such timing advance lack of accuracy will impair the decoding of colliding Msg3 in the same PUSCH resource or will at least require receivers that are able to cope with this. As NR-U targets small cells such timing alignment issue is not seen as crucial, since it is not expected to prevent the gNB from decoding colliding Msg3.

Observation 15: Time alignment lack of accuracy is not seen as crucial in small cells, since it is not expected to prevent the gNB from decoding colliding Msg3.

Therefore, following the above observation, the introduction of multiple DMRS ports in the PUSCH resource for Msg3 transmission is beneficial, as it actually reduces the number of contention failures.

Proposal 18: When operating in small cells, UEs should be allowed to pseudo-randomly select one of the available DMRS ports in the PUSCH resource assigned for their Msg3 transmission.

# 4. RRM and RLM

The NR-U WID [2] lists the following within the physical layer procedure(s) to be specified:

RLM/RRM extensions for NR-U operation due to uncertain and reduced transmission opportunities for DL signals and channels due to LBT failure in line with agreements during the study phase (NR-U TR section 7.2.1.3.2), including configuring different DRS Measurement Time Configuration (DMTCs) for RRM and RLM respectively, identifying the set of RLM-RSs to measure, which set(s) are used for in-sync, out-of-sync evaluations, potential definition of a metric to accurately identify unsuccessful detection of RLM-RS. Support RSSI reporting. Define a metric to measure channel occupancy or medium contention and its corresponding reporting. (RAN1/RAN2)

## Although RRM/RLM extension for NR-U operations may raise similar issues – e.g. which signals are used for measurements - they do not mandatorily lead to the same solutions; on the other hand, some issues may be specific to either RRM or RLM, e.g. the configuration of the SMTC for RRM. In the following we will therefore address RRM and RLM topics separately.

4.1 RRM

In RAN1 #96 the following agreement was made related to RRM:

Agreement: - At least the functionalities of Rel-13 LTE-LAA RSSI and channel occupancy reporting as a baseline should be supported - FFS: - Enhanced RSSI metrics, for e.g., sub-band-level interference measurements in a wideband operation scenario - Reporting of a new medium contention/load metric other than channel occupancy - Any modification of the parameters of the Rel-15 SMTC for operation in unlicensed spectrum

4.1.1 SSB/CSI-RS transmission and SMTC configuration

As for a UE in idle mode CSI-RS measurements are not applicable, the main use of CSI-RS RRM measurements will be related to Connected mode mobility. As such performing CSI-RS transmission outside of the DRS transmission window may lead to additional complexity and may not be compatible with the strict timing requirements for neighboring cells monitoring.

Another issue is related to CSI-RS transmission power, as the network shall adjust CSI-RS transmission power because of e.g. different PSD requirements depending on the transmission bandwidth. To ensure reliable RRM measurements – and e.g. a reliable evaluation of the pathloss - it has been agreed for LTE-LAA that the network should guarantee a constant CRS power transmission inside the DMTC window, such constraint being not valid outside of the DMTC window; hence for LTE-LAA no RRM measurements are performed outside of the DMTC window.

It is therefore proposed to support only SSB/CSI-RS-based RRM measurements inside the DRS transmission window.

Proposal 19: Only SSB/CSI-RS-based RRM measurements inside the DRS transmission window are supported.

The way the NR Rel-15 SMTC configuration needs to be updated for NR-U has to be further discussed, but as per the above proposal it seems clear that the SMTC has to be configured based upon the DRS transmission window characteristics for both serving and neighbor cells.

The following aspects should to be investigated:

- The DRS transmission windows distribution between serving and neighbor cells for both synchronized and unsynchronized networks and the consequence upon SMTC configuration are to be discussed. In particular and for unsynchronized networks the longer DRS transmission window periodicity compared to the NR Rel-15 baseline – mandatory to cope with the impacts of LBT – will require specific SMTC configurations, given that the gNB is not aware of the neighbor cells timing. Several tracks to be further investigated could solve this issue, such as the use of the maximum SMTC duration or the use of a sliding SMTC window, but we think that such improvements are not suitable to ensure reliable mobility, as the intervals between measurements are too large.

- Different SMTC configurations for intra and inter-frequency measurements.

- SMTC duration: should be based upon the DRS Tx window duration with the possible use of SSB-ToMeasure to restrict the set of subframes to be actually measured by the UE.

- SMTC periodicity: should be based upon DRS Tx window periodicity.

Proposal 20: The SMTC configuration for NR-U should assume NR Rel-15 as a baseline. The way it needs to be updated for NR-U is to be further discussed, including the aspects above mentioned.

### 4.1.2 RSSI and other metrics

As agreed during RAN1 #96 meeting at least the functionalities of Rel-13 LTE-LAA RSSI and channel occupancy reporting as a baseline should be supported.

The RSSI - measured by the UE over a given bandwidth – corresponds to the signal strength received from the nodes belonging to the same operator plus the interferences coming from other nodes, including nodes from other operators and/or other RATs, which contribute to increase the interference level. For LTE-LAA a single RSSI measurement has been defined. As the signal strength received from the nodes belonging to the same operator can be directly measured by the UE from all the neighbour cells it monitors, the level of interferences can be derived from both these measurements and the RSSI.

On the other hand, NR-U should be able to perform LBT on multiple 20 MHz sub-bands, for which the interference level coming from other nodes may be different. In order to improve sub-band selection (e.g. in the case several sub-band LBTs are successful at the same time) it is found beneficial to provide the gNB with RSSI measurements provided on a per sub-band basis, as for LTE-LAA.

Proposal 21: To support RSSI measurements on a per sub-band basis.

In our view reporting of a new medium contention/load metric other than channel occupancy is not expected to be needed for NR-U, unless a strong evidence for its related benefits is shown by proponents.

Proposal 22: Reporting of a new medium contention/load metric other than channel occupancy is not expected to be needed for NR-U.

## 4.1.3 Impacts of LBT upon RRM measurements

4.1.3.1 Handling of missing L1 samples

Once initial access is performed, the UE is expected to be implicitly aware of the DRS transmission characteristics in the serving and neighboring cell, including DRS timing. On the other hand, providing the UE with an explicit indication of missing L1 samples is not needed in our view, as such indication will be provided “a posteriori” and does not bring any benefit compared to the L3 filter input rate adaptation we propose below.

Observation 16: It is not needed to provide the UE with explicit indication of DRS transmission.

As the UE is aware of SSB/CSI-RS configuration within the DRS transmission window we believe it is reasonable to assume the UE can detect the presence of the SSB/CSI-RS. Thus, missing L1 samples due to e.g. LBT can be identified and removed. Of course, this means there may be fewer samples in L1 measurement period. However, in agreement with what discussed and concluded in RAN2, we think L3 measurement filter should be able to accommodate this - as it was also concluded for RRM measurements in LTE-LAA.

Proposal 23: Missing L1 samples due to LBT are handled in UE implementation specific manner by adapting the input rate to the L3 filter. No changes to the RAN1/RAN2 Specifications are needed to address impacts of LBT on RRM measurements based on SSB and/or CSI-RS.

4.1.3.2 Cell-quality indicator

In Rel-15 NR the cell-quality indicator is calculated as the linear average of up to N best beams (at a given moment) that are above a certain absolute threshold - before L3 filtering. If one of the best N beams fails because of e.g. LBT, the corresponding L1 measurement may be somehow corrupted. However, this only becomes a relevant issue in case multiple SSB-beams are supported in NR-U. If only single-SSB beam is supported by the gNB, then impact of LBT on cell-quality indicator in NR-U can be handled as for RRM measurements based on SSB and/or configured CSI-RS.

Observation 17: If only single-SSB beam is supported by the gNB then impact of LBT on cell-quality indicator in NR-U can be handled as for RRM measurements based on SSB and/or configured CSI-RS.

On the other hand, to support multi-SSB beams there are two possible ways to handle the cell-quality indicator (see figure 6):

- Option A: use the Rel-15 NR approach, i.e. the cell-quality indicator is periodically computed as the average of the N best beams above a threshold, the final cell-quality indicator being built from these periodic computations.

- Option B: perform for each beam measurement over a given period (including L3 filtering as by Proposal 23), the final cell-quality indicator being built by averaging the N best L3 filtered beam measurements.

Figure 6: Two possible ways to handle the cell-quality indicator

Both approaches guarantee that L3 filtering compensates for the missing SSB/CSI-RS. The main advantage of Option B could be that as individual beam measurements are computed over a given period, the impact of missing SSB/CSI-RS for a given beam is less sensitive than for the Rel-15 NR approach (Option A), leading to a more robust cell-quality indicator. On the other hand, assuming a pseudo-random missing SSB/CSI-RS distribution both approaches are expected to be equivalent.

Given the above and as from a specifications complexity perspective the benefits are obvious, we propose to use the Rel-15 NR approach as a baseline for NR-U (i.e. Option A in Figure 6).

Proposal 24: To use the Rel-15 NR approach as a baseline for cell-quality indicator evaluation:
- When a single-SSB beam is supported by the gNB, as for RRM measurements based on SSB and/or configured CSI-RS.
- When multiple-SSB beams are supported by the gNB, as the average of N best beams above threshold before L3 filtering (Rel-15 NR approach).

4.2 RLM

In RAN1 #96 the following agreement was made related to RLM:

Agreement: - An RLM measurement window for serving cell RLM measurements based on SSBs in the DRS is supported for in-sync and out-of-sync evaluations. - FFS: How RLM measurement window is indicated or determined and relation to DRS transmission window - FFS: Whether or not an SSB can fall outside the measurement window and, if so, whether it can be used for in-sync and out-of-sync evaluations. - FFS: Any relationship of RLM measurements based on CSI-RS to the measurement window. - FFS: Mechanism to handle missing RLM-RS due to LBT failure

## 4.2.1 RLM measurements inside the DRS transmission window

About the current RLM framework in NR, UEs are provided RLM reference signal configurations by implicit determination or explicit configuration from the gNB, implicit configuration meaning same monitoring occasions for both RLM-RS and RMSI; hence related to the implicit RLM-RS configuration we propose to use as RLM-RS both SSB and CSI-RS within the DRS transmission window, the RLM measurement window being equal to the DRS transmission window. The UE shall monitor the downlink quality based on the SSB and/or CSI-RS and assess the downlink radio link quality per every evaluation period.

Proposal 25: SSB/CSI-RS-based RLM measurements inside the DRS transmission window are supported, the RLM measurement window being equal to the DRS transmission window.

However, in NR unlicensed spectrum, the periodical transmission of RLM-RS may be interrupted due to LBT failure as shown in Fig.7.

Figure 7: Periodical RLM-RS transmission in NR-U

Once the gNB misses the RLM-RS transmission due to the failed LBT, it must wait until the next transmission opportunity. In this case, the UE cannot distinguish whether such lack of transmission is due to poor link quality or because the RLM-RS transmission is blocked by LBT; therefore, missing RLM-RS have to be taken into account for both IS and OOS evaluations. If the UE always reports OOS indication to higher layers even if the channel condition is good, the inaccurate measurement result may lead to unnecessary RLF. On the other hand, if the UE always doesn’t report OOS indications when it cannot detect any of its configured RLM-RS resources, this will result in unacceptable delays in declaring RLF. Therefore, the RLM/RLF mechanism may be impacted by the possible lack of reference signals due to LBT.

Hence, we propose that the event of the UE not detecting any RLM-RS transmission within the DRS transmission window should be counted in both the in-sync and out-of-sync evaluations. How to count the missing RLM-RS samples within the DRS transmission window in the in-sync/out-of-sync evaluations is for RAN4 to be discussed.

Proposal 26: RLM-RS samples within the DRS transmission window are taken into account for both in-sync and out-of-sync evaluations. How to count within the DRS transmission window both the detected and missing RLM-RS samples in the in-sync/out-of-sync evaluations is for RAN4 to be discussed.

## 4.2.2 RLM measurements outside of the DRS transmission window

As by proposal 26 LBT blocking within the DRS transmission window may result in unnecessary declaration of RLF and also slow down the RLM recovery. Though as mentioned above for the RRM topic RLM-RS transmission outside the DRS transmission window may be affected by a fluctuant transmission power, they can still be used for RLM measurements purpose (in-sync evaluations only). To allow for faster RLM recovery in case RLM-RS transmission within the DRS transmission is blocked, we propose that detected L1 samples outside the DRS transmission window in correspondence of configured RLM-RS resources can also be used for RLM measurements for in-sync evaluations only.

Another reason to use RLM-RS signals outside of the DRS transmission window for RLM measurements is linked with the RLM-RS density. It seems that the number of detected RLM-RS signals, which relies on the RLM-RS periodicity and the success probability of LBT, may impact the in-sync evaluation performance. The RLM-RS signals used for in-sync evaluations might not have sufficient density when the periodical RLM-RS signal could not always be transmitted periodically at the configured time resource due to the unsuccessful channel access.

Observation 18: The RLM-RS signals used for in-sync evaluation might not have sufficient density when the periodical RLM-RS signals are blocked by failed LBT.

Proposal 27: RLM measurements outside of the DRS transmission window are supported for in-sync evaluations only. How to count outside of the DRS transmission window the detected RLM-RS samples in the in-sync evaluations is for RAN4 to be discussed.

Hence, further enhancements for RLM-RS signal outside of the DRS transmission window need to be investigated, especially for long periodicity of RLM-RS resources. For example, when the gNB could not send the periodical RLM-RS signals for pre-defined time duration (e.g., K=2 consecutive periods), an additional opportunity window could be triggered as shown in Fig.8, where some additional occasions of RLM-RS signal transmission should be introduced.

Proposal 28: The RLM measurement should consider the additional opportunistic RLM-RS transmission in one triggered window in the case no RLM-RS sample within the DRS transmission window has been detected after a pre-defined time duration.

Figure 8: Opportunistic RLM-RS transmission in one triggered window in NR-U

Related to opportunistic RLM-RS, our proposal is to rely upon DMRS/CSI-RS carried by GC-PDCCH/PDSCH channels, bearing in mind that:

- GC-PDCCH channel is always present within a gNB acquired COT, as it is used to provide the UEs with the COT structure. The presence of the GC-PDCCH (i.e. the gNB acquired COT) can be confirmed once the GC-PDCCH DMRS has been detected by the UE.

- PDSCH channel is scheduled by the (GC-)PDCCH and may be carried within a separate gNB acquired COT, depending e.g. upon the k0 value.

Hence the presence of CSI-RS transmission can be determined based on COT structure indication in GC-PDCCH plus the PDSCH scheduling information within the (GC-)PDCCH; otherwise no additional indication to the UE is expected to be needed.

Observation 19: No explicit indication to the UE of opportunistic transmission other than COT structure indication in GC-PDCCH plus the PDSCH scheduling information within the (GC-)PDCCH is expected to be needed.

As a consequence, we propose to use both GC-PDCCH DMRS and GC-PDCCH/PDSCH CSI-RS for in-sync evaluations.

Proposal 29: The UE should use both GC-PDCCH DMRS and GC-PDCCH/PDSCH CSI-RS for in-sync evaluations.

# 5. Paging

The NR-U WID [2] lists the following within the physical layer procedure(s) to be specified:

Paging: specify required NR modifications to enhance paging opportunities by increasing time-domain paging occasions or paging monitoring occasions while taking UE power consumption into account.

In this contribution we discuss RAN1-related aspects for the paging procedure.

Due to the LBT procedure, it is not always possible to send paging message to a UE during the PO when it is supposed to be sent based on NR Rel-15 PO calculation procedures. In line with the above, we discuss proposals to introduce additional POs in time-domain for each DRX cycle.

From UE power consumption point of view as well as from LBT success point of view it would be good to try to align sending of reference signals and paging messages. In this way one LBT procedure may provide opportunities to send multiple downlink signals and UE only can perform multiple activities within one wake up cycle. In NR licensed band operation this behaviour was achieved by UE having same PDCCH monitoring occasions for paging and RMSI. But during the discussion for licensed band operation the capacity of this approach was raised as a concern, hence a possibility to configure different search space for paging to allow larger paging capacity and also freedom to schedule paging was introduced. In our view we should follow similar principles in NR-U and it should be possible to “align” sending of DRS and paging messages for the reasons mentioned above.

Proposal 30: It should be possible to support POs multiplexed with DRS.

On the other hand, as for licensed band operation paging capacity would be limited if the DRS and paging sending would need to happen in the same slot, possibly to a larger extend. It should be therefore possible to also schedule paging message in different slots if there is no possibility for network to schedule enough paging capacity at same time with DRS.

Observation 20: Paging capacity may become a problem if paging messages needs to be always aligned in time with DRS transmission.

Proposal 31: It should be possible to support additional POs in addition to those multiplexed with DRS.

# 6. Conclusions

In this contribution, we have discussed potential changes and enhancements needed for physical layer procedures for NR unlicensed scenarios. Based on the discussion, we make the following proposals and observations:

DRS Transmission

Observation 1: NR-PSS and NR-SSS are transmitted with 10.2 or 7.2 dB lower transmission power, when using 15 or 30 kHz SCS, than downlink signal that is spread across the 20 MHz sub-band.

Observation 2: Single SSB detection performance in NR-U assuming Rel15 structure is relatively poor, supporting cell radius way below 100 m.

Observation 3: It would be beneficial to improve detection performance of the DRS both in single-beam and multi-beam configuration (i.e. DRS transmitted using one beam).

Proposal 1: Support configurable number of QCLed consecutive SSBs in NR-U for configured DRS transmissions.

Proposal 2: The duration of the DRS transmission window is fixed, periodicity is ≥ 20ms.

Proposal 3: Support UE performing RRM measurements of the neighbour cells without need to read the PBCH of the measured cells.

Proposal 4: For a given cell, the number of DMRS sequences used is kept as for NR Rel-15, i.e. up to 8 different DMRS sequences.
Note: The first candidate position of the DRS transmission window is located at the first half slot of a half frame.

Proposal 5: To recover the beam index of the SSB, the UE performs an operation mod(s,Q) where Q denotes the number of transmitted beams and s denotes the DMRS sequence index.

Proposal 6: The UE determines the timing t=Stx*c+s , where c is the cycle index indicated in the PBCH, Stx is the number of cycled/transmitted DMRS sequences from the total number of sequences Stot and given by the number of transmitted beams Q as the maximum integer multiple of the number of transmitted beams per cell Q*i, subject to Q*i≤S_tot, and s denotes the DMRS sequence index.

Proposal 7: To recover the beam index of the SSB, the UE performs an operation mod(floor(s/R),Q) where Q denotes the number of transmitted beams, s denotes the DMRS sequence index and R is the number of repeated SSBs.

Proposal 8: The UE determines the timing t=Stx*c+s , where c is the cycle index indicated in the PBCH, Stx is the number of cycled/transmitted DMRS sequences from the total number of sequences Stot and given by the number of transmitted beams Q as the maximum integer multiple of the number of transmitted beams per cell multiplied by the repetition factor R*Q*i, subject to R*Q*i≤S_tot, and s denotes the DMRS sequence index.

Proposal 9: The cycle index c is carried using the 2 reserved bits of the PBCH transport block.

Observation 4: Actual impact of our proposal upon UE complexity remains to be clarified in our view.

Observation 5: the PBCH MIB payload will remain unchanged over a 160ms TTI if Q and R (if supported) are added.

Proposal 10: For initial cell search Q and R (if supported) are provided to the UE via the PBCH MIB.

Proposal 11: For neighbour cells RRM measurements Q and R (if supported) are provided to the UE via the following signalling:
- For UEs in idle/inactive mode, SIB3/IntraFreqNeighCellInfo IE (for intra-frequency measurements) and SIB4/ InterFreqNeighCellInfo IE (for inter-frequency measurements).
- For UEs in connected mode, dedicated RRC signalling (MeasObjectNR IE).

Observation 6: beam-level neighbour cells RRM measurements should be supported for NR-U.

Observation 7: our proposal allows the support of beam-level neighbour cells RRM measurements for both synchronized and unsynchronized networks.

Observation 8: Our proposal is compliant with all above design targets.

Observation 9: Robustness for RMSI/OSI/paging delivery can be increased by providing UE with configuration of CORESETs and associated search space sets in multiple sub-bands for SI and paging monitoring as well as for RACH procedure. Whether this is achieved with single cell or multiple cells is FFS.

Random Access

Proposal 12: Support multiple PRACH resources in frequency domain spread across multiple 20 MHz sub-bands.

Observation 5: Such proposal is applicable when the BWP corresponds to at least two sub-bands and also for UEs which do not support a bandwidth > 20 MHz. In such a case the UE would not able to perform simultaneous LBT on multiple sub-bands but it may select any of the available sub-bands and perform LBT on the selected sub-band.

Proposal 13: After the UE has selected one sub-band for transmission of Msg1/MsgA, the RA procedure is completed on the same sub-band.

Proposal 14: Support PRACH resources being multiplexed in a flexible manner in UL portion of a shared COT acquired by the gNB for connected mode UEs.

Observation 6: Benefits from 2.b. in idle mode are not clear and should be anyway balanced with the drawbacks indicated in the above note.

Observation 7: Option 2.c. is covered in option 2.a.

Proposal 15: Support option 2.c. as a particular case of option 2.a.

Proposal 16: Do not support multiple Msg1 transmissions from a UE before end of RAR window.

Proposal 17: Remove option 2.e. as it can be supported already by Rel-15 NR.

Observation 8: When two or more UEs select the same PRACH preamble in the same PRACH opportunity, they will be assigned the same PUSCH resources for their Msg3 transmission, resulting in a collision.

Observation 9: PUSCH collisions can be overcome if, when transmitting Msg3, the UE may pseudo-randomly select one of the available DMRS ports in their assigned PUSCH resource.

Observation 10: Time alignment lack of accuracy is not seen as crucial in small cells, since it is not expected to prevent the gNB from decoding colliding Msg3.

Proposal 18: When operating in small cells, UEs should be allowed to pseudo-randomly select one of the available DMRS ports in the PUSCH resource assigned for their Msg3 transmission.

RRM

Proposal 19: Only SSB/CSI-RS-based RRM measurements inside the DRS transmission window are supported.

Proposal 20: The SMTC configuration for NR-U should assume NR Rel-15 as a baseline. The way it needs to be updated for NR-U is to be further discussed, including the aspects above mentioned.

Proposal 21: To support RSSI measurements on a per sub-band basis.

Proposal 22: Reporting of a new medium contention/load metric other than channel occupancy is not expected to be needed for NR-U.

Observation 11: It is not needed to provide the UE with explicit indication of DRS transmission.

Proposal 23: Missing L1 samples due to LBT are handled in UE implementation specific manner by adapting the input rate to the L3 filter. No changes to the RAN1/RAN2 Specifications are needed to address impacts of LBT on RRM measurements based on SSB and/or CSI-RS.

Observation 12: If only single-SSB beam is supported by the gNB then impact of LBT on cell-quality indicator in NR-U can be handled as for RRM measurements based on SSB and/or configured CSI-RS.

Proposal 24: To use the Rel-15 NR approach as a baseline for cell-quality indicator evaluation:
- When a single-SSB beam is supported by the gNB, as for RRM measurements based on SSB and/or configured CSI-RS.
- When multiple-SSB beams are supported by the gNB, as the average of N best beams above threshold before L3 filtering (Rel-15 NR approach).

RLM

Proposal 25: SSB/CSI-RS-based RLM measurements inside the DRS transmission window are supported, the RLM measurement window being equal to the DRS transmission window.

Proposal 26: RLM-RS samples within the DRS transmission window are taken into account for both in-sync and out-of-sync evaluations. How to count within the DRS transmission window both the detected and missing RLM-RS samples in the in-sync/out-of-sync evaluations is for RAN4 to be discussed.

Observation 13: The RLM-RS signals used for in-sync evaluation might not have sufficient density when the periodical RLM-RS signals are blocked by failed LBT.

Proposal 27: RLM measurements outside of the DRS transmission window are supported for in-sync evaluations only. How to count outside of the DRS transmission window the detected RLM-RS samples in the in-sync evaluations is for RAN4 to be discussed.

Proposal 28: The RLM measurement should consider the additional opportunistic RLM-RS transmission in one triggered window in the case no RLM-RS sample within the DRS transmission window has been detected after a pre-defined time duration.

Observation 14: No explicit indication to the UE of opportunistic transmission other than COT structure indication in GC-PDCCH plus the PDSCH scheduling information within the (GC-)PDCCH is expected to be needed.

Proposal 29: The UE should use both GC-PDCCH DMRS and GC-PDCCH/PDSCH CSI-RS for in-sync evaluations.

Paging

Proposal 30: It should be possible to support POs multiplexed with DRS.

Observation 15: Paging capacity may become a problem if paging messages needs to be always aligned in time with DRS transmission.

Proposal 31: It should be possible to support additional POs in addition to those multiplexed with DRS.

# References

- 3GPP TR 38.889 v16.0.0, “Study on NR-Based Access to Unlicensed Spectrum (Release 16)”

- RP-190706, “Revised WID on NR-Based Access to Unlicensed Spectrum (Release 16)”, Qualcomm

- RP-190711, “Revised WID: 2-step RACH for NR”, ZTE, Sanechips

- R1-1906655, “Initial Access Signals and Channels for NR-U”, Nokia, Nokia Shanghai Bell

- R1-1906643, “NR-U uplink signals and channels”, Nokia, Nokia Shanghai Bell

- 3GPP TS 38.213, “Physical layer procedures for control”

- R1-1905786, ”Feature lead summary #1 of Enhancements to initial access procedure”, Charter Communications

- 3GPP TS 38.212, “Multiplexing and channel coding”

- 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”

# Appendix – Link budget analysis

In this section we provide link budget analysis for NR-PSS, NR-SSS and NR-PBCH (SS/PBCH block), and NR PRACH Formats A1 (targeted to small cells) and A2. Simulations assumptions for:

- joint NR-PSS and NR-SSS detection performance are provided in Table A.1.5-1: Simulation assumptions for synchronization signals/channels in [3GPP TR 38.802] and in Table 6

- NR-PBCH are defined in Table A.1.5-1: Simulation assumptions for synchronization signals/channels and Table A.1.5-3: Simulation assumptions for PBCH in [3GPP TR 38.802]

- NR-PRACH are as defined in Table A.1.5-2: Simulation assumptions for RACH in [3GPP TR 38.802].

Table 6 Simulation assumptions for joint NR-PSS and NR-SSS detection

| Parameter | Value |
| --- | --- |
| PSS/SSS length | 127 |
| PSS/SSS period | 5 ms |
| Sub-carrier spacing | 15 kHz |
| Channel model | CDL-C 100 ns scaling |
| UE speed | 30 km/h |
| PSS receiver algorithm | Single-shot time-domain correlator with thresholding |
| SSS receiver algorithm | Single-shot non-coherent exhaustive search with thresholding |
| False alarm probability | < 1 % |

As shown in Table 6, NR-PSS and NR-SSS joint detection performance is analysed for the 15 kHz SCS option only. However, it’s to be noted that 30 kHz SCS option would improve the performance slightly (~1 dB) due to better frequency diversity. The following Figure 9 provides the joint NR-PSS and NR-SSS detection performance showing that the targeted performance is achieved at -3 dB SNR.

Figure 9 Joint NR-PSS and NR-SSS detection performance [R1-1708231].

The following Figure 10 provides NR-PBCH detection performance for 15, 30 and 60 kHz SCS options (only 15 and 30 kHz used in the link budget). It can be observed that targeted detection performance is achieved at -4 dB, -5 dB and -6 dB SNR for 15, 30 and 60 kHz SCS, respectively.

Figure 10 NR-PBCH detection performance.

PRACH link level simulations results for the link budget analysis are taken from [R1-1710889].

Then in the link budget analysis, used system level parameters are inherited from [3GPP TR 38.802], section 4 (Outdoor below 7 GHz). Calculated MCL values for 15 and 30 kHz SCS options for joint PSS&SSS (only 15 kHz), PBCH, PRACH format A1 and PRACH format A2 detection performance are provided in Table 7. Based on the link budget calculations following summary can be made where MCL values are represented together with the difference to the PRACH format A1 (targeted to small cells).

Table 7 MCLs [dB] for the joint NR-PSS/NR-SSS detection, NR-PBCH detection and PRACH formats A1 and A2. In parenthesis the difference in MCL compared to PRACH format A1 is shown.

|  | 15 kHz SCS | 30 kHz SCS |
| --- | --- | --- |
| NR-PSS/NR-SSS | 115 dB (-9.0) | N/A |
| NR-PBCH | 116 dB (-8.0) | 117 dB (-8.6) |
| NR PRACH format A1 | 124 dB(0.0) | 125.6 dB(0.0) |
| NR PRACH format A2 | 126.8 dB (+2.8) | 127.6 dB (+2.0) |

Based on above summary it can be concluded that NR-PSS/NR-SSS and NR-PBCH in NR-U are having roughly 8-9 dB and 11-12 dB worse maximum coupling loss than PRACH format A1 and A2, respectively. Imbalance is mainly coming from the higher noise figure at the receiver in downlink and remarkably higher antenna gain at the receiver in uplink.

Table 8 Link budget analysis for NR-PBCH, NR-PSS, NR-SSS and NR PRACH