3GPP TSG RAN WG1 #97                                  R1-1906674

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.2.1.3

Source: 	LG Electronics

Title: 	Physical layer design of UL signals and channels for NR-U

Document for:	Discussion and decision

- Introduction

In RAN1#96bis meeting [1], following agreements were made on the structure of UL channels and signals in NR-U.

Agreement: For a 20 MHz carrier bandwidth, if enhancements to PF0 and PF1 are supported, a mapping to physical resources of at least one full interlace is supported - FFS: Whether or not to support enhancements to PF0/1. - Companies are encouraged to provide user multiplexing capacity and UCI payload analysis for enabling the decision for relevant use cases Agreement: Support RRC configuration of an SRS resource to start at any OFDM symbol within a slot by extending the RRC parameter startPosition of resourceMapping of SRS-Config for Rel-16 to have a value range 0..13. Agreement: Decisions on which additional PUCCH formats (enhanced or combination of legacy and enhanced) are supported should be at least based on the following. - Which PUCCH format(s) are to be used at least for the following use cases: - HARQ ACK prior to dedicated PUCCH resource configuration - HARQ ACK, SR, CSI and combinations thereof after dedicated PUCCH resource configuration - Specification impact, e.g., UE procedures in 38.213 and 38.212, for all proposed PUCCH formats to be supported - User multiplexing capacity and UCI payload analysis for all proposed PUCCH formats to be supported - In-band and out-of-band emissions

In this contribution, we discuss on the design of UL channels and signals (e.g. PUSCH, PUCCH, SRS) for NR-U operation.

- UL channel structure

This section describes UL channel structure of PUSCH and PUCCH (and SRS) for NR-U considering regulatory requirements in unlicensed spectrum.

  - PUSCH structure

On the structure of PUSCH resource, PRB-level interlace structure in terms of: 1) the interlace interval or the number of interlaces (M) for a given BW, and 2) the number of RBs (N) per each of interlaces, had been agreed for 15 kHz and 30 kHz SCS. Besides, it was also agreed in RAN1#96 not to support sub-PRB interlace design for PUSCH (and PUCCH). Thus, for the case of 60 kHz SCS, PRB-level interlace can be considered as the PUSCH resource structure as well, and it would be beneficial in terms of UL resource efficiency/ flexibility rather than only allowing wideband contiguous allocation (satisfying OCB requirement) or not supporting PUSCH for 60 kHz SCS at all.

Proposal #1: Support PRB-level interlace resource structure for NR-U PUSCH with 60 kHz SCS for commonality with other SCSs.

- Where the interlace interval M = 3 PRBs and the number of PRBs per interlace N = 8 based on UL BW of 20MHz.

  - PUCCH structure

As the conclusion in the SI phase, it may be desirable to design UL waveform of the PUCCH similar to UL waveform of PUSCH as much as possible because multiplexing between PUSCH and PUCCH can be required. Therefore, it may be also preferable to design block-interlace based waveform even for the PUCCH like the PUSCH waveform. Considering both of implementation complexity and standardization efforts, it seems reasonable to reuse the existing PUCCH format(s) in NR as much as possible to design PUCCH format(s) for NR-U operation.

First of all, in case of PUCCH format 2/3 which support multi-RB transmission, it can be relatively easily expanded to block-interlace structure by dividing frequency resource of a PUCCH resource into multiple clusters in the frequency domain. In addition, considering large amount of RBs within a single interlace (compared to RB granularity in NR), it could be beneficial (in terms of resource efficiency) to support multi-UE multiplexing for the interlace-based PUCCH format 2/3. For example, in case of PUCCH format 2 based on FDMed UCI and DMRS (without DFT), frequency domain OCC can be applied per each of RBs belonging to a same interlace, or time domain OCC can be applied for the case of multi-symbol interlace-based PUCCH format 2 (i.e., PF2). Furthermore, RE group level FDM and/or CDM within a RB can also be considered to support PUCCH flexibility in terms of the supportable (maximum) payload size and UE multiplexing capacity. For another example, in case of PUCCH format 3 (i.e., PF3) based on TDMed UCI and DMRS (with DFT), time domain OCC can be applied over multiple symbols.

Firstly, in case of the enhanced PF2 for NR-U with interlaced structure, it is required to consider supporting various UCI size and short latency (and more TX opportunities) as main design target. More specifically, in order to support flexible UE multiplexing capacity according to UCI payload size, at least two aspects need to be considered as:

- Variable DMRS density (e.g. UCI RE-to-DMRS RE ratio) per PRB

- Variable spreading factor (e.g. OCC length) for CDM applied to UCI/DMRS within PRB

For an example, in case with UCI-to-DMRS RE ratio of 2:1 (as for NR PF2) where 8 UCI REs and 4 DMRS REs are in a PRB, following CDM structures can be used (per each of PRBs within an interlace) according to UCI payload size (as shown in Figure 1 (a)).

- Structure 1

  - Use of length-4 OCC, 4 REs as one CDM group, then number of CDM groups for UCI = 2

  - Based on the above, 4 UEs with 4 coded bits (assuming QPSK) can be supported per PRB

- Structure 2

  - Use of length-2 OCC, 2 REs as one CDM group, then number of CDM groups for UCI = 4

  - Based on the above, 2 UEs with 8 coded bits (assuming QPSK) can be supported per PRB

For another example, in case with UCI-to-DMRS RE ratio of 1:1 where 6 UCI REs and 6 DMRS REs are in a PRB, following CDM structures can be used (per each of PRBs belonging to an interlace) according to UCI payload size (as shown in Figure 1 (b)).

- Structure 1

  - Use of length-6 OCC, 6 REs as one CDM group, then number of CDM groups for UCI = 1

  - Based on the above, 6 UEs with 2 coded bits (assuming QPSK) can be supported per PRB

- Structure 2

  - Use of length-2 OCC, 2 REs as one CDM group, then number of CDM groups for UCI = 3

  - Based on the above, 2 UEs with 6 coded bits (assuming QPSK) can be supported per PRB

(a) UCI-to-DMRS RE ratio = 2:1, length-4 OCC for Structure 1, length-2 OCC for Structure 2

(b) UCI-to-DMRS RE ratio = 1:1, length-6 OCC for Structure 1, length-2 OCC for Structure 2

Figure 1. Example of PUCCH format 2 extension

Proposal #2: Support variable DMRS density and CDM capacity for the enhanced PUCCH format 2 (based on FDMed UCI and DMRS), in order to support flexible UE multiplexing capacity according to UCI payload size.

Secondly, in case of the enhanced PF3 for NR-U with interlaced structure, it is required to consider supporting moderate or large UCI size and UL coverage-limited case (based on DFT) as main design target. In this context, it seems to need further discussion on whether to support CDM even for this enhanced PF3 (for example, which way would be better between increase of OFDM symbols for the enhanced PF2 and applying CDM for the enhanced PF3 composed of interlaced PRBs), with consideration of the decision (in SI phase) to avoid design efforts for the case of using DFT-s-OFDM.

Proposal #3: Support the enhanced PUCCH format 3 with DFT processing (based on TDMed UCI and DMRS) targeting moderate or large UCI size and UL coverage-limited case.

Moreover, in case of PUCCH format 0/1 which are sequence based PUCCH format(s), the formats may still be required at least in the initial access before RRC connection as well as for SR transmission after RRC connection in the NR-U operation but it may not be easy to extend PUCCH format 0/1 into a block interlace structure since it is designed to fit in a single PRB. One possible way is to keep the low PAPR/CM property as much as possible by applying different cyclic shift values to each repeated resource while repeating PUCCH format 0/1 in the frequency domain. In the extended PUCCH format 0/1 (i.e., PF0/1), same coded UCI bits (e.g., UCI0) can be repeated over multiple clusters or different coded UCI bits (e.g., UCI0, UCI1 …) can be distributed over multiple clusters as shown in Figure 2. Moreover, also for this structure, RE group (consisting of comb-type REs) level FDM in each RB can be considered for PUCCH multiplexing capacity.

Figure 2. Example of PUCCH format 0/1 extension

In summary, in case of the enhanced PF0 for NR-U with interlaced structure, it is required to consider supporting small UCI size, short latency (and more TX opportunities), and large multiplexing capacity as main design target. More specifically, for the enhanced PF0, multiple sequences with length of 12 are concatenated and different cyclic shift values are applied across the multiple sequences to keep low PAPR/CM property. Note that main usages of this enhanced PF0 (and PF1) would at least include: 1) 1-bit HARQ-ACK transmission corresponding to PDSCH reception including Msg4 in initial access procedure before RRC connection, 2) 1-bit SR transmission after RRC connection, and 3) 1/2-bit HARQ-ACK transmission in case of receiving a single PDSCH only within a bundling window.

Furthermore, in case of the enhanced PF1 with interlaced structure, it is required to consider supporting small UCI size, large multiplexing capacity (based on time-domain OCC), and UL coverage-limited case as main design target. Similarly with the enhance PF0, concatenation of multiple sequences applying different cyclic shift values is used in both UCI and DMRS symbols for the enhanced PF1.

Proposal #4: Support the enhanced PUCCH format 0/1 based on concatenation of multiple sequences (mapped on interlaced PRBs), in order to support small UCI payload with large multiplexing capacity with consideration of UL resource efficiency.

Regarding PUCCH format to support small UCI payload (e.g. 1-bit HARQ-ACK or positive SR), whether to use the enhanced PF0 (and/or PF1) or the enhanced PF2 (and/or PF3) is one of the issues to be decided. Based on the agreement in RAN1#96bis, we summarize the comparison between enhanced PF0 and enhanced PF2 (with duration of one OFDM symbol) as below, in terms of UE multiplexing capacity and potential specification impacts.

- Multiplexing capacity

  - In case of enhanced PF0

    - Support of up to 12 UEs (for SR) or 6 UEs (for HARQ-ACK) with 12 cyclic shifts

  - In case of enhanced PF2

    - Support of up to 4 UEs with length-4 OCC and UCI:DMRS ratio of 2:1 as in Rel-15

- Specification impacts

  - In case of enhanced PF0

    - On TS38.212: no impact

    - On TS38.213: small impact on initial PUCCH resource set

  - In case of enhanced PF2

    - On TS38.212: impacts on PUCCH channel coding for small UCI payload size

      - No channel coding is used for small UCI payload size in Rel-15 NR

    - On TS38.213: large impacts on the composition of initial PUCCH resource set, some impacts on dedicated PUCCH resource (set) configuration/determination

      - PF2 (and PF3) is not used for initial PUCCH resource set in Rel-15 NR

Proposal #5: Support the enhanced PUCCH format 0/1 for small UCI payload with consideration of UE multiplexing capacity as well as potential specification impacts.

  - SRS structure

On the support of SRS transmission for NR-U, block-interlace based structure may be beneficial with consideration of some (unlicensed band-specific) regulations in terms of PSD limitation and OCB requirement. In addition, considering multi-UE multiplexing in a same interlace resource, a set of cyclic shift value and/or transmission comb could be used for generation of the SRS sequence. In this case, the following aspects need to be considered in the design of SRS transmission for NR-U.

- Multiplexing with PUCCH or PUSCH

- Multi-symbol SRS with LBT operation

- Frequency hopping of SRS transmission

Regarding the multi-symbol SRS transmission based on LBT operation, it may be required to consider the case when the LBT (then CCA) is failed for the first symbol of the SRS. In this case, some handing or UE behavior may need to be decided, for example, symbol-level (gradual) dropping or symbol shifting/cycling (without dropping). Besides, regarding the frequency hopping of SRS transmission, it may be necessary to discuss on the hopping BW of SRS transmission and LBT operation during SRS hopping. For example, configuring single LBT-SB as the transmission BW of SRS, the SRS hopping can be done across multiple different LBT-SB, and LBT gap can be provided between the hopping.

Proposal #6: Consider the following aspects for the support of SRS transmission in NR-U.

- Multiplexing with PUCCH or PUSCH

- Multi-symbol SRS with LBT operation

- Frequency hopping of SRS transmission

- UL resource allocation

This section describes resource allocation scheme(s) in time/frequency domain for block-interlace based UL transmission (e.g., PUSCH or PUCCH).

  - Frequency domain

In LTE eLAA, frequency resource allocation in interlace units is defined according to the regulations in the unlicensed band that occupy more than 80% of the system bandwidth. Similarly to LTE eLAA, it is also possible to consider allocating frequency resources on an interlace basis in a frequency resource allocation scheme for PUSCH (or PUCCH) for NR-U operation. However, in recent regulations on unlicensed bands, even if 80% of system bandwidth is not occupied, temporary transmission of the signal is allowed if the transmission band of the signal is 2 MHz or more. Therefore, in NR-U operation, it is possible to consider fine granularity support for resource allocation in frequency domain compared to LTE eLAA. Particularly, the fine granularity is advantageous in supporting various TBSs according to the standalone NR-U operation. One way is to allocate consecutive RBs in VRB domain and apply VRB-to-PRB mapping which maps consecutive VRBs to PRBs on the block-interlace resource(s) in PRB domain as shown in Figure 3. It should be noted that the actual data mapping can be done by frequency-first manner to keep low PAPR/CM property of (block-interlaced) DFT-s-OFDM waveform.

Figure 3. Example of resource allocation in frequency domain: VRB-to-PRB mapping

Additionally, it seems necessary to consider resource allocation method in order to cope with the case where UEs having different maximum BW RF capabilities coexist for the same carrier (or bandwidth part). For instance, the entire UL transmission band can be divided into multiple (LBT) sub-bands, and consecutive (and/or non-consecutive) sub-band index(s) may be selected (based on proper (e.g. RIV or bitmap) signaling) in the sub-band index domain, and then consecutive interlace index(s) in the interlace index domain for the selected sub-band(s) can be allocated (based on proper (e.g. RIV) signaling) for UL transmission as shown in Figure 4.

Figure 4. Example of resource allocation in frequency domain: hierarchical allocation

Proposal #7: Consider to support the following aspects for UL frequency resource allocation in NR-U based on the regulations.

- Dynamic switching between interlaced resource allocation type and contiguous resource allocation type by UL grant DCI.

- Allocation of consecutive (and/or non-consecutive) LBT sub-bands and consecutive PRB-interlace indexes within the allocated LBT sub-bands.

Furthermore (similarly with NR), considering different BW capability and/or different BWP size between UEs, it is required to consider carrier BW which may be derived from a reference RB point (i.e., “Point A” defined in NR) on top of UL active BWP, for resource definition/allocation in frequency domain. For example, assuming that carrier BW (e.g. 80 MHz) is larger than the BW of UL active BWP for a UE (e.g. 40 MHz), first of all, the RB interlaces (in terms of grouping of the RB indexes belonging to a same interlace, indexing of each interlace based on the grouping) can be defined based on the carrier BW. Given that, the available RBs in each of the interlaces for the UE can be determined as the RBs belonging to the UL active BWP, and then actual allocated RBs (within the UL active BWP) to the UE can be smaller than the UL active BWP (e.g. single LBT-SB of 20 MHz) based on the above resource allocation approach. Besides, in this structure, it may necessary to consider how to randomize DMRS sequence parameter/scrambling.

Proposal #8: Support hierarchical BW structure as the following to support block-interlace based UL resource allocation for the UEs with different BW capability.

- RB interlaces (in terms of RB grouping per interlace and indexing of each interlace) is defined based on the carrier BW.

- Available RBs in each of the interlaces for the UE is determined as the RBs belonging to the UL active BWP.

- Actual allocated RBs (within the UL active BWP) to the UE can be smaller than the UL active BWP.

Based on the above structure, considering wideband UL BWP consisting of multiple LBT-SBs, interlace resource allocation could be different between PUSCH and PUCCH. In case of PUSCH, a PUSCH can be scheduled to be transmitted on a LBT-SB or over multiple LBT-SBs according to some parameters such as TBS. In case of PUCCH, a PUCCH can be scheduled to be transmitted one of multiple candidate LBT-SBs based on the (simultaneous) LBT operation for the multiple LBT-SBs. For the PUCCH, allocation of the multiple candidate LBT-SBs may need to be considered, for example, indicating an interlace index and valid LBT-SB index for the interlace.

In addition, considering different transmission BW and UE multiplexing between PUSCH and PUCCH, the DMRS parameter including the sequence length could be different between PUSCH and PUCCH or between different PUCCH formats. In case of PUSCH, for example, DMRS sequence length can be determined based on the amount of allocated interlaces and/or the number of allocated LBT-SBs. On the other hand, in case of PUCCH, for example, DMRS sequence length can be determined based on the size of single interlace in a single LBT-SB (e.g. PUCCH format 3) or the size of single PRB (e.g. PUCCH format 0/1), with consideration of multiplexing between the UE with different resource size.

Moreover, in case when structure of unit resource is differently defined between PUSCH/PUCCH and PRACH, the frequency resources allocated for PUSCH/PUCCH and the resources reserved for PRACH might be overlapped in a same (RACH) slot. In this case, to protect potential PRACH transmission from certain UEs for the purpose of initial access, it can be consider to apply puncturing PUSCH/PUCCH signals in RB level or in RB-interlace level.

  - Time domain

In NR-U operation, the baseline time resource allocation scheme for PUSCH or PUCCH transmission can be to indicate the start symbol and duration (in the slot) of UL transmission as in the existing NR. On the other hand, in NR-U operation, since UE must perform LBT procedure before PUSCH (or PUCCH) transmission, the probability of success in transmission may not be relatively high when the transmission is attempted only in the (single) UL resource scheduled by base station. Therefore, additional techniques for increasing the probability of success of UL transmission may need to be considered. In an intuitive way, it can be considered to set multiple candidate transmission resources for a (single) PUSCH (or PUCCH) triggering and in detail, the set or the specific starting position among multiple candidates can be derived by LBT type, additional DM-RS position, PUCCH/PUSCH resource configuration, etc.

Proposal #9: Consider multiple candidate resources (in time) for a single UL transmission as the time resource allocation in NR-U for better transmission success rate.

- UCI multiplexing

This section describes some considerations on UCI multiplexing especially on PUSCH regarding LBT procedure. In NR-U operation, UE may not be able to start PUSCH transmission at the (scheduled) starting symbol due to LBT failure at UE side. In this regard, similar to LTE eLAA, it is possible to consider delaying PUSCH transmission by puncturing the forward part of PUSCH. Considering the above operation, for UCI on PUSCH, it may not be desirable to map UCI REs in front of PUSCH as in existing NR and some modification may be needed.

Proposal #10: Consider UCI multiplexing/mapping on PUSCH with potential puncturing for a part of the PUSCH due to LBT procedure.

- Conclusion

In this contribution, we discussed on the design of PUSCH/PUCCH and SRS for NR-U operation, and the followings are proposed.

Proposal #1: Support PRB-level interlace resource structure for NR-U PUSCH with 60 kHz SCS for commonality with other SCSs.

- Where the interlace interval M = 3 PRBs and the number of PRBs per interlace N = 8 based on UL BW of 20MHz.

Proposal #2: Support variable DMRS density and CDM capacity for the enhanced PUCCH format 2 (based on FDMed UCI and DMRS), in order to support flexible UE multiplexing capacity according to UCI payload size.

Proposal #3: Support the enhanced PUCCH format 3 with DFT processing (based on TDMed UCI and DMRS) targeting moderate or large UCI size and UL coverage-limited case.

Proposal #4: Support the enhanced PUCCH format 0/1 based on concatenation of multiple sequences (mapped on interlaced PRBs), in order to support small UCI payload with large multiplexing capacity with consideration of UL resource efficiency.

Proposal #5: Support the enhanced PUCCH format 0/1 for small UCI payload with consideration of UE multiplexing capacity as well as potential specification impacts.

Proposal #6: Consider the following aspects for the support of SRS transmission in NR-U.

- Multiplexing with PUCCH or PUSCH

- Multi-symbol SRS with LBT operation

- Frequency hopping of SRS transmission

Proposal #7: Consider to support the following aspects for UL frequency resource allocation in NR-U based on the regulations.

- Dynamic switching between interlaced resource allocation type and contiguous resource allocation type by UL grant DCI.

- Allocation of consecutive (and/or non-consecutive) LBT sub-bands and consecutive PRB-interlace indexes within the allocated LBT sub-bands.

Proposal #8: Support hierarchical BW structure as the following to support block-interlace based UL resource allocation for the UEs with different BW capability.

- RB interlaces (in terms of RB grouping per interlace and indexing of each interlace) is defined based on the carrier BW.

- Available RBs in each of the interlaces for the UE is determined as the RBs belonging to the UL active BWP.

- Actual allocated RBs (within the UL active BWP) to the UE can be smaller than the UL active BWP.

Proposal #9: Consider multiple candidate resources (in time) for a single UL transmission as the time resource allocation in NR-U for better transmission success rate.

Proposal #10: Consider UCI multiplexing/mapping on PUSCH with potential puncturing for a part of the PUSCH due to LBT procedure.

- Reference

- RAN1#96bis chairman’s note