3GPP TSG RAN WG1 Meeting #97			R1-1906686

Reno, USA, May 13th – 17th, 2019

Agenda Item:	6.2.1.6

Source: 	LG Electronics

Title: 	Discussion on MPDCCH performance improvement

Document for:	Discussion and decision

- Introduction

In the RAN #80 meeting, the following Rel-16 MTC enhancement for LTE was approved as part of a work item [1].

Improved DL transmission efficiency and/or UE power consumption:

- Specify MPDCCH performance improvement by using CRS at least for connected mode [RAN1, RAN2, RAN4]

Related to this agenda, the following agreements were made in RAN1 #96bis meeting [2].

Agreement

When CSI-based precoding is configured:

- The UE monitors a set of MPDCCH candidates assuming the mapping between DMRS and CRS ports is based on a previously reported PMI.

  - FFS details, including delay for applicability of PMI.

- A fallback mechanism is supported. The details of the fallback mechanism are FFS.

Agreement

For precoding cycling for 2Tx antennas, predefine one precoder set which consists of 2 existing precoders.  FFS: Which of the two precoder among the existing 2TX precoders are used.

Agreement

For precoding cycling for 4Tx antennas, predefine one precoder set which consists of 4 existing precoders.  FFS: Which of the four precoder among the existing 4TX precoders are used.

Agreement

The precoding cycling in time domain has a granularity of YCH (granularity of frequency hopping) only.

- Precoder cycling can be used even in case frequency hopping is not used as long as YCH is configured

- Discussion

As of RAN1 #95 meeting, precoder cycling is supported for both localized and distributed MPDCCH transmission, and port mapping based on CSI report is supported for localized MPDCCH transmission. And in RAN1 #96 and #96bis meetings, details of precoder cycling such as time/frequency domain granularity and the number of precoders in a precoder set were discussed. We further discuss details and remaining issues of the port mapping between CRS and MPDCCH DMRS for both localized and MPDCCH transmission.

  - Port mapping based on precoder cycling

For localized and distributed MPDCCH, the predefined mapping between CRS ports and MPDCCH ports is based on precoder cycling across time and frequency domain. For time domain granularity of the precoder cycling, it was agreed in RAN1 #96bis meeting to support only YCH which is equal to the DL frequency hopping interval for legacy eMTC and also the number of subframes over which the same precoding for MPDCCH DMRS is assumed. For the granularity of the precoder cycling in frequency direction, there was no convergence as there are trade-offs between space diversity and PRB bundling gain for which companies have different flavors. Taking into account the fact that 2 or 3 PRBs provides better performance due to PRB bundling gain (from our experiment during Rel-13 discussion on PRB bundling) and also the fact that the MPDCCH PRB set consists of 2/4/6 PRBs, it is proposed to support the precoder cycling granularity of 2 PRBs in frequency direction. As granularity of 1 PRB provides better space diversity effect than a multiple of PRBs, and also as the PRBs comprising a MPDCCH PRB set may not be contiguous for distributed MPDCCH transmission, we can further consider the precoder cycling granularity of 1 PRB.

Proposal 1: When precoder cycling is applied in frequency direction, the granularity of precoder cycling is N PRB(s)

- N=2 is supported at least for localized MPDCCH transmission

- FFS for distributed MPDCCH transmission

- FFS whether to support N=1

When the precoder cycling is enabled in both time and frequency direction, the order or pattern of the precoder cycling should be designed in such a way that it avoids the case where the same precoder index(es) repeat(s) in time direction in a short period. The said case can easily be avoided by repeating the same precoding index(es) in time direction within each time domain granularity (YCH), and adding an offset (e.g., 1) to the precoding index(es) across time domain granularities.

Proposal 2: The order of precoder cycling when enabled for both frequency and time directions are as follows:

- Precoder index increases (or decreases) in frequency direction within a set of precoders comprising a precoder set

- All the precoder index(es) in frequency domain repeat(s) in time direction within each time-domain granularity (YCH, frequency hopping interval)

- All the precoder index(es) in frequency domain increase(s) (or decrease(s)) by an offset value in time direction across time domain granularities (e.g., offset value = 1)

- Modulo Np applies to the precoder index if it is equal to or larger than Np (Np is the number of precoders in a precoder set)

  - Port mapping based on CSI report

When an MPDCCH is assigned a port mapping based on CSI report, the eNB can configure the precoder recommended by a UE based on the CSI report. In that case, it is possible for the eNB to miss the CSI report from the UE in which case the UE cannot recover without a fallback mechanism.

Proposal 3:

Support precoder cycling as a fallback mechanism of the mapping based on CSI report to cope with the case where eNB misses a CSI report from a UE or (re)configures the port mapping b/w CRS and DMRS

- FFS how to support precoder cycling as a fallback mechanism (e.g., enable configuring 2 MPDCCH PRB sets, one for mapping based on CSI report and another for precoder cycling)

It was briefly discussed in RAN1 #95 meeting on the port relation between CRS and DMRS when normal subframes and MBSFN subframes coexist within a configured number of repetitions. We realized that there can be potential different understandings among companies, but since then there has been no further discussion on this topic. We suggest the port relation b/w CRS and DMRS configured for Rel-16 UEs capable of using CRS for MPDCCH performance improvement also applies to MBSFN subframe(s) available for MPDCCH/PDSCH transmission. By doing so, when normal subframes and MBSFN subframes coexist within a configured number of repetitions, cross-subframe channel estimation between normal and MPBSFN subframes are possible for MPDCCH DMRS.

Proposal 4:

The port relation b/w CRS and DMRS configured for Rel-16 UEs capable of using CRS for MPDCCH performance improvement also applies to MBSFN subframe(s) available for MPDCCH/PDSCH transmission.

- Conclusion

In this contribution, we discussed details and open issues of the port mapping between CRS and MPDCCH DMRS. Our proposals are as follows.

Proposal 1: When precoder cycling is applied in frequency direction, the granularity of precoder cycling is N PRB(s)

- N=2 is supported at least for localized MPDCCH transmission

- FFS for distributed MPDCCH transmission

- FFS whether to support N=1

Proposal 2: The order of precoder cycling when enabled for both frequency and time directions are as follows:

- Precoder index increases (or decreases) in frequency direction within a set of precoders comprising a precoder set

- All the precoder index(es) in frequency domain repeat(s) in time direction within each time-domain granularity (YCH, frequency hopping interval)

- All the precoder index(es) in frequency domain increase(s) (or decrease(s)) by an offset value in time direction across time domain granularities (e.g., offset value = 1)

- Modulo Np applies to the precoder index if it is equal to or larger than Np (Np is the number of precoders in a precoder set)

Proposal 3:

Support precoder cycling as a fallback mechanism of the mapping based on CSI report to cope with the case where eNB misses a CSI report from a UE or (re)configures the port mapping b/w CRS and DMRS

- FFS how to support precoder cycling as a fallback mechanism (e.g., enable configuring 2 MPDCCH PRB sets, one for mapping based on CSI report and another for precoder cycling)

Proposal 4:

The port relation b/w CRS and DMRS configured for Rel-16 UEs capable of using CRS for MPDCCH performance improvement also applies to MBSFN subframe(s) available for MPDCCH/PDSCH transmission.

- Reference

RP-181450, New WID on Rel-16 MTC enhancements for LTE , Ericsson

Chairman’s Notes RAN1 #96bis