3GPP TSG RAN WG1 #97	   			      		   R1-1906732

Reno, USA, May 13th – 17th, 2019

Agenda item: 7.2.8.5.

Source: LG Electronics

Title: Evaluation results on N3 value for Rel-16 Type II CSI

Document for: Discussion and Decision

- Introduction

In RAN1#96bis meeting, following agreement regarding N3 value for Rel-16 Type II CSI is captured in the chairman’s note as:

Agreement

On the value of N3 for (N3N3=NSB×R)>13:

- For Alt1:

  - Identify alternatives for padding schemes in RAN1#97 (Reno)

  - Select one from the alternatives for padding scheme by RAN1#98 (Prague)

- For Alt2:

  - Identify alternatives in RAN1#97 (Reno)

  - Select one from the alternatives by RAN1#98 (Prague)

In this contribution, we provide our evaluation results for N3 value with various schemes.

- Discussions on N3

In order to reduce the overhead for Type II CSI, DFT-based compression has been introduced. In this codebook framework, precoders for a layer is given by size-P×N3 matrix

W=W1W2WfH

where P=2N1N2, N3 is the size of frequency domain, W2 is size-2L×M matrix, L is the number of combining beams and M is the number of columns in Wf.

Due to some UE implementation issues, there is a proposal that DFT size for N3 is smallest multiple of 2, 3, or 5 which is ≥NSB×R. Thus, if N3>NSB×R, there is vector length mismatch. To alleviate this issue, padding scheme can be considered. According to current RRC configuration, max value of Npad=N3-NSB×R can be 2, so following padding cases can be considered as in Figure 1. For the padding scheme, we may consider zero-padding, based on inter/extra-polation, and just reusing the same CSI measurement (copy) in the nearest FD unit.

For the evaluation, it is assumed that 16-port CSI-RS and low and medium traffic load. Also, UE is equipped with 2 Rx antenna ports, so that maximum rank can be 2, respectively. For payload comparison, we assume parameter β,p∈14, 12 in the simulation. Other simulation assumptions are listed in Annex. For the performance comparison, following 3 padding schemes are considered.

- Reference: always assumes N3=NSB*R

- Scheme 1: applies zero-padding in the last FD unit

- Scheme 2: uses the same CSI measurement of the nearest FD unit for the first FD unit

- Scheme 3: uses the same CSI measurement of the nearest FD unit for the last FD unit

Figure 1. Examples of padding location in frequency domain

Figure 2 and 3 represent the simulation results for 10MHz and 20MHz, respectively. In Figure 2, we assume 55RB system with15 kHz subcarrier spacing and R=1, thus NSB =14 and N3 for padding scheme 1, 2 and 3 is 15. In Figure 3 considers 52RB system with 30 kHz subcarrier spacing and R=2, thus NSB*R =26 and N3 for padding scheme 1, 2 and 3 is 27.

Figure 2. Average UPT results for 10MHz

Figure 3. Average UPT results for 20MHz

Based on the evaluation results, we have following observations as:

Observation 1. Compared to reference scheme which always assume N3=NSB, marginal performance loss (up to 3% and 1% for 10MHz and 20MHz, respectively) in terms of average UPT is observed.

Observation 2. Among the padding schemes, Scheme 1 (zero-padding at the last FD unit) provides slightly better performance.

Proposal 1. Support Alt 1 with Scheme 1 (zero-padding at the last FD unit(s)) on the value of N3 (>13).

- Conclusion

In this contribution, we discuss the Type II CSI enhancement in order to efficiently support MU-MIMO. Based on the discussion above, we have following observations and proposal as:

Observation 1. Compared to reference scheme which always assume N3=NSB, marginal performance loss (up to 3% and 1% for 10MHz and 20MHz, respectively) in terms of average UPT is observed.

Observation 2. Among the padding schemes, Scheme 1 (zero-padding at the last FD unit) provides slightly better performance.

Proposal 1. Support Alt 1 with Scheme 1 (zero-padding at the last FD unit(s)) on the value of N3 (>13).

Annex

Table A-1. Simulation assumptions

| Parameters | Value |
| --- | --- |
| Scenarios | Dense Urban (4GHz with 15/30 kHz SCS), ISD=200m |
| BS Tx Power | 41 dBm for 10MHz, 44 dBM for 20 MHz |
| BS antenna configurations (M,N,P,Mg,Ng,Mp,Np) | Dense Urban: 16ports=(8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| MS antenna configurations | 2 Rx X-pol (0/+90), |
| Etilt angle | 102 degree |
| System bandwidth | 10MHz (55RBs), SB size = 4RBs, 20MHz (52RBs), SB size = 4RBs |
| UE attachment | Based on RSRP |
| Duplex | FDD |
| UE speed | 3km/h for indoor, 30km/h for outdoor |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes (medium ~50% RU) |
| Receiver | Non-ideal channel estimation and interference modeling LMMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | CQI, PMI and RI reporting triggered per 5ms Feedback delay is 5 ms |
| Transmission scheme | MU-MIMO with rank adaptation |
| Wrapping method | Geographical distance based |
| Metrics | Average UE throughput, 5% UE throughput vs. feedback overhead |
| Overhead | PDCCH (2 symbols), TRS (20ms period), DMRS Type 2, NZP CSI-RS for CM, ZP CSI-RS (4Port) for IM, 1 SSB / 20ms |