3GPP TSG RAN WG1 #97	   			                 R1-1906734

Reno, USA, 13th – 17th May, 2019

Agenda item: 7.2.8.5.

Source: LG Electronics

Title: Evaluation results on (L, p) setting for Rel-16 Type II CSI

Document for: Discussion and Decision

- Introduction

In RAN1#96b meeting, following agreement regarding (L, p) parameter setting for Rel-16 Type II CSI is captured in the chairman’s note as:

Agreement

On RI=3-4 extension:

- (L,p) setting: In RAN1#97 (Reno), down select and decide from the following alternatives:

  - Alt2B, Alt3C, Alt6E (see Table 9 from R1-1905629)

In this contribution, we provide our evaluation results for (L, p) parameters and select the setting among above alternatives to support RI=3-4 extension in Rel-16 Type II CSI.

- Parameter settings to RI=3 and 4

In RAN1#96b, three alternatives for the higher-layer setting of SD/FD basis parameters (L, p) are down-selected in order to design Rel-16 Type II CSI framework to RI=3-4 efficiently while the total payload for RI=3-4 should be comparable to that of RI=2.

For above alternatives with the fixed SD basis parameter L, it is noted that FD basis parameter p can be set and/or configured to RI-common or RI-specific. Moreover, it is also possible to consider layer-common of layer-/layer-group-specific parameter setting for a given RI as shown in Table 1 as:

Table 1. Down-selected alternatives (Alt2B, Alt3C and Alt6E) for (L, p) parameter setting

For example, Alt2B applies a common parameters (L, p) for a given RI value, but it has the different p values for layer 2 and 3. Here, the parameters L and p affect the number of SD and FD basis, respectively, and β determines the max number of non-zero coefficients per layer. Therefore, the parameters (L, p, β) are the main factors for determining the payload for CSI feedback. To further investigate the impact on SD/FD basis parameters, we only consider the value of β as 1/2 and 1/4.

The main point is that the parameter setting with RI-common and layer-/layer-group-specific allows a small amount of implementation impact and leads the performance enhancement in terms of performance-overhead trade off, which is depicted in Figure 1 as:

Figure 1. Average UPT vs. overhead trade-off for various parameter setting (L, p)

Figure 1 presents the average UPT with various parameter setting schemes. It is assumed that 16-port CSI-RS and medium traffic load. Also, each UE is equipped with 4 Rx antenna ports and maximum rank 4 transmission is considered. Note that Table 2 lists the parameter setting for the evaluation. To compute M = {6, 5, 3, 2} for Alt3C and Alt6E, the parameter p is chosen as {4/9, 3/9, 2/9, 1/9}, respectively. Also, beam indicating method such as independent DFT beam selection in FD for layer-specific manner is taken into account. Other simulation assumptions are listed in Annex.

In this plot, we can observe that layer-/layer-group-specific parameter setting for p contributes to improving performance-overhead trade-off compared to that of Alt1 which adopts the parameters with a RI-/layer-common manner (vs. Alt1). For Alt3C adopting RI-specific to RI=3-4 and layer-common p setting, the performance is degraded compared to that of Alt2B since the number of FD basis of layer 0-1 to RI=3-4 is set to v0<y0 (Alt2B vs. Alt3C). Hence, by combining RI-common and layer-group-specific configuration, Alt.2B performs well with quite reduced feedback payload (Alt2B). Note that, for Alt6E, it is difficult to optimize the parameters on the various channel environment and leads high UE implemental complexity while this parameter setting has a flexibility to configure p parameters for all layers to RI=3-4.

|  | Layer | Alt1 |  |  | Alt2B |  |  | Alt3C |  |  | Alt6E |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | M | p | L | M | p | L | M | p | L | M | p |
| RI = 1 | 0 | 4 | 7 | 1/2 | 4 | 7 | 1/2 | 4 | 7 | 1/2 | 4 | 7 | 1/2 |
| RI = 2 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| RI = 3 | 0 |  |  |  |  | 7 | 1/2 |  | 5 | 1/3 |  | 6 | 4/9 |
|  | 1 |  |  |  |  |  |  |  |  |  |  | 4 | 1/4 |
|  | 2 |  |  |  |  | 4 | 1/4 |  |  |  |  | 2 | 1/9 |
| RI = 4 | 0 |  |  |  |  | 7 | 1/2 |  |  |  |  | 6 | 4/9 |
|  | 1 |  |  |  |  |  |  |  |  |  |  | 5 | 1/3 |
|  | 2 |  |  |  |  | 4 | 1/4 |  |  |  |  | 4 | 1/4 |
|  | 3 |  |  |  |  |  |  |  |  |  |  | 3 | 2/9 |

Table 2. Parameter setting (L, p) for schemes in Figure 1

Observation 1. As shown in Figure 1, layer-/layer-group-specific parameter setting for p contributes to improving performance-overhead trade-off compared to that of Alt1.

Observation 2. For Alt3C adopting RI-specific to RI=3-4 and layer-common p setting, the performance is degraded compared to that of Alt2B since the number of FD basis of layer 0-1 to RI=3-4 is set to v0<y0.

Observation 3. With RI-common and layer-group-specific configuration, Alt.2B shows the best performance-overhead trade-off.

- Conclusion

In this contribution, we discuss the (L, p) parameters for Rel-16 Type II CSI in order to efficiently support MU-MIMO to RI=3-4. Based on the discussion above, we have following observations as:

Observation 1. As shown in Figure 1, layer-/layer-group-specific parameter setting for p contributes to improving performance-overhead trade-off compared to that of Alt1.

Observation 2. For Alt3C adopting RI-specific to RI=3-4 and layer-common p setting, the performance is degraded compared to that of Alt2B since the number of FD basis of layer 0-1 to RI=3-4 is set to v0<y0.

Observation 3. With RI-common and layer-group-specific configuration, Alt.2B shows the best performance-overhead trade-off.

Annex

Table A-1. Simulation assumptions

| Parameters | Value |
| --- | --- |
| Scenarios | Dense Urban (4GHz with 15kHz SCS), ISD=200m |
| BS Tx Power | 41 dBm |
| BS antenna configurations (M,N,P,Mg,Ng,Mp,Np) | Dense Urban: 16ports=(8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ 32ports=(8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| MS antenna configurations | 2 Rx/4 Rx X-pol (0/+90), |
| Etilt angle | 102 degree |
| System bandwidth | 10MHz (52RBs), SB size = 4RBs |
| UE attachment | Based on RSRP |
| Duplex | FDD |
| UE speed | 3km/h for indoor, 30km/h for outdoor |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes (medium ~50% RU) |
| Receiver | Non-ideal channel estimation and interference modeling LMMSE-IRC receiver |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | CQI, PMI and RI reporting triggered per 5ms Feedback delay is 5 ms |
| Transmission scheme | MU-MIMO with rank adaptation |
| Wrapping method | Geographical distance based |
| Metrics | Average UE throughput, 5% UE throughput vs. feedback overhead |
| Overhead | PDCCH (2 symbols), TRS (20ms period), DMRS Type 2, NZP CSI-RS for CM, ZP CSI-RS (4Port) for IM, 1 SSB / 20ms |