3GPP TSG RAN WG1 Meeting #97																R1-1906823

Reno, USA, May 13th – 17th, 2019

Source:	Intel Corporation

Title:	UE and gNB Measurements for NR Positioning

Agenda item:	7.2.10.3

Document for:	Discussion and Decision

Introduction

Work item on NR Positioning support was approved in [1]. The WID has the following objectives:

Define UE measurements based on DL reference signals applicable for NR positioning. The following UE measurements are specified for serving, reference, and neighboring cells [RAN1] - DL RSTD (reference signal time difference) measurements for NR positioning - DL RSRP (reference signal received power) measurements for NR positioning - UE RX-TX time difference measurements for NR positioning Define gNB measurements based on UL reference signals applicable for NR positioning. The following gNB measurements are specified [RAN1]: - UL RTOA (relative time of arrival) measurements for NR positioning - UL Angle of Arrival (AoA) measurements (including Azimuth and Zenith Angles) for NR positioning - UL RSRP (reference signal received power) measurements for NR positioning - gNB RX-TX time difference measurements for NR positioning

The following agreements were made by RAN1 WG with respect to measurements for NR Positioning at the previous RAN1 WG Meetings:

RAN1#96bis Intra-frequency and inter-frequency DL RSTD measurements (currently supported to be based at least on DL PRS) are supported in RRC_CONNECTED mode At least intra-frequency DL PRS-RSRP measurements are supported in RRC_CONNECTED mode - FFS: Inter-frequency DL PRS-RSRP measurements in RRC_CONNECTED mode The network can indicate one or more of the following for the UE to use to determine a reference (reference time based on the DL PRS Resource ID(s)) for DL RSTD measurements. - A DL PRS Resource ID - A subset of DL PRS Resource IDs from a single DL PRS Resource set - A DL PRS Resource set The UE may use different DL PRS Resource ID(s) (with the condition that the multiple DL PRS Resource IDs belong to a single DL PRS Resource set) or a different DL PRS Resource set for determining the reference for the RSTD measurement, and if it chooses to do so, it should report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource set used to determine the reference

In this document, we discuss details of UE and gNB measurements for NR Positioning. Our views on other NR Positioning aspects are provided in our companion contribution [2]-[4].

Positioning Techniques, Reference Signals, Measurements

## E-CID Reference Signals and Measurements

The following agreement was made by RAN1 WG with respect to E-CID support during the study item phase:

NR should support E-CID based positioning - E-CID downlink measurements should be supported based on at least RRM measurements defined in NR Rel. 15 - Note: - Support of NR Rel-16 signals, measurements and procedures for E-CID are to be discussed when specified - NR Rel-15 signals may be used to define additional UE/gNB measurements to facilitate E-CID support

In our view, this agreement is sufficient and there is no motivation to consider Rel-16 reference signals to support the NR E-CID positioning technique. Our preference, is to keep NR E-CID as a simple positioning technique that can be supported by majority of UEs. Additional measurements can be defined in Rel-16 to complete the full list of E-CID measurements which are supported by LTE E-CID positioning. In particular, NR E-CID based on Rel-15 reference signals can be enhanced by introducing additional measurements to facilitate support of UL-AoA and DL-AoD techniques. The extension of E-CID to incorporate DL-AoD can be done with help of SS-RSRP measurement which is already defined and can be reused for reporting. The extension of E-CID to incorporate UL-AoA, can be done by introducing measurements based on Rel-15 UL PRACH or SRS reference signals without additional specification changes.

  - NR E-CID is supported based on Rel-15 reference signal framework

  - Additional measurement for NR E-CID support are introduced

    - UL-AoA based on Rel-15 UL SRS/PRACH

    - DL-AoD based on Rel-15 SS-RSRP

    - UE/gNB Tx-Rx time difference

    - Rel-15 SS-SINR (to replace RSRQ)

## Use of Rel-15 NR Reference Signals for NR Positioning

Support of the NR Rel-15 reference signals for NR Positioning can be considered in the following contexts:

Use of Rel-15 only reference signals for all or selected NR positioning techniques

  - The main drawback of Option 1 is that it may require implementation of two different procedures at UE and network side for NR Positioning (considering that new DL positioning reference signals and muting are necessary for improved NR positioning performance). From UE and system perspective, having common solution independently of considered use cases is a preferred option.

Use of Rel-15 and Rel-16 reference signals, where use of Rel-15 reference signals is left up to UE implementation (except NR E-CID based positioning)

  - This option can be considered if network provides assistance data and the use of Rel-15 reference signals is left up to UE implementation, so that UE can utilize it for the benefit of NR positioning where it is necessary/feasible. This option, if it is supported, should not impose any requirement on joint processing of Rel-15 and Rel-16 reference signals for positioning.

Use of Rel-16 reference signals only (i.e. Rel-15 reference signals are not in use for NR positioning)

  - This option rules out the use of Rel-15 reference signal for all NR positioning techniques, except E-CID.

Based on analysis of the above options, we have the following proposal for Rel-16 work.

  - Use of Rel-15 reference signals for all NR Positioning techniques (except of NR E-CID support) is left up to UE implementation

  - Network provides assistance signaling to UE in order to facilitate the use of Rel-15 reference signals for the benefit of NR positioning (e.g. quasi-collocation information)

  - NR positioning measurement requirements/test cases do not assume that Rel-15 reference signals are used by UE

## Mapping of Measurements, Reference Signals & Measurements

During the study item phase on NR Positioning, the mapping between positioning techniques, reference signals and UE/gNB measurements was partially discussed but not finalized. In this section (please refer to Table 1), we provide our views on the minimum set of measurements needed for NR Positioning in Rel.16. The table below also provides our suggested mapping between reference signals, measurements and positioning techniques. We suggest to discuss this table in RAN1 WG and inform RAN2 WG on conclusions made by RAN1 WG. Note that this table does not imply that RAN2 is supposed to define dedicated signaling for all identified positioning techniques. Oppositely it is assumed that RAN2 can further discuss signaling details to facilitate support of identified techniques and any combination.

Table 1: Mapping of NR positioning techniques to reference signals and UE/gNB measurements

| NR Positioning Techniques | DL Reference Signals | UE Measurements | UL Reference Signals | gNB Measurements |
| --- | --- | --- | --- | --- |
| DL-TDOA | Rel.16 DL PRS | DL PRS-RSTD | N/A | N/A |
| DL-AoD | Rel.16 DL PRS | DL PRS-RSRP | N/A | N/A |
| UL-TDOA | N/A | N/A | Rel.16 UL SRS | UL SRS-RTOA |
| UL-AoA | N/A | N/A | Rel.16 UL SRS | UL SRS-AoA (Azimuth, Zenith) |
| Multi-cell RTT | Rel.16 DL PRS | Rx-Tx time difference | Rel.16 UL SRS | Rx-Tx time difference |
| E-CID | Rel.15 SSB Rel.15 CSI-RS for BM | Rx-Tx time difference | Rel.15 PRACH Rel.15 UL SRS | Rx-Tx time difference PRACH/SRS – AoA SRS-RSRP, SRS-SINR |

Note: Rel.16 UL SRS designates UL SRS signals with possible enhancements targeted in Rel.16 as a part of NR positioning

Note: Rel.16 DL PRS designate DL PRS signals introduced in Rel.16 for NR positioning

  - Agree on mapping b/w NR positioning techniques, reference signals and measurements as provided in Table 1.

## Additional Measurements for NR Positioning

During the NR Positioning study item, basic measurements were agreed to facilitate support of NR positioning. Additional measurements characterizing radio environment during estimation of signal location parameters are also beneficial. Therefore, we propose to introduce RSRP and SINR measurement for DL PRS and UL SRS.

In NR, SINR measurement was already defined. For instance, SS-SINR can be used for NR E-CID replacing RSRQ measurement. As for other NR Positioning techniques, the SINR measurement can be defined for Rel-16 DL PRS and UL SRS (PRS). In addition, we assume that RSRP measurement can be useful for any of the NR positioning techniques.

  - Define DL PRS and UL PRS SINR measurements (i.e. for Rel-16 DL PRS and UL SRS(PRS))

If the above proposal is agreed, then the overall mapping of NR Positioning techniques, reference signals and measurements can be summarized by the following table (see Table 2).

Table 2: Proposed mapping b/w NR positioning techniques, reference signals and measurements

| NR Positioning Techniques | DL Reference Signals | UE Measurements | UL Reference Signals | gNB Measurements |
| --- | --- | --- | --- | --- |
| DL-TDOA | Rel.16 DL PRS | DL PRS-RSTD DL PRS-RSRP DL PRS-SINR | N/A | N/A |
| DL-AoD | Rel.16 DL PRS | DL PRS-RSRP DL PRS-SINR | N/A | N/A |
| UL-TDOA | N/A | N/A | Rel.16 UL SRS | UL SRS-RTOA UL SRS-RSRP UL SRS-SINR |
| UL-AoA | N/A | N/A | Rel.16 UL SRS | UL SRS-AoA (Azimuth, Zenith) UL SRS-RSRP UL SRS-SINR |
| Multi-cell RTT | Rel.16 DL PRS | Rx-Tx time difference, DL PRS-RSRP DL PRS-SINR DL PRS RSTD1) | Rel.16 UL SRS | Rx-Tx time difference UL SRS-RSRP UL SRS-SINR |
| E-CID | Rel.15 SSB Rel.15 CSI-RS for BM | SS-RSRP SS-SINR Rx–Tx time difference | Rel.15 PRACH Rel.15 UL SRS | Rx-Tx time difference PRACH/SRS – AoA SRS-RSRP, SRS-SINR |

1) DL PRS RSTD measurement can be used for calculation of RTT with neighboring cells in case of accurate network synchronization

Analyzing Table 2, it can be seen that DL-TDOA is a superset of DL-AoD in terms of measurements and reference signals and therefore these techniques can be combined under common NR DL Positioning framework. In addition UL-TDOA and UL-AoA, require common reference signaling and therefore can be also combined under common NR UL Positioning framework.

  - DL-TDOA and DL-AoD can be combined under common NR DL Positioning framework

  - UL-TDOA and UL-AoA can be combined under common NR UL Positioning framework

UE Measurements for NR Positioning

## DL PRS Measurements for NR DL Timing Based Positioning

The following measurements were agreed by RAN1 WG for NR Positioning during the study item phase:

DL RSTD (reference signal time difference) measurements

DL RSRP (reference signal received power) measurements

UE RX-TX time difference measurements

In addition, the following agreements were made with respect to DL PRS measurements

Agreements at RAN1#96bis Intra-frequency and inter-frequency DL RSTD measurements (currently supported to be based at least on DL PRS) are supported in RRC_CONNECTED mode The network can indicate one or more of the following for the UE to use to determine a reference (reference time based on the DL PRS Resource ID(s)) for DL RSTD measurements. - A DL PRS Resource ID - A subset of DL PRS Resource IDs from a single DL PRS Resource Set - A DL PRS Resource Set The UE may use different DL PRS Resource ID(s) (with the condition that the multiple DL PRS Resource IDs belong to a single DL PRS Resource Set) or a different DL PRS Resource Set for determining the reference for the RSTD measurement, and if it chooses to do so, it should report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource Set used to determine the reference

DL PRS-RSTD

For DL PRS-RSTD measurements, a UE need to estimate timing reference based on reference DL PRS Resource(s) and the timing of the first arrival path for target DL PRS Resource(s) and subtract timing reference from the timing of target DL PRS resource. In order to come up with a definition for DL-PRS-RSTD measurements the more discussion is needed on estimation of the timing for DL PRS Resources or DL PRS Resource Sets.

  - DL PRS RSTD definition requires additional discussion on timing measurement across DL PRS Resources within DL PRS Resource Set

DL PRS-RSRP

For DL PRS-RSRP measurements, a UE should be configured with a DL PRS Resource(s) to be used for measurements. The following definition can be used as a starting point for DL-PRS-RSRP. Note that this definition may need to be adjusted subject to further discussion relationship of DL BWPs and DL PRS processing.

| Definition | DL PRS reference signal received power (DL-PRS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry DL PRS signals. For frequency range 1, the reference point for the DL-PRS-RSRP shall be the antenna connector of the UE. For frequency range 2, DL-PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported DL-PRS-RSRP value shall not be lower than the corresponding DL-PRS-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency. |

UE Rx-Tx Time Difference

In order to support multi-cell RTT based positioning UE Tx-Rx time difference measurement needs to be defined. In order to obtain UE Tx-Rx time difference measurements at the UE side for multiple cells, two options can be considered:

Option 1. Ranging (RTT estimation - RTT0) with serving (or reference) cell/TRP and RSTD measurements for neighboring cells/TRPs relative to serving (or reference) cell/TRP

  - This option has benefits in terms of coverage given that majority of measurements are done for DL PRS that are supposed to have significantly large coverage than UL SRS (PRS) due to UE and gNB TX power difference. The drawback of this option is that it requires accurate network synchronization or knowledge of the synchronization error (TX timing offset between cells/TRPs) at the UE side. This TX timing offset can be either provided by the network or directly compensated at gNB side considering that it is known (i.e. can be measured by gNBs).

  - In this case, UE Rx-Tx Time Difference for the m-th TRP is equal to:

UE Rx-Tx Time Difference for the m-th TRP (RTTm) = 2·RSTDm + RTT0 =

= 2·RSTDm + UE Rx-Tx Time Difference for the 0-th TRP (serving or reference)

Option 2. Ranging (RTT estimation) with multiple cells/TRPs

  - The main drawback of Option-2 is that it requires timing measurements from each UE across multiple cells/TRPs. This may become a bottleneck for UL SRS (PRS) processing due to link budget and UL overhead (spectrum efficiency) perspective. It is clear that Option 2 has much lower scaling capability in terms of capacity of positioned users.

  - The main benefit of this Option 2 is that it is not sensitive to gNB sync error. If RTT measurements are available jointly with gNB Tx-Rx time difference measurements (e.g. the estimates of (t3-t2) or (y3-y2) as shown in Figure 1), the inter-cell timing synchronization error can be estimated (either at UE side or gNB side depending on the protocol) and compensated.

  - In this case, UE Rx-Tx Time Difference for the m-th TRP is equal to:

UE Rx-Tx Time Difference for the m-th TRP = RTTm

Figure 1: Principle of measuring of inter-cell time synchronization offset at UE side

  - Two methods are supported by NR for UE Tx-Rx measurements

    - Method 1: UE Tx-Rx measurements are obtained through ranging with serving (or reference) cell/TRP for RTT estimation and RSTD measurements with respect to serving (or reference) cell/TRP

      - Note: Method 1 assumes synchronized network (TX timing offset among TRPs/gNBs is zero or TRP’s/gNB’s timing offsets with respect to serving(reference) cell/TRP are indicated by network)

    - Method 2: UE Tx-Rx measurements are obtained through ranging with multiple cells/TRPs (including serving(reference) and neighbor cells/TRPs)

      - Note: Method 2 assumes that network measures gNB TX-RX time difference for UEs in serving and neighboring cells. Measured gNB TX-RX time differences are reported to UEs or LMF

Additional DL PRS Measurements

The quality of RSTD measurement depends on DL PRS Resource RSRP and SINR metrics. The SINR metric is more suitable in case of interference limited scenarios. The report of RSRP and SINR measurements is beneficial on top of the RSTD measurement itself. In application to RSTD, the DL PRS-RSRP measurement can be defined in the following two ways:

Option 1: RSRP from all multi-path components (RSRP) – traditional RSRP measurement

Option 2: RSRP of the first arrival path component (RSRPFAP)

The following ratios can be considered as an estimate of K-Factor (or metric characterizing frequency selectivity):

RSRPFAP / RSRP

(RSRPFAP / (RSRP - RSRPFAP))

The high values of K-Factor are often good indicators of the LOS type of propagation. The information about LOS propagation conditions is useful for UE positioning and therefore can be considered for NR Positioning.

  - For NR positioning techniques based on DL timing estimation, the following additional measurements based on DL PRS signals are supported

    - DL PRS SINR

    - DL PRS RSRPFAP

      - DL PRS RSRPFAP can be replaced by K-Factor indication

## DL PRS Measurements for NR DL-AoD based Positioning

If DL PRS RSRPFAP measurement is defined it may be possible to estimate DL-AoD based on DL-PRS RSRP or DL PRS RSRPFAP criteria (e.g. Resource ID with max DL PRS-RSRP or max DL PRS-RSRPFAP) can be reported. In addition, UE may be able to report measurements using both criteria. In this case, two hypothesis can be analyzed at NW side. In case of LOS propagation condition both criteria are likely to produce the same results that may further increase confidence of LOS radio-conditions together with K-Factor measurements.

  - DL-AoD can be determined based on either of the two metrics DL PRS RSRP or DL PRS RSRPFAP

  - Support of both mechanisms of DL-AoD determination is beneficial for NR Positioning

  - NR Positioning supports DL-AoD measurements based on both DL PRS RSRP and DL PRS RSRPFAP

    - DL PRS Resource ID with max of DL PRS RSRP value

    - DL PRS Resource ID with max of DL PRS RSRPFAP value

UE Reporting for NR Positioning

## DL PRS Report Configuration

For NR positioning, a UE is expected to perform measurements for DL PRS Resources within DL PRS Resource Sets, which are configured for UE measurements and reporting. A UE behavior is expected to follow DL PRS Measurement Report configuration for UE DL PRS measurements (see Figure 2).

Figure 2: DL PRS Measurement Report and its association with DL PRS Resource Sets and Resources

  - NR supports signaling to configure UE with DL PRS Measurement Report (DPMR) settings

The DL PRS RSRP and SINR measurements are expected to be conducted individually per DL PRS Resource. As for the DL PRS RSTD measurements, taking into account outcome of the discussion on estimation of reference time for RSTD, different options are possible for RSTD measurements:

Option 1. UE is configured to report single RSTD measurement per DL PRS Resource Set

  - Option 1A. Using single DL PRS Resource from DL PRS Resource Set

  - Option 1B. Using subset of DL PRS Resources from DL PRS Resource Set

  - Option 1C. Using all DL PRS Resources from DL PRS Resource Set

Option 2. UE is configured to report M (M ˃ 1) RSTD measurements per DL PRS Resource Set based on M DL PRS Resources

In both options, it is assumed that DL PRS Resource ID(s) (pointers to DL PRS Resources for measurements) are either provided by network or selected by UE if there is no DL PRS Resource IDs provided as a part DL PRS Measurement Report configuration. If UE is expected to select DL PRS Resources for RSTD measurement it may be preconfigured with a criteria for selection of M DL PRS Resources from DL PRS Resource Set and follow this criteria for DL PRS Resource selection for reporting. Independently of specific options agreed, a UE is expected to always perform measurements according to DL PRS Measurement Report configuration provided by network.

  - For DL PRS RSRP and SINR measurements,

    - UE is configured with a list of DL PRS Resource Set IDs and DL PRS Resource IDs for measurement and reporting of measurement results

  - For DL PRS RSTD measurements,

    - UE is configured whether to provide single or multiple (M) RSTD measurements per DL PRS Resource Set

    - If a UE is configured with a list of DL PRS Resource IDs for each configured DL PRS Resource Set ID,

      - UE is expected to use indicated DL PRS Resources for RSTD measurements and reporting

    - Otherwise, if there is no configuration of DL PRS Resource IDs provided by network,

      - UE is expected to autonomously select one or M DL PRS Resources for RSTD measurements and report their IDs jointly with RSTD measurements

Based on discussion above we have following proposal on DL PRS Measurement Report Configuration:

  - DL PRS Measurement Report Configuration contains at least the following information

    - Reference DL PRS Resource ID(s) / Resource Set

    - List of DL PRS Resource Set IDs

    - List of DL PRS Resource IDs for each DL PRS Resource Set ID

    - DL PRS Report Type (e.g. periodic, semi-persistent, aperiodic)

    - DL PRS Measurement Set (e.g. RSTD, RSRP, SINR, RSRPFAP, etc.)

## Granularity of DL PRS Measurements for UE Reporting

The granularity of DL PRS measurements for reporting can be discussed at a later stage once the set of measurements is finalized, as well as more progress on DL PRS design itself is reached. The general rule of thumb here is that reporting granularity should not significantly degrade positioning accuracy.

## UE Capability Consideration

The UE DL PRS Resource processing capabilities are always limited. For instance, UE may have restriction on maximum number of DL PRS Resources and DL PRS Resource Sets that may be configured. In addition, the number of measurements and reports can be also limited, including values of processing delays. For network based positioning solutions, UE is expected to be configured with the DL PRS Measurement Report configurations that are aligned with UE capabilities.

  - UE is not expected to be configured with number of DL PRS Resources /Resource Sets that exceed UE capabilities in terms of DL PRS measurements and reporting

Other details of UE capabilities in terms of DL PRS processing can be discussed at later stages when RAN1 makes more progress on DL PRS design.

gNB Measurements for NR Positioning

In our understanding, measurements supported at gNB side should be symmetrical to measurements supported at UE side (i.e. same set of measurements should be supported) at least for timing based positioning techniques (UL-TDOA and RTT).

## Measurements on UL SRS (PRS)

UL SRS(PRS) RSRP

The following definition of UL SRS(PRS) RSRP measurement can be considered as a starting point for discussion:

| Definition | UL PRS reference signal received power (UL-PRS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry UL PRS signals. For frequency range 1, the reference point for the UL-PRS-RSRP shall be the antenna connector of the UE. For frequency range 2, UL-PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by gNB, the reported UL-PRS-RSRP value shall not be lower than the corresponding UL-PRS-RSRP of any of the individual receiver branches. |
| --- | --- |

UL SRS(PRS) AoA

Considering 3D MIMO processing at gNB, the UL Azimuth (UL-A-AoA) and Zenith (UL-Z-AoA) AoA measurements can be defined for NR Uplink positioning. In addition we think that for UL-A-AoA and UL-Z-AoA measurements can be done for the first arrival path component of channel impulse response (Type-1) or taking into account contribution of all multi-path components (Type-2). Both measurements should be done using UL SRS (PRS).

  - For NR UL-AoA positioning, NR supports two types of AoA measurements

    - Type-1: based on the first arrival path component of channel impulse response (e.g. using UL-SRS-RSRPFAP)

    - Type-2: based UL-SRS-RSRP measurements of all multi-path components

gNB Tx-Rx time difference

For gNB Tx-Rx time difference, the following definition (based on LTE definition) can be used as a starting point for NR positioning discussion. The final definition can be provided once RAN1 makes more progress on timing measurements (e.g. TgNB-RX) including processing of SRS Resources and/or SRS Resources Sets.

| Definition | The gNB Rx – Tx time difference is defined as T gNB-RX – TgNB-TX Where: TgNB-RX is the gNB received timing of uplink radio frame #i, defined by the first detected path in time. For frequency range 1, the reference point for TgNB-RX shall be the Rx antenna connector. For frequency range 2, the reference point for TgNB-RX shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. T gNB-TX is the gNB transmit timing of downlink radio frame #i. The reference point for TgNB-TX shall be the Tx antenna connector. |
| --- | --- |

In order to support multi-cell RTT at UE side, the gNB Tx-Rx time difference can be signalled to UE.

Additional UL SRS(PRS) Measurements

Similar to discussion on DL measurements, it is beneficial to additionally support UL SRS(PRS) SINR and UL SRS(PRS) RSRPFAP or K-Factor indication.

  - For NR positioning techniques based on UL timing estimation, the following measurements are supported for UL SRS (PRS) signals

    - UL SRS(PRS) SINR

    - UL SRS(PRS) RSRPFAP

      - Can be replaced by K-Factor indication

## gNB Reporting

There are two types of gNB reporting that can be considered:

gNB shares measurements to network (LMF/location server). In general, the corresponding signaling for this type of reporting can be developed by RAN2, however RAN1 may need to work and decide on parameters to be included into that signaling.

gNB shares measurements results to UEs (either directly or through LMF/location server). Signaling details for sharing gNB measurements results to UEs can be discussed in RAN2 WG, however our understanding is that RAN1 can provide recommendation on parameters and measurements to be shared.

# Conclusion

In this contribution, we have provided our views on UE and gNB measurements for NR Positioning. In summary, we have following proposals:

  - NR E-CID is supported based on Rel-15 reference signal framework

  - Additional measurement for NR E-CID support are introduced

    - UL-AoA based on Rel-15 UL SRS/PRACH

    - DL-AoD based on Rel-15 SS-RSRP

    - UE/gNB Tx-Rx time difference

    - Rel-15 SS-SINR (to replace RSRQ)

  - Use of Rel-15 reference signals for all NR Positioning techniques (except of NR E-CID support) is left up to UE implementation

  - Network provides assistance signaling to UE in order to facilitate the use of Rel-15 reference signals for the benefit of NR positioning (e.g. quasi-collocation information)

  - NR positioning measurement requirements/test cases do not assume that Rel-15 reference signals are used by UE

  - Agree on mapping b/w NR positioning techniques, reference signals and measurements as provided in Table 1.

  - Define DL PRS and UL PRS SINR measurements (i.e. for Rel-16 DL PRS and UL SRS(PRS))

  - Two methods are supported by NR for UE Tx-Rx measurements

    - Method 1: UE Tx-Rx measurements are obtained through ranging with serving (or reference) cell/TRP for RTT estimation and RSTD measurements with respect to serving (or reference) cell/TRP

      - Note: Method 1 assumes synchronized network (TX timing offset among TRPs/gNBs is zero or TRP’s/gNB’s timing offsets with respect to serving(reference) cell/TRP are indicated by network)

    - Method 2: UE Tx-Rx measurements are obtained through ranging with multiple cells/TRPs (including serving(reference) and neighbor cells/TRPs)

      - Note: Method 2 assumes that network measures gNB TX-RX time difference for UEs in serving and neighboring cells. Measured gNB TX-RX time differences are reported to UEs or LMF

  - For NR positioning techniques based on DL timing estimation, the following additional measurements based on DL PRS signals are supported

    - DL PRS SINR

    - DL PRS RSRPFAP

      - DL PRS RSRPFAP can be replaced by K-Factor indication

  - NR Positioning supports DL-AoD measurements based on both DL PRS RSRP and DL PRS RSRPFAP

    - DL PRS Resource ID with max of DL PRS RSRP value

    - DL PRS Resource ID with max of DL PRS RSRPFAP value

  - NR supports signaling to configure UE with DL PRS Measurement Report (DPMR) settings

  - For DL PRS RSRP and SINR measurements,

    - UE is configured with a list of DL PRS Resource Set IDs and DL PRS Resource IDs for measurement and reporting of measurement results

  - For DL PRS RSTD measurements,

    - UE is configured whether to provide single or multiple (M) RSTD measurements per DL PRS Resource Set

    - If a UE is configured with a list of DL PRS Resource IDs for each configured DL PRS Resource Set ID,

      - UE is expected to use indicated DL PRS Resources for RSTD measurements and reporting

    - Otherwise, if there is no configuration of DL PRS Resource IDs provided by network,

      - UE is expected to autonomously select one or M DL PRS Resources for RSTD measurements and report their IDs jointly with RSTD measurements

  - DL PRS Measurement Report Configuration contains at least the following information

    - Reference DL PRS Resource ID(s) / Resource Set

    - List of DL PRS Resource Set IDs

    - List of DL PRS Resource IDs for each DL PRS Resource Set ID

    - DL PRS Report Type (e.g. periodic, semi-persistent, aperiodic)

    - DL PRS Measurement Set (e.g. RSTD, RSRP, SINR, RSRPFAP, etc.)

  - UE is not expected to be configured with number of DL PRS Resources /Resource Sets that exceed UE capabilities in terms of DL PRS measurements and reporting

  - For NR UL-AoA positioning, NR supports two types of AoA measurements

    - Type-1: based on the first arrival path component of channel impulse response (e.g. using UL-SRS-RSRPFAP)

    - Type-2: based UL-SRS-RSRP measurements of all multi-path components

  - For NR positioning techniques based on UL timing estimation, the following measurements are supported for UL SRS (PRS) signals

    - UL SRS(PRS) SINR

    - UL SRS(PRS) RSRPFAP

      - Can be replaced by K-Factor indication

# References

- RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson

- R1-1906821, “Downlink reference signal design for NR positioning”, Intel Corporation, Reno, USA, May, 2019

- R1-1906822, “Uplink reference signal design for NR positioning”, Intel Corporation, Reno, USA, May, 2019

- R1-1906824, “On physical-layer procedures for NR positioning”, Intel Corporation, Reno, USA, May, 2019