3GPP TSG RAN WG1 #97	R1-1906852

Reno, USA, 13th – 17th May, 2019

Agenda Item:	7.2.10.1

Source: 	Sony

Title:	Considerations on DL reference signals for NR positioning

Document for:	Discussion / Decision

# Introduction

In RAN plenary #83, a new work item “NR positioning support” was approved [1] . One of the work item objectives are [1] :

Specify NR DL and UL reference signals to facilitate support of NR positioning techniques (DL-TDOA, DL-AoD, UL-TDOA, UL-AoA, multi-cell RTT and E-CID) [RAN1]

  - Support E-CID downlink measurements based on at least RRM measurements defined in NR Rel. 15

  - Identify whether and which Rel-15 NR reference signals can be used for different NR positioning techniques

  - Define new DL positioning reference signals applicable at least for DL-TDOA, DL-AoD, RTT

  - Define UL SRS with possible enhancements for positioning which is applicable at least for RTT, UL-TDOA, UL-AoA

# In RAN1 #96bis [2] , the following agreements related to DL positioning reference signals were made:

Agreement:

- A DL PRS Resource Set is defined as a set of DL PRS Resources, where each DL PRS Resource has a DL PRS Resource ID

- The DL PRS Resources in a DL PRS Resource set are associated with the same TRP

- A DL PRS Resource ID in a DL PRS Resource set is associated with a single beam transmitted from a single TRP (A TRP may transmit one or more beams)

- Note: This does not have any implications on whether the TRPs and beams from which signals are transmitted are known to the UE.

- DL PRS sequence is generated using a Gold sequence generator as defined in TS 38.211 Section 5.2.1

- FFS: cinit value for DL PRS

- QPSK modulation is used for the DL PRS signal transmitted using CP-OFDM

- FFS: Whether a DL PRS sequence generated using a different mechanism is also specified.

- Comb-N resource element pattern per DL PRS Resource is supported to map DL PRS sequence to resource elements in frequency domain

- Comb-N pattern can shift in frequency domain across symbols within DL PRS Resource

- FFS values for N. The potential values for down-selection are provided in the set {1,2,3,4,6,8,12}

- FFS relationship between N and number of symbols per DL PRS Resource

- FFS support of staggered and non-staggered patterns and exact definition of staggered pattern

- DL PRS configuration including DL PRS transmission schedule is to be indicated to the UE for DL PRS positioning measurements

- The UE is not expected to perform any blind detection of DL PRS configurations

- Number of DL PRS Sequence IDs is at least 4096

- The DL PRS configuration provided to the UE is independent of the configured DL BWPs of the UE

- Support of numerologies (CP length and sub-carrier spacing) for the DL PRS is the same as for data transmissions in Rel-15.

- DL PRS muting is supported. The UE is expected to be indicated when the DL PRS is muted.

# In this contribution, we discuss our view on downlink reference signals to support NR positioning, particularly on downlink PRS configuration.

# Downlink PRS Configuration

As illustrated in Figure 1, a UE needs to receive multiple beam-forming signals from gNBs or TRPs for the multilateration in OTDOA. In addition, the UE is expected to receive a beam-forming signal form each gNB/TRP that can indicate a certain direction. This can be further exploited in order to improve the positioning accuracy. This operation can be facilitated by introducing beam-identity on each beam-formed carrying PRS signal. Considering the positioning measurement report is the legacy RSTD information and the beam information, the positioning measurement error can be minimized. This particularly applies in a multipath fading scenario where the quality of RSTD measurement can be degraded and lead to higher positioning measurement error. Thus, the usage of beam information minimizes the measurement error. In addition, the UE positioning estimation by LS can be performed with less than three gNBs. This is particularly the case when the positioning measurements quality of one or two gNBs are significantly better than the others.

Figure 1: Illustration of TDOA with beam-forming signal from gNBs.

As illustrated in Figure 1, each PRS is transmitted in a certain beam direction. A gNB typically need to cover multiple directions. For example, in NR rel.15 initial access, the transmission of SSBs in an SSB burst set can be with beam-sweeping operation that can cover multiple directions. Similarly, the gNB can perform beam-sweeping for PRS transmission as agreed in the previous meeting. Here, we need to define a multiplexing (resource mapping) of beam-based PRS transmission. In RAN1 meeting 96bis, it was agreed that a DL PRS Resource Set is defined as a set of DL PRS Resources, where each DL PRS Resource has a DL PRS Resource ID. A set of PRS resources (e.g. resource elements) can be configured to represent a certain beam direction as illustrated in Figure 2 below. Here, the multiplexing of beam-based PRS transmission from multiple gNBs is also illustrated in a dedicated NR DL PRS resources. To facilitate the PRS multiplexing scheme, we propose:

Proposal 1: DL PRS resource has at least the following attributes: resource ID, frequency shift, time offset, and duration of DL PRS.

Proposal 2: The NR DL PRS resource set configuration (e.g. in one or multiple slot) is designed to accommodate multi-beam transmission (e.g. beam-sweeping) from one or more gNBs.

Proposal 3: Beam configuration such as the maximum number of beams used for PRS transmission is part of DL PRS resource set configuration.

There may be a gap between set of resources to accommodate beam-switching operation. The transmission of PRS in all beam directions should be confined within a known duration of time. For example, within a certain burst duration. The PRS-burst can be consecutively repeated or periodically transmitted with a certain interval. The UE knows the PRS configuration. Based on the received PRS resources the UE can identify the gNB beam direction of the associated positioning measurement (RSTD).

Figure 2: Illustration of beam-based PRS multiplexing

There may also be a need to minimize UE positioning measurement time. This can be facilitated by multiplexing of beam-based PRS transmission from a gNB in frequency domain and thus, minimize the PRS burst duration. This is illustrated in Figure 3. Here, a PRS transmission with repetition is also illustrated. As comparison, the illustration in Figure 2 represents time-domain multiplexing operation. In practice, depending on the PRS bandwidth size and the number of beams, the beam-based PRS transmission can be multiplexed in both frequency and time domain. On the arrangement, it can be frequency domain first then followed by time-domain multiplexing. Furthermore, each frequency allocation for PRS transmission can perform different beam-sweeping pattern. There are some benefits by allowing such operation. A UE with single RX and capable of wideband reception can performs positioning measurements relatively shorter than the case where the gNB only perform time domain multiplexing. Another example, a UE with multi RX beam and capable of wideband reception can perform simultaneous measurements on each RX beam.

Note that the PRS transmission with frequency domain multiplexing may also subject to gNB limitations. The frequency domain multiplexing with the need to cover multiple beam directions is typically supported with digital beam-forming operation.

Figure 3: Beam based PRS transmission with frequency domain multiplexing

Proposal 4: DL PRS resource that has a DL PRS resource ID can be multiplexed in either time-domain and/or frequency-domain depending on the gNB capability.

In RAN1 meeting 96bis, it was agreed that The DL PRS configuration provided to the UE is independent of the configured DL BWPs of the UE. In addition, the DL PRS numerologies is the same as for NR data transmission in Rel-15. In our view, NR needs to support a similar use case as in LTE positioning, where multiple UEs may need to perform positioning measurements based on the PRS from multiple base-stations. In order to accommodate this scenario, at least a DL PRS resource pool with a specific numerology is needed. This resource pool can accommodate DL PRS resources from multiple gNBs.

Proposal 5: NR supports DL PRS resource pool that has a specific bandwidth and duration.

# Conclusion

In this contribution, we discussed our view on DL reference signals for NR positioning. We have made the following proposals:

Proposal 1: DL PRS resource has at least the following attributes: resource ID, frequency shift, time offset, and duration of DL PRS.

Proposal 2: The NR DL PRS resource set configuration (e.g. in one or multiple slot) is designed to accommodate multi-beam transmission (e.g. beam-sweeping) from one or more gNBs.

Proposal 3: Beam configuration such as the maximum number of beams used for PRS transmission is part of DL PRS resource set configuration.

Proposal 4: DL PRS resource that has a DL PRS resource ID can be multiplexed in either time-domain and/or frequency-domain depending on the gNB capability.

Proposal 5: NR supports DL PRS resource pool that has a specific bandwidth and duration.

# References

- RP-190752, “New WID: NR Positioning Support”

- RAN1 Meeting #96bis Chairman Notes