3GPP TSG RAN WG1 #97	R1-1906855

Reno, USA, 13th – 17th May, 2019

Agenda Item:	7.2.10.4

Source: 	Sony

Title:	On physical-layer procedures to support UE and gNB measurements

Document for:	Discussion / Decision

# Introduction

A new work item “NR positioning support” was approved In RAN plenary #83 [1] . The following are some of the objectives [1] :

Define UE measurements based on DL reference signals applicable for NR positioning. The following UE measurements are specified for serving, reference, and neighboring cells [RAN1]

  - DL RSTD (reference signal time difference) measurements for NR positioning

  - DL RSRP (reference signal received power) measurements for NR positioning

  - UE RX-TX time difference measurements for NR positioning

Define gNB measurements based on UL reference signals applicable for NR positioning. The following gNB measurements are specified [RAN1]:

  - UL RTOA (relative time of arrival) measurements for NR positioning

  - UL Angle of Arrival (AoA) measurements (including Azimuth and Zenith Angles) for NR positioning

  - UL RSRP (reference signal received power) measurements for NR positioning

  - gNB RX-TX time difference measurements for NR positioning

If necessary, define physical-layer procedures to support UE/gNB measurements for NR positioning [RAN1]

# During study item phase, NR positioning techniques (e.g. DL-TDOA, DL-AoD, UL-TDOA, UL-AoA) were presented and evaluated. The preliminary results are shown in [2] .

# The following were some agreements made during in RAN1 meeting #96bis [3] :

Agreement:

- The DL PRS configuration provided to the UE is independent of the configured DL BWPs of the UE.

- For positioning purposes, to assist UE to perform Rx beamforming, consider one or more of the following options:

- Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell.

  - Discuss further which DL reference signal can be used (e.g., SSB, CSI-RS, DL-PRS).

- Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter.

- Option 3: The UE may use a fixed Rx beam to receive DL PRS resources which are transmitted with different downlink spatial domain transmission filter.

  - FFS: Whether this option can be achieved by Option 1

- FFS on specification impacts.

- For positioning purposes, with regards to the UL SRS transmission power, at least the following options have been identified:

- Option 1: The UL SRS Tx power is constant (i.e., no power control is supported).

  - FFS: The value of the constant Tx power and how it is determined

- Option 2: The UL SRS Tx power is based on the existing power control procedure.

- Option 3: The UL SRS Tx power is determined by modifying the existing power control procedure, for example:

  - Support configuring a DL reference signal of a neighboring cell to be used for SRS path loss estimation.

    - FFS: Which reference signals can be used (e.g. a CSI-RS, or a SSB, or DL PRS).

  - FFS: which power control modes are supported (e.g., open loop only, or open loop and closed loop).

- For positioning purposes, with regard to UL Beam management/alignment towards serving and neighbouring cells, consider one or more of at least the following options:

- Option 1:  Support configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS.

  - FFS: Which Reference DL RS can be used (e.g., SSB/CSI-RS/DL-PRS).

- Option 2: Support UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources.

- Option 3: The Tx beam is fixed for UL SRS transmissions across multiple UL SRS Resources, for both FR1 and FR2.

  - FFS: How the fixed beam is selected

# In this contribution, we discuss our view on physical-layer procedures to support UE and gNB measurements, particularly related to the above recent agreements.

# Discussions

The operation of NR in FR2 frequency range offers wide bandwidth. However, the transmission in such high frequency requires a high directional antenna / beam forming to compensate path-loss in radio propagation channel. The usage of beam-sweeping for synchronization Signal Block (SSB) transmission to cover gNB cell coverage has been introduced in NR rel.15. A similar approach can be used for NR positioning in which the transmission of PRS can be configured with a beam sweeping operation.	In LTE positioning, the UE receives PRS configuration of each surrounding eNBs. Due to the transmission of NR PRS with beam forming and possible beam sweeping operation, a UE needs to receive the gNBs beam configuration prior to the UE positioning measurements. Beam configuration may include beam identity (e.g. PRS index) of a PRS resource-set in which it can be further exploited to obtain the PRS angle of departure (AoD).

Proposal 1: The gNB beam configuration of the PRS transmission is part of PRS configuration and it is provided to the UE.

The UE is expected to perform Rx beamforming for DL PRS reception and some options were identified in RAN1 96bis as shown in the previous section. We consider Option 1 and Option 2 can be supported. However, RAN1 needs to study on the specific use-cases for each option. On option 1, a UE can be configured to one of the cells and a fix RX beam can be used. A UE can also use any of these options as long as the positioning measurement results can be obtained within a specific measurement time.

Proposal 2: To assist UE to perform Rx beamforming for PRS reception, the following options are supported:

•	Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell.

•	Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter.

On uplink-based positioning, beam alignment between UE and gNBs is essential. The beam configuration of uplink SRS can be based on the DL measurements. In case the UE does not support beam correspondence, the UE can perform UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS resources.

Proposal 3: For positioning purposes, with regard to UL Beam management/alignment towards serving and neighbouring cells, the following options are supported:

- Option 1:  Support configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS.

  - FFS: Which Reference DL RS can be used (e.g., SSB/CSI-RS/DL-PRS).

- Option 2: Support UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources.

In legacy NR, a UE can be configured with 4 DL BWP configurations. These DL BWPs are UE-specific and there is only one active DL BWP. The design of DL PRS transmission should support positioning of multiple UEs (i.e. to reduce PRS overhead) as in legacy LTE system. Hence, the DL PRS transmission can be a cell-specific configuration. The bandwidth of DL PRS can be bigger than the largest UE-specific DL BWP. The UE should be able to measure the DL PRS resource outside of the UE specific DL BWP configuration as long as it is within the UE maximum supported bandwidth. The measurement based on wideband DL-PRS can results in a better accuracy than narrowband DL-PRS.

Proposal 4: The UE can measure the DL PRS resource outside the DL BWP configuration.

The initial target of NR positioning for both regulatory and commercial requirements are described in [2] . The report also mentioned that it is expected that no single solution would fit all of the requirements for various use-cases and scenarios. In downlink-based positioning (e.g. DL-TDOA), an accurate positioning result typically requires wide bandwidth and/or long reference signal (e.g. PRS) transmission. Long PRS transmission may, however, increase the latency. Yet, some use-cases require high accuracy and short latency. In practice, we consider the majority of use-cases would not require high accuracy and only require reasonable latency. Nevertheless, the solution should cover both the more demanding use cases with very high accuracy requirement (e.g. sub 1m), as well as the uses cases with less demanding requirements

Observation 1: NR positioning is expected to support demanding requirements (high accuracy and low latency) for some UEs and relaxed requirements for typical applications in most UEs.

Based on the above observation, an adaptive/flexible approach to positioning techniques may be used to address the multiple requirements. A flexible approach can include the combination of RAT dependent and RAT independent techniques which is not the scope of this work item [1] . Another consideration is the ability to flexibly configure OTDOA positioning with a goal of achieving the positioning requirements. It has been agreed that the flexibility can be achieved by the possibility of allocating on-demand NR DL PRS resource allocation. By doing so, the UE may receive more reference signals in order to obtain an improved positioning measurement for higher positioning accuracy. A location server is assumed to know the required positioning accuracy. It can trigger (e.g. via LPPa protocol) the selected gNBs to allocate on demand resource allocation for NR DL PRS transmission.

Proposal 5: On-demand PRS resources can be provided to the UE(s) in addition to the PRS resources that is periodically transmitted by the gNB.

Another aspect which we would like to propose is on supporting the combined downlink and uplink-based positioning (and vice-versa) to improve the positioning accuracy. In this case, the uplink-based positioning (UL-TDOA) can be operated in conjunction with the downlink-based positioning (DL-TDOA). We can expect downlink PRS signal is periodically transmitted by the gNBs. Here, the SRS resources for UL-TDOA can be allocated after the DL-PRS resources. The time-gap between the different uplink and downlink resources shall be relatively small so that the obtained measurements can be utilized to a single positioning estimate. This is illustrated in Figure 1. Two positioning measurement results represents the two directions and can be obtained and utilized by the location server for positioning estimation. Furthermore, the UE needs to receive the configuration of both positioning configurations including the timing relationship (i.e T-gap) between downlink and uplink positioning reference signals.

T-gap

Resources for UL pos

Resources for DL pos

Figure 1:Illustration of DL-PRS and UL-SRS to support DL/UL positioning

Proposal 6: On supporting DL & UL based positioning measurements, a timing relationship between DL and UL positioning reference signals shall be provided.

# Conclusion

In this contribution, we discussed our view on some aspects related to physical-layer procedures to support UE and gNB measurements. We have made the following observation and proposals:

Proposal 1: The gNB beam configuration of the PRS transmission is part of PRS configuration and it is provided to the UE.

Proposal 2: To assist UE to perform Rx beamforming for PRS reception, the following options are supported:

•	Option 1: The DL PRS can be configured to be QCLed Type D with a DL Reference Signal from a serving or neighboring cell.

•	Option 2: The UE may perform a Rx beam sweeping on DL PRS resources which are transmitted with the same downlink spatial domain transmission filter.

Proposal 3: For positioning purposes, with regard to UL Beam management/alignment towards serving and neighbouring cells, the following options are supported:

- Option 1:  Support configuration of a spatial relation between a reference DL RS from serving or neighbouring cells and the target SRS.

  - FFS: Which Reference DL RS can be used (e.g., SSB/CSI-RS/DL-PRS).

- Option 2: Support UE Tx beam-sweeping on UL SRS transmissions across multiple UL SRS Resources.

Proposal 4: The UE can measure the DL PRS resource outside the DL BWP configuration.

Observation 1: NR positioning is expected to support demanding requirements (high accuracy and low latency) for some UEs and relaxed requirements for typical applications in most UEs.

Proposal 5: On-demand PRS resources can be provided to the UE(s) in addition to the PRS resources that is periodically transmitted by the gNB.

Proposal 6: On supporting DL & UL based positioning measurements, a timing relationship between DL and UL positioning reference signals shall be provided.

# References

- RP-190752, “New WID: NR Positioning Support”

- TR38.855,” Study on NR positioning support”

- Chairman notes RAN1 Meeting #96bis