3GPP TSG RAN WG1 #97		R1-1906863

Reno, USA, 13th – 17th May 2019

Source:	Panasonic

Title: 	Discussion on channel structure for 2-step RACH

Agenda Item:		7.2.1.1

Document for:	Discussion

Introduction

A work item on two step RACH for NR was approved [1]. One of objectives of this work item is to specify the channel structure of Msg.A, which consists of RACH preamble and PUSCH carrying payload. The design principle based on WID is as follows.

- Only reuse the Rel-15 NR PRACH Preambles design.

- Only reuse the Rel-15 NR PUSCH including Rel-15 DMRS for transmission of payload of msgA

  - No new CP length and no sub-PRB guard subcarrier(s)

- Specify the mapping between the PRACH preamble and the time-frequency resource of PUSCH in msgA+DMRS

  - PRACH Preamble and PUSCH in a msgA is TDMed

- Specify the supported MCS(s) and time-frequency resource size(s) of PUSCH in msgA

  - Consider the msgA payload contents determined by RAN2

The agreements made in previous RAN1 meeting are summarized in Appendix A. In this document, we provide our view on channel structure for 2-step RACH.

This document is the update of R1-1904357 [2].

Discussion

2-step RACH

The basic procedure of 4-step RACH and 2-step RACH are illustrated in Fig.1. In general, 2-step RACH procedure is expected to provide the benefits to reduce the latency and/or DL overhead than 4-step RACH procedure by simplified a RACH procedure. In 2-step RACH, Msg.A includes both RACH preamble and PUSCH carrying the equivalent contents of Msg.3 of 4-step RACH. Msg.B includes the equivalent contents of both Msg.2 and Msg.4 of 4-step RACH, although Msg.4 includes RRC message or only MAC contention resolution is discussed in our contribution [3].

(a) 4-step RACH procedure                                      (b) 2-step RACH procedure

Fig.1 RACH procedure

Configuration of PUSCH occasion in Msg.A

In 4-step RACH, the PRACH preamble configuration is provided to the UE in the system information. In RRC CONNECTED, the PRACH preamble configuration can be provided via the dedicated information. The system information also provides information for the UE to determine the association between the RACH preamble resources and DL signal/channel for Msg.2 and the latter Msgs. Based on the DL measurement and corresponding association, the UE can determine a RACH preamble resource set. In 4-step RACH, UE determines whether transform precoding for Msg.3 PUSCH is used or not based on RRC parameter msg3-transformPrecoding. UE also determines the subcarrier spacing for Msg.3 PUSCH from the RRC parameter msg3-SubcarrierSpacing. UE determines the time/frequency resources and other transmission parameters for Msg.3 PUSCH such as whether or not frequency hopping is enabled, MCS, and TPC command via UL grant in RAR. In 2-step RACH, UE needs to determine time/frequency resources and other transmission parameters for PUSCH in Msg.A without UL grant. Therefore, in addition to RACH preamble resource configuration, the gNB needs to provide physical resources and transmission parameters for PUSCH in Msg.A to UE.

For time/frequency resources of PUSCH in Msg.A (PUSCH occasion), two options on how to specify/configure PUSCH occasion were agreed in RAN1#96. One is PUSCH occasions are separately configured from PRACH occasions (Option 1). The other is the relative location of the PUSCH occasion is specified/configured with respect to the associated PRACH occasion (Option 2).

In Option 1, one of possibilities is to reuse configured grant operation for the configuration of PUSCH occasion signalling. At least for RRC CONNECTED mode, Rel.15 configuration of PRACH and the configured grant configuration for PUSCH occasion can have fully flexible operation. In addition, if Type 2 configured grant is used, to change PUSCH resource by activation/deactivation is possible. For RRC IDLE mode, predetermined PUSCH occasion configuration is provided to the UE in the system information. The resource indication signalling can be from configured grant resource signalling. Even in Option 1, some association between PRACH preamble and PUSCH occasion may be necessary. One of possibilities is that UE transmits Msg.A payload in the first available PUSCH occasion after PRACH preamble transmission. Another possibility is similar to the association between SSB and RACH occasion, to define a mapping rule to determine the association between SSB and PUSCH occasion. For latter scheme, in case SSB to RACH occasion of 4-step RACH, all SSB needs to have the preparation of 4-step RACH are necessary as it is a kind of minimum function. In case of 2-step RACH, it should be allowed that only some SSB support 2-step RACH. Option 1 can have flexibility for PUSCH resource utilization from network perspective, it may complicate 2-step RACH operation and gNB receiver.

In Option 2, since the association between PRACH preamble and PUSCH occasion are more direct relation, the 2-step RACH operation including gNB receiver can be simplified at the cost of less flexibility. From signalling perspective, even if Option 2 is baseline design, some part of PUSCH resource indication signalling can be from configured grant operation. In Option 2, some alternatives on the relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions were identified. Within the alternatives, configurable transmission gap between PRACH preamble in PRACH occasion(s) to PUSCH occasion including zero should be considered (i.e., Alt.3 or Alt.4). This can ensure gNB processing time if necessary. Note that zero gap does not mean contiguous transmission between PRACH preamble and PUSCH. Even if transmission gap between PRACH preamble in PRACH occasion(s) to PUSCH occasion is zero, there could still be guard period between transmissions of PRACH preamble and PUSCH as preamble format itself.

Observation 1:

- For time/frequency resources of PUSCH in Msg.A (PUSCH occasion),

  - Option 1 (PUSCH occasions are separately configured from PRACH occasions) using configured grant configuration can provide flexible operation for 2-step RACH, while it may complicate 2-step RACH operation and gNB receiver.

  - Option 2 (the relative location of the PUSCH occasion is specified/configured with respect to the associated PRACH occasion) can be simplify the 2-step RACH operation including gNB receiver at the cost of less flexibility.

Proposal 1: If Option 2 is supported, configurable transmission gap between PRACH preamble in PRACH occasion(s) to PUSCH occasion including zero should be considered.

On PUSCH numerology, it was agreed in RAN1#96bis, when the PRACH and PUSCH for Msg.A transmission are in different slots, the numerology for Msg.A PUSCH follow the numerology configured for the UL BWP for Msg.A transmission. FFS point is whether to support PRACH and PUSCH in the same slot for Msg.A transmission and if supported there are three options.

- Option 1: the numerology for msgA PUSCH follows that of msgA preamble

- Option 2: gNB configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP

- Option 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission

In our view, Option 3 is the simplest way. If different numerologies within a slot is supported (i.e., Option 2), whether to have UL transmission with different numerologies within a slot should be UE capability/feature.

Observation 2: For PUSCH numerology is PRACH and PUSCH in the same slot for Msg.A transmission is supported, Option 3 (a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission) is the simplest.

Observation 3: Whether to have UL transmission with different numerologies within a slot should be UE capability/feature.

On PUSCH waveform, following two options were identified in the feature lead summary [4].

- Option 1: Follow the same waveform of msg3 as msg3-Transformprecoder

- Option 2: Separate configured, which may or may not be the same as msg3

Separate configuration between CP-OFDM for 2-step RACH and DFT-s-OFDM for 4-step RACH may be possibility if different RACH procedure can be selected in cell. For example, 2-step RACH is used rather around cell center area based on RSRP/RSRQ criterion discussed in RAN2. While, in order to support HARQ soft combining, the same waveform of Msg.3 would be beneficial if fallback to Msg.3 PUSCH is supported.

Observation 4: If 2-step RACH is used rather around cell center area based on RSRP/RSRQ criterion discussion in RAN2, separate configuration between CP-OFDM for 2-step RACH and DFT-s-OFDM for 4-step RACH may be possibility.

Observation 5: If fallback to Msg.3 PUSCH is supported, to follow the same waveform of Msg.3 is beneficial in order to support HARQ soft combining.

Mapping between PRACH preamble resource and PUSCH resource unit

The definition of PUSCH resource unit for 2-step RACH was agreed in RAN1#96bis. PUSCH resource unit is the PUSCH occasion and DMRS port / DMRS sequence used for an Msg.A payload transmission. For the mapping between preambles in each RACH occasion and associated PUSCH resource unit, the design should consider the resource utilization efficiency and decoding complexity at gNB. The gNB will configure the RACH preamble resource configuration considering the resource utilization and collision probability. As the mapping between RACH preamble and the PUSCH resource unit, at least support one-to-one and multiple(N)-to-one was working assumption. For one-to-one mapping, a large amount of PUSCH resources needs to be reserved for the PUSCH in Msg.A. On the other hand, multiple(N)-to-one mapping would cause high collision probability of PUSCH depending on the N value. Such high collision case should be standardized in NOMA. Multiple(N)-to-one mapping with low collision probability which does not require NOMA such as advanced receiver could be considered.

Proposal 2: Working assumption of supporting one-to-one and multiple-to-one mapping between preambles in each RACH occasion and associated PUSCH resource unit should be confirmed.

TBS and time/frequency resource size of PUSCH

In addition, based on the WID, UP data transmission in RRC CONNECTED mode as in Rel.15 NR is supported. In order to achieve a certain level of flexibility by allowing to accommodate variable size of UP data packets, it would be beneficial that the network can configure a set of TB sizes for the PUSCH transmission to the UE. RAN2 LS [5] mentioned that no upper bound for the content size is defined by RAN2 but any limited based on RAN1 design would be good to be known to be able to design the signalling options properly. Based on the data volume in UE buffer, UE selects a TBS/MCS and time/frequency resource size for PUSCH in Msg.A from the configured parameter sets. The selected TBS/MCS and time/frequency resource size for PUSCH in Msg.A are indicated by the selection of RACH preamble resource set. Alternatively, the selected TBS for PUSCH in Msg.A are indicated in UCI which multiplexed on PUSCH or conveyed in a separate encoded control channel (e.g., PUCCH).

Proposal 3: In 2-step RACH, allow gNB to configure a set of TBSs, MCS, and time/frequency resource size for PUSCH in Msg.A. The UE may select a set of these parameters based on the configuration and buffer size.

Proposal 4: The indication of UE selected parameter set is indicated by the selection of RACH preamble resource set or UCI transmitted together with PUSCH.

Conclusion

In this contribution, we discussed channel structure for 2-step RACH and made following observation and proposals.

Configuration of PUSCH occasion in Msg.A

Observation 1:

- For time/frequency resources of PUSCH in Msg.A (PUSCH occasion),

  - Option 1 (PUSCH occasions are separately configured from PRACH occasions) using configured grant configuration can provide flexible operation for 2-step RACH, while it may complicate 2-step RACH operation and gNB receiver.

  - Option 2 (the relative location of the PUSCH occasion is specified/configured with respect to the associated PRACH occasion) can be simplify the 2-step RACH operation including gNB receiver at the cost of less flexibility.

Proposal 1: If Option 2 is supported, configurable transmission gap between PRACH preamble in PRACH occasion(s) to PUSCH occasion including zero should be considered.

Observation 2: For PUSCH numerology is PRACH and PUSCH in the same slot for Msg.A transmission is supported, Option 3 (a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission) is the simplest.

Observation 3: Whether to have UL transmission with different numerologies within a slot should be UE capability/feature.

Observation 4: If 2-step RACH is used rather around cell center area based on RSRP/RSRQ criterion discussion in RAN2, separate configuration between CP-OFDM for 2-step RACH and DFT-s-OFDM for 4-step RACH may be possibility.

Observation 5: If fallback to Msg.3 PUSCH is supported, to follow the same waveform of Msg.3 is beneficial in order to support HARQ soft combining.

Mapping between PRACH preamble resource and PUSCH resource unit

Proposal 2: Working assumption of supporting one-to-one and multiple-to-one mapping between preambles in each RACH occasion and associated PUSCH resource unit should be confirmed.

TBS and time/frequency resource size of PUSCH

Proposal 3: In 2-step RACH, allow gNB to configure a set of TBSs, MCS, and time/frequency resource size for PUSCH in Msg.A. The UE may select a set of these parameters based on the configuration and buffer size.

Proposal 4: The indication of UE selected parameter set is indicated by the selection of RACH preamble resource set or UCI transmitted together with PUSCH.

Reference

[1] RP-182894, “New work item: 2-step RACH for NR,” ZTE Corporation, Sanechips, RAN#82

[2] R1-1904357, “Discussion on channel structure for 2-step RACH,” Panasonic, RAN1#96bis

[3] R1-1906864, “Discussion on 2-step RACH procedure,” Panasonic, RAN1#97

[4] R1-1905793, “Updated summary of 7.2.1.1 Channel Structure for Two-step RACH,” ZTE, RAN1#96bis

[5] R1-1905854, “LS on MsgA content size,” RAN2, RAN1#96bis

- Appendix A: Previous agreements

RAN1#96

Agreements:

- PUSCH occasion for 2-step RACH is defined as

  - the time-frequency resource for payload transmission

- Consider the following methods for PUSCH occasion of msgA transmission:

  - Opt 1: PUSCH occasions are separately configured from PRACH occasions

    - For one PUSCH occasion, it is derived based on:

      - Alt 1: reuse the resource allocation for NR configured grant in principle

      - Alt 2: other potential configurations (e.g., reuse semi-static SFI + BWP, reuse PRACH RO, etc.)

    - FFS detailed association rule between the PRACH and PUSCH for msgA transmission

  - Opt 2: Specify/configure the relative location (in time and/or frequency) of the PUSCH occasion with respect to the associated PRACH occasion

    - Alt 1: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single specification fixed value.

    - Alt 2: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasions can have different values.

    - Alt 3: Time/frequency relation between PRACH preambles in PRACH occasion(s) and PUSCH occasions are single semi-statically configured value.

    - Alt 4: Time/frequency relation between each PRACH preamble in PRACH occasion(s) to the PUSCH occasion is semi-statically configured value. Different preambles in different PRACH occasions can have different values.

    - Note: The time and frequency relation is not required to be the same alternative.

  - FFS detailed mapping between preamble and PUSCH resource + DMRS

Agreements:

- Both DFT-s-OFDM and CP-OFDM are supported for the payload transmission in msgA

  - FFS how to indicate/configure the waveform

- Consider the following numerology for msgA PUSCH (for possible down-selection)

  - Alt 1: follow the numerology configured for the UL BWP

    - FFS initial vs. active UL BWP

  - Alt 2: same as msgA preamble numerology at least for some cases

    - E.g., when short preamble is used (L=139)

RAN1#96bis

Agreements:

- One or more PUSCH occasion(s) within an msgA PUSCH configuration period are configured.

  - FFS msgA PUSCH configuration period, e.g.

    - For opt.1 with separate PUSCH configuration, msgA PUSCH configuration period may or may not be the same as PRACH configuration period

    - For opt.2 PUSCH configuration with relative location, msgA PUSCH configuration period is the PRACH configuration period.

Agreements:

- PUSCH resource unit for 2-step RACH is defined as

  - The PUSCH occasion and DMRS port / DMRS sequence used for an msgA payload transmission.

    - FFS support only one or both of DMRS port / DMRS sequence

    - The DMRS sequence generation mechanism should follow Rel.15.

Working assumption:

- At least support one-to-one and multiple-to-one mapping between preambles in each RO and associated PUSCH resource unit.

  - Configurable number of preambles (including one or multiple) mapped to one PUSCH resource unit

  - FFS one-to-multiple mapping

- Companies are strongly encouraged to perform additional evaluations/analysis

Agreements:

- Support the PRACH and PUSCH for msgA transmission in different slots. In this case, the numerology for msgA PUSCH follow the numerology configured for the UL BWP for msgA transmission.

  - FFS whether to support PRACH and PUSCH in the same slot for msgA transmission. If supported, down-select from the following option.

    - Opt 1: the numerology for msgA PUSCH follows that of msgA preamble

    - Opt 2: gNB configure whether the numerology for msgA PUSCH follows that of msgA preamble or UL BWP

    - Opt 3: a UE is not expected to be configured with different numerology among PRACH preamble, msgA PUSCH and UL BWP for msgA transmission

    - Note: in Rel.15 the PRACH and PUSCH transmitted in the same slot for a UE are not supported.