3GPP TSG RAN WG1 #97		R1-1906868

Reno, USA, 13th – 17th May 2019

Source:	Panasonic

Title: 	Discussion on URLLC enhancements for grant-free transmission

Agenda Item:		7.2.6.6

Document for:	Discussion

Introduction

A work item on physical layer enhancements for NR URLLC was approved [1]. One of objectives of this work item is to specify enhanced UL configured grant (grant-free) transmissions, especially multiple active configured grant type 1 and type 2 configurations for a given BWP of a serving cell. The agreements/observations related to multiple active configured grant configurations made in previous RAN1 meetings are summarized in Appendix A. In this document, we provide our view on enhancement of UL grant-free transmission.

This document is update of R1-1905096 [2].

Discussion

On the parameters/configurations, it was agreed in RAN1#96bis that separate RRC parameters for different configured grant configurations (for both type 1 and type 2 configured grants) for a given BWP of a serving cell is supported. Separate RRC parameter configuration is suitable at least for handling different services/traffic types. On the other hand, if the use case of multiple active configured grant configurations is for keeping reliability and reducing the alignment latency, almost all parameters except for starting offset could be the same. From specification point of view, enabling to configure all parameters independently would be sufficient and for the use case for keeping reliability and reducing the alignment latency, to configure almost all parameters same can be up to network implementation. On the other hand, issues of configuring all parameters independently would be overhead increase in higher layer signalling and in dynamic signalling for activation/deactivation of Type 2 configured grant.

Observation 1: For keeping reliability and reducing the latency usage, almost all parameters except for starting offset could be the same.

One of approaches proposed by several companies is to introduce a concept of a group of configured grant configurations [3]. In this approach, one or more multiple configured grant configuration group(s) can be configured. In each configured grant configuration group, one or multiple configured grant configurations can be included. For a given configured grant configuration group, most parameters should be the same except for the starting offset. Across different configured grant configuration groups, parameters should be configured independently. In this concept, most of parameters except for starting offset can be the same for all configured grant configurations within a group, intended for supporting the use case for keeping reliability and reducing the latency. Since parameters can be independently configured across different configured grant configuration groups, different service or traffic types can also be accommodated. Then, to consider such concept seems reasonable.

Note that the similar discussion on to have multiple starting offsets in configured grant is discussed in NR-U agenda of configured grant [4]. The similar solution should be taken between URLLC and NR-U.

Proposal 1: To introduce a concept of a group of configured grant configurations.

For activation/deactivation of particular configuration for the case of Type 2 configured grant by dynamic DCI indication, to support separate activation or release for different configured grant Type 2 configurations for a given BWP of a serving cell was agreed in RAN1#96bis. Similar to the higher layer configuration aspects, this would be more suitable for handling different services/traffic types due to flexibility in parameter setting. On the other hand, single DCI can reduce DL control overhead. At least for deactivation (release), single DCI (joint signalling) can be used. If the concept of group of configured grant configurations is introduced, the activation can also be done by single DCI within the same configured grant configuration group.

Proposal 2: For Type 2 configured grant, at least for deactivation (release), single DCI (joint signalling) can be used.

Proposal 3: If the concept of group of configured grant configurations is introduced, the activation should also be done by single DCI within the same configured grant configuration group.

How UE select which configuration to use and when a traffic occurs at the UE should be RAN2 discussion similar to what SR resource is selected. If there is no overlap of the resource among configurations, which configuration is used is known by the gNB. On the other hand, for a given resource, if there is overlap, for example, 1) initial transmission of the first configuration and initial transmission of the second configuration are overlapped, 2) non-initial transmission of the first configuration and initial transmission of the second configuration are overlapped, gNB needs to distinguish them by some mechanisms. DMRS based differentiation or UCI which includes configuration ID on configured grant PUSCH should be considered.

Proposal 4: As there can be overlap of the resource among configurations, some mechanism, such as DMRS based differentiation or UCI which includes configuration ID should be used for the distinction.

Conclusion

In this contribution, we discussed enhancement for UL configured grant transmission in Rel.16 URLLC and made following observations and proposals.

Observation 1: For keeping reliability and reducing the latency usage, almost all parameters except for starting offset could be the same.

Proposal 1: To introduce a concept of a group of configured grant configurations.

Proposal 2: For Type 2 configured grant, at least for deactivation (release), single DCI (joint signalling) can be used.

Proposal 3: If the concept of group of configured grant configurations is introduced, the activation should also be done by single DCI within the same configured grant configuration group.

Proposal 4: As there can be overlap of the resource among configurations, some mechanism, such as DMRS based differentiation or UCI which includes configuration ID should be used for the distinction.

Reference

[1] RP-190726, “New WID: Physical Layer Enhancements for NR Ultra-Reliable and Low Latency Communication (URLLC),” Huawei, HiSilicon, RAN#83

[2] R1-1905096, “Discussion on URLLC enhancements for grant-free transmission,” Panasonic, RAN1#96bis.

[3] R1-1903707, “Offline discussions for enhanced configured grant PUSCH transmissions,” NTT DOCOMO, RAN1#96

[4] R1-1906869, “Configured grant enhancements for NR-U,” Panasonic, RAN1#97

- Appendix A: Agreements/Observations in previous meetings

RAN1#94

Agreements:

- Study further whether/how multiple active configured grants for a BWP of a serving cell.

  - Identify potential specification impacts and options for both Type 1 and Type 2

    - At least activation/deactivation mechanism for Type 2

      - E.g., whether each configuration is activated/deactivated or multiple configurations are activated/deactivated.

    - Study how to support repetitions with multiple configurations for a BWP of a serving cell

    - FFS HARQ process ID determination for both Type 1 and Type 2

    - FFS other specification impacts for both Type 1 and Type 2

  - Study the performance impacts

RAN1#94bis

Agreements:

- To study further from at least the following:

  - Option 1: multiple active configured grant configurations for a BWP of a serving cell

  - Option 2: repetition(s) across the boundary of a period P

  - Option 3: one retransmission cross boundary of a period P

  - FFS the UE behaviour when repetitions are collided with the resource which are not available for UL transmission

  - Note: Switch grant free to grant based retransmission which is available in Rel.15

RAN1#95

Agreements:

- Multiple active configured grant configurations for a given BWP of a serving cell should be supported at least for different services/traffic types and/or for enhancing reliability and reducing latency.

  - FFS details

  - Note: it is understood that the above may be related to RAN2-led work on intra-UE multiplexing

RAN1 AH1901

Agreements:

- In Rel-16, for both Type 1 and Type 2 configured grant and when multiple active configurations are configured in a BWP, transmission of a TB based on the configured grant is associated with a single active configuration, even if the transmission is repeated.

RAN1#96bis

Agreements:

- Support separate RRC parameters for different configured grant configurations (for both type 1 and type 2 configured grants) for a given BWP of a serving cell.

  - FFS whether or not some parameters can be common among different configured grant configurations.

Agreements:

- Support separate activation for different configured grant Type 2 configurations for a given BWP of a serving cell.

  - FFS whether or not to support joint activation in a DCI for two or more configured grant Type 2 configurations.

- Support separate release for different configured grant Type 2 configurations for a given BWP of a serving cell.

  - FFS whether or not to support joint release in a DCI for two or more configured grant Type 2 configurations/