3GPP TSG RAN WG1 Meeting #97    			                                               R1-1906893

Reno, USA, May 13th – 17th, 2019

Agenda item:	6.2.1.2

Source:	Samsung

Title:	Discussion on transmission in preconfigured UL resources for MTC

Document for:	Discussion and Decision

# Introduction

In RAN1#96Bis Meeting, following agreements on transmission in preconfigured UL resources for MTC were made [1]:

- Working Assumption#1

In idle mode, updating PUR configurations and/or PUR parameters via L1 signalling after a PUR transmission is supported

- FFS: Which PUR configurations and PUR parameters will be signaled via L1

- FFS: Definition of PUR configurations and PUR parameters

The working assumption will be automatically confirmed if for some cases L2/L3 signaling is not needed. If RAN2 decides that L2/L3 signaling is needed for all cases, the working assumption will be reverted.

- Working Assumption#2

For dedicated PUR

- During the PUR search space monitoring, the UE monitors for DCI scrambled with a RNTI assuming that the RNTI is not shared with any other UE

  - Note: It is up to RAN2 to decide how the RNTI is signaled to UE or derived

- FFS if the UE monitors any additional RNTI which may be shared with other UEs.

- Note: The same RNTI may be used over non-overlapping time and/or frequency resources

Send an LS to RAN2 to include two above working assumptions. Ask whether the first bullet in working assumption #2 is feasible. If it is concluded that working assumption #2 feasible, the working assumption #2 will be automatically confirmed.

Agreement

The UE monitors the MPDCCH for at least a time period after a PUR transmission

- FFS: Details of the time period

- FFS: UE behaviour if nothing is received in the time period

- FFS: If and how often UE monitors MPDCCH after a PUR allocation in which it has not transmitted

Agreement

The value(s) of RSRP threshold(s) is UE specific

Agreement

The power control parameters within the PUR configuration, shall at least include:

- Target UL power level (P_0) for the PUR transmission

Agreement

For dedicated PUR in idle mode, the PUR configuration is configured by UE-specific RRC signaling.

In this contribution, we discussed some issues to specify support for transmission in preconfigured UL resources for MTC.

# Transmission in preconfigured UL resources

## In RRC-connected mode

Transmission in preconfigured UL resources means grant-free UL transmission, i.e., no dynamic scheduling signaling. There may be multiple possibilities considering diverse signaling for UL resources configuration, e.g., L1, L2 and/or L3 signaling. In 5G NR system, two types of grant-free UL transmission are specified, wherein, Type 2 grant-free UL transmission is the same as UL SPS in LTE system and Type 1 grant-free UL transmission introduces semi-static resource configuration in RRC without L1 signaling. Comparing with Type 2, Type 1 can improve latency performance, e.g., arrived data can be immediately transmitted in preconfigured UL resources and needn’t wait L1 signaling for resources activation. But, the flexibility of resource allocation in Type 1 is worse than Type 2 since L1 signaling is supported for resource reconfiguration in Type 2. And, there may be resource wasting during the ambiguity time of RRC signaling carrying resource configuration in Type 1. For a BL/CE UE, considering latency performance is not important, existing UL SPS mechanism in RRC-connected mode is more practical than other grant-free transmission like Type 1.

For CE Mode A, UL SPS has been supported in Rel-13, and it directly reuses LTE mechanism without much modification, including SPS parameters and activation/deactivation of SPS resources via a DCI scrambled with SPS C-RNTI. The functionality of current UL SPS is enough for CE Mode A and new transmission in preconfigured UL resources needn’t be introduced for CE Mode A.

Proposal #1: For CE Mode A, current UL SPS is enough and new transmission in preconfigured UL resources needn’t be introduced.

For CE Mode B, UL SPS or other new transmission in preconfigured UL resources may not be needed because the benefit is not obvious. The preconfigured UL resources may cause resource wasting since the reserved resources may not be always used for data transmission, e.g., no periodic data traffic. In a large CE level, the preconfigured transmission would need more repetitions. Thus, more resources may be wasted if no data arrivals. Therefore, transmission in preconfigured UL resources needn’t be supported for CE Mode B.

Proposal #2: For CE Mode B, transmission in preconfigured UL resources needn’t be supported.

## In RRC-idle mode

In this section, some issues on transmission in preconfigured UL resources in RRC-idle mode are discussed, e.g., dedicated resources or shared resources, validity period of dedicated PUR, skipping UL transmission, activation/deactivation of preconfigured UL resources, TA acquisition and update, and fallback mechanism.

### Dedicated resources or shared resources

In the last meeting, dedicated PUR is agreed and shared PUR is FFS. There are two types of shared PUR, i.e., CFS (Contention-free Shared) PUR and CBS (Contention-based Shared) PUR. For CFS PUR, orthogonal multiple access scheme is used for multiple UEs sharing the same time-frequency resource, e.g., MU-MIMO and CDM. For MU-MIMO, it can be transparently supported using dedicated PUR with UE specific DMRS configuration. How many UEs are configured to share the same time-frequency resource using MU-MIMO is up to eNB configuration and transparent for UE. Therefore, CFS PUR using MU-MIMO with UE specific DMRS pattern needn’t be specified with explicit specification change. For CDM, OCC may be required and much specification effort will be expected, e.g., how to use OCC with either subframe level or symbol level. In the other hand, in RRC-idle mode, it is difficult to use closed loop power control to achieve approximate receiving power for CDMed UEs, which is a key factor to obtain robust multi-user detection performance for CDM. Therefore, CFS PUR using CDM needn’t be considered due to much specification effort and difficulty of power control.

Proposal #3: CFS PUR using MU-MIMO needn’t be specified with explicit specification change.

Proposal #4: CFS PUR using CDM/OCC is not considered.

For CBS PUR, contention resolution is required to ensure the reliability of PUSCH, and it may cause much more specification effort. Transmission in CBS PUR with contention resolution may be like two-step PRACH procedure. For example, in the first step, UE ID is reported in the contention based PUSCH transmitted in the CBS PUR, and in the second step, contention resolution signaling carrying the reported UE ID will be monitored. From UE’s perspective, additional power consumption may be caused for monitoring of contention resolution signaling. Moreover, assuming only orthogonal multiple-access schemes can be used, the capacity of configurable CBS PUR is very limited comparing with PRACH capacity. Thus, the probability of PUSCH collision will be very large, and successful decoding of collided PUSCH may be be more challenged comparing with PRACH collision. Since CBS PUR may cause much more specification effort, more UE power consumption, and more challenges for eNB receiver, CBS PUR is not supported.

Proposal #5: CBS PUR is not supported.

### Validity period of dedicated PUR

In the last meeting, it was agreed to support UE-specific RRC signalling for configuration of dedicated PUR. This means dedicated PUR can be configured in RRC-connected mode and used in RRC-idle mode. Similarly, UE-specific RRC signalling can also be used to reconfigure/release dedicated PUR. However, some UE with dedicated PUR may not be able to release the allocation, e.g., goes out of coverage, breaks, or lose power. Therefore, validity period should be introduced for dedicated PUR. If validity period expires, dedicated PUR shouldn’t be used, and eNB can release the allocation and used for other UE. The timer of validity period can be started when receiving dedicated PUR configuration, or when firstly switching into RRC-idle mode after receiving dedicated PUR configuration. Validity period can be configurable, and indicated in UE-specific RRC signaling or system information.

Proposal #6: Validity period should be introduced for dedicated PUR.

On validity of dedicated PUR, another problem is whether dedicated PUR can be used if the camped cell is changed in RRC-idle mode. For a UE in cell edge, cell reselection may often happen, and the issue is worth to be discussed. One option is that dedicated PUR cannot be used in the new camped cell, i.e., dedicated PUR is cell specific. For efficient resource utilization, the UE should release the dedicated PUR, e.g., UE reports the information of dedicated PUR in the new camped cell. Thus, information exchange related to dedicated PUR may be required between neighboring cells. Another option is that dedicated PUR can still be used in the new camped cell. For example, dedicated PUR may be reserved by multiple cells, i.e., dedicated PUR is cell-group specific. Thus, even the camped cell providing dedicated PUR is changed, the dedicated PUR can still be used. The problem needs to be further discussed to clarify UE behavior.

Proposal #7: FFS whether dedicated PUR can be used if camped cell is changed.

### Skipping UL transmission

In the last RAN1#95 Meeting, one agreement is that “For dedicated PUR in idle mode, the UE may skip UL transmissions. And FFS on resource release mechanism and whether or not to support mechanism to disable skipping by eNB”. To provide the flexibility, skipping UL transmission in dedicated PUR can be enabled or disabled by network. When skipping UL transmission in dedicated PUR is enabled, UE power consumption can be saved. But if a UE is not mandated to transmit on every dedicated PUR, the UE may request more resources than it really needs. For example, a UE may request a dedicated PUR for every 10 seconds but only transmits every 1 hour. This would be very spectrally inefficient. To avoid this, the maximum number of continuous skipping transmission in dedicated PUR should be restricted. If the allowed maximum number of continuous skipping is reached, either dedicated PUR is implicitly released, or a PUSCH carrying zero MAC SDU needs to be transmitted if no any UL data arrivals.

Proposal #8: Maximum number of continuous UL skipping should be specified.

### Activation/deactivation of preconfigured UL resources

In some case, eNB may want to temporarily turn off transmission in PUR since the cell’s traffic load is relatively high. For example, the cell’s traffic load status may be changed from the time when configuring the PUR. Although a dedicated PUR is within a validity period, eNB may want to take back the PUR for dynamical scheduling, i.e., the PUR is activated. When the cell’s traffic load is mitigated, transmission in PUR can be turned on again, i.e., the PUR is activated. Since the cell’s traffic load is not dependent on a certain UE, taking back of PUR should be used for all of UEs with dedicated PUR provided by the cell. Therefore, eNB can turn on or turn off the functionality of transmission in PUR using a broadcast signaling, i.e., preconfigured UL resources can be activated/deactivated in system information.

Proposal #9: Preconfigured UL resources can be activated/deactivated in system information.

### TA acquisition and update

During HARQ procedure of transmission in PUR, a small range of TA may be able to be detected using PUSCH/DMRS, wherein the detectable range using PUSCH/DMRS is smaller than that using PRACH. Thus, TA can be updated within a small range for TA tracking. To save signalling overhead, MAC CE for TA update can be excluded, and the updated TA can be indicated in DCI, e.g., 2~4bits. Although a small error of TA may not impact the decoding of PUSCH, accumulation of the small error would cause significant impact and PUSCH may not be decodable. Therefore, TA update is helpful even within a small range. Continuous TA update in each PUR can maintain a valid TA during a long time.

Proposal #10: TA can be updated within a small range in DCI during HARQ procedure.

### Fallback mechanism

For transmission in preconfigured UL resources, fallback mechanism to RACH/EDT procedure were agreed in the last meeting, but, the details is still FFS. There may be several cases to use fallback mechanism to RACH/EDT procedure. In the first use case, preconfigured UL resources cannot be used to send data. For example, there is no valid TA, the arrival data volume exceeds the allowed maximum TBS of preconfigured UL transmission, or the preconfigured repetition number is not enough due to CE level change, and so on. Thus, UE can fall back to RACH/EDT procedure to send data. In the second use case, transmission in preconfigured UL resources is not successfully decoded, e.g., ACK is not received during a preconfigured window. Thus, UE can fall back to RACH/EDT procedure to resend data. In the third use case, during the HARQ procedure, eNB may trigger UE to fall back to RACH/EDT procedure. For example, the used TA by UE may have a large error and successful decoding cannot be ensured via HARQ combination. Above use cases should be further discussed to conclude which use case can be supported. For the first two use cases, fallback is triggered by UE and no new signalling is required. For the last one use case, fallback is triggered by eNB, and an indication signalling needs to be introduced. If fallback is triggered by eNB, using DCI to indicate fallback to EDT/RACH procedure can be considered.

Proposal #11: Further discuss the use cases of fallback mechanism, and using DCI to indicate fallback to EDT/RACH procedure can be considered for fallback triggered by eNB.

# 3. Conclusion

In this paper, we discussed some issues to specify support for transmission in preconfigured UL resources for MTC. Based on above discussion, we have the following proposals:

For transmission in PUR in RRC-connected mode,

Proposal #1: For CE Mode A, current UL SPS is enough and new transmission in preconfigured UL resources needn’t be introduced.

Proposal #2: For CE Mode B, transmission in preconfigured UL resources needn’t be supported.

For transmission in PUR in RRC-idle mode,

Proposal #3: CFS PUR using MU-MIMO needn’t be specified with explicit specification change.

Proposal #4: CFS PUR using CDM/OCC is not considered.

Proposal #5: CBS PUR is not supported.

Proposal #6: Validity period should be introduced for dedicated PUR.

Proposal #7: FFS whether dedicated PUR can be used if camped cell is changed.

Proposal #8: Maximum number of continuous UL skipping should be specified.

Proposal #9: Preconfigured UL resources can be activated/deactivated in system information.

Proposal #10: TA can be updated within a small range in DCI during HARQ procedure.

Proposal #11: Further discuss the use case of fallback mechanism, and using DCI to indicate fallback to EDT/RACH procedure can be considered for fallback triggered by eNB.

# 4. Reference

- Final Chairman’s notes for RAN1#96Bis Meeting