3GPP TSG RAN WG1 Meeting #97	R1-1905xxx

# Reno, USA,  May 13th – 17th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | XX | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on NR PHR over LTE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In R1-1901487(R2-1819101), RAN2 sent an LS to ask RAN1 to take the following RAN2 agreements into account and update RAN1 specification(s) accordingly if needed: - After a PHR is triggered, if dualConnectivityPHR is configured, UE needs to report power headroom informaion of all activated LTE and NR serving cells. For a NR serving cell, whether UE reports its PH value basd on real transmission or reference format is determined based on UL transmissions that have been scheduled or configured until 4 ms prior to the TTI in which the PHR MAC CE is transmitted. - For a band combination in which a UE is not capable of dynamic power sharing, the UE may omit reporting power headroom information for serving cells in the other MAC entity. When NR power headroom is reported over LTE, NR PH value has to include TPC accumulation fb,f,c(i,l) as specified in TS38.213. In this case, if a PUSCH transmission is configured by ConfiguredGrantConfig, TPC accumulation is performed until TPC commands received up to the product of a number of symbols per slot and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c. Since the minimum of the values of k2 can be smaller than the time required for LTE processing, UE may not have enough time to prepare LTE MAC CE carrying NR PHR. This is also applied for Type 3 PHR calculation when SRS transmission is semi-persistent or periodic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - If UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and NR Type 1 PHR is reported in LTE MAC CE, the UE performs TPC accumulation for NR PHR calculation until the TPC commands received up to 4 ms prior to the LTE subframe. - If UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and NR Type 3 PHR is reported in LTE MAC CE, if SRS transmission is semi-persistent or periodic, the UE performs TPC accumulation for NR PHR calculation until the TPC commands received up to 4 ms prior to the LTE subframe. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case where a UE capable of dynamic power sharing in EN-DC reports NR PHR over LTE, timing for TPC accmulation for NR PHR calculation is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.1, 7.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR only impacts NR PHR via LTE for EN-DC only when UE is capable of dynamic power sharing and if dualConnectivityPHR is configured after a PHR triggered. If the network is implemented based on the CR and the UE is not, the UE calculates PH using the configured k2 and the UE may omit NR PHR. If the UE is implemented based on the CR and the network is not, no inter-operability issue. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

7.7.1	Type 1 PH report

If a UE determines that a Type 1 power headroom report for an activated serving cell is based on an actual PUSCH transmission then, for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell , the UE computes the Type 1 power headroom report as

[dB]

where , , , , ,  and  are defined in Subclause 7.1.1. If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and after a PHR is triggered, if dualConnectivityPHR is configured, for a PUSCH transmission configured by ConfiguredGrantConfig in a slot on active UL BWP b of carrier f of serving cell c, fb,f,c(i,l)  is the accumulation of the TPC commands up to 4 ms prior to the subframe in which the E-UTRA power headroom report is transmitted.

If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration  on active UL BWP  of carrier  of serving cell  is smaller than a SCS configuration  on active UL BWP  of carrier  of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP  that overlaps with multiple slots on active UL BWP , the UE provides a Type 1 power headroom report for the first PUSCH, if any, on the first slot of the multiple slots on active UL BWP  that fully overlaps with the slot on active UL BWP .

If a UE is configured with multiple cells for PUSCH transmissions, the UE does not consider for computation of a Type 1 power headroom report in a first PUSCH transmission that includes an initial transmission of transport block on active UL BWP  of carrier  of serving cell , a second PUSCH transmission on active UL BWP  of carrier  of serving cell  that overlaps with the first PUSCH transmission if

-	the second PUSCH transmission is scheduled by a DCI format 0_0 or a DCI format 0_1 in a PDCCH received in a second PDCCH monitoring occasion, and

-	the second PDCCH monitoring occasion is after a first PDCCH monitoring occasion where the UE detects the earliest DCI format 0_0 or DCI format 0_1 scheduling an initial transmission of a transport block after a power headroom report was triggered

or

-	the second PUSCH transmission is after the first uplink symbol of the first PUSCH transmission minus T'proc,2=Tproc,2 where Tproc,2 is determined according to [6, TS 38.214] assuming d2,1 =1, d2,2=0, and with µDL corresponding to the subcarrier spacing of the active downlink BWP of the scheduling cell for a configured grant if the first PUSCH transmission is on a configured grant after a power headroom report was triggered.

If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion  on active UL BWP  of carrier  of serving cell , the UE computes the Type 1 power headroom report as

[dB]

where  is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS 38.101-3]. The remaining parameters are defined in Subclause 7.1.1 where  and  are obtained using  and p0-PUSCH-AlphaSetId = 0,  is obtained using PathlossReferenceRS-Id = 0, and . If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and after a PHR is triggered, if dualConnectivityPHR is configured, fb,f,c(i,l) in a slot on active UL BWP b of carrier f of serving cell c is the accumulation of the TPC commands up to 4 ms prior to the subframe in which the E-UTRA power headroom report is transmitted.

If a UE is configured with two UL carriers for a serving cell and the UE determines a Type 1 power headroom report for the serving cell based on a reference PUSCH transmission, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pusch-Config. If the UE is provided pusch-Config for both UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pucch-Config. If pucch-Config is not provided to the UE for any of the two UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the non-supplementary UL carrier.

*** Unchanged text is omitted ***

7.7.3	Type 3 PH report

If a UE determines that a Type 3 power headroom report for an activated serving cell is based on an actual SRS transmission then, for SRS transmission occasion  on active UL BWP  of carrier  of serving cell  and if the UE is not configured for PUSCH transmissions on carrier  of serving cell , the UE computes a Type 3 power headroom report as

[dB]

where , , , ,  and  are defined in Subclause 7.3.1. If UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and NR Type 3 PHR is reported in LTE MAC CE, if SRS transmission is semi-persistent or periodic, the UE performs TPC accumulation for NR PHR calculation until the TPC commands received up to 4 ms prior to the LTE subframe. If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and after a PHR is triggered, if dualConnectivityPHR is configured, for semi-persistent or periodic SRS transmission in a slot on active UL BWP b of carrier f of serving cell c, hb,f,c(i)  is the accumulation of the TPC commands up to 4 ms prior to the subframe in which the E-UTRA power headroom report is transmitted.

If the UE determines that a Type 3 power headroom report for an activated serving cell is based on a reference SRS transmission then, for SRS transmission occasion  on UL BWP  of carrier  of serving cell , and if the UE is not configured for PUSCH transmissions on UL BWP  of carrier  of serving cell , the UE computes a Type 3 power headroom report as

[dB]

where  is a SRS resource set corresponding to SRS-ResourceSetId = 0 for UL BWP  and , ,  and  are defined in Subclause 7.3.1 with corresponding values obtained from SRS-ResourceSetId = 0 for UL BWP .  is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB and TC =0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS 38.101-3]. If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and after a PHR is triggered, if dualConnectivityPHR is configured, hb,f,c(i) in a slot on active UL BWP b of carrier f of serving cell c is the accumulation of the TPC commands up to 4 ms prior to the subframe in which the E-UTRA power headroom report is transmitted.

If a UE is configured with two UL carriers for a serving cell and the UE determines a Type 3 power headroom report for the serving cell based on a reference SRS transmission, the UE computes a Type 3 power headroom report for the serving cell assuming a reference SRS transmission on the UL carrier provided by pucch-Config. If pucch-Config is not provided to the UE for any of the two UL carriers, the UE computes a Type 3 power headroom report for the serving cell assuming a reference SRS transmission on the non-supplementary UL carrier.

*** Unchanged text is omitted ***