3GPP TSG RAN WG1 #97	R1-1906963

Reno, USA, May 13th – 17th, 2019

Agenda item:	7.2.7.2

Source: 	Samsung

Title: 	Evaluation results on spectral efficiency for IMT-2020 self-evaluation

Document for:	Discussion and Decision

# Introduction

In this contribution, evaluation results on spectral efficiency in Indoor Hotspot-eMBB scenario Configuration C and Dense-Urban scenario Configuration B are provided for IMT-2020 self-evaluation.

# Evaluation methodology and requirement

In order to evaluate the spectral efficiency using system-level simulation, following methodologies are provided in [1].

Average spectral efficiency Let Ri (T) denote the number of correctly received bits by user i (i = 1,…N) (downlink) or from user i (uplink) in a system comprising a user population of N users and M Transmission Reception Points (TRxPs). Further, let W denote the channel bandwidth and T the time over which the data bits are received. The average spectral efficiency may be estimated by running system-level simulations over number of drops Ndrops. Each drop gives a value of i=1NRi(T) denoted as: R 1T, … RNdrops (T) and the estimated average spectral efficiency resulting is given by: SEavg= j=1NdropsR(j) (T)Ndrops T.W.M= j=1Ndropsi=1NRij(T)Ndrops T.W.M where SEavg is the estimated average spectral efficiency and will approach the actual average with an increasing number of Ndrops and Rij (T) is the simulated total number of correctly received bits for user i in drop j. 5th percentile user spectral efficiency 5th percentile user spectral efficiency is the 5th percentile point of the cumulative distribution function (CDF) of the normalized user throughput, estimated from all possible user locations. Let user i in drop j correctly decode Rij (T) accumulated bits in [0, T]. For non-scheduled duration of user i zero bits are accumulated. During this total time user i receives accumulated service time of Ti ≤ T, where the service time is the time duration between the first packet arrival and when the last packet of the burst is correctly decoded. In case of full buffer, Ti = T. Hence the rate normalised by service time Ti and channel bandwidth W of user i in drop j, ri(j), is: ri(j)= Rij (T)Ti.W Running Ndrops simulations leads to Ndrops × N values of ri(j) of which the lowest 5th percentile point of the CDF is used to estimate the 5th percentile user spectral efficiency.

The requirements of IMT-2020 on spectral efficiency in Indoor Hotspot-eMBB scenario and Dense Urban-eMBB scenario are listed in Table 1. The detailed evaluation assumptions can be found in Appendix.

Table 1. Requirement of spectral efficiency

| Test environnements |  |  | Average SE (bit/s/Hz) |  | 5% SE (bit/s/Hz) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor Hotspot-eMBB | DL | 9 |  | 3 |  |
|  | UL | 6.75 |  | 0.21 |  |
| Dense Urban-eMBB | DL | 7.8 |  | 0.225 |  |
|  | UL | 5.4 |  | 0.15 |  |

# Evaluation results

Evaluation results for average spectral efficiency and 5% spectral efficiency is evaluated in Indoor Hotspot - eMBB scenario with configuration C is summarized in Table 2. Based on the evaluation results, NR achieves minimum performance requirements of average and 5% spectral efficiency for Indoor Hotspot-eMBB with configuration C. It is noted that OFDM waveform is applied for the evaluation.

Table 2. Evaluation results of spectral efficiency for Indoor Hotspot-eMBB

| Test environnements | Configuration |  | Average SE (bit/s/Hz) | 5% SE (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| Indoor Hotspot - eMBB | Config. C (70GHz) | DL | 9.98 | 0.348 |
|  |  | UL | 6.85 | 0.312 |

Additionally, evaluation results for average spectral efficiency and 5% spectral efficiency is evaluated in Dense Urban - eMBB scenario with configuration B. It is also noted that for Dense Urban-eMBB Configuration B, additional assumption of a certain admission control scheme is used. For example, users with worse pathloss (e.g., larger than maximum pathloss of around 125dB) are not attached so that the users attached the cell can be served with better spectral efficiency. Based on the evaluation results, NR achieves minimum performance requirements of average and 5% spectral efficiency for Dense Urban-eMBB Configuration B if additional evaluation assumption is used.

Table 3. Evaluation results of spectral efficiency for Dense Urban-eMBB

| Test environnements | Configuration |  | Average SE (bit/s/Hz) | 5% SE (bit/s/Hz) |
| --- | --- | --- | --- | --- |
| Dense Urban-eMBB | Config. B (30GHz) | DL | 8.37 | 0.26 |
|  |  | UL | 5.75 | 0.22 |

Based on the evaluation results, we draw following observation:

Observation:

- For Indoor Hotspot-eMBB with configuration C, NR achieves minimum performance requirements of average and 5% spectral efficiency for ITU-R self-evaluation.

- For Dense Urban-eMBB with configuration B, NR achieves minimum performance requirements of average and 5% spectral efficiency for ITU-R self-evaluation if a certain level of admission control is applied.

# Conclusion

In this contribution, performance requirements and evaluation results with detailed evaluation assumptions are provided. Based on the evaluation results, we draw following observation:

Observation:

- For Indoor Hotspot-eMBB with configuration C, NR achieves minimum performance requirements of average TRP and 5% UE spectral efficiency for ITU-R self-evaluation.

- For Dense Urban-eMBB with configuration B, NR achieves minimum performance requirements of average and 5% spectral efficiency for ITU-R self-evaluation if a certain level of admission control is applied.

# Appendix

Table 4. Evaluation assumptions for DL Indoor Hotspot-eMBB scenario

| Parameter |  | Value |
| --- | --- | --- |
| ISD |  | 20m (12TRxP) |
| Carrier Frequency |  | 70 GHz |
| System BW |  | 100MHz |
| Subcarrier spacing |  | 120 kHz |
| Symbols number per slot |  | 14 |
| BS Antenna Configuration |  | (M,N,P,Mg,Ng, Mp, Np) = (8,16,2,1,1,2,2) |
| Number of antenna elements per UE |  | (M,N,P,Mg,Ng, Mp, Np) = (2,4,2,1,2,1,1) |
| Total BS Tx Power |  | 23 dBm |
| TRxP number per site |  | 1 |
| Mechanic tilt |  | 180° in GCS (pointing to the ground) |
| Electronic tilt |  | According to Zenith angle in "Beam set at TRxP" |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-MIMO with PF |
| ACK/NACK delay |  | Next available UL slot |
| MIMO mode |  | MU-MIMO with rank 2 adaptation per user Maximum MU layer = 12 |
| Guard band ratio |  | 5.5% (for 100 MHz) |
| BS receiver type |  | MMSE-IRC |
| CSI feedback |  | 25 slots period |
|  |  | NR Type II CSI |
| Overhead | PDCCH | 2 symbols |
|  | DMRS | Type II, based on MU-layer (dynamic in simulation) |
|  | CSI-RS | 32 ports per 5 slots |
|  | SSB | 8 SSB per 20 ms |
|  | TRS | 2 consecutive slots per 20 ms, 1 port |
|  | PTRS | 2 ports PT-RS, (L,K) = (1,4) L is time density and K is frequency density |
|  | PUCCH | 1 symbol |
|  | SRS | 2 symbols per 10 slots |
| Power control parameter |  | P0=-80, alpha = 0.8 |
| Channel estimation |  | Non-ideal |
| Waveform |  | OFDM |

Table 5. Evaluation assumptions for Dense Urban-eMBB scenario

| Parameter |  | Value |
| --- | --- | --- |
| ISD |  | 200m |
| Carrier Frequency |  | 30 GHz |
| System BW |  | 100MHz |
| Subcarrier spacing |  | 120 kHz |
| Symbols number per slot |  | 14 |
| BS Antenna Configuration |  | (M,N,P,Mg,Ng, Mp, Np) = (4,8,2,2,2,1,1) |
| Number of antenna elements per UE |  | (M,N,P,Mg,Ng, Mp, Np) = (2,4,2,1,2,1,1) |
| Total BS Tx Power |  | 37 dBm |
| TRxP number per site |  | 1 |
| Mechanic tilt |  | 90° in GCS (pointing to the ground) |
| Electronic tilt |  | According to Zenith angle in "Beam set at TRxP" |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR 36.873) from port 0 |
| Scheduling |  | MU-MIMO with PF |
| ACK/NACK delay |  | Next available UL slot |
| MIMO mode |  | MU-MIMO with rank 2 adaptation per user Maximum MU layer = 12 |
| Guard band ratio |  | 5.5% (for 100 MHz) |
| BS receiver type |  | MMSE-IRC |
| CSI feedback |  | 25 slots period |
|  |  | NR Type II CSI |
| Overhead | PDCCH | 2 symbols |
|  | DMRS | Type II, based on MU-layer (dynamic in simulation) |
|  | CSI-RS | 32 ports per 5 slots |
|  | SSB | 8 SSB per 20 ms |
|  | TRS | 2 consecutive slots per 20 ms, 1 port |
|  | PTRS | 2 ports PT-RS, (L,K) = (1,4) L is time density and K is frequency density |
|  | PUCCH | 1 symbol |
|  | SRS | 2 symbols per 10 slots |
| Power control parameter |  | P0=-80, alpha = 0.8 |
| Channel estimation |  | Non-ideal |
| Waveform |  | OFDM |