3GPP TSG RAN WG1 97			R1-1906965

Reno, USA, 13th – 17th May, 2019

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	Feature lead summary for MU-MIMO CSI

Document for:	Discussion and Decision

# Introduction

Based on the agreements made in RAN1#96bis [2] and the updated timeline (see section 3) informed with the WID [1], the following items will be summarized in this contributions to facilitate progress based on the submitted contributions ([6]-[48]) combined with the outcomes of offline email discussions led by the FL [4][5]:

- RI=3 and 4: p setting

- RI=3 and 4: K0 setting

- UCI design: Table 2 of R1-1905629

- UCI design: FFS issues in Table 1 of R1-1905629

- Values of N3 for # units>13

Summary

## RI=3 and 4: p setting

The following was agreed in RAN1#96bis [2]:“On RI=3-4 extension:

- (L,p) setting: In RAN1#97 (Reno), down select and decide from the following alternatives: Alt2B, Alt3C, Alt6E (see R1-1905629)”

| Alt2B: RI={1,2,3,4} common, layer-group specific RI; Layer; L; p. 1; 0; x0; y0. 2; 0. 1. 3; 0; y0. 1. 2; y2. 4; 0; y0. 1. 2; y2. 3. | Alt3C: RI={3,4} common, layer common RI; Layer; L; p. 1; 0; x0; y0. 2; 0. 1. 3; 0; v0. 1. 2. 4; 0. 1. 2. 3. |
| --- | --- |
| Alt6E: RI={3,4} specific, layer specific RI; Layer; L; p. 1; 0; x0; y0. 2; 0. 1. 3; 0; v3,0. 1; v3,1. 2; v3,2. 4; 0; v4,0. 1; v4,1. 2; v4,2. 3; v4,3. |  |

The views from different companies and available SLS results can be summarized below.

Table 1 RI=3-4 p setting: summary of companies’ views

| Category | No. companies | Companies – also note the configuration preference for parameters other than x0 and y0: fixed relation vs. higher-layer configured |
| --- | --- | --- |
| Alt2B | 6 | CATT, Fraunhofer/HHI, LGE (fixed), MediaTek, Qualcomm |
| Alt3C | 11 | Ericsson (configured), Intel, vivo, MotM/Lenovo, NEC, Nokia/NSB (configured), OPPO, Samsung (configured), ZTE (configured) |
| Alt6E | 6 | Fraunhofer/HHI, Huawei/HiSi (fixed), Intel, NTT Docomo |

Table 2 RI=3-4 p setting: summary of observation from SLS

| Company | Metric | Key observation |
| --- | --- | --- |
| Ericsson | UPT vs. overhead | - Alt 3C and Alt 6E have similar performance/overhead tradeoff while Alt 2B is inferior |
| Huawei/HiSi | UPT vs. overhead | - Observation 1: Alt2B/3C/6E have similar performance with difference less than 1%. - Observation 2: For rank 3 and 4, the overhead of Alt2B is larger than that of Alt3C and Alt6E, and Alt2B has a larger dynamic range for the overhead of rank 2/3/4. - Alt6E has the smallest dynamic range with the following table for the overhead of rank 2/3/4. RI; Layer; L; p. 1; 0; L0; p0. 2; 0. 1. 3; 0; p0. 1; p0/2. 2. 4; 0; p0/2. 1. 2. 3. - Observation 3: All the three alternatives 2B/3C/6E may have a nested RI and PMI search procedure up to UE implementation. - Observation 4: for (y0, v0) = (1/4, 1/4) compared to (1/4, 1/8), the maximal overhead of rank 4 reporting is increased around 80 bits but the performance gain is only 0.3%. |
| Intel | UPT vs. overhead | - Alt2B has higher overhead while it doesn’t provide performance gains over other alternatives - Alt3C and Alt6E provide similar performance and overhead |
| LGE | UPT vs. overhead | - Observation 1. As shown in Figure 1, layer-/layer-group-specific parameter setting for p contributes to improving performance-overhead trade-off compared to that of Alt1. - Observation 2. For Alt3C adopting RI-specific to RI=3-4 and layer-common p setting, the performance is degraded compared to that of Alt2B since the number of FD basis of layer 0-1 to RI=3-4 is set to v0<y0. - Observation 3. With RI-common and layer-group-specific configuration, Alt.2B shows the best performance-overhead trade-off. |
| MotM/Lenovo | UPT vs. overhead | - Alt2B requires nearly 100 bits additional rank 4 overhead compared to 3C and 6E for the same average user packet throughput and cell edge throughput. |
| Nokia/NSB | UPT vs. overhead | - Alt 2B, Alt 3C and Alt 6E have very similar performance - Joint configuration of y0 and v0 may be advisable, to ensure consistency of the FD basis configuration across layers and RIs. - A limited number of configurations should be supported, possibly not larger than 2. It is not reasonable, nor physically meaningful, to consider configurations for which v0>y0. - y0,v0 values such that the resulting K0 is smaller than 2L may yield lower PMI accuracy than what is achieved by Rel-15 WB-only PMI, i.e., when ‘subbandAmplitude’ set to false. In fact, if K0<2L then the maximum number of reported NZC would always be lower than what is reported by its “low overhead” Rel-15 PMI counterpart, i.e., up to 2L non-zero wideband amplitude values. |
| OPPO | UPT vs. overhead | - Observation 1: Alt 2B outperforms Alt 3C/Alt 6E about 2% with additional 50 bits; Alt 3C/ Alt 6E show similar performance, and the gain of further optimization offered by Alt6E seems marginal. |
| Qualcomm | UPT vs. overhead | - Observation 1: Alt2B and Alt3C with M2=M/2 achieves similar performance with Alt6E. |
| Samsung | UPT vs. overhead | - Alt2B is worse than Alt3C/6E - Alt3C is slightly better than Alt6E in Rel. 15 Type II overhead regime - Alt3C with v0∈18,14 can achieve good performance-overhead tradeoffs in all overhead regimes (low, medium, high). - y0,v0=14,14 can bring 2% additional gain over y0,v0=14,18 with slight increase in overhead. |
| vivo | UPT vs. overhead | - The performance of Alt 3C is slightly worse than Alt 2B with the cost of more CSI overhead for case 0, while the average UPT performance of Alt 3C has a bit improvement in comparison with that of Alt 2B with similar overhead. - The average UPT performance of Alt 3C has a significant improvement in contrast to that of Alt 2B for case 1 with similar overhead. - Overall, Alt 3C provides reasonable balance between overhead and performance gain in contrast to Alt 2B. - Alt 2B and Alt 3C are special cases of Alt 6E. - More than 1% performance gain is achieved by Alt 3C for case 0. - Almost 4% performance gain is obtained for Alt 3C with 2K0 = 56 for case 1. - For the case 1 with 2K0=28, the performance of Alt 3C is slightly better than that of Alt 2B. - Alt 3C shows a considerable gain without increasing overhead in comparison with Alt 2B. |
| ZTE | UPT vs. overhead | - Among Alt 2B, Alt 3C and Alt 6E - Alt 2B costs larger overhead than the other two, without clear performance gain. - Alt 6E does not provide performance gain over Alt 3C. The performance of these two is very close. In some cases, Alt 3C even performs slightly better. It implies that layer-specific setting does not provide gain over layer-common setting. - Alt 3C is simpler than the other two in terms of codebook structure and configuration parameters. |

As evident, Alt3C represents the majority view. It has been demonstrated in several Tdocs via SLS results that, Alt3C, although the simplest among the three, performs similarly to Alt6E (the most complex scheme) and significantly outperforms Alt2B (the second most complex scheme).

Observation: On RI=3-4 p parameter setting, Alt3C represents the majority view.

- Most companies supporting Alt3C also propose that the second parameter v0 be higher layer configurable with v0≤y0.

- Note: It has been agreed that y0∈12,14 is higher-layer configurable (as a part of RI=1-2 agreement)

Proposal: Alt3C (illustrated in the table below) is supported where the parameter p=v0 for RI=3-4 is higher-layer configured in conjunction with the parameter p=y0 for RI=1-2.

- The parameters (y0,v0) take value from 12,14,14,14,14,18

| RI | Layer | L | p |
| --- | --- | --- | --- |
| 1 | 0 | x0 | y0 |
| 2 | 0 |  |  |
|  | 1 |  |  |
| 3 | 0 |  | v0 |
|  | 1 |  |  |
|  | 2 |  |  |
| 4 | 0 |  |  |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |

## RI=3 and 4: K0

The following was agreed in RAN1#96bis [2]:

“On RI=3-4 extension, with the agreed total max # NZ coefficients across all layers ≤ 2K0 where the K0 value (hence β) set for RI{1,2}, the scheme for determining the # NZC per layer will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt0. KNZ,i is unrestricted as long as i=0RI-1KNZ,i≤2K0

- Alt1. KNZ,i≤K0 as long as i=0RI-1KNZ,i≤2K0 “

The views from different companies and available SLS results can be summarized below.

Table 3 RI=3-4 K0 setting: summary of companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt0. Unrestricted | 7 | CATT, Intel, LGE, OPPO, Qualcomm, Spreadtrum, vivo |
| Alt1. Per-layer upper bound | 13 | Ericsson, Fraunhofer/HHI, Huawei/HiSi, MotM/Lenovo, NEC, Nokia/NSB (without additional restrictions on KNZ,i), NTT Docomo, Samsung, ZTE (without defining ) |

Table 4 RI=3-4 K0 setting: summary of observation from SLS

| Company | Metric | Key observation |
| --- | --- | --- |
| Huawei/HiSi | UPT | Observation 4: The gain or benefit of Alt0 over Alt 1 is unclear yet. Alt 1 with fixed ratio/distribution of NZ coefficients across all ranks and all layers seems to have better performance and robustness of quantization from the perspective of UE implementation. |
| Qualcomm | UPT vs. overhead | Observation 2: Alt1 with K0 max NNZC per layer achieves 1~2% loss compared to Alt0 with no constraint on max NNZC per layer. |
| Samsung | UPT vs. overhead | For NZ coefficient selection, there is no clear gain with Alt0 (unrestricted), it can be worse than Alt1, and has additional UE complexity. |
| ZTE | UPT vs. overhead | - Alt 0 (do not support per-layer K0 restriction) and Alt 1-2 (support per-layer K0 restriction w/o β value for rank 3-4) performs quite similarly at least for Alt 3C, and Alt 1-1 (support per-layer K0 restriction by adjusting β) has a bit performance loss. - Complexity and simplicity of Alt 0 and Alt 1-2 is similar. |

As evident, Alt1 represents the majority view. The main argument for Alt1 (over Alt0 where no restriction on per-layer NNZC) is the lack of benefit for Alt0. That is, there is no perceivable performance gain yet it may come at the expense of UE complexity (since the main advantage of independent processing across layers can be compromised with Alt0). On the other hand, Qualcomm claims that both Alt0 and Alt1 require joint search across all layers because layer-independent processing with Alt1 cannot guarantee that the total NNZC is no larger than 2K0. This might add some additional complexity for Alt1. Qualcomm also conjectures that this issue could be somewhat related to SCI design (Alt3.3 vs. Alt3.4).

Observation: On the restriction on the number of non-zero coefficients (NNZC) for RI=3-4, Alt1 (per-layer upper bound) represents the majority view.

- With Alt1, the need for additional restriction (such as some additional formulation/restriction for β) is unclear

- Note: It has been agreed that the total number of non-zero coefficients across all the layers shall be less than or equal to 2K0 where K0=β×2LM (β is higher-layer configured)

Proposal: For RI=3-4, given the value of K0, the number of non-zero coefficients per layer shall be less than or equal to K0

## UCI design: Table 2 of R1-190562

The following was agreed in RAN1#96bis [2]: “In RAN1#96bis, continue to discuss the need for supporting the proposed UCI parameters in Table 2 of R1-1905629

- At least until RAN1#97 (Reno), additional proposals on UCI parameter can be made.”

In an offline discussion, it was pointed out that the following parameters can be removed from Table 2 of R1-1905629 for the following reason [4]:

- RI: discussed jointly with # NZC indicator

- N3’: discussed jointly with FD basis selection indicator

- Indication of zero pol reference amplitude values: n/a since there is no “zero” in reference amplitude alphabet

- Indication of non-zero coefficient polarization: discussed jointly with bitmap design for RI=3-4

- Indication of polarization with SCI: discussed jointly with bitmap design for RI=3-4

- FD oversampling (rotation) Q3: WA assumption is reverted, no need

- Indication of intermediate FD basis set: discussed jointly with FD basis selection indicator

- (N1’ N2’): no longer applicable since SD basis selection with log2N1N2L-bit indicator has been agreed

- The size of bitmap(s) Nb: discussed jointly with bitmap design for RI=3-4

On the blue highlighted items, note that this is only applicable for RI=3-4 since we have agreed that the size of the bitmap for RI=1-2 is 2LM per layer. This leaves us with the following two candidate parameters in Table 2 of R1-1905629:

- M’ (UCI part 1), and

- Basis sufficiency indicator (BSI, UCI part 1).

During offline discussion, the following offline observation, conclusion, and agreements were made (details can be found in [4]):

Offline observation: On the candidate UCI parameters listed in Table 2 of R1-1905629: - ​Indication of zero pol reference amplitude values: not applicable since there is no “zero” in reference amplitude alphabet - ​​FD oversampling (rotation) Q3: not applicable since WA assumption on O3=4 is reverted - (N1’ N2’): no longer applicable since SD basis selection with log2N1N2L-bit indicator has been agreed - There is no consensus on the need for “basis sufficiency indicator” (BSI) and M' reporting - On the benefit of M’, one company has shared SLS results (UPT v. overhead) and argued that more technical discussion is still needed Offline conclusion: On the candidate UCI parameters listed in Table 2 of R1-1905629, the following parameters are not supported as a consequence of the previous agreements in RAN1#96bis: - Indication of zero pol reference amplitude values, - FD oversampling (rotation) Q3, - (N1’ N2’) Offline agreement: On the candidate UCI parameters listed in Table 2 of R1-1905629, whether to support or not support the following parameters is to be decided in RAN1#97 (Reno): - M’, - Basis sufficiency indicator (BSI)

On the blue-highlighted offline agreement, the views from different companies and available SLS results can be summarized below.

Table 5 List of UCI parameters for further discussion (remaining parameters from Table 2 of R1-1905629)

| Parameter | Location | Details/description | Companies’ views |
| --- | --- | --- | --- |
| M’ | UCI part 1 | Whether to report M’ ≤ M, e.g. # bits, values | Support (6): Ericsson (joint encoding with NNZC), Fraunhofer IIS/HHI (joint encoding with NNZC), LGE (joint encoding with NNZC), NTT Docomo, vivo Against (7): Huawei/HiSi, NEC, Nokia/NSB, Qualcomm, ZTE |
| “Basis sufficiency” indication (BSI) | UCI part 1 | Whether to support this indication. Functionalities: - Indicating the sufficiency of the configured p and/or β values; - FFS: indicating UCI part 2 is absence or partially absence or fully present. Currently there are two alternatives: Alt1: implicit indication via # NZC=0 or # NZC>0; Alt2: 1-bit explicit indication | Support (5): Fraunhofer IIS/HHI, Qualcomm, NTT Docomo, LGE Against (9): CATT, Ericsson, MotM/Lenovo, NEC, Nokia/NSB, Samsung, ZTE |

Table 6 M’: summary of observation from SLS

| Company | Metric | Key observation |
| --- | --- | --- |
| vivo | UPT vs. overhead | - The overhead of parameter combination of (beta = 0.5, M = 4) is smaller than that of (beta = 0.25, M = 7) without applying restriction of NNZC<=2K0. - The performance of beta = 0.5 and M = 4 is slightly better than that of beta = 0.25 and M = 7. - M = 7 with 2K0 = 17 and M = 4 with 2K0 = 24 provides similar balance between overhead and performance for L = 2. - The performance of M = 7 with 2K0 = 9 is worse than that of M = 4 with 2K0 = 16 for L = 2. - The performance gap between M =4 and M = 7 becomes much larger, around 5%, with almost same overhead for L = 4. |

- On reporting M’, several main concerns from the companies against the proposal can be summarized as follows (see also [4]):

- The additional overhead in UCI part 1: The response from the proponents was two-fold. First, it was argued (e.g. by vivo) that the additional overhead in UCI part 1 is outweighed by the overhead reduction in UCI part 2. It should be kept in mind, however, that UCI part 1 requires lower coding rate since it is used to determine the payload of part 2 and includes higher priority UCI parameters. Second, some (not all) proponents propose that the hypotheses for M’ could be jointly encoded with other UCI parameters such as the number of NZ coefficients indicator (NNZCI). While this proposal could partially address the concern, it should be kept in mind that the number of NZ coefficients is not restricted to a certain set of values. Therefore, being able to utilized the unused code points from NNZCI could be opportunistic at best.

- Allowing the UE to determine its own M’ in place of the configured M: This concern is primarily on the performance impact since it is highly dependent on UE implementation. For example, it is mentioned by ZTE that if the UE deliberately selects a small value of M’ to reduce UCI payload with less regard on performance, it could impact the network performance.

Observation: On the candidate UCI parameters listed in Table 2 of R1-1905629:

- Three parameters are excluded from consideration due to the previous agreements in RAN1#96bis, and two parameters: indication of zero pol reference amplitude values, FD oversampling, and (N1’ N2’)

- Even after Tdoc submission, there is still no consensus to support two parameters: M’ and BSI

- The other parameters have to be decided along with other aspects of UCI design

Proposal: On the candidate UCI parameters listed in Table 2 of R1-1905629:

- [Offline conclusion] The following parameters are not supported as a consequence of the previous agreements in RAN1#96bis:

  - Indication of zero pol reference amplitude values,

  - FD oversampling (rotation) Q3,

  - (N1’ N2’)

- The following parameters are not supported due to lack of consensus:

  - M’

  - Basis sufficiency indicator (BSI)

## UCI design: Table 1 of R1-190562 FFS

The agreement in RAN1#96bis [2], along with the summary of companies’ views are given below.

# NZC indicator

The scheme for indicating the number of NZ coefficients (NZC) will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt1.1: RI + # NZC summed across layers where # NZC = {0, 1, 2, …, 2K0} (if sufficiency indicator is supported) or {1, 2, …, 2K0}

- Alt1.2: Per-layer # NZC without RI where # NZC = {0, 1, 2, …, K0}

- Alt1.3: RI + differential of # NZC summed across layers

  - Differential means fraction of 2K0 with smaller number of possible values compared to the regular # NZC (in Alt1.1)

- Alt1.4: RI + per-layer differential # NZC

  - Differential means fraction of K0 with smaller number of possible values compared to the regular # NZC (in Alt1.2)

Table 7 UCI, # NZC: summary of companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt1.1 RI+joint | 20 | Apple, CATT, Ericsson, Fraunhofer/HHI, Huawei/HiSi, Intel, LGE, MotM/Lenovo, NEC, Nokia/NSB, NTT Docomo, OPPO, Samsung, Spreadtrum, vivo, ZTE |
| Alt1.2 Per layer | 1 | Qualcomm |
| Alt1.3 RI + diff joint | 0 | -- |
| Alt1.4 Diff per layer | 0 | -- |

RI=3-4 bitmap

For RI=3-4, the bitmap design will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt2.1: 2LMi bits per layer, i=0,1,…,(RI-1)

- Alt2.2: One joint bitmap 1 for all layers, where an indicator bit is 1 if at least one of the RI layers has non-zero coefficient (UCI part 2) + Additional bitmap 2 (or, alternatively, a combinatorial indicator) indicating which layer(s) have either non-zero or zero coefficient(s) (UCI part 2) + Bitmap 2 (or, alternatively, a combinatorial indicator) size indicator (UCI part 1)

- Alt2.2B: Bitmaps 1 for each layer, where an indicator bit is 1 if at least one of the RI beams has non-zero coefficient (UCI part 2) + Additional bitmap 2 (or, alternatively, a combinatorial indicator) indicating which layer(s) have either non-zero or zero coefficient(s) (UCI part 2) + Bitmap 2 (or, alternatively, a combinatorial indicator) size indicator (UCI part 1)

- Alt2.3: LMi bits for the layer in which the weaker polarization is dropped (else 2LMi bits) + up to 4-bit bitmap to indicate the layer where the weaker polarization is dropped (UCI part 1); , i=0,1,…,(RI-1)

Table 8 UCI, bitmap for RI=3-4: summary of companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt2.1 2LMi | 11 | Apple, Ericsson, Intel, NEC, Nokia/NSB, NTT Docomo, Qualcomm, vivo, ZTE, Samsung |
| Alt2.2 Joint | 0 | -- |
| Alt2.2B Joint B | 5 | CATT, LGE, MotM/Lenovo, Spreadtrum |
| Alt2.3 Drop weak | 4 | Fraunhofer/HHI, Huawei/HiSi |

SCI for RI>1

For RI=1, strongest coefficient indicator (SCI) is a log2KNZ-bit indicator. For RI>1, SCI design will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt3.1 (applicable to Alt1.2): Per-layer SCI, where SCIi is a log2KNZ,i–bit indicator (i=0,1,…,(RI-1))

- Alt3.2 (applicable to Alt1.1): Per-layer SCI, where SCIi is a log2mini=0RI-1KNZ,i,2LMi–bit indicator

- Alt3.3: Per-layer SCI, where SCIi is a log22K0–bit or log2K0 indicator (i=0,1,…,(RI-1))

- Alt3.4: Per-layer SCI, where SCIi is a log22L–bit (i=0,1,…,(RI-1))

Table 9 UCI, SCI: summary of companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt3.1 per layer Knz | 1 | Qualcomm (1st preference) |
| Alt3.2 joint Knz | 2 | Spreadtrum, vivo |
| Alt3.3 per layer 2K0 or K0 | 10 | CATT (log2min2K0,2LMi or log2minK0,2LMi bits), Fraunhofer/HHI, Huawei/HiSi, LGE, NEC, Qualcomm (2nd preference), Samsung, ZTE (if # NZC per layer is restricted for RI=3-4) |
| Alt3.4 per layer 2L | 7 | Ericsson, Intel, MotM/Lenovo, Nokia/NSB, OPPO |

FD basis subset selection indicator

On FD basis subset selection indicator, the design will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt5.1: FD basis subset selection indicator is per layer where it is a log2N3Mi-bit indicator or log2N3-1Mi-1-bit indicator or size-N3 bitmap, (i=0,1,…,(RI-1))

- Alt5.2: Two-step FD basis subset selection where

  - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) and N3’ is either reported in UCI part 1 or fixed in specification or higher-layer configured, and the intermediate set in UCI part 2

    - Minitial indicated by log2N3 (or other values) bits indicates starting point of the intermediate FD basis set. The remaining FD basis in this intermediate set is given by mod(Minitial+n,N3), n=0,1,..,N3’-1

  - The 2nd step uses either N3’-bit bitmap or log2N3'Mi-bit indicator to indicate the FD basis used for each layer

- Alt5.3: Two-step FD basis subset selection where

  - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) selected from multiple higher-layer configured intermediate sets and the value of N3’ is indicated in UCI part 1

  - The 2nd step uses log2N3'Mi-bit indicator to indicate the FD basis used for each layer

- Alt5.4: FD basis subset is selected as mod(Mi_initial + n,N3), n=0,1,..,Mi–1

- Alt5.5: The intermediate FD basis subset of size N3' is higher layer configured per rank, and N3' is not reported in UCI part 1.

  - FFS: FD basis subset and size N3' per rank

  - The UE reports N3'-bit bitmap or log2N3'Mi or log2N3'Mi-1 bit indicator to indicate the FD basis used for each layer

- Alt5.6: Two-step FD basis subset selection where

  - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) and the value of N3’ is either fixed or higher-layer configured

    - The FD basis in this intermediate set is reported either by N3-bit bitmap or  log2N3N3' bit indicator

  - The 2nd step uses either N3’-bit bitmap or log2N3'Mi-bit indicator to indicate the FD basis used for each layer

- Alt5.7: Two-step FD basis subset selection where

  - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) and the value of N3’ is indicated in UCI part 1

    - The FD basis in this intermediate set is the union of FD basis for all layers, and is reported bylog2N3N3' bit indicator

  - The 2nd step uses either N3’-bit bitmap or log2N3'Mi-bit indicator to indicate the FD basis used for each layer

- Alt5.8:

  - For RI > 2, two-step FD basis subset selection

    - The 1st (intermediate) step uses an intermediate set of size-N3’ (N3’=N3/2)

      - Intermediate set is the union of FD basis for all layers, and is reported by size-N3 bitmap

    - The 2nd step uses size-N3’ bitmap to indicate the FD basis used for each layer

  - For RI < 3, FD basis subset selection indicator is per layer where it is a size-N3 bitmap

During the offline discussion, the following offline agreement was made to reformulate the problem (see [5] for details):

Offline agreement 1: The two-step FD basis subset selection is described as follows: - The 1st (intermediate) step uses an intermediate FD basis set of size-N3' (N3'≤ N3) - The value of N3' is RI- and layer-common - The intermediate FD basis subset is RI- and layer-common - The 2nd step uses an indicator to indicate the FD basis used for each layer - FFS (to be resolved in RAN1#97): for the indicator, select between a N3'-bit bitmap and X-bit combinatorial indicator where X is either log2N3'Mi or log2N3'-1Mi-1. - FFS (to be resolved in RAN1#97): select one of the following alternatives on N3' setting mechanism: - 1) reported in UCI part 1 - 2) higher-layer configured - 3) fixed - FFS (to be resolved in RAN1#97): select one of the following alternatives on size-N3' intermediate subset (IntS) - 1) IntS is adjacent and fully parameterized with Minitial, indicating that the intermediate set consists of FD bases modMinitial+n,N3, n=0,1,⋯N3'-1, - FFS (to be resolved in RAN1#97): whether Minitial is reported in UCI part 2, higher-layer configured, or fixed - 2) IntS is selected freely from N3 FD bases, a combinatorial indicator is reported in UCI part 2 - FFS (to be resolved in RAN1#97): exact bitwidth, either log2N3N3' or log2N3-1N3'-1 Offline agreement 2: In RAN1#97, decide on FD basis subset selection scheme from the following alternatives: - Free selection (Alt 5.1 in RAN1#96b) - Fixed selection (Alt 5.4 in RAN1#96b) - Two-step selection (the final outcome of proposal 1)

Table 10 UCI FD subset selection: summary of companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| One-step free selection | 4 | Nokia/NSB (R=1), Spreadtrum, ZTE |
| One-step fixed selection | 0 | -- |
| Two-step selection | 16 | CATT, Ericsson (at least for R=2), Fraunhofer/HHI, Huawei/HiSi, LGE, MotM/Lenovo, NEC, Nokia/NSB (R=2), OPPO, Qualcomm, Samsung, vivo |

Among the supporters of two-step FD basis subset selection, views on the design aspects are summarized as follows.

Table 11 UCI FD subset selection, two-step: summary of companies’ views

| Category | Scheme | Companies |
| --- | --- | --- |
| N3' setting mechanism | Reported in UCI part 1 (8) | CATT, Huawei/HiSi, MotM/Lenovo, NEC, Qualcomm, vivo |
|  | Higher-layer configured (2) | Fraunhofer/HHI |
|  | Fixed (4) | Ericsson, LGE, OPPO, Samsung |
| IntS design | Adjacent/contiguous (window-based), Minitial reported in UCI part 2 (6) | CATT, Ericsson, MotM/Lenovo, Samsung, vivo |
|  | Adjacent/contiguous (window-based), Minitial higher-layer configured (2) | Fraunhofer/HHI |
|  | Adjacent/contiguous (window-based), Minitial fixed (4) | Fraunhofer/HHI, NEC, Qualcomm |
|  | Free design, combinatorial log2N3N3' bits (5) | CATT, Huawei/HiSi, LGE, Samsung |
|  | Free design, combinatorial log2N3-1N3'-1 bits (2) | NEC, OPPO |
| 2nd step indicator | N3'-bit bitmap (1) | Qualcomm |
|  | Combinatorial log2N3'Mi bits (10) | CATT, Fraunhofer/HHI, Huawei/HiSi, LGE, NEC, Qualcomm, Samsung, vivo |
|  | Combinatorial log2N3'-1Mi-1 bits (3) | OPPO, MotM/Lenovo |

The available SLS results can be summarized below.

Table 12 UCI: summary of observation from SLS

| Company | Metric | Key observation |
| --- | --- | --- |
| Ericsson | UPT vs. overhead | - Similar performance for one-step and two-step FD-basis indication - For R=2, the overhead savings with the adjacent IntS selection is substantial, around 25 bits. Both one-step and arbitrary two-step FD-basis selection result in similar overhead. - For R=1, all three schemes have similar overhead |
| Fraunhofer/HHI | UPT vs. overhead | - Observation: Fixing Minit by Minit= N3-N3'2+ 1 is sufficient to select the FD basis vectors that are selected with the highest probability. - Observation: Configuring the IntS in Alt5.5 suffices to obtain similar performance to that of Alt5.1 and Alt5.6 with a lower feedback overhead. - Observation: For a large value N3', the freely selected IntS is contained in the configured IntS. |
| Huawei/HiSi | Overhead analysis, N3’ value distribution analysis | - Observation 2: For Alt 5.7, the intermediate FD basis set has larger probability with small value of N3’. Compared to Alt5.1, Alt 5.2 and Alt 5.8, Alt 5.7 has lower average overhead for small value of N3 and higher rank for small value of N3 and higher rank. |
| MotM/Lenovo | UPT vs. overhead | - Alt5.2’ and Alt5.7 have 5-6 and 1-2 bits less average overhead than Alt5.1. - Alt5.2 and Alt5.6 average UPT and cell edge performance is inferior to Alt5.1, 5.2’, and 5.7 by 0.1% to 0.2% and 0 to 0.5% in cell edge. - Alt5.4 has similar average user throughput performance but degraded cell edge performance compared to Alt5.1, between 0.6% and 1.3%, and 14-18 bits less average overhead. |
| Qualcomm | UPT vs. overhead | - Observation 4: Window-based approach achieves similar performance as the combinatorial-based approach. - Observation 5: Combinatorial-based intermediate set may need more overhead than one-stage FD basis report. |
| Samsung | UPT vs. overhead | - When N3 is small (e.g. R=1), - there is no noticeable difference in overhead saving between Alt5.1, 5.2, and 5.6 - Alt5.2 suffers from up to 1% performance loss - Alt5.6 shows some performance gain over Alt5.1 in Rel.15 Type II overhead regime - When N3 is large (e.g. R=2), there is ~12-24 bits overhead saving and slight performance gain with Alt5.2 - The FD index distribution is concentrated towards the two ends (the first few and the last few FD components) - The concentration of FD indices increases as N3 increases, i.e., the distribution is more concentrated for N3-26 than for N3=13 - For window-size N3'=8, the probability of X=N3-N3' FD indices in the middle being selected by the UE is ~10% and ~5% for N3=13 and N3=26, respectively. - For window-size N3'=8, the probability that the X=N3-N3' FD components outside of the window has more power than the X FD components inside the window is ~15-20% and ~5-8% for N3=13, and N3=26, respectively. - Observation 1: - Alt3.4 can be worse in overhead than Alt3.3 if N_SB is small - The overhead saving with Alt3.4 is small (0-2% of the total payload) only if N_SB is large - Observation 2: There is no noticeable difference in performance-overhead tradeoffs achieved by Alt3.3 and Alt3.4 |
| vivo | UPT vs. overhead | - Observation 1: For M = 4, - When 2K0 =32, alt 1 with intermediate set length equal to 6 and 7 achieves same performance with 7 bits less overhead in contrast to alt 2 with same intermediate length. - When 2K0 =48, alt 1 has a similar performance with alt 2. - Both of alt 1 and alt 2 can achieve similar performance, and degradation of less than 1% compared to alt 0. - Observation 2: For M = 7, - Almost same performance is obtained among three FD basis selection schemes in both of 2K0 = 28 and 2K0 = 56. - Alt 1 reduces almost half of w3 indication overhead compared with alt 0. - Observation 3: For R = 2, - W3 indication overhead of alt0 is close to 60 bits and 80 bits for selecting 4 and 7 FD basis per layer respectively. - Alt 1 reduces almost half of w3 indication overhead compared with alt 0. - Only for M = 4 and 2K0 = 32, alt 1 shows better balance of performance and overhead than alt 2. - Alt 1 and alt 2 provides similar performance as well as overhead. - Both alt 1 and alt 2 can achieve similar performance of alt 0 by increasing length of intermediate set. |

On the bitmap design for RI=3-4, Huawei/HiSi argued that if Alt2.1 (the majority view) is adopted, some additional restriction in the specification should be added to avoid odd reporting behavior, e.g. each bitmap shall indicate at least one non-zero coefficient per polarization. In addition, NTT Docomo, while preferring Alt2.1, suggests that some information-theoretically optimal “smart” coding should be used to compress the raw bitmaps and discussed in RAN1#97. This, however, goes against the agreement in RAN1#96bis (that the RI=3-4 bitmap design should be chosen from the four identified alternatives).

Among all the UCI parameter design issues, FD basis subset selection is arguably the most complex issue. While two-step selection represents the super-majority view, there are three design aspects that have to be finalized.

Among the three aspects, the most pivotal one would be the intermediate subset (IntS) design where two subset design alternatives have been proposed: window-based (parametrized by a starting index Minitial and subset size N3') and free selection (combinatorial, selecting N3' out of N3). Here several companies show that for larger N3 values, the window-based design performs the best while for smaller N3 values, the free-selection-based design performs the best (the difference in performance is within at most ~1% in average UPT for a given overhead – which may be significant (for some companies) considering that the gain of Rel.16 Type II with DFT-based compression is thus far ~6-7% over Rel.15 Type II). This observation is expected since the window-based design incurs some UPT loss over the free-selection-based design. But for larger N3 values, the overhead saving due to the reduced IntS indicator overhead from the window-based design may offset its UPT loss. Given that the window-based design with fixed/reported Minitial and the free-selection-based design with log2N3'Mi–bit indicator receive more or less comparable majority support, it seems fitting to support the free-selection-based design for smaller N3 values and window-based design for larger N3 values. This could also be a good compromise between the two designs, i.e. taking the best from both proposals.

However, it should also be noted that the performance difference between one-step and two-step schemes is within at most ~1% average UPT for the same overhead, For any value of N3, the performance of one-step free selection tends to be between free-selection-based and window-based two-step selection design. That is, if <1% difference in average UPT is not perceived to be significant, the one-step free selection scheme performs almost as well as the best of the two-step schemes (for different values of N3) – moreover with a simpler single-scheme solution.

Related to this issue, Qualcomm conjectures that there may not be UPT difference among one-stage and two-stage schemes if N3' is reported for the two-stage scheme (regardless of the IntS design). Yet the payload of the window-based IntS would be the lowest of all. If N3' is configured or fixed, window-based IntS may be inferior to one-stage or two-stage with freely selected IntS. But Qualcomm conjectures that this loss could be compensated with fixing Minitial or choosing a large enough N3' (note: increasing N3' would obviously reduce the overhead saving benefit of the two-step scheme and alter the baseline). Overall, Qualcomm opines that there is no need for supporting two schemes for FD basis subset selection. Another aspect is the setting of N3' (the intermediate subset IntS size). Among the three alternatives (reporting in UCI part 1, higher-layer configuration, and fixed relation with N3), the first and the third alternatives are more popular than the second. However, the advantage of one scheme over the other two has not been clearly articulated in the submitted Tdocs. For instance, reporting may offer N3' a bit more flexibility to the UE to select the size of IntS but it incurs significant increase in the payload of UCI part 1. Regarding N3' reporting, a few companies also raised some concern on the performance benefit. Since it is difficult to guarantee (either via RAN4 testing or specificatio) whether the UE will select a good N3', the scheme would provide no overhead reduction gain (if the reported N3' is too large) or cause performance/UPT loss (if the reported N3' is too small). Given the trade-off, it seems that having from a small number of N3' values (e.g. two) can provide some degree of flexibility without the drawback raised on the reporting of N3'.

For the two-step approach, some companies (e.g. Ericsson, Fraunhofer) argue that some aspects could be tied with SCI design when the cyclic shift (CS) principle akin to the previously agreed O3 issue is used to reduce overhead. For example, Alt3.4 for SCI (RI>1) and any combinatorial indicator for FD basis subset selection with the “minus one” alternative (log2N3-1Mi-1 for one-step, or log2N3-1N3'-1 and log2N3'-1Mi-1 for two-step) requires the UE to apply CS operation to align the strongest coefficient with FD component 0 – which also implies that fixing Minitial to zero (by relying on the UE applying CS to shift the starting point of window-based IntS to zero) may no longer be applicable. Conversely, if Minitial is fixed to zero by the (possibly transparent) CS operation, Alt3.4 may no longer be applicable for the SCI. Therefore, whether/how to use the CS operation for reducing the payload for various indicators may need to be discussed jointly.

Closely related to this issue is the benefit the use of this CS operation for SCI and/or FD subset selection. While the CS operation can reduce the payload and the same mathematical principle as that for the previously agreed O3 issue (in RAN1#96bis) applies, several companies argue that the “risk-return” factor can differ starkly. As an example, for O3, several companies have shown that even if the UE does not apply the (transparent) CS operation (e.g. to effect a different O3 from 1 for maximizing performance), the performance impact is marginal. Therefore, the return (allowing the UE to select its own O3 if a "smart" algorithm is used) outweighs the risk (even if the UE doesn't use this algorithm, the performance loss of O3=1 is small relative to O3=4). However, whether the same could be said to the SCI and/or FD indicator issues is unclear and demands further assessment. For instance, for the SCI issue, if the CS operation is made transparent and a UE does not implement the CS operation, the impact on performance could be significant as several companies had demonstrated during the quantization discussion (cf. Alt4 on FD=0 component). In this case, some additional “cost” in terms of spec impact (e.g. specifying some UCI mapping operation) may be needed. How this is done relative to the strongest coefficient may need more discussion. Then the question is how much overhead reduction can be attained and whether it is worth the cost.

Regarding the use of the CS operation, Nokia/NSB argues that no clear proposal on specification support for FD basis windowing has been made, e.g..: 1) the applied CS; 2) UCI mapping for the LCC. This may be needed to avoid large performance degradation due to unspecified UE behavior. This is unlike Alt3.4 where multiple proponents of Alt3.4 have proposed example text proposal(s) for UCI mapping rule the LCC (no bit-level interleaving required). Note that other SCI schemes also require some UCI mapping rule. Nokia/NSB further argued that Alt3.4 is superior to Alt3.3 in terms of overhead saving for: 1) number of configurations where Alt3.4 has lower overhead than Alt3.3 (especially for, but not limited to, the number of subbands ≥ 13); 2) maximum number of bits saved and; 3) practical relevance of the configurations where Alt3.4 saves over Alt3.3.

In regard of joint consideration on SCI and FD basis subset selection, Fraunhofer also argues that Alt3.3 + window-based IntS with Minitial=0 (relying on the UE to apply CS operation to “center” the FD coefficient profile) is superior to Alt3.4 + window-based IntS with reported Minitial (“We found that for RI = 4, beta = ¾, M = 4, N3 = 13, L = 4, setup 2 requires 5 bits more than setup 1. For RI = 4, beta = ½, M =4, N3 = 13, L = 4, setup 2 requires 9 bits more than setup 1.”).

Observation: For the agreed UCI parameters in Table 1 of R1-1905629:

- On NNZCI, Alt1.1 (RI + total NNZC across layers) represents the super-majority view.

- On bitmap for RI=3-4, Alt2.1 (2LMi bits for layer i) represents the majority view

- On SCI for RI>1, Alt3.3 (per layer 2K0 or K0 hypotheses, or possibly 2LMi if 2LMi<K0) and Alt3.4 (per layer 2L hypotheses) are almost equally supported and represent the stronger majority views

  - For UCI part 2, Alt3.4 offers some overhead reduction over Alt3.3 when number of subbands ≥ 13, yet incurs some overhead increase over Alt3.3 for a few cases when number of subbands < 13.

  - Alt3.4 may require a UCI mapping rule that applies cyclic shift with the strongest coefficient as a reference. It was also argued, e.g. by Nokia/NSB, that Alt3.3 may require a UCI mapping rule that specifies the order in which the NZC are counted to calculate the SCI. Otherwise the performance of either scheme would depend on UE implementation.

- On FD basis subset selection scheme, the two-step selection represents the super-majority view

  - The setting mechanism for N3' needs further discussion: select one from reporting in UCI part 1, higher-layer configuration, and fixed relation (with N3)

  - The window-based design with fixed/reported Minitial and the free-selection-based design with log2N3'Mi–bit indicator receive more or less comparable support. The window-based design tends to perform slightly better than free-selection-based design for larger N3 values while the opposite happens for smaller N3 values (see, e.g. [34][39][43]).

    - For a given overhead, difference in average UPT between one-step and two-step schemes (with window-based and free-selection-based designs) is within ~1% with one-step performing within <1% compared with the best of the two-step schemes for each scenario.

  - For the 2nd step indicator, reporting a log2N3'Mi-bit combinatorial indicator in UCI part 2 represents the super-majority view

Proposal: For further details on the agreed UCI parameters in Table 1 of R1-1905629:

- RI (∈1,…,RIMAX) and KNZ,TOT (the total number of non-zero coefficients summed across all the layers, where KNZ,TOT∈1,2,…,2K0 are reported in UCI part 1

- For RI=3-4, RI bitmaps, each with size-2LMi (i=0,1,…,RI-1, where i denotes the i-th layer) are reported in UCI part 2

  - FFS: Specification impact on how to prevent reporting all zero LC coefficients for a polarization

- On SCI for RI>1 (reported in UCI part 2), discuss further and decide between the two following alternatives in RAN1#97 (Reno):

  - Alt3.3: Per-layer SCI, where SCIi is a log22K0–bit or log2K0 indicator (i=0,1,…,(RI-1)), including further reducing the bitwidth if applicable (to log2min2K0,2LMi or log2minK0,2LMi)

  - Alt3.4: Per-layer SCI, where SCIi is a log22L–bit (i=0,1,…,(RI-1))

- The following FD basis subset selection scheme is supported:

  - For N3≤19, one-step free selection (cf. Alt5.1 in RAN1#96bis) is used

    - FFS (to be finalized in RAN1#97 Reno): The subset selection is indicated by an X1-bit combinatorial indicator (for each layer) in UCI part 2 where, depending on the outcome of SCI, X1 is either log2N3Mi (if Alt3.3 is selected) or log2N3-1Mi-1 (if Alt3.4 is selected)

  - For N3>19, IntS is window-based and fully parameterized with Minitial, indicating that the intermediate set consists of FD bases modMinitial+n,N3, n=0,1,⋯N3'-1

    - The value N3'=αM where α is higher-layer configured from two possible values

      - FFS (to be finalized in RAN1#97 Reno): the two values of α

      - FFS (to be finalized in RAN1#98 Prague): the supported parameter combinations for L,p,β,α

    - FFS (to be finalized in RAN1#97 Reno): whether Minitial is reported in UCI part 2 (along with its possible values and bitwidth) or fixed

  - The 2nd step subset selection is indicated by an X2-bit combinatorial indicator (for each layer) in UCI part 2

    - FFS (to be finalized in RAN1#97 Reno): depending on the outcome of SCI, X2 is either log2N3'Mi (if Alt3.3 is selected) or log2N3'-1Mi-1 (if Alt3.4 is selected)

## N3 values

The following was agreed in RAN1#96bis [2]: “On the value of N3 for N3=NSB×R>13:

- For Alt1:

  - Identify alternatives for padding schemes in RAN1#97 (Reno)

  - Select one from the alternatives for padding scheme by RAN1#98 (Prague)

- For Alt2:

  - Identify alternatives in RAN1#97 (Reno)

  - Select one from the alternatives by RAN1#98 (Prague)”

The views from different companies and available SLS results can be summarized below.

Table 13 N3: summary of companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| Alt0 N3=NSB×R | 9 | CATT, Ericsson, Huawei/HiSi, NEC, Nokia/NSB, Qualcomm, Samsung |
| Alt1 (padding) | 4 | Intel, LGE, MediaTek, NTT Docomo |
| Alt2 (segmentation) | 2 | MediaTek, ZTE |
| Alt0 for 13<N3=NSB×R≤19, Alt1 for N3=NSB×R>19 | 1 | Ericsson |

Table 14 Alt1 N3 candidates: summary of companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| Extrapolation (numerous variations) | 4 | Intel, Mediatek, NTT Docomo, Qualcomm |
| Zero padding | 1 | LGE |

Table 15 Alt2 N3 candidates: summary of companies’ views

| Category | No. companies | Companies |
| --- | --- | --- |
| S1: 1, …, Y; S2: NSB×R-Y+1, …, NSB×R | 1 | ZTE |
| S1: 1, …, N3; S2: Ns-N3+1, …, Ns | 1 | MediaTek |

Table 16 N3: summary of observation from SLS

| Company | Metric | Key observation |
| --- | --- | --- |
| Ericsson | UPT vs. overhead | - DFT padding results in around 2% loss compared to No DFT padding - Segmentation approach has inferior performance/overhead tradeoff compared to approaches without segmentation |
| Huawei/HiSi | UPT vs. overhead | - Observation 2: Although the difference among padding schemes is not significant, the variance of performance gain can be up to 2%. And segmentation shows performance loss. |
| Intel | UPT vs. overhead | - Type II CSI DFT-based compression with padding of FD coefficients provides similar performance comparing to the case without padding (with DFT size equal to the number of FD compression units) - Type II CSI DFT-based compression with segmentation does not provide performance gains over other cases while it has slightly higher overhead |
| LGE | UPT vs. overhead | - Observation 1. Compared to reference scheme which always assume N3=NSB, marginal performance loss (up to 3% and 1% for 10MHz and 20MHz, respectively) in terms of average UPT is observed. - Observation 2. Among the padding schemes, Scheme 1 (zero-padding at the last FD unit) provides slightly better performance. |
| Samsung | UPT vs. overhead | - Alt0 (N3=NSB×R) achieves the best performance-overhead trade-off - Alt1 (N3 is a multiple of 2, 3, or 5) incurs large performance loss due to possible misalignment/mismatch between an assumed precoder by the gNB and an actual precoder used by the UE while calculating CQI - Alt2 (two segments) results in high overhead, and performs worse than Alt0 in terms of performance-overhead trade-off |
| ZTE | UPT vs. overhead | - Observation 2: Segmentation can provide performance gain and better Performance-Overhead trade-off over padding. The performance gain of segmentation over padding can be more than 5%. |

Observation: On the value of N3 for N3=NSB×R>13:

- Most companies are still proposing Alt0 N3=NSB×R mainly due to some concern on performance loss from Alt1/Alt2

- For Alt1, extrapolation represents the majority view although there are numerous variations mentioned in, e.g. MediaTek Tdoc.

- For Alt2, two specific schemes (where the segmentations overlap between the two) were proposed, each supported by a single company

Proposal: On the value of N3 for N3=NSB×R>13:

- For Alt1 (padding), consider only extrapolation-based scheme and decide on the final specific design alternatives in RAN1#97 (Reno) for down selection in RAN1#98 (Prague)

- For Alt2 (two segments), the following alternatives will be considered for down selection in RAN1#98 (Prague):

  - Alt2.1: S1: 1, …, Y;     S2: NSB×R-Y+1, …, NSB×R

  - Alt2.2: S1: 1, …, N3;  S2: Ns-N3+1, …, Ns

## Miscellaneous

Miscellaneous (other) issues were also mentioned in companies’ contributions such as:

- Other detailed UCI design issues: UCI omission for DFT-based compression

- CBSR

- Reducing the number of supported parameter combinations (too premature to discuss before RI=3-4 codebook and parameter setting are finalized)

- Other (non-DFT-based) compression schemes

- UE capability issues

- Adjustment of layer-/layer-group-specific quantization level to RI=3-4

Since this meeting will focus on the issues listed in section 1, a summary of such miscellaneous issues is not included in this FL summary.

Updated timeline and work plan

The timeline (with a set of milestones for each RAN1 meeting is updated with more details. The updated parts are highlighted in blue.

Table 17 Proposed timeline along with the milestones

| 97 (05/19) | 98 (08/19) – early projection |
| --- | --- |
| - For RI=3 and 4, finalize the following: - Choose parameter (L, p) setting Alt2B vs 3C vs 6E – including configurability vs. fixed relation - K0/ setting, select from two alternatives - Refine the previous agreement on UCI design details: - Resolve 5 FFS items from “Table 1” (agreed UCI parameters) - Decide support for parameters in “Table 2” (proposed) - Identify alternatives for Alt1 and Alt2 of N3 for # units>13 | - For RI=3 and 4, finalize the remaining details, if any - Finalize remaining details on the following for UCI details - Content of part 1 - Content of part 2 - Select one scheme for Alt1 and one scheme for Alt2 of N3 for # units>13 - Identify other UCI-related issues and potential solutions (e.g. UCI omission) - Identify alternatives for CBSR. - Identify alternatives for reducing supported combinations of compression parameters |

| 98B (10/19) – early projection | 99 (11/19) – early projection |
| --- | --- |
| - Identify other UCI-related issues and potential solutions (e.g. UCI omission) - Finalize CBSR. - Finalize reducing supported combinations of compression parameters. - Select one scheme for Alt1 and one scheme for Alt2 of N3 for # units>13 - Discuss and (if possible) conclude on the support for “other schemes” - Discuss and identify UE capabilities | - (Pre-)maintenance on Type II overhead reduction, i.e. finalize remaining issues on previous agreements - Finalize UE capabilities |

# References

- RP-181425 Enhancements on MIMO for NR

- 3GPP RAN1, RAN1#96bis, Chairman Notes

- R1-1905629, Samsung, Feature lead summary for MU-MIMO CSI – revision on selected issues

- R1-1906966, Samsung, Feature lead summary on offline email discussion for MU-MIMO CSI: UCI parameters

- R1-1906967, Samsung, Feature lead summary on offline email discussion for MU-MIMO CSI: FS basis subset selection

- R1-1906028, Huawei, HiSilicon, Discussion on CSI enhancement

- R1-1906032, Huawei, HiSilicon, Discussion on UE complexity of DFT-based compression codebook

- R1-1906033, Huawei, HiSilicon, Codebook subset restriction for DFT-based compression codebook

- R1-1906034, Huawei, HiSilicon, Parameter combination reduction for DFT-based compression codebook

- R1-1906602, Huawei, HiSilicon, Evaluation results of DFT-based compression codebook for rank 1 and 2

- R1-1906603, Huawei, HiSilicon, Evaluation results of DFT-based compression codebook for rank 3 and 4

- R1-1907526, Huawei, HiSilicon, UCI design for DFT-based compression codebook

- R1-1906158, vivo, Discussion on remaining issues of MU CSI enhancements

- R1-1906162, vivo, Evaluation of L,p parameters setting for rank>2

- R1-1906163, vivo, Evaluation of FD basis selection scheme

- R1-1906165, vivo, Evaluation of CSI omission scheme

- R1-1906223, NTT DOCOMO, Type II CSI Enhancement for MU-MIMO Support

- R1-1906235, ZTE, CSI enhancements for MU-MIMO support

- R1-1906240, ZTE, Evaluation results on alternatives for higher rank Type II compression CSI

- R1-1906286, OPPO, Enhancements on overhead reduction and rank extension for type II CSI feedback

- R1-1906344, CATT, Discussions on Type II CSI enhancement

- R1-1906348, CATT, Phase precoding for Type II CSI

- R1-1906360, Spreadtrum Communications, Discussion on Type II CSI enhancement for MU-MIMO

- R1-1906397, NEC, Discussion on CSI Reporting

- R1-1906535, MediaTek Inc., CSI enhancement for MU-MIMO Support

- R1-1906729, LG Electronics, Discussions on overhead reduction for Type II codebook

- R1-1906733, LG Electronics, Evaluation results on N3 value for Rel-16 Type II CSI

- R1-1906734, LG Electronics, Evaluation results on (L,p) setting for Rel-16 Type II CSI

- R1-1906735, LG Electronics, Evaluation results on overhead reduction with quantization for rank 3 and 4

- R1-1906814, Intel Corporation, On CSI Enhancements for MU-MIMO

- R1-1907708, Samsung, CSI enhancement for MU-MIMO

- R1-1906972, Samsung, SLS evaluation on MU-MIMO CSI: N3 value

- R1-1906973, Samsung, SLS evaluation on MU-MIMO CSI: rank 3-4 parameter setting and subset selection

- R1-1907709, Samsung, SLS evaluation on MU-MIMO CSI: FD basis subset indicator

- R1-1906975, Samsung, On UCI design for DFT-based compression

- R1-1906976, Samsung, On CBSR for DFT-based compression

- R1-1907710, Samsung, On strongest coefficient indicator

- R1-1907053, Fraunhofer IIS, Fraunhofer HHI, Enhancements on Type-II CSI reporting

- R1-1907074, Ericsson, On CSI Enhancements for MU-MIMO

- R1-1907075, Ericsson, On CBSR for Type II CSI enhancements

- R1-1907076, Ericsson, On CSI omission procedure

- R1-1907077, Ericsson, Evaluation results on the value of N3 for Type II CSI enhancements

- R1-1907078, Ericsson, Evaluation results for different parameter settings for Type II CSI enhancements

- R1-1907245, Motorola Mobility, Lenovo, Type II MU CSI Enhancements

- R1-1907288, Qualcomm Inc., CSI enhancement for MU-MIMO support

- R1-1907315, Nokia, Nokia Shanghai Bell, MU-CSI Rank extension parameter setting and UCI design

- R1-1907319, Nokia, Nokia Shanghai Bell, On UCI reporting of SCI and FD basis

- R1-1907719, Nokia, Nokia Shanghai Bell, On the proposals for Rel-16 codebook strongest coefficient indicator

- R1-1907341, Apple Inc., Consideration on CSI enhancement for MU-MIMO support