3GPP TSG RAN WG1 meeting #97	R1-1906972

Reno, USA, 13th – 17th May, 2019

Agenda item:	7.2.8.5

Source: 	Samsung

Title: 	SLS evaluation on MU-MIMO CSI: N3 value

Document for:	Discussion and Decision

# Introduction

In RAN1 AH 1901, the following agreement was made about the DFT-based FD compression parameter N3.

Agreement Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R≤19, Values of N3: For R∈1,2 and NSB is # CQI subbands, when NSB×R>19, downselect among the following alternatives in RAN1#96 - Alt1: N3N3 is smallest multiple of 2, 3, or 5 which is ≥NSB×R - Alt2: N3N3 is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

The following agreement was made in RAN1#96.

Agreement On the values of N3, further discuss and clarify/refine both of the available alternatives with as the evaluation baseline

This contribution provides simulation results to compare the performance of the two alternatives for N3, and makes conclusion in support of our proposal in [3]. This contribution is a revision of [4].

# Simulation results for N3

For performance evaluation, the non-full-buffer system-level evaluation is carried out for Dense Urban (Macro only) channel model in medium (50% target RU) traffic loading scenario, and dynamic switching between SU-MIMO and MU-MIMO is considered in the simulation. The results are provided for 16 antenna ports at the gNB. The relevant simulation assumptions and parameters are according to the agreed assumptions in RAN1#94bis, and are enlisted in Table 1 in Appendix. As reference, Rel. 15 Type II with L = 2, WB+SB amplitude, and 8-PSK phase is considered.

We compare the two alternatives (Alt1 and Alt2 in Section 1) for N3. For comparison, (Alt0) N3=NSB×R is also considered in this evaluation. The results are provided in Figure 1 (NSB=21, R=1) and Figure 2 (NSB=11, R=2) for the following parameters and assumptions.

- Spatial compression: L = 4

- Frequency compression: M = 6

- Rank 1 only

- N3=24 for Alt1 and Alt2

- Padding in Alt1 is random between the following three padding options.

  - Option 0: N3-NSB×R FD units are padded before the NSB×R FD units corresponding to the CQI SBs

  - Option 1: N3-NSB×R FD units are padded after the NSB×R FD units corresponding to the CQI SBs

  - Option 2: N3-NSB×R/2 FD units are padded before the NSB×R FD units corresponding to the CQI SBs, and the remaining N3-NSB×R-N3-NSB×R/2 FD units are padded after.

- For Alt2, N3/2 FD units are allocated to the first segment, and the remaining N3-N3/2 FD units are allocated to the second segment. In order to align the number of FD units in the CQI SBs and the total number of FD units in two segments, some of FD units in the two segments can overlap.

We can observe the following.

Observation: When NSB×R>13,

- Alt0 (N3=NSB×R) achieves the best performance-overhead trade-off

- Alt1 (N3 is a multiple of 2, 3, or 5) incurs large performance loss due to possible misalignment/mismatch between an assumed precoder by the gNB and an actual precoder used by the UE while calculating CQI

- Alt2 (two segments) results in high overhead, and performs worse than Alt0 in terms of performance-overhead trade-off

Figure 1: Performance-overhead trade-off for NSB=21 and R=1

Figure 2: Performance-overhead trade-off for NSB=11 and R=2

# Conclusions

In this contribution, simulation results are provided to compare the performance of the two alternatives for N3. The observations made are summarized as follows.

Observation: When NSB×R>13

- Alt0 (N3=NSB×R) achieves the best performance-overhead trade-off

- Alt1 (N3 is a multiple of 2, 3, or 5) incurs large performance loss due to possible misalignment/mismatch between an assumed precoder by the gNB and an actual precoder used by the UE while calculating CQI

- Alt2 (two segments) results in high overhead, and performs worse than Alt0 in terms of performance-overhead trade-off

# References

- Chairman’s notes, RAN1 AH 1901

- Chairman’s notes, RAN1#96

- R1-1906964, “CSI enhancement for MU-MIMO,” Samsung

- R1-1904454, “SLS evaluation on MU-MIMO CSI: N3 value” Samsung

# Appendix

Table 1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1, 4GHz, 20 MHz BW |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS SB size = 4 and #SBs = 13 |
| Simulation bandwidth |  | 10 MHz,15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation |
| MIMO layers |  | Up to 4 MU layers |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 50% for 10 MHz BW, 30% for 100 MHz BW |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |