3GPP TSG RAN WG1 meeting #97	 R1-1906974

Reno, USA, 13th – 17th May, 2019

Agenda item:	7.2.8.5

Source: 	Samsung

Title: 	SLS evaluation on MU-MIMO CSI: FD basis subset indicator

Document for:	Discussion and Decision

# Introduction

The following agreement was made in RAN1#96bis [1].

Agreement On FD basis subset selection indicator, the design will be chosen from the following alternatives in RAN1#97 (Reno): - Alt5.1: FD basis subset selection indicator is per layer where it is a log2N3Mi-bit indicator or log2N3-1Mi-1-bit indicator or size-N3 bitmap, (i=0, 1, …, (RI-1)) - Alt5.2: Two-step FD basis subset selection where - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) and N3’ is either reported in UCI part 1 or fixed in specification or higher-layer configured, and the intermediate set in UCI part 2 - Minitial indicated by log2N3 (or other values) bits indicates starting point of the intermediate FD basis set. The FD basis in this intermediate set is given by mod(Minitial+n,N3), n=0,1,..,N3’-1 - The 2nd step uses either N3’-bit bitmap or log2N3'Mi-bit indicator to indicate the FD basis used for each layer - Alt5.3: Two-step FD basis subset selection where - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) selected from multiple higher-layer configured intermediate sets and the value of N3’ is indicated in UCI part 1 - The 2nd step uses log2N3'Mi-bit indicator to indicate the FD basis used for each layer - Alt5.4: FD basis subset is selected as mod(Mi_initial + n,N3), n=0,1,..,Mi–1 - The subset selection is done per layer - Alt5.5: The intermediate FD basis subset of size is higher layer configured per rank, and is not reported in UCI part 1. - FFS: FD basis subset of size N3' per rank - The UE reports N3'-bit bitmap or log2N3'Mi or log2N3'Mi-1 bit indicator to indicate the FD basis used for each layer - Alt5.6: Two-step FD basis subset selection where - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) and the value of N3’ is either fixed or higher-layer configured - The FD basis in this intermediate set is reported either by N3-bit bitmap or log2N3N3' bit indicator - The 2nd step uses either N3’-bit bitmap or log2N3'Mi-bit indicator to indicate the FD basis used for each layer - Alt5.7: Two-step FD basis subset selection where - The 1st (intermediate) step uses an intermediate FD basis set of size-N3’ (N3’≤ N3) and the value of N3’ is indicated in UCI part 1 - The FD basis in this intermediate set is the union of FD basis for all layers, and is reported bylog2N3N3' bit indicator - The 2nd step uses either N3’-bit bitmap or log2N3'Mi-bit indicator to indicate the FD basis used for each layer - Alt5.8: - For RI > 2, two-step FD basis subset selection - The 1st (intermediate) step uses an intermediate set of size-N3’ (N3’=N3/2) - Intermediate set is the union of FD basis for all layers, and is reported by size-N3 bitmap - The 2nd step uses size-N3’ bitmap to indicate the FD basis used for each layer - For RI < 3, FD basis subset selection indicator is per layer where it is a size- N3 bitmap

This contribution provides simulation results for FD basis subset selection according to some of the alternatives (Alt5.1, 5.2, and 5.6) in support of proposals made in the companion contribution [2].

# Simulation results for FD basis subset selection

For performance evaluation, the non-full-buffer system-level evaluation is carried out for Dense Urban (Macro only) channel model in low (20% target RU) traffic loading scenario, and SU-MIMO is considered in the simulation. The results are provided for 16 antenna ports at the gNB. The relevant simulation assumptions and parameters are according to the agreed assumptions in RAN1#94bis, and are enlisted in Table 1 in Appendix. As reference scheme, Rel. 15 Type II for rank 1-2 and Rel. 15 Type I for rank 3-4 is considered in this evaluation. For comparison, simple extension of Rel. 15 Type II codebook up to rank 4 is also considered.

The performance-overhead trade-offs of the three FD basis subset selection alternatives (Alt5.1, 5.2, and 5.6) are compared. The results are provided in Figure 1 through Figure 6 for the following parameters.

- Spatial compression: L = 4

- Frequency compression: M = 7 (i.e., p=12)

- β∈{14,12,34}

- N3=13 (R=1) and N3=26 (R=2)

We can observe the following.

Observation 1: For FD basis subset selection,

- When N3 is small (e.g. R=1),

  - there is no noticeable difference in overhead saving between Alt5.1, 5.2, and 5.6

  - Alt5.2 suffers from up to 1% performance loss

  - Alt5.6 shows some performance gain over Alt5.1 in Rel.15 Type II overhead regime

- When N3 is large (e.g. R=2), there is ~12-24 bits overhead saving and slight performance gain with Alt5.2

Figure 1: Performance-overhead trade-off for FD basis subset selection

Figure 2: Performance-overhead trade-off for FD basis subset selection

Figure 3: Performance-overhead trade-off for FD basis subset selection

Figure 4: Performance-overhead trade-off for FD basis subset selection

Figure 5: Performance-overhead trade-off for FD basis subset selection

Figure 6: Performance-overhead trade-off for FD basis subset selection

# Conclusions

In this contribution, simulation results are provided for three FD basis subset selection alternatives (Alt5.1, 5.2, and 5.6). The observations made are summarized as follows.

- For FD basis subset selection,

  - When N3 is small (e.g. R=1),

    - there is no noticeable difference in overhead saving between Alt5.1, 5.2, and 5.6

    - Alt5.2 suffers from up to 1% performance loss

    - Alt5.6 shows some performance gain over Alt5.1 in Rel.15 Type II overhead regime

  - When N3 is large (e.g. R=2), there is ~12-24 bits overhead saving and slight performance gain with Alt5.2

# References

- Chairman’s notes, RAN1#96bis

- R1-1906964, “CSI enhancement for MU-MIMO,” Samsung

# Appendix

Table 1: Simulation assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1, 4GHz with 13 SBs, 10 MHz BW |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS SB size = 4 and #SBs = 13 |
| Simulation bandwidth |  | 10 MHz,15kHz SCS |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU |
| MIMO layers |  | Up to 4 layers |
| CSI feedback |  | Feedback assumption - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | DMRS, CSI-RS, PDCCH |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 20% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput vs CSI feedback overhead (bits) |
| Baseline for performance evaluation |  | Rel-15 Type II Codebook |