3GPP TSG RAN WG1 RAN1 Meeting #97	R1-1906983

Reno, US, 13th – 17th May, 2019

Agenda Item:		7.2.9.3

Source:		Samsung

Title:		Evaluation results for power saving schemes

Document for:		Discussion and decision

# Introduction

In RP meeting#83, WID for UE power saving is agreed with below WI objectives [1]:

- Specify power saving techniques with UE adaptation with focus in RRC_CONNECTED mode [RAN1, RAN4]

  - Specify the power saving techniques with power saving signal/channel

    - Specify the PDCCH-based power saving signal/channel triggering UE adaptation in RRC_CONNECTED

    - Note: this objective shall not duplicate DRX operation and impact to DRX is studied at RAN2

    - Note: Any change of PDCCH channel coding and payload interleaver  is not in the scope

  - Specify the procedure of cross-slot scheduling power saving techniques

    - Note: The procedure is in addition to Rel-15 cross-slot scheduling procedure

- Evaluate the required switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers [RAN4]

  - Note: Switching on/off the RF is part of the evaluation

Note:

- These objectives are RAN1/RAN4 focus and do not consider RAN2 impact.

- The objectives are subject to further update in RAN#84.  The update will be based on recommendations from the completion of RAN2 study and remaining RAN1 recommendations based on the conclusion of RAN1 study.

In this contribution, we provide evaluation results for the power saving schemes proposed in [3].

# Evaluation on power saving gain

According to the potential functionalities of power saving signal/channel in [3], the power consumption model for the following two cases need to be defined:

- PDCCH-based power saving channel with DCI size larger than 1

- PDCCH DMRS based power saving signal without DCI

The suggested relative power value for each of case is summarized in Table 1.

Table 1.  Power consumption model for power saving signal/channel

| PoSS structure | Relative power | Note |
| --- | --- | --- |
| PDCCH-based with DCI size >0 | 70 – 100 | Same as PDCCH-only state. Depending on the number of BDs, the power level can be varied from 70 to 100. |
| PDCCH DMRS based without DCI | 55 – 70 | The minimum power 55 is obtained by adding 10 more power to the power for micro sleep (45). Depending on the configurations of the sequences, it can be scaled up to 70. |

Proposal 1. Consider the power consumption model for power saving signal/channel detection in Table 1.

For the simulation results in  the following, we consider FTP3 traffic model with mean of arrivals equals to 200ms, and power model for different activities in [2]. The baseline for No C-DRX case is PDCCH monitoring periodicity of 0.5ms, and the baseline for C-DRX case has CDRX configuration as: (C-DRX cycle, On duration, InActivity) = (160ms,8ms,100ms).

## 2.1 Adaptation on PDCCH monitoring periodicity/duration

In this sub-section, we evaluate the performance for using power saving signal/channel to trigger adaptation on PDCCH monitoring periodicity and duration. The baseline has fixed PDCCH monitoring periodicity of 1 slot and duration of 1 slot.

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Samsung | Trigger dynamic scaling on PDCCH monitoring periodicity, T, and duration, D. 1st DCI field (1bit): “1”: T = min(2*T, T_max); “0”: T = max(T/2, 1). 2nd DCI field(1bit): “1”: D = min(2*D, T); “0”: T = max(D/2, 1). | 51.1%-88.5% | #1: 51.1% #2: 69.2% #3: 74.1% #4: 88.5% | #1: 420.6Mbps #2: 170.1Mbps #3: 82Mbps #4: 39.1Mbps | Negligible, PDCCH-based power saving signal has sane monitoring periodicity as PDCCH monitoring periodicity. | No C-DRX Different cases have different value of T_max, Case #1: 5ms Case #2: 10ms Case #3: 20ms Case $4: 40ms | Numerical simulation One PDSCH over 10 symbols in one slot is capable of carrying 868584 information bits. |
|  |  | 45.8%-79.8% | #1: 45.8% #2: 63.2% #3: 68.0% #4: 79.8% | #1: 14.1Mbps #2: 15..1Mbps #3: 15.0Mbps #4: 13.2Mbps |  | w/ C-DRX Different cases have different value of T_max, Case #1: 5ms Case #2: 10ms Case #3: 20ms Case $4: 40ms |  |

Observation 1: The power saving scheme of dynamic scaling on PDCCH monitoring periodicity and duration shows 51.1%-88.5% and 45.8%-79.8% power saving gain for no C-DRX configured case and with C-DRX configured case, respectively.

## 2.2 Adaptation on PDCCH candidates per CCE AL

In this sub-section, we evaluate the performance for using power saving signal/channel to trigger adaptation on the number of blind decoding per PDCCH monitoring occasion. The baseline has fixed number of blind decoding of 36.

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Samsung | Trigger adaptation on PDCCH candidates per activated CCE AL. DCI field; Scalar on PDDCH candidate per activated CCE AL. 00; 25%. 01; 50%. 10; 75%. 11; 100%. | 5.4%-22.3% |  | 1.735Gbps | Negligible, PDCCH-based power saving signal has sane monitoring periodicity as PDCCH monitoring periodicity T | No C-DRX | Numerical simulation One PDSCH over 10 symbols in one slot is capable of carrying 868584 information bits. |
|  |  | 4.9%-21.1% |  | 19.1Mbps |  | w/ C-DRX |  |

Observation 2: The power saving scheme of dynamic scaling on PDCCH candidates per CCE AL shows 5.4%-22.3% and 4.9%-21.1% power saving gain for no C-DRX configured case and with C-DRX configured case, respectively.

## 2.3 Adaptation (de)activation on search space sets per PDCCH monitoring periodicities

In this sub-section, we evaluate the performance for using power saving signal/channel to trigger (de)activation of all the search spaces in one and/or a number of next K consecutive PDCCH monitoring periodicities.

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Samsung | Trigger (de)activation of search space per PDCCH monitoring periodicity Filed 1(1bit): “1”: deactivate current PDCCH monitoring periodicity “0”: activate current PDCCH monitoring periodicity Filed 2(1bit): “1”: deactivate next K PDCCH monitoring periodicities “0”: activate next K PDCCH monitoring periodicities | 29.78%-48.39% | #1: 29.78% #2: 42.19% #3: 46.32% #4: 48.39% | #1: 1.74Gbps #2: 1.15Gbps #3: 0.87Gbps #4: 0.69Gbps | Negligible, PDCCH-based power saving signal has sane monitoring periodicity as PDCCH monitoring periodicity T | No C-DRX K is different for different cases Case #1:0 Case #2: 1 Case #3: 2 Case #4: 3 | Numerical simulation One PDSCH over 10 symbols in one slot is capable of carrying 868584 information bits. |
|  |  | 27.85%-45.4% | #1: 27.85% #2: 39.50% #3: 43.2% #4: 45.4% | #1: 19.37Mbps #2: 19.20Mbps #3: 19.25Mbps #4: 18.86Mbps |  | w/ C-DRX K is different for different cases Case #1:0 Case #2: 1 Case #3: 2 Case #4: 3 |  |

Observation 3: The power saving scheme of (de)activation search space sets per PDCCH monitoring periodicity shows 29.78%-48.39% and 27.85%-45.4% power saving gain for no C-DRX configured case and with C-DRX configured case, respectively.

## 2.4 Adaptation (de)activation on search space sets per DRX cycles

In this sub-section, we evaluate the performance for using power saving signal/channel to trigger (de)activation of all the search spaces in one and/or a number of next K consecutive C-DRX ON duration.

| Company | Power saving scheme | Power saving gain | Power saving gaing for each configuration | UPT/Latency | Estimated Overhead | Evaluation methodology/baseline assumption | Note (include UE throughput) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Samsung | Trigger (de)activation of search space per DRX cycles Filed 1(1bit): “1”: deactivate current DRX ON duration “0”: activate current DRX ON duration Filed 2(1bit): “1”: deactivate DRX on duration in next K DRX cycles “0”: activate DRX on duration in next K DRX cycles | 9.37%-48.66% | #1: 9.37% #2: 26.47% #3: 39.29% #4: 48.66% | #1: 17.46Mbps #2: 8.21Mbps #3: 5.16Mbps #4: 3.66Mbps | Negligible, PDCCH-based power saving signal is on the first PDCCH monitoring occasion in the DRX ON duration | K is different for different cases Case #1:0 Case #2: 1 Case #3: 2 Case #4: 3 | Numerical simulation One PDSCH over 10 symbols in one slot is capable of carrying 868584 information bits. |

Observation 4: The power saving scheme of (de)activation search space sets per DRX cycle shows 9.37%-48.66% power saving gain.

# Conclusions

This contribution considers evaluation results for power saving schemes in [3]. Our proposals and observations as follows.

Proposal 1: Support implicit triggering of adaptation on PDCCH monitoring periodicity and duration according to real-time detection results of DCI formats for scheduling PDSCH/PUSCH

Observation 1: The power saving scheme of dynamic scaling on PDCCH monitoring periodicity and duration shows 51.1%-88.5% and 45.8%-79.8% power saving gain for no C-DRX configured case and with C-DRX configured case, respectively.

Observation 2: The power saving scheme of dynamic scaling on PDCCH candidates per CCE AL shows 5.4%-22.3% and 4.9%-21.1% power saving gain for no C-DRX configured case and with C-DRX configured case, respectively.

Observation 3: The power saving scheme of (de)activation search space sets per PDCCH monitoring periodicity shows 29.78%-48.39% and 27.85%-45.4% power saving gain for no C-DRX configured case and with C-DRX configured case, respectively.

Observation 4: The power saving scheme of (de)activation search space sets per DRX cycle shows 9.37%-48.66% power saving gain.

# References

[1] RP-190727, “UE Power Saving in NR”

[2] 3GPP TR 38.840, “Study on UE Power Saving”

[3] R1-1906980, “On PDCCH-based power saving signal/channel”, Samsung, RAN1#97, May, 2019