3GPP TSG-RAN WG1 Meeting #97 	R1-1906985

Reno, USA, May 13th – 17th, 2019

Agenda item:	6.1.1

Source: 	Qualcomm Incorporated

Title: 	Signaling of FD-MIMO capabilities

Document for:	Discussion and Decision

# Background

In RAN1#96b, the following agreement was reached on signalling of FD-MIMO capabilities:

Agreement:

Adopt the following baseband capability signalling for Rel-13 FD-MIMO:

For a UE configured with FD-MIMO, for a band combination for which the UE does not report FD-MIMO capabilities, a configuration related to a set of CCs is supported by the UE if the following inequality is met

i ∈ configured CCswi∙li≤y

where

-	 li is the maximum number of DL layers configured for CC i

-	wi=x0,  if CC i is configured with FD-MIMO and li=2x1,  if CC i is configured with FD-MIMO and li=4x2,  if CC i is configured with FD-MIMO and li=81,   if CC i  is not configured with FD-MIMO

-	y is the “total number of weighted layers” the UE can process

Notes:

-	Legacy capability signalling can still be used after this capability is introduced (to e.g. signal a subset of supported band combinations).

-	Detailed design of the capability signalling is left to RAN2

Send LS to RAN2 to introduce the following parameters in UE capability signalling (per UE):

-	xi (i=0,1,2): Relative weight of processing FD-MIMO with {2, 4, 8} layers with respect to non-FD-MIMO with the same number of layers. xi has a range of {1, 1.25, 1.5, 1.75, 2, 2.5, 3, 4}

-	y: “Total number of weighted layers”, which is an integer between 2 and 128.

The above signalling allows for greatly reducing the capability exchange for LTE CA. In the following, we present our views on how the new capability applies for EN-DC case.

# Applicability for EN-DC

The basic intuition behind the recently introduced FD-MIMO capability is as follows:

- Operating a CC with FD-MIMO is more computationally expensive than operating a CC without FD-MIMO.

- The scaling is nonlinear with the number of layers

- The UE has a total baseband capability in terms of “weighted layers” to be shared between CC, including CC with FD-MIMO and CC without FD-MIMO

One key difference in operation in EN-DC is that the baseband processing capability is shared between LTE and NR. Depending on the band combination, the split may be biased towards LTE (more LTE bandwidth/carriers) or NR (more NR bandwidth carriers).

In general, therefore, the capability for EN-DC may be expressed as follows:

i ∈LTE configured CCswi∙li +NRbb≤y

Where NRbb is the baseband computational complexity (expressed in terms of “weighted layers”) that is used by NR in a particular band combination. An alternative way of expressing this is as follows:

i ∈ LTE configured CCswi∙li ≤yLTE

Where yLTE is the baseband computational capacity dedicated to LTE in a particular band combination. Thus, a simple extension of the agreement in R1#96b is to introduce the total number of weighted layers y per EN-DC band combination.

Proposal: For EN-DC, introduce capability signalling of y (“Total number of weighted layers”) per band combination.