3GPP TSG-RAN WG1 #97	R1-1907000

Reno, USA, May 13th – 17th, 2019

Agenda item:    6.2.2.1

Source:             Qualcomm Incorporated

Title:                  UE-group wake-up signal for NB-IoT

Document for:  Discussion and Decision

# Introduction

In RAN1#96bis meeting, we have the following agreements for UE-group NWUS:

Agreement

Up to 2 time-multiplexed WUS resources, for both legacy WUS and group WUS, may be configured. FFS whether a group WUS resource may be shared with legacy WUS or not.

Agreement

Group WUS location in relation to legacy WUS may be configured such that:

- If one group WUS resource is configured, that group WUS resource may be configured to coincide with the legacy WUS resource or to occur immediately before the legacy WUS resource, and,

- If two group WUS resources are configured, the first group WUS resource coincides with the legacy WUS resource and the second group WUS resource occurs immediately before the first group WUS resource.

Agreement

Group WUS sequence design is down selected between the following in RAN1#97

- Legacy WUS with shifted scrambling codes

- Legacy WUS with time-frequency short OCCs

- Phase shifted legacy WUS using a suitable subset of the phase increments.

- Legacy WUS with Gold codes

Companies are encouraged to provide evaluation results

Agreement

- Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation.

- Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap.

In RAN1#96bis meeting, we also have the agreements for UE-group MWUS, which should be applied to NWUS as well:

Agreement

If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

In this contribution, we further discuss UE-group NWUS from the following aspects:

- Multiplexing of WUS

  - Load balancing of WUS resources

  - Number of UE groups

- Configuration of UE grouping

  - Paging carriers for Rel16 WUS

  - Weighting factors for WUS resources

  - Other parameters

- Group WUS sequence

# Multiplexing of WUS

In Release 15, WUS does not support UE grouping and all UEs that support WUS will use legacy WUS. As population of UEs supporting WUS grouping increases, resources for new WUS supporting UE grouping becomes over utilised relative to that of legacy WUS resource. To mitigate this, it is desirable for some UEs supporting grouping to be grouped together with legacy UEs and share resources allocated for legacy WUS.

Considering the ratio of legacy UEs, we first divide all the UEs supporting WUS grouping belong to one of two groups G1 and G2, monitoring one of two TDMed WUS resources respectively, as illustrated in Figure 1.

Figure 1 Up to 2 WUS resources for legacy and UE-group WUS

- New UE in G1 (new UE1), who monitor the legacy WUS resource same with legacy UEs, can be further divided into G1 subgroups. Legacy UEs detect legacy WUS only, unaware of UE-group WUS. New UE1 will detect G1 common WUS (legacy WUS) and G1 UE-group WUS.

  - G1 common WUS (legacy WUS) is sent when at least one legacy UE or more than one G1 subgroup needs to be paged.

  - G1 UE-group WUS is sent when only new UE1 from one subgroup needs to be paged.

- New UE in G2 (new UE2), who are allocated WUS time/freq resources TDMed with that of legacy WUS resource, can be further divided into G2 subgroups. New UE2 will detect G2 common WUS and G2 UE-group WUS.

  - G2 common WUS is sent when more than one G2 subgroup needs to be paged.

  - G2 UE-group WUS is sent when only new UE2 from one subgroup needs to be paged.

A legacy UE will wake up if successfully detect legacy WUS, or if falsely detect a legacy sequence with probability of pFA. A new UE1 will wake up if successfully detect its G1 UE-group WUS, G1 common WUS (legacy WUS), or if false detection of any of 2 new sequences with probability of (1-1-pFAmin(G1,2)). Similarly, a new UE2 will wake up if successfully detects its G2 UE-group WUS or G2 common WUS or if falsely detect any of 2 sequences with probability of (1-1-pFAmin(G2,2)). Table 1 summarizes the distribution of legacy UE0, new UE1 and new UE2, as well as the detection probability and false alarm probability for the following cases.

- Case1: N=1 WUS resource is legacy WUS resource shared by legacy UE0 and all new UEs (X=0)

- Case 2: N=2 WUS resources include legacy WUS resource not sharing for with new UEs (X=1)

- Case 3: N=2 WUS resources include legacy WUS resource shared by legacy UE0 and new UE1 (0<X<1 with total X UEs in G1)

Table 1 Detection/false alarm probability of legacy UE0/new UE1/new UE2

|  | Legacy UE0 | New UE1 | New UE2 |
| --- | --- | --- | --- |
| Case 1 | R0 | 1-R0 | 0 |
| Case 2 | R0 | 0 | 1-R0 |
| Case 3 | R0 | R1=max⁡(X%-R0,0) | R2=1-R0-R1 |
| Detection prob | pUE0(1-pMISS) | pUE1(1-pMISS) | pUE2(1-pMISS) |
| False alarm | pFA | (1-1-pFAmin(G1,2)) | (1-1-pFAmin(G2,2)) |

The probabilities of WUS to be detected by legacy UE0, new UE1 and new UE2 are

pUE0=1-1-piNUE01-piNUE1+G11-piNUE1G1(G1-1)(1-1-piNUE1G1)

pUE1=1-1-piNUE01-piNUE1+(G1-1)1-piNUE1G1(G1-1)(1-1-piNUE1G1)

pUE2=1-1-piNUE2+(G2-1)1-piNUE2G2(G2-1)(1-1-piNUE2G2)

where p is the total paging probability and pi is the paging probability per UE with relationship of  p=1- 1-piNUE. The UE ratio R0=NUE0/NUE , R1=NUE1/NUE, and Ri+1=NUEi+1/NUE with total number of UEs as NUE=NUE0+NUE1+NUE2…. In simulation, we assume NUE=100 per PO (but similar results for different NUE if assuming same p). Note that we count the false wake-up of new UE1 if only legacy UEs have paging, e.g., 1-piNUE0 in the calculation of pUE1, since we assume legacy WUS as the common WUS.

In case of X%=100%, it is single-sequence CDM only and all the new UEs are in G1 and will use legacy WUS as its common WUS; in case of X%=0%, new UEs will not monitor legacy WUS but only monitor new WUS in time resources TDMed with legacy WUS. The WUS for new UEs are multiplexed by single-sequence CDM only. If the legacy UE ratio R0 is larger than X%, all the new UEs are in G2 group, i.e., R1=0, only detecting new WUS instead of legacy WUS. Legacy UEs will not be waken up if there is paging for new UEs only.

The legacy UE0’s average power is PUE0=(pUE01-pMISS+pFA+β), which may be increased if there is new UE1 in G1. While, the new UE1’s average power is PUE1=(pUE11-pMISS+(1-1-pFAmin(G1,2))+β), which would be higher than that of new UE2, i.e., PUE2=(pUE21-pMISS+(1-1-pFAmin(G2,2))+β). This is because of the impact of legacy UE0 in pUE1. β is the scaling factor for WUS maximum duration relative to Rmax. Assuming total paging probability per PO of p=0.1, 0.2 or 0.5,  β=1/16, pFA=1%, pmiss=1%, we have compared the normalized average power consumption per UE for different ratio R0 of legacy UEs. The average power consumption of all UEs, including legacy UE0, new UE1, and new UE2, as P=R0PUE0+R1PUE1+R2PUE2.

In Figure 2, we change the ratio X to balance the UEs in G1 and G2. By using UE grouping, the UEs’ power can be reduced significantly especially if there are less legacy UEs. Case 1 with N=1 WUS resource has worse performance than Case 3 with N=2 WUS resource. If Case 2 with X=0, all the new UEs are in G2 and separated from legacy UEs (TDM only), which costs higher average power when ratio of legacy UEs is less than 20%. Relatively, the use of X%=50% (TDM+CDM) for Case 3 achieves best performance by equally balancing G1 and G2 group.

- p=0.1					(b) p=0.2

Figure 2 Average power consumption per UE with G1=G2=2

Observation 1: Case 3 with load balancing amongst two WUS resources can significantly improve power saving by up to 13~22% than Case 1 and Case 2 for low paging p=0.1~0.2 and up to 44~59% relative to no UE grouping.

Figure 3 investigates the performance by using different number G of subgroups per WUS resource with different paging probability. Assuming there are N=1 or 2 WUS resources, one of WUS resources coincides with the legacy WUS resource shared by legacy UEs and new UE1. The total number of groups will be equal to (NxG).

- p=0.1					(b) p=0.2

- p=0.5

Figure 3 Average power consumption per UE with 1 or 2 WUE resources 
and different number of UE groups per WUS resource

Table 2a summarizes the power saving gain relative to no grouping (Rel15 WUS only) assuming no legacy UEs for p=0.1, 0.2 or 0.5, respectively. In case of no legacy UEs, the legacy WUS resource will be fully used by new UEs and all the new UEs will be distributed equally into 2 WUS resources if configured.

Table 2a Power saving gain of different number of UE groups per WUS resource (No legacy UEs)

- p=0.1

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | 0% | +22% | +36% | +42% | +46% |
| Total number of groups (2xG) | 2G=4 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +28% | +37% | +44% | +48% | +49% |

- p=0.2

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | 0% | +31% | +47% | +54% | +58% |
| Total number of groups (2xG) | 2G=4 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +35% | +50% | +59% | +62% | +65% |

- p=0.5

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | 0% | +34% | +48% | +54% | +56% |
| Total number of groups (2xG) | 2G=4 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +36% | +58% | +68% | +73% | +75% |

Table 2b summarizes the power saving gain relative to no grouping (Rel15 WUS only) assuming 25% legacy UEs for p=0.1, 0.2 or 0.5, respectively. Legacy WUS is shared by legacy WUS and new WUS and 25% legacy UEs and new UE1 of 75%-(N-1)/N are allocated to monitor legacy WUS resource, and the remaining 1/N new UE2 monitor the new group WUS resource with N=1 or 2 WUS resources.  Here we assume the common WUS for new UE1 is the legacy WUS in G1 and another common WUS for new UE2 is a non-legacy WUS in G2.

Table 2b Power saving gain of different number of UE groups per WUS resource (25% legacy UEs)

- p=0.1

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | +11% | +22% | +29% | +33% | +35% |
| Total number of groups (2xG) | 2G=4 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +32% | +37% | +41% | +43% | +44% |

- p=0.2

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | +13% | +29% | +38% | +42% | +44% |
| Total number of groups (2xG) | 2G=4 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +39% | +48% | +54% | +56% | +58% |

- p=0.5

| Total number of groups (1xG) | G=1 | G=2 | G=4 | G=8 | G=16 |
| --- | --- | --- | --- | --- | --- |
| Gain of N=1 WUS resource | +15% | +31% | +38% | +41% | +43% |
| Total number of groups (2xG) | 2G=4 | 2G=4 | 2G=8 | 2G=16 | 2G=32 |
| Gain of N=2 WUS resources | +42% | +55% | +61% | +64% | +65% |

Observation 2: More total number (NxG) of UE groups achieves larger power saving gain until (NxG)=16 achieving almost saturated gain.

- G: number of subgroups per WUS resource

- N: number of WUS resources

Observation 3: Assuming NxG=16 UE groups (to see saturated power saving gain), the larger number N of WUS resources the better power saving gain.

- In case of no legacy UEs, N=1 is sufficient for p=0.1, 0.2 but N=2 is much better for p=0.5.

  - p=0.1: N=1 +46%  N=2 +48% (+2%)

  - p=0.2: N=1 +58%  N=2 +62% (+4%)

  - p=0.5: N=1 +56%  N=2 +73% (+17%)

- In case of 25% legacy UEs, N=2 is always better than N=1 and larger gain especially for p=0.5.

  - p=0.1: N=1 +35%  N=2 +43% (+8%)

  - p=0.2: N=1 +44%  N=2 +56% (+12%)

  - p=0.5: N=1 +43%  N=2 +64% (+21%)

Based on the above analysis, we propose

Proposal 1: The number of WUS resources and the number of UE groups per WUS resource can be configured based on the paging probability.

- Number of WUS resources: N=1 or 2

- Number of UE groups per WUS resource: G=1, 2, 4, 8, or 16

- Total number of UE groups: NxG<=16

# Configuration of Rel16 WUS

In RAN1#96bis meeting, we had following agreement on the common WUS configuration for group MWUS.

Agreement

If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

When the eNB can schedule the paging of legacy and new UEs in different POs, a common WUS can be configured as a non-legacy WUS for the new UEs. If there are more than one group per WUS resource, we prefer to let eNB schedule the paging with minimal latency and use a common WUS for all the group WUS UEs. If there is only one group, common WUS is not needed. From the Table 2a and 2b in Sect. 2, the configuration of more than one groups significantly saving the power even in case of low paging probability.

Proposal 2: If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

In NB-IoT, there can be multiple carriers configured for paging. The eNB can distribute the paging for non-anchor carriers based on the UE ID and weighting factor for paging carriers as:

- floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(n) with W=W(0) + W(1) + … + W(n)

The paging probability may be different per carrier, depending on the number of UEs per paging carrier. In Rel15 WUS configuration, WUS configuration is cell-specific. If Rel15 WUS is on, all the paging carriers are used for Rel15 WUS. However, for Rel16 WUS, the group WUS could be configured as cell-specific or carrier-specific considering different paging probability and the time/freq/power resources per carrier.

In addition, if carrier-specific on/off is supported for Rel16 WUS, it may be beneficial to change the way a Rel16 UE selects its paging carrier. For the UEs using group WUS, the number Nn of paging carriers is the number of carriers that support group WUS.

For example, there are carrier #0-5 for Rel-15 WUS UEs and Nn=6 for legacy UEs with

- floor(UE_ID/(N*Ns)) mod W < W(0) + W(1) + … + W(5) with W=W(0) + W(1) + … + W(5)

When there are less new UEs, only the limited number of carriers are used for new UEs, e.g., the carrier#0-2 are used to send UE-group WUS. Therefore, Nn=3 for new UEs with

- floor(UE_ID/(N*Ns)) mod W’ < W(0) + W(1) + W(2) with W’=W(0) + W(1) + W(2)

It could be a possible way to change the relative ratio between legacy UEs and new UEs by carrier selection.

Proposal 3: The group WUS configuration can be different for paging carriers.

- FFS carrier selection for group WUS.

Another aspect is how to load balancing amongst the different WUS resources. From the observation in Sect. 2, load balancing to uniform UE distribution maximize the power saving gain. We could balance the number of UEs per WUS resource, e.g., in a manner similar to that developed in Rel-14 for paging in non-anchor carriers. For the distribution of new UEs in the two WUS resources, we can set weighting factors for group WUS resources as WG(i,G1) and WG(i,G2) and WG(i)=WG(i,G1)+WG(i,G2) on i-th paging carrier supporting group WUS. The new UEs of WG(i,G1)/WG(i) are allocated to monitor the legacy WUS resource and remaining new UEs of WG(i,G2)/WG(i) belong to the new WUS resource.

For example,

- If only 1/4 UEs are legacy, the weighting factors can be WG(i,G1)=1, WG(i,G2)=2 and WG(i)=WG(i,G1)+WG(i,G2)=3. The new UEs of WG(i,G1)/WG(i)=1/3 are allocated to monitor the legacy WUS resource and remaining new UEs of WG(i,G2)/WG(i)=2/3 belong to the new WUS resource. In total, there are 1/2 UEs per WUS resource.

- If 1/2 UEs are legacy, the legacy WUS resource is not used for group WUS. The weighting factors can be WG(i,G1)=0, WG(i,G2)=1 and WG(i)=WG(i,G1)+WG(i,G2)=1. No new UEs, i.e., WG(i,G1)/WG(i)=0, are allocated to monitor the legacy WUS resource and all the new UEs, i.e., WG(i,G2)/WG(i)=1 are belong to the new WUS resource. In total, there are 1/2 UEs per WUS resource.

Proposal 4: Support weighting factor configuration for UE distribution amongst the different WUS resources configured for a carrier.

The UE only monitors one WUS resource. Per WUS resource, there is only one sequence sent from eMB at one time, e.g., a UE group WUS, a common WUS for all group WUSs. If a common WUS is configured, the UE will detect two WUS sequences. More than two WUS sequence detection will increase UE complexity as well as the false detection probability.

Proposal 5: Each UE monitors up to 2 WUS sequences.

In RAN2 agreements, coverage-based grouping is not supported. The legacy WUS can be shared by Rel15 WUS and Rel16 WUS. Therefore, the parameters for legacy WUS, i.e., the max duration for new WUS sequence and the transmit power, should be applied to Rel16 WUS sequence as well.

Proposal 6: UE assumes transmit power for Rel16 WUS sequence is same as legacy WUS sequence.

Proposal 7: Max WUS duration for Rel16 WUS sequence is same as legacy WUS sequence.

# Group WUS sequence

In previous RAN1 meetings, we have following agreements for UE-group WUS sequence design:

Agreement

Group WUS sequence design is down selected between the following in RAN1#97

- Legacy WUS with shifted scrambling codes

- Legacy WUS with time-frequency short OCCs

- Phase shifted legacy WUS using a suitable subset of the phase increments.

- Legacy WUS with Gold codes

Companies are encouraged to provide evaluation results

In the following, we have compared the above schemes for group WUS sequence.

Assuming wm is the legacy WUS sequence with no UE grouping, NUEgroup is the number of UE groups, NIDUEgroup={0,…NUEgroup-1} is the UE group ID, we have compared the following alternatives for UE-group WUS sequence design:

  - Gold sequence cover code

    - wNIDUEgroupm=wmCgold(NIDUEgroup),

  - CGold is 127-length binary Gold sequence defined as CGold=1-2x0m mod 1271-2x1(m+m1)mod 127  with

    - x0i+ 7=x0i+4+x0imod 2 using [0000001] as initialization values

    - x1i+ 7=x1i+1+x1imod 2  using [0000001] as initialization values

    - m1=NIDUEgroup127/NUEgroup

  - Phase shift [2]

    - wNIDUEgroupm=wmexp⁡{-j2πm(α+NIDUEgroupβ)132} with α=0, ∆=7 or α=2, ∆=13

  - TF-OCC [3]:

    - wNIDUEgroupm=wmCOCC(f, mod(NIDUEgroup+s,11)),

  - COCC is the RE-level OCC cover codes with m=12s+f  with

    - f: subcarrier index per symbol, f=0, …11

    - s: symbol index per a subframe for WUS, s=0, …10.

  - Scrambling phase shift [3]

    - cn_f,n_s,NIDUEgroup(i)=cn_f,n_s(i+2*132M*NIDUEgroup) ,

  - cn_f,n_s(i) is the scrambling sequence for WUS with i=0,1, …,2*132M-1.

As discussed in Sect. 2, we need 16 different WUS sequences plus 2 common sequences if configured for up to 2 WUS resources, i.e., in total 18 sequences. Here, we compare different sequence designs for 2, 4, 8, or 16 UE groups.

The CDF of intra-cell cross-correlation of UE-group WUS is given in Figure 4. The maximum cross-correlation peak within the assumed timing drift window, e.g., [-11,11] samples, [-37,37] samples, and [-135,135] samples at 1.92 MHz sampling rate, respectively. The frequency error is 50Hz. The cross-correlation is the correlation of different UE-group WUS sequences as well as UE-group WUS vs. legacy WUS sequences. We evaluate all the intra-cell correlation of the UE-group WUS as well as legacy WUS with cell ID=0,…503.

The worst cross-correlation of each WUS sequence design is compared in Table 3a. Also, we investigated the impact of configurable UE group number with the worst cross-correlation compared in Table 3b.

Table 3a Worst cross-correlation of Intra-cell UE-group WUS and legacy WUS 
assuming 16 UE groups, CFO=50Hz, and different TimeOffset

| Time offset (samples) | [0,0]Ts | [-11,11]Ts | [-37,37]Ts | [-135,135]Ts |
| --- | --- | --- | --- | --- |
| Gold cover | -16.4dB | -12.4dB | -10.8dB | -10.2dB |
| Phase shift (DELTA1327) | -87.7dB | -17.5dB | -10.8dB | -2.0dB |
| Phase shift (DELTA13213) | 0dB | 0.01dB | 0.01dB | 0.015dB |
| TF-OCC | 0dB | 0dB | 0dB | 0dB |
| Scrambling phase | -10.4dB | -9.7dB | -9.4dB | -9.35dB |

Table 3b Worst cross-correlation of Intra-cell UE-group WUS and legacy WUS 
assuming TimeOffset=[-135,135]Ts, CFO=50Hz and different number of UE groups

| Number of UE groups | 2 groups | 4 groups | 8 groups | 16 groups |
| --- | --- | --- | --- | --- |
| Gold cover | -11.7dB | -11.0dB | -10.5dB | -10.2dB |
| Phase shift (DELTA1327) | -11.9dB | -11.1dB | -10.6dB | -2.0dB |
| Phase shift (DELTA13213) | -11.2dB | -10.9dB | -10.4dB | 0dB |
| TF-OCC | -11.8dB | -11.6dB | -9.97dB | 0dB |
| Scrambling phase | -11.9dB | -10.5dB | -9.93dB | -9.35dB |

(a) 16 UE groups, TimeOffset=0		      (b) 16 UE groups, TimeOffset=[-11,11]Ts

(c) 16 UE groups, TimeOffset=[-37,37]Ts		(d) 16 UE groups, TimeOffset=[-135,135]Ts

Figure 4 Intra-cell cross correlation of UE-group WUS and legacy WUS

Observation 4:

- Phase-based (DELTA13213) and TF-OCC-based UE group WUS don’t support no more than 10 group WUS sequences.

- Phase-based (DELTA1327) UE-group WUS support up to 18 different sequences but shows significantly degraded cross-correlation in case of large timing error.

- Gold cover-code-based and Scrambling-phase-based UE-group WUS support 18 and even more than 18 different sequences and have much more stable performance in presence of variant timing error.

- Gold cover-code-based sequence is 1~4dB better than Scrambling phase one in case of 16 UE groups with variant timing error

Proposal 8: Group WUS sequence is the legacy WUS sequence with Gold cover codes.

# Summary

The proposals made in this contribution are summarized below.

For WUS multiplexing:

Proposal 1: The number of WUS resources and the number of UE groups per WUS resource can be configured based on the paging probability.

- Number of WUS resources: N=1 or 2

- Number of UE groups per WUS resource: G=1, 2, 4, 8, or 16

- Total number of UE groups: NxG<=16

For Rel16 WUS configuration:

Proposal 2: If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

Proposal 3: The group WUS configuration can be different for paging carriers.

- FFS carrier selection for group WUS.

Proposal 4: Support weighting factor configuration for UE distribution amongst the different WUS resources configured for a carrier.

Proposal 5: Each UE monitors up to 2 WUS sequences.

Proposal 6: UE assumes transmit power for Rel16 WUS sequence is same as legacy WUS sequence.

Proposal 7: Max WUS duration for Rel16 WUS sequence is same as legacy WUS sequence.

For group WUS sequence design:

Proposal 8: Group WUS sequence is the legacy WUS sequence with Gold cover codes.

References

- 3GPP RAN1 Chairman’s Notes, RAN1#96bis, Xi’an, China, April 8th – 12th, 2019.

- R1-1903891, “UE-group wake-up signal in NB-IoT”, Ericsson, RAN1#96bis, Xi’an, China, April 8th – 12th, 2019

- R1-1905529, “UE-group wake-up signal for NB-IoT”, Intel, RAN1#96bis, Xi’an, China, April 8th – 12th, 2019.