3GPP TSG RAN WG1 #97		                        R1-1907012

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.4.1

Source: 	LG Electronics

Title: 	Discussion on physical layer structure for NR sidelink

Document for:	Discussion and decision

- Introduction

In RAN1#96bis meeting [1], followings are agreed for physical layer structure for Rel-16 NR sidelink:

Agreements: - Polar code adopted for Rel-15 NR DCI is applied to PSCCH. - LDPC codes used for Rel-15 NR DL-SCH is applied to a transport block delivered by PSSCH. Agreements: - The starting symbol and the number of symbols for a PSCCH are assumed to be known to the receiving UE before decoding the PSCCH. Agreements: - For the purpose of evaluation of PSCCH design, RAN1 assumes 60 bits, 90 bits, 120 bits as the total SCI sizes including 24 bits CRC. - Other sizes are not precluded. Agreements: - QPSK is used for PSCCH. Working assumption: - Transmission of 1 TB with up to 2 layers in a PSSCH is supported. Agreements: - At least for transmission perspective of a UE in a carrier, at least TDM between PSCCH/PSSCH and PSFCH is allowed for a PSFCH format for sidelink in a slot. - FFS the details of the corresponding PSFCH format - FFS whether it is also applicable from system/resource pool perspective or not - i.e., in this case, there is no simultaneous transmission of PSCCH and PSFCH and there is no simultaneous transmission of PSSCH and PSFCH. - FFS FDM between PSCCH/PSSCH and a PSFCH format which uses last symbol(s) available for sidelink in a slot - FFS TDM/FDM between PSCCH/PSSCH and other PSFCH format(s), if supported, which is/are different from the PSFCH format which uses last symbol(s) available for sidelink in a slot Agreements: - NR V2X supports an initial transmission of a TB without reservation, based on sensing and resource selection procedure - NR V2X supports reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB, based on sensing and resource selection procedure - This functionality can be enabled/disabled by (pre-)configuration FFS Standalone PSCCH transmissions for resource reservations are supported in NR V2X Agreements: - Support at least Sidelink CSI-RS for CQI/RI measurement - Sidelink CSI-RS is confined within the PSSCH transmission

In this contribution, we discuss issues on physical layer structure and physical channel format for NR sidelink including SCI, PSCCH, PSSCH, PSFCH, and measurement RS. In addition, we discuss SL BWP design and resource pool design.

Discussion

  - Physical layer structure

    - SCI design

To design PSCCH especially for the physical layer mapping, it is necessary to investigate the amount of SCI payload size and target coverage first. In this section, we list up the potential candidates of SCI field with tentative field size.

Layer-1 IDs can be derived from upper layer IDs (e.g. Layer-2 IDs), but their size need to be further reduced to save SCI overhead. In our view, it can be a starting point that each Layer-1 ID has 8-bit length. In addition, Layer-1 destination ID can be used to distinguish cast type (e.g. broadcast, groupcast, unicast) if necessary, therefore, we do not see the necessity of having additional explicit indication of cast type in SCI. Meanwhile, it is necessary to clarify the necessity of having indication of cast type first. Regarding HARQ process related SCI fields and MCS field, bit field size of Uu link can be a starting point. Meanwhile, bit field size of HARQ process ID can be determined based on the maximum HARQ RTT considering HARQ feedback timing and preparation time for retransmission scheduling. Considering asynchronous HARQ operation, in our view, Layer-1 source ID, Layer-1 destination ID, HARQ process number, NDI, and RV are needed for suitable HARQ combining regardless of the existence of HARQ feedback and cast type.

Regarding the resource allocation, two types of resource allocation mechanisms can be considered: one is used to allocate sub-channel(s) for a single PSSCH transmission, and the other is used to allocate sub-channel(s) for more than one PSSCH transmissions simultaneously to support blind retransmissions of a TB or to support resource reservation in advance. For example, assuming that the starting sub-channel of PSSCH is derived by the associated PSCCH resource, resource allocation field for a single PSSCH transmission can indicate the number of sub-channels to be used for PSSCH transmission. In this case, the bit field size for resource allocation of a single PSSCH is given by log2NsubSL where NsubSL is the number of sub-channels within a resource pool. Meanwhile, for resource allocation field(s) for multiple PSSCH (re)transmissions, it can be considered that resource allocation field(s) in SCI format 1 in LTE sidelink is a starting point. In other words, 4 bits is used to indicate slot offset between two PSSCH transmissions indicated by the same SCI, and log2NsubSLNsubSL+1/2 bits used to indicate frequency resource of two PSSCH transmissions simultaneously. For the periodic traffic, additional 4 bits can be used to indicate periodicity of the periodic traffic. Regarding retransmission index if necessary, in our view, RV field can be used to indicate whether the scheduled PSSCH is initial transmission or retransmission. Considering sensing operation, priority field in LTE SCI format 1 can be reused.

According to 5G NR V2X WID, Rel-16 NR sidelink will support CQI/RI reporting for unicast and multi-rank PSSCH transmission with up to two antenna ports. Regarding CSI measurement/reporting field, SCI can indicate at least whether aperiodic SL-CSI-RS is transmitted or not when PSSCH is transmitted, and UE receiving SL-CSI-RS can transmit CQI/RI reporting via PSSCH. To avoid ambiguity on the physical layer mapping on PSSCH, SCI can also indicate whether CQI/RI reporting is included or not in PSSCH for unicast. In short, bit field size for CQI/RI reporting is at least 2 bits. Detailed description is in our companion contribution [2]. For broadcast and groupcast, bit field for CQI/RI reporting will not be present in SCI. Regarding antenna ports field, it can be used to indicate rank of PSSCH transmission for unicast, and DMRS port(s) to be used for spatial multiplexing. Considering that up to two antenna port and MU-MIMO operation is not main target for NR sidelink, the bit field size of antenna ports can be 1 bit to indicate rank of PSSCH transmission. For broadcast and groupcast, since CQI/RI reporting is not supported, it is unclear how TX UE decide the rank of PSSCH dynamically. Instead, it can be considered that the rank of PSSCH for broadcast or groupcast is (pre)configured. In a similar manner, we do not see the necessity of having DMRS sequence initialization field in SCI.

Regarding TX-RX distance based HARQ feedback for groupcast, it is necessary that TX UE provide information related to UE location via SCI to RX UEs. In our companion contribution [2], it can be considered that Layer-1 ID(s) in SCI include the necessary information to derive TX-RX distance at RX UE side. Alternatively, the concept of Zone ID can be introduced and, the Zone ID is present in SCI for groupcast. Table 1 summarizes the set of SCI fields to be considered for broadcast/groupcast/unicast.

According to HARQ feedback mechanism in our companion contribution [2], SAI (sidelink assignment index) field would be useful to adjust HARQ feedback payload size efficiently. To be specific, during SL slots associated with the same PSFCH slot, a TX UE transmit only one PSCCH/PSSCH with SAI=1 to a RX UE, the RX UE would transmit PSFCH with a single HARQ bit. On the other hand, a RX UE receives one or more PSCCH/PSSCH with SAI>1, the RX UE would transmit PSFCH with multiple HARQ bits. In this case, PSFCH resources are separate based on the SAI value, and the RX UE select PSFCH resource to be transmitted for HARQ feedback based on the SAI value in the latest received SCI.

Observation 1: For implicit/explicit indication of cast type, the technical benefit of having such indication is clarified first.

Observation 2: For asynchronous HARQ operation, regardless of whether HARQ feedback is enabled or which cast type is associated, SCI needs to include Layer-1 source ID, Layer-1 destination ID, HARQ process number, NDI, and RV fields.

Observation 3: It would be beneficial to implicitly derive the starting sub-channel of PSSCH based on the associated PSCCH resource in terms of SCI overhead reduction.

Observation 4: Considering MU-MIMO operation is not a main target for NR sidelink, DMRS sequence initialization field may not be needed in SCI, and 1-bit field is needed to indicate rank of PSSCH.

Observation 5: Since CQI/RI reporting is not supported for broadcast and groupcast, SCI for broadcast and groupcast would not include CQI/RI reporting field, antenna port/rank field for PSSCH.

Observation 6: To adjust HARQ feedback payload size efficiently, it can be considered that SAI (sidelink assignment index) field in SCI indicates the information on the number of PSCCH/PSSCH transmissions associated with the same PSFCH slot.

Depending on the SCI payload size, the required number of RB and/or symbols for PSCCH transmission could be significantly large, and it can cause latency problem and/or large sub-channel size. To reduce SCI payload size to be conveyed on PSCCH, it can be considered that total SCI payload is divided into two parts, and one part is conveyed by PSCCH, and other part is piggybacked on PSSCH. In this case, it is necessary to further investigate which SCI field will be conveyed by PSCCH or other channel(s). For instance, at least SCI fields to be used for sensing operation can be conveyed by PSCCH.

Proposal 1: For NR sidelink, the set of SCI fields includes at least followings.

- SCI format can consist of parts of the above SCI fields depending on the (pre)configuration.

- FFS: whether all the SCI fields in a SCI format are conveyed on PSCCH.

Table 1: Example of the set of SCI fields with tentative size

| SCI field | Broadcast |  | Groupcast |  | Unicast |  |
| --- | --- | --- | --- | --- | --- | --- |
| L1-source ID | [8] |  | [8] |  | [8] |  |
| L1-destination ID | [8] |  | [8] |  | [8] |  |
| Time-and-Frequency resource location | Frequency resource assignment | log2NsubSLNsubSL+12 | Frequency resource assignment | log2NsubSLNsubSL+12 | Frequency resource assignment | log2NsubSLNsubSL+12 |
|  | Time gap between inTx and ReTx | [4] | Time gap between inTx and ReTx | [4] | Time gap between inTx and ReTx | [4] |
|  | Resource reservation | [4] | Resource reservation | [4] | Resource reservation | [4] |
| HARQ process number | [4] |  | [4] |  | [4] |  |
| NDI | 1 |  | 1 |  | 1 |  |
| RV | 2 |  | 2 |  | 2 |  |
| MCS | 5 |  | 5 |  | 5 |  |
| Priority field | [3] |  | [3] |  | [3] |  |
| CQI/RI reporting field |  |  |  |  | [2] |  |
| Antenna port(s) for PSSCH |  |  |  |  | [1] |  |
| Sidelink assignment index |  |  | [2] |  | [2] |  |

    - PSCCH design

      - PSCCH structure

The number of RB and symbols of PSCCH resource may be predetermined or may be flexible according to service or application or slot structure or PSCCH/PSSCH association option.

Considering that PSCCH resource is confined within the associated PSSCH resource (i.e. Option 3), it is necessary that the frequency resource of PSCCH is mapped within a sub-channel to support PSSCH with a single sub-channel allocation as shown in Figure 1. In this case, depending on the symbol duration of PSCCH and SCI payload size, the required number of RBs for PSCCH mapping could be large.

In this contribution, we provide our link level simulation results for SCI payload size of 60/90/120 bits including 24-bit CRC. In our link-level simulation, it is assumed that carrier frequency of 6GHz, CDL Urban NLOS, UE speed of 140kmph, subcarrier spacing of 30 kHz. Furthermore, NR PDCCH DMRS pattern is assumed to be used for PSCCH DMRS pattern. Regarding transmission scheme, precoder cycling is used.

First of all, we compare detection performance of PSCCH mapped on 3 OFDM symbols which is the maximum symbol duration of NR PDCCH across different SCI payload size and different RB allocation. As shown in Figure 1, to achieve BLER of 10-2, when the number of RB for PSCCH is 4 RB, the required SINR needs to be much higher than 10 dB due to the lack of coding gain regardless of the SCI payload size. On the other hand, when the number of RB for PSCCH is 10 RB, the required SINR to achieve BLER of 10-2 is reduced into 2.5dB, 4dB, or 5dB for SCI payload size of 60bits, 90bit, or 120bits, respectively.

Figure 1: Detection performance of PSCCH with symbol duration of 3.

Observation 7: When the number of symbol for PSCCH is 3, to achieve BLER of 10-2 at small SINR region (e.g. 2.5dB~5dB), the required number of RB for PSCCH is at least 10.

Next, we compare detection performance of PSCCH mapped on 6 OFDM symbols to with reduced number of RBs for PSCCH. We consider 5 RB for PSCCH with symbol duration of 6 to have the same coding rate of the case where PSCCH is mapped on 10 RB in symbol duration of 3. As shown in Figure 2, it is observed that for a given SCI payload size, the detection performance of PSCCH with 3 symbol duration and 10 RB allocation outperforms over that of PSCCH with 6 symbol duration and 5 RB allocation by roughly 2dB. In this case, it can be considered that the frequency diversity gain is more dominant than the time diversity gain for PSCCH detection performance in our evaluation results. In that point of view, to improve detection performance of PSCCH with smaller number of RBs (e.g. 4 RBs), the number of symbols for PSCCH may needs to be further increased.

Figure 2: Comparison between symbol duration of 3 and 6 for PSCCH.

Observation 8: For a given coding rate, large number of RB for PSCCH can achieve better detection performance compared to small number of RB for PSCCH due to the frequency diversity.

According to the evaluation results, when the symbol duration of PSCCH is 3, the number of RB needs to be 10 RB, in this case, the sub-channel size needs to be larger than 10 RB. The large value of the sub-channel will reduce scheduling flexibility. Furthermore, considering 4 MHz bandwidth with subcarrier spacing of 30 kHz, the number of sub-channels within a resource pool could be extremely small (In this example, the number of sub-channel is 1).

Figure 3: Example of PSCCH/PSSCH mapping.

Alternatively, it can be considered to allow frequency resource of PSCCH is mapped across multiple sub-channels. However, in this case, it is necessary to ensure that the allocated sub-channels for PSSCH always covers frequency resource of PSCCH, which is another scheduling restriction. Furthermore, PSSCH with a single sub-channel will not be supported.

Proposal 2: At least for PSSCH with a single sub-channel allocation, NR sidelink supports that frequency resource of a PSCCH is mapped within a sub-channel.

For efficient resource allocation with a reasonable sub-channel size, it is necessary to reduce the number of RBs to be used for PSCCH. To do this, first of all, it can be considered to increase the symbol duration of PSCCH. However, depending on the symbol duration, tight latency requirement may not be met.

Another approach is to reduce the size of SCI payload to be conveyed on PSCCH. For instance, total SCI payload can be divided into two part: 1st SCI is to help sensing operation of all the UEs and 2nd SCI is used to indicate how to decode PSSCH in details for a legitimate UE based on the L1-destination ID which will be given by the 1st SCI. For instance, the 1st SCI can include 8-bit Layer-1 destination ID, resource allocation field (including 4-bit time gap between initial and retransmission, and 4-bit resource reservation), 3-bit priority field, and 2-bit 2nd SCI indicator to indicate the form/contents of 2nd SCI. In addition, considering sensing operation where UE measure RSRP of PSSCH DMRS, 1st SCI may need to have SCI fields related to DMRS ports and/or pattern. Moreover, when the 2nd SCI is piggybacked on the associated PSSCH resource, it can be considered that MCS field is present in the 1st SCI as well. Considering specification work for channel coding design including distributed CRC of NR polar coding, it is preferred to use 24-bit CRC for both 1st SCI and 2nd SCI. In this case, the total payload size to be conveyed on PSCCH would be roughly 60 bits. Meanwhile, 2nd SCI can include remaining SCI fields required for PSSCH decoding/other purposes (e.g. CQI/RI measurement/reporting) and 24-bit CRC. The 2nd SCI can be mapped across multiple sub-channels depending on PSSCH allocation with smaller symbol duration as shown in Figure 4.

Figure 4: Example of 2nd SCI mapping.

In the last meeting, it is discussed that the possibility of applying CRC masking of 2nd SCI with Layer-1 source ID to reduce payload size of the 2nd SCI. In this case, a RX UE will perform CRC de-masking with some candidates of Layer-1 source ID before CRC check. However, in case of broadcast or connection-less groupcast, a RX UE may not know all the Layer-1 source IDs to be considered, and it would not be feasible to use Layer-1 source ID for CRC masking of the 2nd SCI. Furthermore, if the number of Layer-1 source ID to be considered for CRC (de-)masking, the false alarm probability of PSCCH could be problematic.

Observation 9: Considering specification work for channel coding, it is preferred to reuse 24-bit CRC for both 1st SCI and 2nd SCI if 2-stage SCI is supported.

Observation 10: If Layer-1 source ID is used for CRC masking for 2nd SCI, RX UE needs to know the candidates of CRC masking sequences before CRC check, while it may not know the Layer-1 source ID especially for connection-less groupcast.

Observation 11: Depending on the number of CRC masking sequences, false alarm probability could be significantly high.

Proposal 3: If 2-stage SCI is introduced, it is not supported that Layer-1 source ID is scrambled with CRC of 2nd SCI.

The DMRS of the PSCCH can be designed similar to the NR PDCCH. Three REs per RB are mapped to DMRS. Depending on the time-domain pattern of PSCCH DMRS, it is possible to further increase the number of REs to be used for PSCCH data mapping. To be specific, PSCCH DMRS can be transmitted in a subset of symbols for PSCCH as shown in Figure 5. Considering high speed environment in NR V2X, PSCCH DMRS can be mapped in the middle of the symbol duration for PSCCH.

Proposal 4: For PSCCH DMRS pattern in frequency domain, NR PDCCH DMRS pattern in frequency domain is reused.

- FFS: PSCCH DMRS pattern in time domain.

Figure 5: Example of PSCCH DMRS pattern.

According to the design criteria for PSCCH DMRS sequence generation in LTE, it is taken into account that the sequence randomization between NR Uu link and NR sidelink, multiple PSCCH DMRS can be CDMed for potential collision handling, and all the UEs can decode SCI conveyed on PSCCH at least for sensing operation. Unlike LTE, since NR sidelink uses CP-OFDM only, the DMRS sequence will be pseudo random sequence instead of CAZAK sequence. In this case, it can be considered to carefully redesign the random seed of the NR PDCCH DMRS sequence.

Proposal 5: For PSCCH DMRS sequence generation, the formula for NR PDCCH DMRS sequence is a starting point and the at least following considerations need to be taken into account:

- DMRS sequence randomization between NR Uu link and NR sidelink.

- All the UE can decode PSCCH at least for sensing operation.

- Multiple PSCCH DMRS can be CDMed at least for capacity increment.

In a similar manner, PSCCH scrambling sequence can be designed considering the sequence randomization between NR Uu link and NR sidelink, and all the UEs can decode SCI conveyed on PSCCH at least for sensing operation.

Proposal 6: For PSCCH scrambling sequence generation, the formula for NR PDCCH scrambling sequence is a starting point and the at least followings need to be considered:

- Scrambling sequence randomization between NR Uu link and NR sidelink.

- All the UE can decode PSCCH at least for sensing operation.

Regarding PSCCH transmission scheme, PDCCH transmission scheme can be considered as a starting point. In this case, 1 TX antenna port with transparent diversity scheme can be considered as a baseline.

      - Multiplexing PSCCH and the associated PSSCH

Regarding the transient time according to PSCCH/PSSCH configuration options, RAN4 answers RAN1’s question as follows [3]:

- Questions : RAN1 would like to ask the following questions for options 1A, 1B, and 3 when the total transmit power does not change between the last symbol containing PSCCH and the following symbol: - Q1: Whether transient period is needed (if so, how much) if power spectral density is different between the last symbol containing PSCCH and the following symbol - A1: Yes, in case of RB configuration or total transmit power change, transient period is needed, which could result in phase discontinuity for PSSCH. Basically, RAN4 define transient period as shown in section 6.3.3 in TS38.101-1 between transmit OFF power and transmit ON power symbols (transmit ON/OFF) between continuous ON-power transmissions with power change or RB hopping is applied. - Q2: Whether transient period is needed (if so, how much) if PSCCH and PSSCH use different frequency location and/or resource size in option 1B and 3 - A2: The answer as follow according to PSCCH/PSSCH configurations - Option 1B could result in a different RB configuration (location and/or Size), power spectral density for control than that for data. This could result in a different MPR and AMPR for control and data. Hence, this option needs a transient period. - Option 3: This option will not need any transient period. In RAN4, MPR and AMPR should be derived for the entire slot and a single value of MPR/AMPR should be used for the entire slot. - Q3: Whether there is any impact on the above questions due to different assumptions of amplifiers (e.g., for MIMO purpose, etc.) - A3: In the case where PSCCH is rank-1 and transmitted from one antenna only, while PSSCH is transmitted from both antennas, the transient period will be needed. It will result in phase discontinuity for PSSCH. In the case where PSCCH and PSSCH are transmitted from same the antennas connectors, the answers to the above questions are not impacted. - Q4: Whether there are any other cases that require transient period (if so, how much) - A4: RAN4 can provide additional analysis result once sidelink physical design becomes clearer

In this section, we compare Option 1b and Option 3 in terms of specification work load, transient period, and phase discontinuity.

Considering PSCCH coverage, EPRE boosting can be considered for Option 3 while the total transmit power does not change and PSCCH and PSSCH are transmitted from the same set of antennas to avoid the transient period. In this case, EPRE of PSSCH could be different across different symbols depending on whether symbol contains PSCCH or not. For QAM demodulation, it is necessary to define mechanism to ensure that RX UE knows the PSSCH EPRE ratio between different symbols. In addition, since UE transmit power will be shared by PSCCH and PSSCH, the PSCCH coverage could be restricted especially when the number of allocated sub-channels for PSSCH is large. In this case, the specification work load for SL power control/allocation could be relatively large.

Meanwhile, in case of Option 1b, the specification work for SL power control would be much simple since power control/allocation for PSCCH and PSSCH can be performed independently. Furthermore, since PSCCH transmission can use maximum UE transmit power, it would be beneficial in terms of the coverage. However, in general, Option 1b will cause the transient time due to the different RB allocation between PSCCH and PSSCH.

Regarding multi-rank transmission, if PSCCH is transmitted from one antenna only, and if PSSCH is transmitted from two antennas, both options need to have the transient time. In Option 3, UE transmit power will be unevenly distributed for the region where PSCCH and PSSCH are FDMed to ensure that EPRE of PSSCH from each antenna is the same. It is necessary to investigate its impact on the power control with respect to potential MPR. Alternatively, it can be considered that the concept of virtualization in which PSCCH uses the same number of physical antenna ports with that of PSSCH while the number of logical antenna ports could be different between PSCCH and PSSCH.

For Option 3, there are three possible cases satisfying the definition as in Figure 6. Basically the motivation of Option 3 is to decrease the code rate of PSSCH while PSCCH is transmitted in the former part of the resource for fast decoding. The difference between the second and third cases is the channel for which AGC operation is applied. However, it seems that the motivation of the first case in which part of PSCCH transmission is allowed in the time resources non-overlapping with PSSCH is unclear considering the motivation of Option 3. Considering the required number of RB can be large in Option 3, it is preferred that PSCCH mapping avoid AGC symbol. Otherwise, it would be difficult to design PSCCH to be confined within a sub-channel with reasonable size.

Figure 6: Possible scenarios of supporting Option 3

In addition, we consider pre-emption indication which is for avoiding resource collision, and it can be transmitted via PSCCH or PSFCH. If pre-emption indication is transmitted via PSCCH without associated PSSCH within a slot in option 1b or 3, additional AGC operation may be required. As an alternative to avoid additional AGC operation, dummy signal can be transmitted in PSSCH with corresponding PSCCH. Another approach would be to transmit pre-emption indication via PSFCH for which AGC operation is defined.

Observation 12: Option 3 can avoid the transient time by keeping the total transmit power and restricting antenna to be used for PSCCH/PSSCH transmission.

Observation 13: Option 1b will cause the transient time, but PSCCH coverage can be enhanced.

Observation 14: For Option 3, the motivation of supporting part of PSCCH transmission in the time resources non-overlapping with PSSCH is unclear.

Proposal 7: When PSCCH resource is confined within PSSCH resource (i.e. Option 3), PSCCH symbol starts after symbol used for AGC.

Proposal 8: If pre-emption indication is supported, further investigation is necessary on which resource region (e.g., PSCCH, PSFCH) is used for its transmission considering AGC aspect.

    - PSFCH format for SFCI

Similar to NR Uu, self-contained feedback channel structure can be supported in NR sidelink. If the feedback channel is configured in some ending symbols in a slot, AGC/switching period should be taken into account as mentioned above. For small payload size of HARQ feedback and service with low latency requirement, NR PUCCH format 0 is a starting point for the PSFCH format. In our view, for coverage enhancement, it can be considered that the sequence-based PSFCH format is repeated in more than 2 symbols. Meanwhile, applying time-domain OCC needs to be carefully investigated considering AGC symbol and channel variations in time domain. To be specific, if UE needs to perform AGC operation, and if OCC sequence applied to symbol group containing AGC symbol, the orthogonally of the OCC sequence will be broken.

On the other hand, considering the periodicity of PSFCH resource can be set to be more than one slot, in this case, it can be considered that multiple HARQ feedbacks associated with different PSCCH/PSSCH are simultaneously transmitted via a single PSFCH. Therefore, HARQ feedback payload size could be increased as well. In this case, NR PUCCH format 2 can be a starting point for PSFCH format to support large payload size of HARQ feedback with low latency requirement.

Next, further study is needed to handle the overhead due to additional AGC for PSFCH. For instance, if PSFCH to be FDMed have different symbol duration and different starting symbol index, UE may needs to perform AGC multiple times in a slot. Alternatively, it can be considered to align at least starting symbol index of PSFCH in a given slot.

Proposal 9: NR PUCCH format 0 and NR PUCCH format 2 are starting points (e.g., for service with low latency requirement), and the symbol repetition number larger than 2 can be considered further if the coverage/performance enhancement is necessary.

- FFS how to handle the overhead due to additional AGC for PSFCH.

Regarding groupcast HARQ feedback, when multiple RX UEs transmit NACK in the common PSFCH resource, there could be the destructive channel sum effect which causes an error in detecting NACK. To mitigate this problem, one possible solution is that the randomized sequence per RX UE can be transmitted. We conducted evaluation on NACK-to-ACK error performance considering destructive channel sum effect. To be specific, we assume the carrier frequency, subcarrier spacing, UE velocity, and sequence length to be 6 GHz, 30 kHz, 60 km/h, and 1 RB, respectively. Furthermore, ACK-to-NACK error was assumed to be 0.01 considering NACK-based HARQ feedback mechanism. In order to verify the destructive channel sum effect, the NACK-to-ACK error performance of proposed scheme in which two UEs respectively transmit a randomized sequence was compared to that of baseline scheme in which two UEs transmit the same sequence. In Figure 7, it can be found that the proposed scheme provides better performance in terms of NACK-to-ACK error.

Figure 7: NACK-to-ACK error performance considering destructive channel sum effect

Also when the threshold is used for the energy detection of NACK transmission(s) from RX UE(s), further study is necessary on how to guarantee the NACK detection performance regardless the fluctuation of external interference. For example, to resolve this problem, the resource for determining the threshold (i.e., null RE) can be configured as described in Figure 8. In other words, by using null RE, the TX UE can determine the threshold that is more appropriate to a given channel environment. This means that it is possible to satisfy the minimum requirement even under time-varying channel environment.

Figure 8: Example of null RE for NACK-based HARQ transmission.

Observation 15: For only NACK based SL HARQ feedback in groupcast,

- Randomized sequence selection per receiver UE could be beneficial to mitigate the destructive channel sum effect of HARQ-NACK transmissions from multiple receiver UEs.

- Using null resource element could be beneficial to determine the presence of HARQ-NACK transmissions for receiver UEs.

Regarding multiplexing PSFCH with PSCCH/PSSCH, it is agreed that at least TDM is supported at least for transmission perspective of a UE. It is motivated that non-contiguous transmission of multiple sidelink channel/signal will cause high specification work especially on power control, and high UE complexity while there is no case where non-contiguous UL transmission in NR Uu link. In that point of view, it is natural to support TDM between PSCCH/PSSCH and PSFCH in a slot regardless of PSFCH format or symbol duration of PSFCH format. Next, considering FDM between PSCCH/PSSCH and PSFCH for system/resource pool perspective, it may need to adopt additional AGC during the reception of PSSCH before the duration in which PSFCH is FDMed. If PSFCH to be FDMed have different symbol duration, AGC could be problem during PSFCH decoding. Thus, fixed symbol duration is used for a given PSFCH format in a given slot. Another issue is collision avoidance between PSSCH and PSFCH. Because a certain PSSCH can be transmitted until the end of a slot, the resource of PSFCH needs to be selected in order to avoid the resource of PSSCH accordingly.

Observation 16: To support FDM between PSCCH/PSSCH and PSFCH in a resource pool, it is necessary to carefully investigate how to reduce additional AGC during PSSCH reception and how to avoid collision between PSCCH/PSSCH and PSFCH.

Alternatively, it can be considered that it is not specified how to handle the above issues to support FDM of PSCCH/PSSCH with PSFCH in the perspective of resource pool. In other words, it is up to network choice whether PSCCH/PSSCH resources are FDMed with PSFCH resource.

Proposal 10: For reception perspective of a UE, it is assumed not to perform AGC in the middle of PSSCH reception.

Proposal 11: The ending symbol location/index of PSSCH is (pre)configured per resource pool.

    - PSSCH and PSSCH DMRS design

According to 5G V2X WID, PSSCH transmission with up to two antenna ports is supported. Meanwhile, in the last meeting, it was discussed that the number of TB for a PSSCH. In our view, to support 2 TB for a PSSCH, it is necessary to carefully investigate the impact on SCI design and SFCI design. To be specific, SCI may need to have two set of MCS, NDI, and RV. In this case, SCI payload size could be further increased and it can affect the PSCCH coverage. In a similar manner, HARQ feedback payload size would be twice compared to 1 TB case. Especially for the case where a single PSFCH include multiple HARQ feedback for multiple PSCCH/PSSCH, it can make the PSFCH coverage worsened. Meanwhile, this issue was already well discussed in NR Uu link design, and the result is that 1 TB is supported up to 4 layers. Considering NR sidelink supports up to two antenna ports, the benefit of having 2 TB for a PSSCH is unclear.

Proposal 12: Confirm the following working assumption:

- Working assumption:

  - Transmission of 1 TB with up to 2 layers in a PSSCH is supported.

In NR structure, two DMRS types are supported for PDSCH/PUSCH DMRS. DMRS type 1 targets to cover up to roughly 1000ns delay spread (which cause frequency selectivity). Meanwhile, DMRS type 2 targets to support MU-MIMO and more antenna ports (12 APs). However, in NR V2X structure, the number of antenna ports will be limited (e.g. up to 2), and MU-MIMO is not a main target. Thus, for a carrier with a given numerology, there is no clear motivation/benefit to support multiple DM-RS patterns in frequency domain for PSSCH. Meanwhile, considering that PSCCH resource will be confined within PSSCH resource, for PSSCH DMRS pattern in time-domain design, it is necessary to make a decision on the form of PSCCH especially on symbol duration in advance.

Observation 17: For a carrier with a given numerology, there is no clear motivation/benefit to support multiple DM-RS patterns in frequency domain for PSSCH.

For fast decoding and low latency, DMRS is located in the front symbol (i.e. front-loaded DMRS) in NR structure. In addition, for high speed UE to cope with the high Doppler Effect, additional DMRS can be configured and the position and number of front-loaded and additional DMRS symbols are listed in TS 38.211. In LTE V2X structure, there are 4 DMRS symbols located in one subframe. This DMRS structure of LTE V2X (i.e. 4-V (4 vertical) DMRS structure) already can be configured by using the listed table in NR. As described above, the flexible subcarrier spacing and frequency range (i.e., carrier frequency) should be considered in NR V2X structure. One thing to consider in the sidelink is that the symbol position of the DMRS can be shifted because the first symbol DMRS can be affected by AGC. In addition, considering AGC issue, it would be beneficial to align at least starting symbol of PSSCHs in a given slot. Moreover, further study is needed for the symbol duration of PSSCH (e.g., fixed or variable symbol duration, range of the possible values).

Proposal 13: For NR PSSCH DMRS pattern in frequency domain, NR PDSCH DMRS configuration 1 is reused.

- FFS: NR PSSCH DMRS pattern in time domain

Under the condition of high speed and high frequency range, the H (horizontal) DMRS structure (which is similar to PT-RS structure of NR) can be used for NR V2X. Furthermore, for the flexibility of NR V2X structure, both V-type and H-type DMRS structure can be used by configuring PT-RS properly. The phase tracking for the high speed UE can be performed by using the H-type DMRS and the signal can be demodulated by using the V-type DMRS. These DMRS/PT-RS pattern used for PSSCH can be indicated via PSCCH or (pre)configuration.

    - Sidelink CSI-RS design

According to the agreements, sidelink CSI-RS will be transmitted together with PSSCH for unicast, and the physical resources for sidelink CSI-RS will be confined within PSSCH resource in both time and frequency domain. Considering motivation of having sidelink CSI-RS for CQI/RI reporting, the number of antenna ports for sideilnk CSI-RS could be different with that of PSSCH and could be up to two. According to DL CSI-RS in NR Uu link, depending on the configuration for CSI-RS, DL CSI-RS can be mapped on some portion of RBs (e.g. every 2 RB) to reduce overall CSI-RS density. Furthermore, for 2 antenna ports, DL CSI-RS resources for different antenna ports are mapped in CDM manner.

Regarding multiplexing sidelink CSI-RS resources of different CSI-RS, FDM would be beneficial to boost EPRE of the CSI-RS. In this case, the accuracy of the CQI/RI reporting for sidelink could be further improved at the expense of spectral efficiency loss of PSSCH due to the reduced number of data RE. On the other hand, CDM can be considered to increase spectral efficiency of PSSCH at the expense of the degradation on the accuracy of the CQI/RI reporting for sidelink.

Observation 18: On multiplexing scheme (e.g. FDM, CDM) for sideilnk CSI-RS resources for different antenna ports and density of sidelink CSI-RS resources, it is necessary to investigate the impact on the accuracy of CQI/RI reporting for sidelink and the spectral efficiency of PSSCH.

Regarding density of sidelink CSI-RS, it is necessary to carefully investigate how amount of resources are needed for accurate CQI/RI measurement even if the number of allocated RBs for PSSCH is small. In this case, it can be considered that only sidelink CSI-RS is mapped on a certain symbol without multiplexing with PSSCH data RE to increase overall transmit power of the sidelink CSI-RS. Furthermore, sidelink CSI-RS would be mapped on every RBs of PSSCH. Meanwhile, when the number of allocated RBs for PSSCH is large or modulation order of the PSSCH is high, high density of sidelink CSI-RS would be inefficient in terms of spectral efficiency of the PSSCH. Alternatively, it can be considered that the pattern of sidelink CSI-RS can be dynamically changed implicitly or explicitly via SCI.

Observation 19: On physical resource mapping for sidelink PT-RS, it is necessary to investigate the impact on spectral efficiency of a TB on PSSCH considering PT-RS overhead, and channel estimation performance after phase compensation.

Proposal 14: For sidelink CSI-RS transmission with multiple antenna ports, down select one of followings for multiplexing scheme:

- Option 1: FDM of sidelink CSI-RS resources with different antenna ports.

- Option 2: CDM of sidelink CSI-RS resources with different antenna ports.

- FFS: Multiplexing scheme is (pre)configured or indicated by SCI.

- FFS: Whether sidelink CSI-RS is FDMed with PSSCH in the same symbol.

    - Sidelink PT-RS design

In FR2, PSSCH can include sidelink PT-RS for phase compensation. Considering the case where PSCCH resource is confined within PSSCH resource (i.e. Option 3), PSSCH resource can be partitioned into two parts: one is RB or sub-channel group with overlapping with PSCCH resource, PT-RS may need to be mapped on the symbol duration where PSCCH and PSSCH are FDMed for accurate channel estimation especially when the symbol duration of PSCCH is relatively large (e.g. 6 symbols). In this case, to minimize PT-RS overhead, it can be considered that RB or subcarrier used for PT-RS is selected from RBs or subcarriers which are not overlapped with PSCCH in frequency domain. Moreover, on the frequency gap between PT-RS resources needs to be determined considering the frequency range of PSCCH.

Observation 20: On physical resource mapping for sidelink PT-RS, it is necessary to investigate the impact on spectral efficiency of a TB on PSSCH considering PT-RS overhead, and channel estimation performance after phase compensation.

    - Bandwidth part and resource pool

In RAN1 AdHoc 1901 meeting, it was agreed that configuration for SL BWP is separated from Uu BWP configuration signaling. Furthermore, one remaining issue is the relation between SL and active/initial Uu BWP. Considering dynamic active BWP switching in NR Uu link, depending on UE capability, the switching delay of roughly 1 ms or 3ms is assumed for BWP switching. Meanwhile, UE may needs to operate in both active Uu BWP and SL BWP. In this case, the switching delay between active Uu BWP and SL BWP is not desirable considering latency requirement of Uu/SL services/applications and Uu/SL resource utilization. To be specific, on Uu/SL resource utilization, UE does not receive/transmit any signals during the transient time for BWP switching. In this case, frequent BWP switching will make resource utilization inefficient.

To avoid these switching delay, first of all, it is assumed that the numerology of configured SL BWP is the same as that of active UL BWP in the same carrier at a given time. Next, it can be further considered that RF retuning is not needed to switch between active UL BWP and configured SL BWP. In other words, it can be considered that UE’s RF setting covers both active UL BWP and configured SL BWP. In this case, even though SL BWP and active UL BWP have different center frequency of BWP and BWP size, UE may not apply the switching delay.

Regarding active DL BWP, for paired spectrum, it can be taken into account that separate RF chains between active DL BWP and configured SL BWP as in LTE V2X. On the other hand, for unpaired spectrum, it can be assumed that UE’s RF setting covers both active DL BWP and configured SL BWP together with active UL BWP. Note that, in NR Uu link, for unpaired spectrum, UE expects that the center frequency of active DL BWP is aligned with that of active UL BWP.

In NR Uu link, initial BWP and other configured BWP can be configured with different numerologies. Especially, since initial DL BWP does not support SCS of 60 kHz, if we always assume that both initial BWP and other configured BWP always have the same numerologies, extended CP cannot be used anymore. In that point of view, it is not desirable to restrict the same numerology setting between initial Uu BWP and active Uu BWP(s). Meanwhile, UE may operate in initial DL BWP to monitor broadcast messages, and operate in initial UL BWP to perform RACH procedure. In addition, if default BWP is not configured, UE may switch initial DL/UL BWP if bwp-inactivityTimer expires for power saving. In our view, network can configure default BWP to ensure that there is no switching delay between the default BWP and configured SL BWP. In those points of views, rather than handling issues related to switching between initial Uu BWP and SL BWP, it can be considered that UE expects that the configured SL BWP is deactivated if initial Uu BWP has different numerology with SL BWP and if the initial Uu BWP is active. In this case, UE can (re)start operating in SL BWP after switching to active Uu BWP of which numerology is the same as that of SL BWP.

Proposal 15: UE is not expected to have switching latency between SL BWP and active Uu BWP(s) in the same carrier at a given time.

Observation 21: It is not desirable to restrict the same numerology setting between initial Uu BWP and active Uu BWP(s).

Proposal 16: UE assumes that SL TX/RX operation is deactivated when it switches to initial Uu BWP of which numerology is different from SL BWP.

Since only one SL BWP is configured in a carrier for NR V2X UE, in our view, the information of SL BWP does need to be informed by PSBCH. Instead, (pre)configuration can indicate starting RB and length of SL BWP in terms of ARFCN (absolute radio frequency channel number) and the offset with respect to the indicated ARFCN. Meanwhile, since SL-SSB will be confined within SL BWP, it can be considered that the starting RB of SL-SSB is (pre)configured in terms of the offset with respect to the lowest RB of SL BWP to efficiently reduce signaling overhead.

Regarding resource pool, at least UL symbols configured by cell-specific higher layer signaling in NR Uu can be used for NR SL. Further discussion is needed whether to use flexible symbols for NR SL considering the dynamic nature of the usage for flexible symbols in NR Uu. In RAN1 AdHoc 1901 meeting, it was agreed that down select of two options is needed for frequency domain resources of a resource pool for PSSCH. For a given carrier, SL BWP will be fixed in frequency domain while Uu BWP will be RRC configured. In this case, the overlapping portion of SL BWP and active UL BWP can be varying depending on the Uu BWP configuration. For instance, SL BWP can be super set of active UL BWP in frequency domain. In this case, considering resource utilization efficiency, it would be beneficial to allow that resource pool can consists of non-contiguous PRBs. Meanwhile, a single PSCCH or PSSCH mapping does not need to be non-contiguous.

Proposal 17: In shared licensed carrier, at least UL symbols configured by cell-specific higher layer signaling in NR Uu is used for NR SL.

Proposal 18: It is supported that the resource pool can consist of non-contiguous PRBs (e.g., for the increment of resource utilization efficiency in the shared licensed carrier).

Since resource pool will be confined within SL BWP, the starting RB and length of the resource pool can be (pre)configured in terms of the offset with respect to the lowest RB of SL BWP and the number of RB or sub-channels.

    - AGC and switching period consideration

In the LTE V2X, the first symbol and last symbol in one TTI are used to AGC and TX/RX switching operation, respectively. Thus, the first and the last symbols among 14 symbols are not used for demodulation. However if the AGC or TX/RX switching of 1 symbol is considered in the NR structure, the overhead becomes too large. According to tentative assumption of AGC/switching period agreed in RAN1 #94, AGC/switching period can be much less than one symbol depending on numerology. For example, AGC and TX/RX switching times are 15 and 13 us respectively for FR1 in the RAN1 tentative assumption, and each of them is shorter than the half of the one OFDM symbol time of 30 kHz subcarrier spacing. This implies that even when AGC or TX/RX switching takes place in a symbol, only half of the symbol time would be needed and the other half symbol can be used for data or reference signal transmission/reception. To enhance efficiency, the comb type RE mapping on the AGC and/or TX/RX switching symbol can be considered to utilize the partial REs for data or RS mapping in the guard symbols. Another way is to place the AGC and the switching period in one symbol. For example, the switching period is placed in the front part of the first symbol in the next slot and the AGC period is placed in the last part of the first symbol in the slot.

Proposal 19: Comb type RE mapping on AGC and/or TX/RX switching symbol or placing AGC and switching period within one symbol can be considered to enhance resource utilization.

Comb type RE mapping is especially useful when the number of OFDM symbols constituting each channel is small. When control and data are TDMed, it can be greatly affected by AGC when control is mapped to the first symbol. Figure 9 compares the link performance when the first symbol is affected by AGC when PSCCH is transmitted on 2 or 3 OFDM symbols. In two OFDM symbols case, about 3dB gain at 1% BLER for the proposed comb type RE mapping is achieved and in three OFDM symbol case, about 1dB gain is achieved.

(a) # of OFDM symbols: 2, RB size: 8 RBs          (b) # of OFDM symbols: 3, RB size: 6 RBs

Figure 9: PSCCH BLER performance comparison with or without comb-like RE mapping (assumption: carrier frequency: 6 GHz, mobility: 60 km/h, channel coding: polar, payload size: 48 bits)

Meanwhile, it is necessary to investigate how to minimize performance degradation on PSSCH decoding due to puncturing AGC symbol(s). To be specific, PSSCH decoding performance could be further optimized depending on which part of coded bits of a TB is mapped on AGC symbol(s) to be punctured. First of all, considering LDPC coding, it would be beneficial that the systematic bits of each code block of a TB is not punctured. Next, depending on the TB size, all the coded bits of a certain code block of a TB could be mapped on AGC symbol(s). In this case, when RX UE does not use AGC symbol(s) for demodulation, the RX UE shall fail to decode the code block of a TB associated with AGC symbol(s), and it cause PSSCH decoding failure even through the UE successes to decode other code blocks. In that point of view, it would be necessary to avoid the case where AGC symbol(s) contains large portion of coded bits of a certain set of code block(s) of a TB. For simplicity, it can be considered that AGC training symbol is mapped on AGC period. However, depending on the scenario, it is not always needed that RX UE performs AGC. In this case, it can be considered that some portion of each coded code block of a TB is mapped on AGC symbol(s).

Observation 22: In the perspective of LDPC decoding performance, it would be beneficial that the systematic bits of a TB is not mapped on AGC symbol.

Observation 23: When TB size is large, it is not preferred that all the coded bits of a certain code block is punctured due to AGC operation.

Proposal 20: For physical resource mapping of a TB conveyed on PSSCH, followings need to be considered:

- Which part of coded bits (e.g. systematic bits or parity bits) of a code block of a TB is mapped on AGC symbol(s).

- Which code block(s) of a TB is mapped on AGC symbol(s).

- Conclusions

In this contribution, some aspects of physical layer structure for NR V2X sidelink was discussed. Based on the above discussion, our proposals and observations are summarized as follows:

Observation 1: For implicit/explicit indication of cast type, the technical benefit of having such indication is clarified first.

Observation 2: For asynchronous HARQ operation, regardless of whether HARQ feedback is enabled or which cast type is associated, SCI needs to include Layer-1 source ID, Layer-1 destination ID, HARQ process number, NDI, and RV fields.

Observation 3: It would be beneficial to implicitly derive the starting sub-channel of PSSCH based on the associated PSCCH resource in terms of SCI overhead reduction.

Observation 4: Considering MU-MIMO operation is not a main target for NR sidelink, DMRS sequence initialization field may not be needed in SCI, and 1-bit field is needed to indicate rank of PSSCH.

Observation 5: Since CQI/RI reporting is not supported for broadcast and groupcast, SCI for broadcast and groupcast would not include CQI/RI reporting field, antenna port/rank field for PSSCH.

Observation 6: To adjust HARQ feedback payload size efficiently, it can be considered that SAI (sidelink assignment index) field in SCI indicates the information on the number of PSCCH/PSSCH transmissions associated with the same PSFCH slot.

Observation 7: When the number of symbol for PSCCH is 3, to achieve BLER of 10-2 at small SINR region (e.g. 2.5dB~5dB), the required number of RB for PSCCH is at least 10.

Observation 8: For a given coding rate, large number of RB for PSCCH can achieve better detection performance compared to small number of RB for PSCCH due to the frequency diversity.

Observation 9: Considering specification work for channel coding, it is preferred to reuse 24-bit CRC for both 1st SCI and 2nd SCI if 2-stage SCI is supported.

Observation 10: If Layer-1 source ID is used for CRC masking for 2nd SCI, RX UE needs to know the candidates of CRC masking sequences before CRC check, while it may not know the Layer-1 source ID especially for connection-less groupcast.

Observation 11: Depending on the number of CRC masking sequences, false alarm probability could be significantly high.

Observation 12: Option 3 can avoid the transient time by keeping the total transmit power and restricting antenna to be used for PSCCH/PSSCH transmission.

Observation 13: Option 1b will cause the transient time, but PSCCH coverage can be enhanced.

Observation 14: For Option 3, the motivation of supporting part of PSCCH transmission in the time resources non-overlapping with PSSCH is unclear.

Observation 15: For only NACK based SL HARQ feedback in groupcast,

- Randomized sequence selection per receiver UE could be beneficial to mitigate the destructive channel sum effect of HARQ-NACK transmissions from multiple receiver UEs.

- Using null resource element could be beneficial to determine the presence of HARQ-NACK transmissions for receiver UEs.

Observation 16: To support FDM between PSCCH/PSSCH and PSFCH in a resource pool, it is necessary to carefully investigate how to reduce additional AGC during PSSCH reception and how to avoid collision between PSCCH/PSSCH and PSFCH.

Observation 17: For a carrier with a given numerology, there is no clear motivation/benefit to support multiple DM-RS patterns in frequency domain for PSSCH.

Observation 18: On multiplexing scheme (e.g. FDM, CDM) for sideilnk CSI-RS resources for different antenna ports and density of sidelink CSI-RS resources, it is necessary to investigate the impact on the accuracy of CQI/RI reporting for sidelink and the spectral efficiency of PSSCH.

Observation 19: On physical resource mapping for sidelink PT-RS, it is necessary to investigate the impact on spectral efficiency of a TB on PSSCH considering PT-RS overhead, and channel estimation performance after phase compensation.

Observation 20: On physical resource mapping for sidelink PT-RS, it is necessary to investigate the impact on spectral efficiency of a TB on PSSCH considering PT-RS overhead, and channel estimation performance after phase compensation.

Observation 21: It is not desirable to restrict the same numerology setting between initial Uu BWP and active Uu BWP(s).

Observation 22: In the perspective of LDPC decoding performance, it would be beneficial that the systematic bits of a TB is not mapped on AGC symbol.

Observation 23: When TB size is large, it is not preferred that all the coded bits of a certain code block is punctured due to AGC operation.

Proposal 1: For NR sidelink, the set of SCI fields includes at least followings.

- SCI format can consist of parts of the above SCI fields depending on the (pre)configuration.

- FFS: whether all the SCI fields in a SCI format are conveyed on PSCCH.

Table 1: Example of the set of SCI fields with tentative size

| SCI field | Broadcast |  | Groupcast |  | Unicast |  |
| --- | --- | --- | --- | --- | --- | --- |
| L1-source ID | [8] |  | [8] |  | [8] |  |
| L1-destination ID | [8] |  | [8] |  | [8] |  |
| Time-and-Frequency resource location | Frequency resource assignment | log2NsubSLNsubSL+12 | Frequency resource assignment | log2NsubSLNsubSL+12 | Frequency resource assignment | log2NsubSLNsubSL+12 |
|  | Time gap between inTx and ReTx | [4] | Time gap between inTx and ReTx | [4] | Time gap between inTx and ReTx | [4] |
|  | Resource reservation | [4] | Resource reservation | [4] | Resource reservation | [4] |
| HARQ process number | [4] |  | [4] |  | [4] |  |
| NDI | 1 |  | 1 |  | 1 |  |
| RV | 2 |  | 2 |  | 2 |  |
| MCS | 5 |  | 5 |  | 5 |  |
| Priority field | [3] |  | [3] |  | [3] |  |
| CQI/RI reporting field |  |  |  |  | [2] |  |
| Antenna port(s) for PSSCH |  |  |  |  | [1] |  |
| Sidelink assignment index |  |  | [2] |  | [2] |  |

Proposal 2: At least for PSSCH with a single sub-channel allocation, NR sidelink supports that frequency resource of a PSCCH is mapped within a sub-channel.

Proposal 3: If 2-stage SCI is introduced, it is not supported that Layer-1 source ID is scrambled with CRC of 2nd SCI.

Proposal 4: For PSCCH DMRS pattern in frequency domain, NR PDCCH DMRS pattern in frequency domain is reused.

- FFS: PSCCH DMRS pattern in time domain.

Proposal 5: For PSCCH DMRS sequence generation, the formula for NR PDCCH DMRS sequence is a starting point and the at least following considerations need to be taken into account:

- DMRS sequence randomization between NR Uu link and NR sidelink.

- All the UE can decode PSCCH at least for sensing operation.

- Multiple PSCCH DMRS can be CDMed at least for capacity increment.

Proposal 6: For PSCCH scrambling sequence generation, the formula for NR PDCCH scrambling sequence is a starting point and the at least followings need to be considered:

- Scrambling sequence randomization between NR Uu link and NR sidelink.

- All the UE can decode PSCCH at least for sensing operation.

Proposal 7: When PSCCH resource is confined within PSSCH resource (i.e. Option 3), PSCCH symbol starts after symbol used for AGC.

Proposal 8: If pre-emption indication is supported, further investigation is necessary on which resource region (e.g., PSCCH, PSFCH) is used for its transmission considering AGC aspect.

Proposal 9: NR PUCCH format 0 and NR PUCCH format 2 are starting points (e.g., for service with low latency requirement), and the symbol repetition number larger than 2 can be considered further if the coverage/performance enhancement is necessary.

- FFS how to handle the overhead due to additional AGC for PSFCH.

Proposal 10: For reception perspective of a UE, it is assumed not to perform AGC in the middle of PSSCH reception.

Proposal 11: The ending symbol location/index of PSSCH is (pre)configured per resource pool.

Proposal 12: Confirm the following working assumption:

- Working assumption:

  - Transmission of 1 TB with up to 2 layers in a PSSCH is supported.

Proposal 13: For NR PSSCH DMRS pattern in frequency domain, NR PDSCH DMRS configuration 1 is reused.

- FFS: NR PSSCH DMRS pattern in time domain

Proposal 14: For sidelink CSI-RS transmission with multiple antenna ports, down select one of followings for multiplexing scheme:

- Option 1: FDM of sidelink CSI-RS resources with different antenna ports.

- Option 2: CDM of sidelink CSI-RS resources with different antenna ports.

- FFS: Multiplexing scheme is (pre)configured or indicated by SCI.

- FFS: Whether sidelink CSI-RS is FDMed with PSSCH in the same symbol.

Proposal 15: UE is not expected to have switching latency between SL BWP and active Uu BWP(s) in the same carrier at a given time.

Proposal 16: UE assumes that SL TX/RX operation is deactivated when it switches to initial Uu BWP of which numerology is different from SL BWP.

Proposal 17: In shared licensed carrier, at least UL symbols configured by cell-specific higher layer signaling in NR Uu is used for NR SL.

Proposal 18: It is supported that the resource pool can consist of non-contiguous PRBs (e.g., for the increment of resource utilization efficiency in the shared licensed carrier).

Proposal 19: Comb type RE mapping on AGC and/or TX/RX switching symbol or placing AGC and switching period within one symbol can be considered to enhance resource utilization.

Proposal 20: For physical resource mapping of a TB conveyed on PSSCH, followings need to be considered:

- Which part of coded bits (e.g. systematic bits or parity bits) of a code block of a TB is mapped on AGC symbol(s).

- Which code block(s) of a TB is mapped on AGC symbol(s).

- Reference

[1] RAN1#96bis Chairman’s Notes, Xian, China, 8th – 12nd April, 2019.

[2] R1-1907018, “Discussion on physical layer procedure for NR sidelink,” LG Electronics, Reno, USA, 13th – 17th May, 2019.

[3] R4-1902514, “Reply LS on NR UE transient period in sidelink channels,” RAN4, Athens, Greece, 25th February – 1st March, 2019.