3GPP TSG RAN WG1 Meeting RAN1#97	R1-1907074

Reno, US, May 13th-17th, 2019

Source:	Ericsson

Title:	On CSI enhancements for MU-MIMO

Agenda Item:	7.2.8.1

Document for:	Discussion and Decision

# Introduction

A large part of the enhanced Type II codebook design has already been finalized, at least for ranks 1-2. What remains are some minor issues, parameter settings for rank 3-4 and corresponding UCI design.

We summarize our understanding of the agreements achieved so far as well as open discussion points as follows.

Summary of agreements:

- Precoder vectors for all FD-units/subbands for a layer is given by size-P×N3 matrix W=w0 …w(N3-1)=W1W2WfH

  - P=2N1N2= #SD dimensions (number of antenna ports)

  - N3=NSB×R = #FD dimensions (number of PMI subbands)

    - The value R={1,2} (PMI subband size indicator) is RRC configured

      - FFS if R=2 is associated with UE capability or processing relaxations

    - NSB is the number of CQI subbands

    - This applies for NSB×R≤13,

      - For NSB×R>13, downselect between padding, segmentation or same behavior

      - FFS how to handle edge subbands

  - Precoder normalization: the precoding matrix for given rank and unit of N3 is normalized to norm 1/sqrt(rank)

- Spatial domain (SD) compression by W1

  - L spatial domain basis vectors (mapped to the two polarizations, so 2L in total) selected

  - Compression in spatial domain using  W1=v0v1…vL-100v0v1…vL-1, where vii=0L-1 are N1N2×1 orthogonal DFT vectors (same as Rel. 15 Type II)

  - SD-basis selection is layer-common

  - The value of L=2,4,6 (number of “beams”, SD-basis vectors) is RRC configured

    - L=6 only supported for limited parameter setting:

      - 32 Tx, R=1, p,β∈14,14,14,12,14,34,(12,14)

      - Optional UE capability, FFS UE processing relaxations

- Frequency-domain (FD) compression by Wf

  - Compression via Wf=fk0 fk1…fkM-1, where fkmm=0M-1 are M size-N3×1 orthogonal DFT vectors

  - Nominal number of FD-components M=p×N3/R, with p={14,12} is RRC configured

    - For layers 0 and 1, nominal value of M applied directly

    - For layers 3 and 4, nominal value of M is mapped to smaller actual value

  - FD-basis selection is layer-specific

  - FFS if FD-bases for layers are selected directly or using two-step procedure

- Linear combination by W2  (for a layer)

  - W2=c0,0⋯c0,M-1⋮⋱⋮c2L-1,0⋯c2L-1,M-1  is composed of K=2LM  linear combination coefficients

  - Coefficient subset selection

    - Only a subset KNZ≤K0<2LM coefficients are non-zero and reported

      - The 2LM-KNZ non-reported coefficients are zero and not reported

      - The nominal maximum number of non-zero coefficients per layer K0=β×2LM

        - β∈{14,12,34} is RRC configured

        - This applies for RI={1,2}

      - For RI={3,4}, total max # NZ coefficients across all layers ≤ 2K0

        - FFS if any restrictions on division of coefficients among layers

    - Coefficient subset selection is indicated with a size-2LM bitmap with KNZ ones in UCI Part 2

    - Coefficient subset selection is layer-specific

    - Indication of KNZ for all layers is given in UCI Part 1, so that UCI Part 2 payload can be known

      - FFS if joint or separate indication across layers

  - Coefficient quantization according to cl,m=preflL×pl,m×φ(l,m)

  - Strongest coefficient: An indicator for the strongest coefficient index, SCI, l*,m*

    - Strongest coefficient cl*,m*=1 (hence its amplitude/phase is not reported)

    - FFS bitwidth of SCI indicator

  - Two polarization-specific reference amplitudes pref0, pref1

    - For the polarization associated with the strongest coefficient prefl*L=1 and hence not reported

    - For the other polarization, the reference amplitude is quantized to 4 bits:

      - The alphabet is {1,1214,1414,1814,…,121414 , "reserved"} (-1.5dB step size)

  - For cl,m,  l,m≠(l*,m*):

    - For each polarization, differential amplitudes pl,m of the coefficients calculated relative to the associated polarization-specific reference amplitude and quantized to 3 bits

      - The alphabet is {1,12,12,122,14,142,18,182} (-3dB step size)

    - Each phase φ(l,m) is quantized to either 8PSK (3-bit) or 16PSK (4-bit) (configurable)

## Open issues

In our understanding, the following open issues remain to be resolved:

- Higher rank codebook

  - Decide FD-basis parametrization

  - Decide NZC distribution among layers

- UCI design

  - Resolve FFSs

    - Indication of NNZC in UCI Part 1

    - Strongest coefficient indication

    - Bitmap design

    - FD basis selection

  - Address new proposals

    - M’

    - BSI

- CSI omission procedure

- Reducing supported combinations of compression parameters

- Decide on the value of N3 for NSB×R > 13

- Associated UE capability

- Codebook subset restriction (CBSR)

We present our view on theses open issues in this contribution. In addition, we present our view on potential CBSR schemes for the Type II CSI enhancement in [1]. A discussion on CSI omission procedure is given in [2]. Evaluation results on the value of N3 is given in [3]. Evaluation results and discussion regarding reducing the number of supported parameter combinations is given in [2].

# High rank extension

## FD basis parametrization

In RAN1#96bis, MIMO experts managed to downselect to the following FD-basis parameter settings for the high-rank codebook to the following three alternatives:

| Alt2B RI; Layer; L; p. 1; 0; x0; y0. 2; 0. 1. 3; 0; y0. 1. 2; y2. 4; 0; y0. 1. 2; y2. 3. | Alt3C RI; Layer; L; p. 1; 0; x0; y0. 2; 0. 1. 3; 0; v0. 1. 2. 4; 0. 1. 2. 3. |
| --- | --- |
| Alt6E RI; Layer; L; p. 1; 0; x0; y0. 2; 0. 1. 3; 0; v3,0. 1; v3,1. 2; v3,2. 4; 0; v4,0. 1; v4,1. 2; v4,2. 3; v4,3. |  |

As can be seen, Alt 6E is the most flexible solution, where each RI and layer hypothesis potentially can be assigned a different number of FD-basis vectors. Alt 2B on the other hand is a bit simpler, where the same number of configured FD-basis vectors for RI=1,2 is used for the first two layers regardless of rank, and a smaller number of FD-basis vectors is used the last two layers. While this solution allows the for rank-nested PMI search where the coefficients for the first two layers need not be re-calculated for RI=3,4, there may be concern with the overhead of the bitmaps, which is increased for RI=3,4, compared to RI=2. After all, the motivation for reducing the FD-basis size for higher rank reporting was that to reduce the bitmap size. Such a reduction in bitmap size is achieved by Alt 3C, where vo=y02 can be selected to assure the bitmap size for RI=3,4, does not exceed that of RI=2.

To evaluate the performance, we consider the following parametrizations for the three alternatives:

| Alt 2B RI; Layer idx; L; P; K0. 1; 0; 4; p0; K0. 2; 0. 1. 3; 0; p0; 45K0. 1. 2; p0/2; 25K0. 4; 0; p0; 23K0. 1. 2; p0/2; 13K0. 3. | Alt 3C RI; Layer idx; L; p; NNZC. 1; 0; 4; p0; K0. 2; 0. 1. 3; 0; p0/2; 23K0. 1. 2. 4; 0; K02. 1. 2. 3. | Alt 6E RI; Layer idx; L; p; K0. 1; 0; 4; p0; K0. 2; 0. 1. 3; 0; p0; K0. 1; p0/2; K02. 2. 4; 0; p0/2; K02. 1. 2. 3. |
| --- | --- | --- |

The NZ coefficient allocation is fixed per layer and follows the principle that the number of coefficients scales linearly with the number of allocated FD-basis vectors. Performance is evaluated for L=4,p0=12 and K0=β×2LM ,β={14,12} and plotted against rank-4 overhead in Figure 1 below. As expected, Alt 2B results in worst performance/overhead tradeoff due to the additional bitmap size, while Alt 3C and 6E have quite similar performance (the rank-4 overhead is the same in this case).  Thus, the additional complexity of Alt 6E does not seem motivated.

Alt 3C and Alt 6E have similar performance/overhead tradeoff while Alt 2B is inferior

Figure 1: Comparison between Alt 2B, Alt 3C and Alt 6E

We therefore propose to adopt Alt 3C. The next issue is the value range for v0. Since y0={12,14} has already been agreed and it seems reasonable to use half of the FD-basis vectors for RI=3,4 compared to RI=1,2, we suggest adopting v0={14,18} as the value range.

For FD-basis parametrization, support Alt 3C with a value range v0={14,18}

## Distribution of non-zero coefficients

In RAN1#96bis, the following was agreed:

Agreement

On RI=3-4 extension, with the agreed total max # NZ coefficients across all layers ≤ 2K0 where the K0 value (hence β) set for RI{1,2}, the scheme for determining the # NZC per layer will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt0. KNZ,i is unrestricted as long as i=0RI-1KNZ,i≤2K0i=0RI-1KNZ,i≤2K0

- Alt1. KNZ,i≤K0 as long as i=0RI-1KNZ,i≤2K0i=0RI-1KNZ,i≤2K0

Regardless of which alternative is selected, the UE can perform either a joint determination of the # NZ coefficients across the layers for RI=3,4 or, it can a priori assign a fixed distribution of coefficients among layers, e.g. [K0/2 K0/2 K0/2 K0/2] and determine the coefficients independently for each layer. This is up to UE implementation. The restriction of Alt 1 is somewhat arbitrary since even if there was such a restriction in the specification, the UE could ignore it and anyway signal arbitrary # NZ coefficients distribution among layers by setting the corresponding bitmaps.

However, with the completely unrestricted distribution of coefficients according to Alt 0, the UE could create a “pseudo-rank-1” precoder with increased PMI granularity by selecting RI=3 and assigning [2K0-2 1 1] coefficients for each layer, thus effectively doubling the # coefficients for rank-1. This does not make the codebook design look consistent. That is, since there is a limitation on K0 coefficients per layer for rank-1 and rank-2, it makes sense to maintain that restriction for higher ranks as well.

For distribution of 2K0 coefficients across layers for RI=3,4, adopt Alt1 with a restriction of at most K0 coefficients for each layer

# UCI design

## Number of non-zero coefficients and RI

In RAN1#96bis, the following agreement was achieved regarding the indication of # NZC and RI in UCI Part 1:

Agreement

The scheme for indicating the number of NZ coefficients (NZC) will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt1.1: RI + # NZC summed across layers where # NZC = {0, 1, 2, …, 2K0} (if sufficiency indicator is supported) or {1, 2, …, 2K0}

- Alt1.2: Per-layer # NZC without RI where # NZC = {0, 1, 2, …, K0}

- Alt1.3: RI + differential of # NZC summed across layers

  - Differential means fraction of 2K0 with smaller number of possible values compared to the regular # NZC (in Alt1.1)

- Alt1.4: RI + per-layer differential # NZC

  - Differential means fraction of K0 with smaller number of possible values compared to the regular # NZC (in Alt1.2)

The parameter setting for the FD basis, i.e. the number of selected FD basis vectors MR,l may in the general case (TBD) be both rank-specific and layer-specific as indicated by the sub-indices (R,l) which denote the rank and layer index respectively, whereas the SD-basis is layer and rank-common.

For each layer l=0,…,R-1, there are thus 2LMR,l possible LC coefficients in the transformed domain and it can be assumed that a number KNZl≤K0(R,l)<2LMR,l are non-zero coefficients which are reported.

Alt 1.2

In Alt 1.1, the number of non-zero coefficients (NNZC) for each layer KNZl is indicated with a layer-wise separate indicator in UCI Part 1. Since the rank selection of the UE is not known to the gNB prior to decoding UCI Part 1, the UCI Part 1 payload needs to be rank-independent. This means that there always needs to be 4 per-layer NNZCIs present in UCI Part 1, regardless of UEs rank selection.

In this case, the indication of the NNZCs has the bitwidth 4×log2K0, assuming that the maximum number of coefficients are capped to K0 per layer even for ranks 3-4. This enables the UCI Part 1 payload to be constant irrespective of UE’s rank selection.

Alt 1.1

Since the purpose of including the number of NNZC indicator(s) in UCI Part 1 is only to determine the payload of UCI Part 2, the total number of non-zero coefficients across all layers could be indicated instead of having layer-wise indication. That is, a single UCI parameter indicating the sum of the NNZC across layers is indicated in UCI Part 1.

That is KNZ(TOT)=l=0R-1KNZ(l) is indicated, where R is the selected rank by the UE. Since per-layer NZC bitmaps are included in UCI Part 2, the gNB can infer the distribution of the NZCs among the layers and there is no ambiguity. The required bitwidth for KNZ(TOT) indication is thus log22K0. In addition, the RI would need to be conveyed as well.

A comparison between the two schemes is presented in Table 1 for different values of K0. Clearly, Alt 1.1 is superior.

Table 1: Overhead calculation for NNZC indication

| K0 | Alt 1.1 | Alt 1.2 |
| --- | --- | --- |
| 7 | 6 | 12 |
| 14 | 7 | 16 |
| 28 | 8 | 20 |
| 42 | 9 | 24 |

Regarding the “differential” proposals of Alt 1.3 and Alt 1.4, this will likely not reduce the overhead significantly compared to Alt 1.1 but will on the other hand risk reducing performance and increases UE complexity.

- Joint number of non-zero coefficient indication across layers can reduce overhead with 15 bits for K0=42 compared to layer-wise non-zero coefficient indication

- Support Alt 1.1, joint number of non-zero coefficient indication across layers and separate RI encoding in UCI Part 1

## Strongest coefficient indication

The following was agreed with respect to SCI design in RAN1#96bis:

Agreement

For RI=1, strongest coefficient indicator (SCI) is a log2KNZlog2KNZ-bit indicator. For RI>1, SCI design will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt3.1 (applicable to Alt1.2): Per-layer SCI, where SCIi is a –bit indicator (i=0, 1, …, (RI-1))

- Alt3.2 (applicable to Alt1.1): Per-layer SCI, where SCIi is a log2mini=0RI-1KNZ,i,2LMilog2mini=0RI-1KNZ,i,2LMi–bit indicator

- Alt3.3: Per-layer SCI, where SCIi is a log22K0log22K0–bit or log2K0log2K0 indicator (i=0, 1, …, (RI-1))

- Alt3.4: Per-layer SCI, where SCIi is a log22Llog22L–bit (i=0, 1, …, (RI-1))

The difference between the above alternatives is that Alt 3.4 utilizes the assumption that the strongest coefficient is present in the first FD-component, with index 0, while the other alternatives assumes that the strongest coefficient can be present in any FD-component. Hence, only the SD-component index rather than the coefficient index needs to be signalled, which reduces the overhead significantly.

The question is then, can it be guaranteed that the strongest coefficient occurs in a fixed FD-component? We try to answer this question using the following analysis.

Say that the UE has derived an optimal FD-basis and corresponding LC coefficients. Then we know from basic properties of the discrete Fourier transform and Grassmannian matrix theory that if the UE would apply a linear phase ramp exp(j*2*pi*n*k/N) over the columns n of the precoder vectors before FD-compression (where k is an integer <N3):

- It would result in that the transformed coefficients are cyclically shifted. I.e. the columns of the full W2-tilde matrix are perturbed

- The precoding properties are not affected by applying the linear phase ramp since it is just a scalar phase rotation of the precoder vector

Thus, there exists a set of N3 space-frequency precoder matrices  W=W1W2(k)WfH(k), k=0,…,N3-1 which have identical precoding properties and which only differ in that the FD-bases Wf(k) are different and that the columns of W2k are perturbated.

That is, regardless of which FD-coefficient the strongest coefficient belongs to in UE’s initial PMI search, a cyclic shift of the input before FD-compression can be applied to generate an equivalent set of coefficients where the strongest coefficient is located in the first FD-component.

UE can guarantee that the strongest coefficient occurs in FD-component 0 by applying linear phase ramp before FD compression, resulting in cyclically shifted coefficients in the transformed domain with identical precoding properties

Since this can be guaranteed, it seems obvious that Alt 3.4 should be supported.

For SCI indication, support Alt 3.4

Note that the UE does not actually need to perform the cyclic shift operation, instead there can be mapping defined in the specification.

For instance, let the determined FD basis subset be kmlml=0Ml-1 and the calculated LC coefficients be {ci,m}, where the strongest coefficient ci*,m*  can be in any FD-component.

Instead of reporting the indices kmlml=0Ml-1  directly, a set of modulo-shifted FD-basis indices k'ml=modkml-kml*,N3.ml=0Ml-1  can be reported in conjunction with that the UE perturbs the order in which the LC coefficients are mapped in UCI as

c0,mod(0+m*,Ml),c0,mod(1+m*,Ml), …,c1,mod(0+m*,Ml),c1,mod(1+m*,Ml),,…,…,c2L-1,mod(Ml-1+m*,Ml)

In this case, only i* needs to be reported as the SCI, whereas m* is only used to implicitly determine the UCI encoding and does not need to be reported. The UCI encoding procedure results in the indication of an equivalent precoder where the strongest coefficient is always at the first FD-component. From gNB perspective, it simply reads the CSI report as usual.

UCI encoding rule can be introduced to ensure that strongest coefficient always occurs in FD-component 0

## Bitmap design

The following was agreed with respect to bitmap design in RAN1#96bis:

Agreement

For RI=3-4, the bitmap design will be chosen from the following alternatives in RAN1#97 (Reno):

- Alt2.1: 2LMi bits per layer, i=0, 1, …, (RI-1)i=0,1,…,(RI-1)i=0,1,…,(RI-1)

- Alt2.2: One joint bitmap 1 for all layers, where an indicator bit is 1 if at least one of the RI layers has non-zero coefficient (UCI part 2) + Additional bitmap 2 (or, alternatively, a combinatorial indicator) indicating which layer(s) have either non-zero or zero coefficient(s) (UCI part 2) + Bitmap 2 (or, alternatively, a combinatorial indicator) size indicator (UCI part 1)

- Alt2.2B: Bitmaps 1 for each layer, where an indicator bit is 1 if at least one of the RI beams has non-zero coefficient (UCI part 2) + Additional bitmap 2 (or, alternatively, a combinatorial indicator) indicating which layer(s) have either non-zero or zero coefficient(s) (UCI part 2) + Bitmap 2 (or, alternatively, a combinatorial indicator) size indicator (UCI part 1)

- Alt2.3: LMi bits for the layer in which the weaker polarization is dropped (else 2LMi bits) + up to 4-bit bitmap to indicate the layer where the weaker polarization is dropped (UCI part 1); i=0, 1, …, (RI-1)

Alt 2.1 is a straightforward extension of the principle for RI=1,2 and consists of an independent bitmap for each layer. Alt 2.2 on the other hand introduces special handling for RI=3,4, compared to RI=1,2 with a two-step bitmap indication. The motivation is to reduce average overhead. However, depending on the configured number of non-zero coefficients, the overhead can actually be increased! In our view, the potential overhead savings are not significant enough to consider this alternative and the additional UE complexity does not seem motivated.

Alt 2.3 introduces additional indicators in UCI Part 1 to indicate if the entire polarization of a layer is dropped, whereby the bitmap can be cut in half. In our understanding, the likelihood of such an event occurring is rather low, i.e. it is a corner case which the UCI design does not need to be optimized towards. On average, in our estimation, the total overhead will be increased compared to Alt 2.1 due to the additional overhead in UCI Part 1.

Therefore, we suggest adopting the straightforward approach of Alt 2.1.

For NZC bitmap design, adopt Alt 2.1 with independent bitmap per layer

## FD basis selection

In RAN1#96bis, a number of alternatives where identified for FD-basis selection, in subsequent offline email discussion, these alternatives were boiled down to a more compact description as per the following offline agreements:

Offline agreement 1: The two-step FD basis subset selection is described as follows:

- The 1st (intermediate) step uses an intermediate FD basis set of size-N3' (N3'≤ N3)

  - The value of N3' is RI- and layer-common

  - The intermediate FD basis subset is RI- and layer-common

- The 2nd step uses an indicator to indicate the FD basis used for each layer

  - FFS (to be resolved in RAN1#97): for the indicator, select between a N3'-bit bitmap and X-bit combinatorial indicator where X is either log2N3'Mi or log2N3'-1Mi-1.

- FFS (to be resolved in RAN1#97): select one of the following alternatives on N3' setting mechanism:

  - 1) reported in UCI part 1

  - 2) higher-layer configured

  - 3) fixed

- FFS (to be resolved in RAN1#97): select one of the following alternatives on size-N3' intermediate subset (IntS)

  - 1) IntS is adjacent and fully parameterized with Minitial, indicating that the intermediate set consists of FD bases modMinitial+n,N3, n=0,1,⋯N3'-1,

    - FFS (to be resolved in RAN1#97): whether Minitial is reported in UCI part 2, higher-layer configured, or fixed

  - 2) IntS is selected freely from N3 FD bases, a combinatorial indicator is reported in UCI part 2

    - FFS (to be resolved in RAN1#97): exact bitwidth, either log2N3N3'  or log2N3-1N3'-1

Offline agreement 2: In RAN1#97, decide on FD basis subset selection scheme from the following alternatives:

- Free selection (Alt 5.1 in RAN1#96b)

- Fixed selection (Alt 5.4 in RAN1#96b)

- Two-step selection (the final outcome of proposal 1)

In our view, if two-step FD-basis indication is supported, the value of N3’ should be fixed in specification according to a predefined formula, e.g. N3'=1.5⋅M in order to reduce the number of configuration parameters and the spec complexity.

To investigate if two-step FD-basis selection results in any performance loss compared to one-step selection, we perform evaluations comparing:

- One-step selection

- Arbitrary two-step selection: IntS is selected freely from N3 FD bases

- Adjacent two-step selection: IntS is adjacent and fully parameterized with Minitial,

We consider 13 CQI subbands and R=2, p={12,14} and using different values of N3’. The results are presented in Table 2 below. Overall, the performance of all three schemes are very similar. There is some minor loss for 2-step indication if N3’ is small, however it is almost negligible, with adjacent 2-step selection having a minor loss compared to arbitrary 2-step. However, this difference is negligible as well.

Table 2: Comparison between one-step and two-step FD-basis indication

|  | Scheme |  | Average UPT gain [%] |
| --- | --- | --- | --- |
|  | Type I |  | 0 |
| p=1/2 | One-step |  | 7,2 |
|  | Arbitrary 2-step N3'=7 |  | 7,0 |
|  | Arbitrary 2-step N3'=9 |  | 7,1 |
|  | Adjacent 2-step N3'=7 |  | 6,7 |
|  | Adjacent 2-step N3'=9 |  | 7,2 |
| p=1/4 | One-step |  | 5,9 |
|  | Arbitrary 2-step N3'=4 |  | 5,3 |
|  | Arbitrary 2-step N3'=6 |  | 5,3 |
|  | Arbitrary 2-step N3'=8 |  | 5,6 |
|  | Arbitrary 2-step N3'=10 |  | 5,7 |
|  | Adjacent 2-step N3'=4 |  | 5,0 |
|  | Adjacent 2-step N3'=6 |  | 5,0 |
|  | Adjacent 2-step N3'=8 |  | 5,2 |
|  | Adjacent 2-step N3'=10 |  | 5,6 |

Similar performance for one-step and two-step FD-basis indication

To compare the overhead for one-step selection with the two options for two-step selection, we consider that N3’ is selected according to N3'=1.5⋅M and sweep the number of CQI subbands between 5-19, R=1,2 and we assume p=12. The resulting overhead is shown as a function of N3 in Figure 2.

For R=2, the overhead savings with the adjacent IntS selection is substantial, around 25 bits. Both one-step and arbitrary two-step FD-basis selection result in similar overhead.

For R=1, all three schemes have similar overhead

Figure 2: Overhead for intermediary subset indication

Based on these observations, at least for R=2, there is a benefit with adjacent IntS selection since the overhead can be substantially reduced while maintaining the performance. However, for R=1, the benefit is unclear compared to one-step indication. Since the overhead and performance for all schemes are similar in this case, we should consider the alternative with least spec impact.

Support two-step FD-basis selection using adjacent IntS selection with fixed N3’, at least for R=2

## M’

It has been proposed to introduce a parameter in UCI Part 1 for the UE to dynamically select the size of the FD-basis. This means that the UE can determine that only a number M'<M FD-basis vectors is needed in order to describe the channel, which is smaller than the number of configured FD-basis vectors M. This would be as a consequence of the UE first determining that all the coefficients for a certain FD-basis vector are zero, so that a column of the NZC bitmap would be all ‘0’s. The benefit with this proposal would be that the zero columns of the bitmap in UCI Part 2 could be dropped, which would reduce the overhead.

However, the potential overhead savings are not enormous and there is already large overhead savings from dropping the reporting of the coefficients themselves. That is, if the corresponding coefficients for M-M'  FD-basis vectors are not reported the overhead is reduced with 2L× M-M'×6 bits (assuming 8-PSK). If additionally, M’ is indicated in UCI Part 1, the bitmap in UCI Part 2 can be reduced in size, but this only saves 2L× M-M' additional bits. I.e. 85% of the overhead reduction is already achieved. It does therefore not seem warranted to introduce additional overhead in UCI Part 1 when the lions’ share of the overhead saving can be achieved solely by NNZC indication.

It is not warranted to introduce additional overhead in UCI Part 1 for M’ indication, since most of the overhead savings is already achieved via NNZC indication

However, there may be a way to introduce M’ indication without increasing the UCI Part 1 overhead.

Since the indicator of the total NNZC summed across layers is independently encoded into a field in UCI Part 1 (according to Alt 1.1), the bitwidth of that field is  log22K0. For M=7 and L=4, which is a typical configuration, K0 may take the values 14, 28 and 42. The bitwidth of the field is thus 5, 6 and 7 bits respectively for the three values. However, the value range that requires to be indicated for the NNZC indicator is only KNZ(TOT)=1,…,2K0, that is, 2K0 possible values. The mapping from a codepoint of an independently encoded NNZC indicator field to the NNZC value would thus be according to Table 3 for the example of 2K0=84, which results in a bitfield of width log284=7  bits and thus containing 2log22K0=27=128 codepoints. There thus exists 2log22K0-2K0 reserved codepoints which can not be used to convey any information and are, in a sense, wasted.

Table 3: Example mapping of independently encoded NNZC Indicator field to NNZC value

| NNZC Indicator field | NNZC Value |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| … | … |
| 83 | 84 |
| 84 | reserved |
| … | … |
| 127 | reserved |

There could be many unused codepoints of the NNZC Indicator field

It therefore seems appropriate to utilize these unused codepoints to convey some useful information, such as an indication of M’. That is, the NNZC value and FD-basis size indication M’ could be jointly encoded into a “joint NNZC and M’ Indicator field”. An example of this is given in Table 4, where the first 2K0=84 codepoints are the same as for an independent NNZC Indicator field, i.e. M’=M and the FD basis size is the same as the configured one, while the remaining 2log22K0-2K0=44 codepoints indicates for instance an half-size FD-basis, i.e. M’=M/2.

Note that since 2log22K0-2K0<2K0 in the general case, all possible values of KNZ(TOT) cannot be indicated in conjunction with indicating M’=M/2. However, that is likely okay, since if the UE reports M’<M it by necessity does not have many non-zero coefficients. Hence, the range of NNZC that can be indicated together with M’<M can be biased towards the lower end. Another limitation is that if K0 is a power of two, there are no unused codepoints and hence M’<M cannot be indicated. However, this only occurs for a small subset of possible configurations, so it should not be a large issue, especially due to that the feature of M’ indication is not super-critical to have for a functional codebook, it merely provides an additional overhead optimization. If the gNB is very keen on using this feature it can make sure to configure β such that K0 is not a power of two.

Table 4: Example mapping of joint NNZC and M’ Indicator field

| Joint NNZC and M’ Indicator field | NNZC Value | M’ |
| --- | --- | --- |
| 0 | 1 | M |
| 1 | 2 | M |
| … | … | … |
| 83 | 84 | M |
| 84 | 1 | M/2 |
| … | … | M/2 |
| 127 | 44 | M/2 |

Support M’ indication by utilizing unused codepoints of the NNZC indicator field in UCI Part 1, by jointly encoding the NNZC and M’ values

## Basis sufficiency/insufficiency indication

There have been proposals to introduce an indicator in UCI Part 1 to indicate either the sufficiency or insufficiency of the FD-basis or number of non-zero coefficients. The motivation is that the UE can dynamically indicate either if more or less FD-basis vectors are required which could prompt the gNB to either reconfigure (using RRC) the CSI Report Setting to a codebook with more or less FD-basis vectors, or, choose to trigger another CSI Report Setting which has already been pre-configured to the UE. From a principal perspective, one can argue whether it is proper to introduce dynamic L1 signalling, present in every CSI report, for the purpose of informing the gNB to perform RRC reconfiguration. Such functionality, if needed, should be solved via other means in our view.

It is improper to introduce dynamic L1 signalling with the purpose of triggering L3 reconfiguration

Additionally, it is not clear to us why the UE explicitly needs to indicate basis sufficiency/insufficiency. This information can be inferred by the gNB implicitly from the CSI report. For instance, if the basis is “sufficient” and too many FD-basis vectors have been configured, the gNB can directly infer this from the reported NZC bitmaps and/or reported amplitude values. That is, if the UE consistently reports many zero or low power coefficients for a subset of the FD-bases, the gNB can infer that p is configured too high and switch to triggering either Type I CSI or Type II CSI with smaller p. Note that in typical operation, the gNB will preconfigure the UE with at least a Type I CSI Report Setting and a Type II CSI report setting and dynamically switch which one is triggered depending on a number of factors such as cell load, UL coverage, etc.

Similarly, for basis insufficiency, this can also be inferred by the gNB from the CSI report for instance by observing that the UE consistently reports many non-zero and/or high-power coefficients.

Basis insufficiency/sufficiency can be inferred by the gNB directly from the reported NZC bitmaps and/or amplitude coefficients

Therefore, we do not support introducing a basis sufficiency/insufficiency indictor in UCI Part 1.

Do not support Basis Sufficiency/Insufficiency Indication in UCI Part 1

# CSI omission procedure

In our companion contribution [2], we discuss principles for CSI omission procedure in Rel-16, and propose the following:

Codebook design choices should not consider if CSI omission are facilitated by them or not, but should be made solely based on performance, overhead and complexity

The CSI omission procedure should not require CSI recalculation based on the available PUSCH allocation

For Rel-16 CSI omission procedure, consider dropping half of the LC coefficients according to FD-beam-index first, then SD-beam index and last layer-index

For Rel-16 CSI omission procedure, consider dropping the portion of the NZC bitmaps which is not required to interpret the non-omitted LC coefficients

# Reducing supported combinations of compression parameters

As is also further elaborated in our companion contribution [4], some combinations of compression parameters do not offer favourable performance overhead trade-off. The following observations can be made:

- For 32Tx, performance of L=2 is not competitive, especially L,p=(2,12) which has inferior performance to L,p=(4,14) with similar overhead

- For 32 Tx, generally β=34 results in quite limited performance gain compared to β=12, at the cost of quite a large overhead increase

Based on these observations, we can conclude that some codebook configurations likely can be pruned since they do not are not competitive enough. We make the following proposals:

- Consider not supporting L=2, at least L,p=(2,12) can be removed

- Consider not supporting β=34, since it has limited performance gain

# Value of N3 for NSB×R>13

This issue is discussed in our companion contribution [3]. Where we observed the following:

- DFT padding results in around 2% loss compared to No DFT padding

- Segmentation approach has inferior performance/overhead tradeoff compared to approaches without segmentation

Based on these observations, we propose a compromise solution:

- When NSB×R>19NSB×R≤19,support free selection of N3. For NSB×R>19, N3N3 is selected as the smallest multiple of 2, 3, or 5 which is  ≥NSB×R≥NSB×R

# Associated UE capability

While the UE capability discussion generally is handled by the end of the release, we have already achieved some agreement on for instance that L=6 is an optional UE capability. Therefore, we would like to start the discussion on some capability-related issues already now.

As also highlighted in [5], the UE capability signalling for CSI calculation in Rel-15 is rather complicated and getting the signalling structure in place have caused a lot of headache for RAN2, where it for instance was realized that the RAN1 indicated capability structure did not result in feasible overhead and a re-design of the entire signalling was required at a late stage (the remnants of which can be seen by the large amount of “dummified” CSI capability signalling in 38.331). Basically, the current structure is as follows:

  - The maximum number of simultaneous CSI reports (i.e. number of CPUs) and simultaneous CSI-RS/IM ports/resources are reported per band combination

  - The maximum number of configured CSI Report Settings per BWP and the maximum number of configured CSI-RS/IM ports/resources are reported per band along with the maximum number of simultaneous CSI reports and simultaneous CSI-RS/IM ports/resources in a CC

    - If the band is within an FR1-FR2 band combination, this signalling can be overridden with another signalling

  - The supported codebooks are signalled per band in CodebookParameters. For each codebook Type (i.e. Type I SP, Type I MP, Type II, Type II port selection), a list of triplets (maxNumberTxPortsPerResource, maxNumberResourcesPerBand, totalNumberTxPortsPerBand) is signalled.

For instance, the UE can indicate that is supports:

  - Type I codebook with 32 ports in a resource, 2 simultaneous resources and max 64 total ports across all CCs in Band #1

  - Type I codebook with 24 ports in a resource, 1 simultaneous resource and max 24 total ports across all CCs in Band #2

  - Type II codebook with 32 ports in a resource, 1 simultaneous resource and max 32 total ports across all CCs in Band #1

  - At most 5 simultaneous CSI reports in a band combination consisting of (Band #1, Band #2)

In this example, the UE can be triggered with simultaneous Type I report on Band #2 and Type II report for Band #1, but not simultaneous Type I and Type II reports on Band #1. This construction may lead the UE to being required to underreport its capability. Furthermore, it is not entirely obvious how one should interpret capability of simultaneous resources/ports associated with a codebook type within a band when the UE is triggered with a CSI report with mixes of codebook types.

Rel-15 CSI capability signalling is rather complicated, consider in Rel-16 if simplifications can be made

Another issue is that the maximum number of configured CSI Report Settings per BWP a UE can indicate as its capability is rather limited in Rel-15, it is capped at four CSI Report Settings per time-domain behaviour. In practical deployment, it is beneficial for the gNB to dynamically vary the CSI granularity (and thus the payload) depending on the UL UCI coverage. For instance, a UE on the cell edge may only have UL coverage to reliably transmit a Type I WB PMI/CQI report whereas a cell-centre UE can afford to transmit a Type II report with L,p,β=(4,12,34). For instance, the gNB could dynamically switch between following codebook configurations depending on the UE’s UL coverage:

- Type I WB CQI / WB PMI

- Type I SB CQI / WB PMI

- Type I SB CQI / SB PMI

- Type II  L,p,β=(2,14,12).

- Type II  L,p,β=2,12,12

- Type II  L,p,β=4,14,12

- Type II  L,p,β=(4,12,12)

However, since the UE can report at most 4 configured CSI Report Settings as its capability, the gNB needs to RRC reconfigure the UE as it moves within the cell. This is clearly not desirable.

For Rel-16 CSI capability, increase the maximum number of configured CSI Report Settings a UE can report

# Conclusion

Based on the discussion in this contribution we make the following observations:

Observation 1	Alt 3C and Alt 6E have similar performance/overhead tradeoff while Alt 2B is inferior

Observation 1	Joint number of non-zero coefficient indication across layers can reduce overhead with 15 bits for K0=42 compared to layer-wise non-zero coefficient indication

Observation 2	UE can guarantee that the strongest coefficient occurs in FD-component 0 by applying linear phase ramp before FD compression, resulting in cyclically shifted coefficients in the transformed domain with identical precoding properties

Observation 3	UCI encoding rule can be introduced to ensure that strongest coefficient always occurs in FD-component 0

Observation 4	Similar performance for one-step and two-step FD-basis indication

Observation 5	For R=2, the overhead savings with the adjacent IntS selection is substantial, around 25 bits. Both one-step and arbitrary two-step FD-basis selection result in similar overhead.

Observation 6	For R=1, all three schemes have similar overhead

Observation 7	It is not warranted to introduce additional overhead in UCI Part 1 for M’ indication, since most of the overhead savings is already achieved via NNZC indication

Observation 8	There could be many unused codepoints of the NNZC Indicator field

Observation 9	It is improper to introduce dynamic L1 signalling with the purpose of triggering L3 reconfiguration

Observation 10	Basis insufficiency/sufficiency can be inferred by the gNB directly from the reported NZC bitmaps and/or amplitude coefficients

Observation 11	For 32Tx, performance of L=2 is not competitive, especially L,p=(2,12) which has inferior performance to L,p=(4,14) with similar overhead

Observation 12	For 32 Tx, generally β=34 results in quite limited performance gain compared to β=12, at the cost of quite a large overhead increase

Observation 13	DFT padding results in around 2% loss compared to No DFT padding

Observation 14	Segmentation approach has inferior performance/overhead tradeoff compared to approaches without segmentation

Observation 15	Rel-15 CSI capability signalling is rather complicated, consider in Rel-16 if simplifications can be made

Based on these observations, we make the following proposals:

Proposal 1	For FD-basis parametrization, support Alt 3C with a value range v0={14,18}

Proposal 2	For distribution of 2K0 coefficients across layers for RI=3,4, adopt Alt1 with a restriction of at most K0 coefficients for each layer

Proposal 3	Support Alt 1.1, joint number of non-zero coefficient indication across layers and separate RI encoding in UCI Part 1

Proposal 4	For SCI indication, support Alt 3.4

Proposal 5	For NZC bitmap design, adopt Alt 2.1 with independent bitmap per layer

Proposal 6	Support two-step FD-basis selection using adjacent IntS selection with fixed N3’, at least for R=2

Proposal 7	Support M’ indication by utilizing unused codepoints of the NNZC indicator field in UCI Part 1, by jointly encoding the NNZC and M’ values

Proposal 8	Do not support Basis Sufficiency/Insufficiency Indication in UCI Part 1

Proposal 9	Codebook design choices should not consider if CSI omission are facilitated by them or not, but should be made solely based on performance, overhead and complexity

Proposal 10	The CSI omission procedure should not require CSI recalculation based on the available PUSCH allocation

Proposal 11	For Rel-16 CSI omission procedure, consider dropping half of the LC coefficients according to FD-beam-index first, then SD-beam index and last layer-index

Proposal 12	For Rel-16 CSI omission procedure, consider dropping the portion of the NZC bitmaps which is not required to interpret the non-omitted LC coefficients

Proposal 13	Consider not supporting L=2, at least L,p=(2,12) can be removed

Proposal 14	Consider not supporting β=34, since it has limited performance gain

Proposal 15	When NSB×R≤19,support free selection of N3. For NSB×R>19, N3 is selected as the smallest multiple of 2, 3, or 5 which is  ≥NSB×R

Proposal 16	For Rel-16 CSI capability, increase the maximum number of configured CSI Report Settings a UE can report

# References

R1-1907075 “On CBSR for Type II CSI enhancements”, Ericsson, 3GPP TSG RAN WG1 Meeting RAN1#97, Reno, USA, May 13th-17th 2019

R1-1907076, “On CSI omission procedure”, Ericsson, 3GPP TSG RAN WG1 Meeting RAN1#97, Reno, USA, May 13th-17th 2019

R1-1907077, “Evaluation results on the value of N3 for Type II CSI enhancements”,	 Ericsson, 3GPP TSG RAN1#97, Reno, USA, May 13th-17th 2019

R1-1907078, “Evaluation results for different parameter settings for Type II CSI enhancements”, Ericsson, RAN1#97, Reno, USA, May 13th-17th 2019

R1-1904312, “On CSI Enhancements for MU-MIMO “, Intel, 3GPP TSG RAN WG1 #96b, Xi'an, China, 8th – 12th April, 2019

# Appendix: Simulation assumptions

For system level evaluations, the agreed assumptions from RAN1#94bis are used.  The remaining evaluation assumptions are given in the table below.

Table 1: SLS assumptions for CSI enhancement

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Scenario | Dense Urban (Macro only) |
| Carrier frequency | 4GHz |
| Inter-site distance | 200m |
| Channel model | According to the TR 38.901 |
| Antenna setup and port layouts at gNB | 32 ports: (8,4,2,1,1,4,4), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ 100 deg tilt |
| Antenna setup and port layouts at UE | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) 4RX: (1,2,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2,3,4) |
| BS Tx power | 41 dBm |
| BS antenna height | 25m |
| UE antenna height & gain | Follow TR36.873 |
| UE receiver noise figure | 9dB |
| Modulation | Up to 256QAM |
| Coding on PDSCH | LDPC Max code-block size=8448bit |
| Simulation bandwidth | 10 MHz with 15kHz SCS |
| Frame structure | Slot Format 0 (all downlink) for all slots |
| MIMO scheme | SU/MU-MIMO with rank adaptation |
| MIMO layers | Maximum 8 layers |
| CSI feedback | - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead | DMRS overhead included CSI-RS overhead included TRS overhead included |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) | - 70 % for CSI overhead reduction |
| UE distribution | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC as the baseline receiver |
| Feedback assumption | Realistic |
| Channel estimation | Realistic |