3GPP TSG RAN WG1 Meeting RAN1#97	R1-1907075

Reno, US, May 13th-17th, 2019

Source:	Ericsson

Title:	On CBSR for Type II CSI enhancements

Agenda Item:	7.2.8.5

Document for:	Discussion and Decision

# Introduction

Codebook Subset Restriction (CBSR) was introduced in Rel-8 for the 2Tx and 4Tx codebooks in LTE and have been supported for each new codebook ever since. In RAN1#95, it was agreed to follow this principle and introduce CBSR also for the enhanced Type II codebook:

Agreement

For Rel-16 NR, agree on Alt1 (DFT-based compression) in Table 1 of R1-1813002 as the adopted Type II rank 1-2 overhead reduction (compression) scheme as formulated in Alt1.1 of R1-1813002

- Note: The same DFT-based compression scheme is extended for Type II port selection codebook

- Codebook subset restriction (CBSR) is supported when DFT-based compression is utilized for Type II codebooks with overhead reduction (compression) scheme

  - FFS: detailed signaling mechanism

In this contribution we discuss potential CBSR designs for the enhanced Type II codebooks.

# Overview of Rel-15 CBSR mechanisms

In NR Rel-15, CBSR is supported for both Type I and Type II codebooks, but with slightly different implementations. Note that we consider the 2Tx codebook CBSR as a special case which is not included in the below analysis. Some general principles for Rel-15 CBSR, that should be followed also for Rel-16 CBSR are:

- CBSR signalling is rank-agnostic, i.e. a single CBSR is given which is applicable to all transmission ranks

- Reported rank is restricted with separate rank restriction signalling

- “W1 restriction” is based on beam restriction of vk,l, if at least one restricted beam is included in the precoder, the precoder is restricted

- “W2 restriction” is not supported (except in the sense that for Type I rank 1-2 codebook with Config 2, the used vk,l can be subselected from intermediary subset in W1)

- CBSR does not change PMI payload

The W1 restriction is useful for controlling inter-cell interference levels, particularly in elevation direction, which is important for both Type I and Type II codebooks.

While the Type I CBSR uses beam restriction directly, with a size N1N2O1O2 bitmap being signalled where each bit in the bitmap corresponds to the restriction of a 2D DFT beam vk,l, the Type II CBSR uses joint beam and wideband amplitude restriction where the beam restriction is constrained so that only P=4 size N1*N2 beam groups can be restricted. This is a quite complicated scheme which is illustrated in Figure 1.

Figure 1: Illustration of Rel-15 Type II CBSR mechanism

# Discussion on Rel-16 CBSR

One immediate observation is that the Rel-15 Type II CBSR design cannot be directly applied to the Rel-16 codebook, since there is no per-beam WB amplitude coefficient (rather there are up to M amplitude coefficients for each beam, each corresponding to an FD-basis vector).

Rel-15 Type II CBSR cannot be applied to Rel-16 Type II codebook since there is no wideband amplitude coefficient

While the spec could of course define a mapping between the set of up to M amplitude coefficients for a beam and layer, to a reference amplitude value (in lieu of the WB amplitude value), which could be used in the Rel-15 CBSR mechanism, this does not seem to be very useful.

Another option is to consider joint beam and FD-basis vector restriction. However, it is not very likely that the gNB can determine a joint restriction that has any practical utility as a method of controlling inter-cell interference (which after all is the motivation for introducing CBSR). Therefore, we propose to consider only beam-based CBSR for Rel-16. In order to minimize the specification work, we can simply reuse the Rel-15 Type I CBSR, which can be directly applied to the Rel-16 Type II codebook.

For CBSR for Rel-16 Type II enhancement, consider the following:

- The Rel-15 Type I CBSR (beam restriction) is reused

  - A bitmap of N1N2O1O2 bits is signalled, where each bit corresponds to a beam vk,l. If at least one restricted beam is included in the precoder, the precoder is restricted

  - The CBSR signalling is rank-agnostic, i.e. a single CBSR is given which is applicable to all transmission ranks

- Reported rank is restricted with separate rank restriction signalling

- CBSR does not change PMI payload

# Conclusion

Based on the discussion in this contribution we make the following observations:

Observation 1	Rel-15 Type II CBSR cannot be applied to Rel-16 Type II codebook since there is no wideband amplitude coefficient

Based on these observations, we propose the following:

Proposal 1	For CBSR for Rel-16 Type II enhancement, consider the following:

	The Rel-15 Type I CBSR (beam restriction) is reused

	A bitmap of N1N2O1O2 bits is signalled, where each bit corresponds to a beam vk,l. If at least one restricted beam is included in the precoder, the precoder is restricted

	The CBSR signalling is rank-agnostic, i.e. a single CBSR is given which is applicable to all transmission ranks

	Reported rank is restricted with separate rank restriction signalling

	CBSR does not change PMI payload