3GPP TSG-RAN WG1 #97 			               R1-1907126

Reno, USA, 13th – 17th May 2019                                        Revision of R1-1904804

Agenda Item:	7.2.11.2

Source: 	ZTE, Sanechips, BJTU

Title:	Discussion on LOS probability model in IIOT scenario

Document for:	Discussion and Decision

# Introduction

In RAN#81, a new RAN1-led SI on Channel modeling for Indoor Industrial scenarios [1] was approved.

The study item aims to develop a channel model to support studies on URLLC/IIOT enhancements for industrial scenarios and use cases. In order to achieve this, the study item should fulfil the following objectives:

- Determine a suitable description of the scenario and frequency bands up to 100GHz that should be supported.

  - The LS from 5G-ACIA in RP-181521 may be used as one of the references to start the discussion.

- Review existing literature and new propagation measurements in industrial environments

- Assess key differences compared to existing channel models such as the model in TR 38.901

- Define a new industrial propagation scenario and determine propagation parameters and, if required, new model components. Use 38.901 as the starting point.

  - Priority should be given to channel modeling for frequency ranges below 52.6GHz, which can be captured in the TR upon completion of the corresponding model.

In [8], the LOS probability model is proposed for BSs above clutters. This document further addresses the LOS probability model for the BS clutter-embedded as well as BS above clutter.

# LOS probability simulation and analysis

The positions of base stations and UE in the simulation are shown in Figure 2.1 and Table 2-1.

Figure 2.1 XY locations of transmitters

Table 2-1 Configuration for IIOT channel model simulation

| Position of Base stations | Base stations with height selected from {2m, 10m, 15m and 22m} are located in 5 different XY positions: BS2_1:[45 20 2]; BS2_2:[45 60 2]; BS2_3: [90 40 2]; BS2_4:[135 20 2]; BS2_5: [135 60 2] BS10_1:[45 20 10]; BS10_2:[45 60 10]; BS10_3: [90 40 10]; BS10_4:[135 20 10]; BS10_5: [135 60 10] BS15_1:[45 20 15]; BS15_2:[45 60 15]; BS15_3: [90 40 15]; BS15_4:[135 20 15]; BS15_5: [135 60 15] BS22_1:[45 20 22]; BS22_2:[45 60 22]; BS22_3: [90 40 22]; BS22_4:[135 20 22]; BS22_5: [135 60 22] * Refer to Figure 3.2 for the locations of gNB in XOY plane, where AntN refer to BSh_N for h={2,10,15,22}m and N={1,2,3,4,5}, respectively. |
| --- | --- |
| Position of UE’s | 1600 UEs are uniformly distributed with UE height =1.5m. |
| Carrier Frequency | 2.3GHz; 4.9GHz; 28GHz; 52.6GHz; 72GHz; 100GHz |

Simulation result for BS embedded in clutter

For each of the BS height = 2m, 5 base stations are deployed in the workshop as shown in Figure 2.1 and Table 2-1. The LOS/NLOS data are counted in each of the 4 sub-areas shown in Appendix.

The LOS probability statistics for each sub-area in BS height =2m are shown in Figure 2.2.

Figure 2.2 LOS probability in BS height 2m

Based on the simulation results above, Area1~ Area4 are fitted by piece-wise functions as following:

(2-1)

where ka, da and pa are listed in Table2-2 and Figure2.4 considering the sub scenario definition in [7].

Table 2-2 Fitting Result of ka in IIOT-B-x sub scenarios

|  | IIOT-B-H (Area1) | IIOT-B-L (Area4) | IIOT-B-M (Area2+Area3) |
| --- | --- | --- | --- |
| ka | 33 | 78 | 64 (averaged from 62 and 66 per area) |
| da | 1m | 1m | 1m |
| pa | 1 | 1 | 1 |

Figure2.4 Fitting Result of ka in Area1~Area4

Simulation result for BS above clutter

For each of the BS heights {10m,15m,22m}, 5 base stations are deployed in the workshop as shown in Figure 2.1 and Table 2-1. The LOS/NLOS data are counted in each of the 4 sub-areas shown in Appendix.

The LOS probability statistics for each sub-area and for each BS height are shown in Figure 2.5~2.7.

Figure 2.5 LOS probability in BS height 10m

Figure 2.6 LOS probability in BS height 15m

Figure 2.7 LOS probability in BS height 22m

Based on the simulation results, Area1~ Area4 are fitted by piecewise functions as following:

(2-2)

where ka, da and pa are listed in Table2-3 and Figure2.8 considering the sub scenario definition in [7].

Table 2-3 Fitting Result of ka in IIOT-A-x sub scenarios

|  |  | IIOT-A-H (Area1) | IIOT-A-L (Area4) | IIOT-A-M (Area2+Area3) |  |
| --- | --- | --- | --- | --- | --- |
| ka | BS @ 10m | 104 | 503 | 101 | 277 |
|  | BS @ 15m | 94 | 530 | 148 | 314 |
|  | BS @ 22m | 58 | 330 | 164 | 77 |
|  | average | 85 | 454 | 180 |  |
| da | Per area | 1m | 7m | 6.25m | 6.96m |
|  | Per density | 1m | 7m | 6.6m |  |
| pa |  | 0.6 | 1 | 1 |  |

BS height=10m                   BS height=15m                   BS height=22m

Figure 2.8 Fitting result for ka in all Areas and in BS height={10m,15m and 22m}

Observation 1: LOS probability decreases as Tx-Rx 2D distance increases.

Observation 2: LOS probabilities are different in sub-areas with different clutter densities.

Proposal 1: LOS probability is dependent on Tx-Rx 2D distance and clutter density.

# LOS probability model

According to the analysis in section 2, the following LOS probability model in IIOT scenario is proposed:

(3-1)

Where

- is the 2D distance between transmitter and receiver;

- is the breakout distance shown in Table 3-1;

- is the breakout LOS probability shown in Table 3-1;

- is the exponential coefficient for corresponding sub scenario shown in Table 3-1.

Table 3-1 Configurations for LOS probability in IIOT sub-scenarios

| SubSce | IIOT-A-H | IIOT-A-L | IIOT-A-M | IIOT-B-H | IIOT-B-L | IIOT-B-M |
| --- | --- | --- | --- | --- | --- | --- |
| ksubsce | 85 | 454 | 180 | 33 | 78 | 64 |
| dsubsce | 1m | 7m | 6.6m | 1m | 1m | 1m |
| psubsce | 0.6 | 1 | 1 | 1 | 1 | 1 |

Proposal 2: The parameters in different IIOT sub scenarios should be differentiated.

Proposal 3: Capture the LOS probability model proposed in the TR.

# Conclusion

In this contribution, the LOS probability model are proposed based on simulation results in an indoor industry scenario with the following observations and proposals:

Observation 1: LOS probability decreases as Tx-Rx 2D distance increases.

Observation 2: LOS probabilities are different in sub-areas with different clutter densities.

Proposal 1: LOS probability is dependent on Tx-Rx 2D distance and clutter density.

Proposal 2: The parameters in different IIOT sub scenarios should be differentiated.

Proposal 3: Capture the LOS probability model proposed in the TR.

# References

RP-182138, SID on Channel Modeling for Indoor Industrial Scenarios, Ericsson, 3GPP TSG-RAN Meeting #81, Gold Coast, Australia, September 10th – 13th 2018.

R1-1811585, Work plan for the SI on Channel Modeling for Indoor Industrial Scenarios, Ericsson, RAN1#94bis, Chengdu, China, Oct 8–12, 2018.

R1-1811586, Skeleton TP for updates to TR 38.901, Ericsson, RAN1#94bis, Chengdu, China, Oct 8–12 2018

RP-181521, LS on Channel Model for Indoor Industrial Scenarios, 5G-ACIA, 3GPP TSG-RAN Meeting #81, Gold Coast, Australia, September 10th – 13th 2018.

R1-1902112, About IIOT scenario, ZTE, Sanechips, BJTU, Tongji University

R1-1812699, IIOT channel modeling for industrial indoor scenario, ZTE, Sanechips

R1-1904116, Discussion on scenarios for IIOT channel model, ZTE, Sanechips, BJTU, Tongji University

R1-1902117, LOS probability model in IIOT scenario , ZTE, Sanechips, BJTU, Tongji University

# Appendix. Sub-area partition in the given IIoT scenario (from [5])