3GPP TSG-RAN WG1 Meeting #97	R1-1907135

Reno, US, May 13th – 17th 2019

Agenda Item:	7.2.4.2.1

Source:	Ericsson

Title:	Uu-based sidelink resource allocation

Document for:	Discussion, Decision

# 1	Introduction

During RAN1#96bis the following has been agreed [1]:

Agreements: - A dynamic grant provides resources for one or multiple sidelink transmissions of a single TB. - A configured grant (type-1, type-2) provides a set of resources in a periodic manner for multiple sidelink transmissions. - UE decides which TB to transmit in each of the occasions indicated by a given configured grant. - FFS: whether different transmissions of a TB can take place across multiple configured grants. - Other restrictions on what can be transmitted in a given configured grant (e.g., based on QoS, destination UE, etc.) are up to RAN2.

In addition, the following was captured in the Chairman’s notes:

R1-1905834 Revisit the issue 2.4 as in x5834 in RAN1#97

During RAN2#105bis, the following was agreed [2]:

Agreements on SL configured grant: 1: Multiple active configured sidelink grants should be supported in NR sidelink. 2: A confirmation for activation/deactivation of SL configured grant type-2 is needed. Details are FFS.

In this paper we discuss remaining aspects of Mode-1 resource allocation, including the issues left open in the above agreements.

# 2	General aspects of the HARQ procedure in Mode 1

In RAN1#96, the following was agreed:

Agreements: - In mode 1 for unicast and groupcast, it is supported for the transmitter UE via Uu link to report an indication to gNB to indicate the need for retransmission of a TB transmitted by the transmitter UE. - FFS the format of the indication, e.g., in the form of HARQ ACK/NACK, or in the form of SR/BSR, etc. - RAN1 continues discussion on whether to support report from the receiver UE - No inter-BS communication will be considered. To discuss aspects related to 1st sub-bullet & 2nd bullet during this week -revisit later Agreements: - Sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16.

The above agreements introduce support for HARQ ACK/NACK from UE to gNB but prevent that the signaling is in the form of HARQ ACK/NACK. Although the agreement does not specify the alternative to be supported, it mentions the possibility of using SR/BSR. In our view, not using ACK/NACK poses multiple problems:

- The gNB must be aware that the resources allocated are for the retransmission of a TB (e.g., to allocate the right amount of resources, etc.). Uplink resources for ACK/NACK transmission, associated with a SL grant trivially solve this problem. That is, if the gNB receives a NACK it knows which SL grant carried the transmission. In contrast:

  - SR resources are not associated with a retransmission since they are meant for requesting resources of new transmissions.

  - BSRs do not contain information about retransmissions. Nor do they contain details about the TBs to be transmitted.

- In the current specification, transmission of SR/BSR is triggered when a new packet arrives at the TX buffer but not when a retransmission is needed.

Some companies discussed the possibility of having dedicated SR resources, which would effectively turn SR into ACK/NACK. In our view, RAN1 should avoid creating ambiguous and confusing specifications.

Although most of these issues are in the scope of RAN2, the problem originates from the above agreements by RAN1. For this reason, we propose to revert the agreement and ask RAN1 to reconsider that HARQ ACK/NACK report from the transmitting UE to gNB is supported.

Sidelink HARQ ACK/NACK report from UE to gNB is supported in Rel-16.

- Note: this reverts the agreement that sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16.

# 3	Dynamic grant

## 3.1	Dynamic grant for broadcast, groupcast, and multicast

In LTE, only broadcast transmission with up to two transmissions per TB is supported. Consequently, a grant for one/two transmissions of the same TB can be easily conveyed in DCI. NR, however, includes the following new features that complicate the procedures for dynamic grant:

- Larger number of scheduled retransmissions in a single grant (e.g., blind retransmissions for broadcast).

- HARQ feedback and retransmissions (for unicast and groupcast).

In sidelink, retransmissions play a very important role when it comes to achieving high reliability. Not only by providing HARQ combining gain but also by allowing half-duplex UEs to receive each other. For this reason, we believe that dynamic grants should also allow for scheduling retransmissions.

In uplink/downlink it is possible to configure (using RRC) the use of retransmissions. That is, each grant is repeated over K consecutive slots. This approach is, unfortunately, not suitable for sidelink as it would lead to persistent half-duplex problems; it is necessary that the gNB has the possibility to schedule different sets of slots to different UEs. In our view, it is sufficient that the gNB can provide a fixed number of repetitions with an arbitrary (but fixed) gap between them. For this purpose, we propose that DCI carries the following information:

- Gap in number of slots between transmission and first retransmission.

- Number of repetitions.

The use of consecutive repetitions (like in PDSCH or PUSCH) leads to severe half duplex issues.

Dynamic sidelink grants are valid for K retransmissions of a TB, equally spaced in time.

- K and the spacing between repetitions are signaled in DCI.

In our view, a dynamic grant should not only provide resources for PSCCH and PSSCH transmission but also indicate whether it has an associated PSFCH resource that can be used by the receiver to convey feedback. In other words, for dynamic grant, whether a PSSCH transmission is (N)ACKed or not is up to the gNB.

Sidelink dynamic grants indicate whether the receiver is expected to transmit HARQ feedback in the corresponding PSFCH resource or not.

Besides this, and assuming that HARQ feedback is conveyed to the gNB in the form of ACK/NACK, we do not think that there is any need to associate a dynamic grant to a specific sidelink HARQ process. Consequently, DCI need not carry other HARQ information (e.g., HARQ process ID, NDI, etc.).

A dynamic sidelink grant is not associated with a specific sidelink HARQ process. No additional HARQ information is carried in DCI.

## 3.2	Signalling aspects

The contents of DCI format 5A in LTE is summarized in Table 1.

Table 1. LTE DCI format 5A contents.

| Field | Size |
| --- | --- |
| Carrier indicator | 3 bits |
| Lowest index of the subchannel allocation to the initial transmission | < 5 bits (for up to 20 sub-channels) |
| SCI format 1 field - Frequency resource location of initial transmission and retransmission | < 8 bits (for up to 20 sub-channels) |
| SCI format 1 field - Time gap between initial transmission and retransmission | 4 bits |
| SL index (for TDD configuration 0-6) | 2 bits |
| SL SPS configuration index (for SPS only) | 3 bits |
| Activation/release indication (for SPS only) | 1 bit |

That is, in total up to 22 information bits are carried by DCI for dynamic grant and up to 26 bits are carried by DCI for an SPS grant. Zero-padding is used to ensure that the DCI size is the same for dynamic and SPS grants. Zero padding is also used to align the size of DCI format 5A with the size of DCI format 0.

For NR, two basic DCI formats are defined: 0-0, known as the fallback format, and 0-1, known as the non-fallback format. DCI format 0-0 can configure a smaller set of parameters and, consequently, has a smaller size than DCI format 0-1, which can, in principle, configure any supported feature. We summarize the contents for each of the formats in Table 2.

Table 2. NR DCI contents for formats 0-0 and 0-1.

| Field |  | Format 0-0 | Format 0-1 |
| --- | --- | --- | --- |
| Identifier |  | 1 bit | 1 bit |
| Resource information | CFI | – | 0 or 3 bits |
|  | UL/SUL | 0 or 1 bit | 0 or 1 bit |
|  | BWP indicator | – | 0-2 bits |
|  | Freq.-domain allocation | Variable (only type 1) | variable |
|  | Time-domain allocation | 0-4 bits | 0-4 bits |
|  | Frequency hopping | 0 or 1 bit | 0 or 1 bit |
| TB-related | MCS | 5 bits | 5 bits |
|  | NDI | 1 bit | 1 bit |
|  | RV | 2 bis | 2 bis |
| HARQ-related | Process number | 4 bits | 4 bits |
|  | DAI | – | 1-4 bits |
|  | CBGTI | – | 0, 2, 4, or 6 bits |
| Multi-antenna related | DMRS seq initialization | – | 1 bit |
|  | Antenna ports | – | 2-5 bits |
|  | SRI | – | Variable |
|  | Precoding information | – | 0-6 bits |
|  | PTRS-DMRS assoc. | – | 0 or 2 bits |
|  | SRS request | – | 2 bits |
|  | CSI request | – | 0-6 bits |
| Power control | PUSCH power control | 2 bits | 2 bits |
|  | Beta offset | – | 0 or 2 bits |

In NR, DCI will also be used to carry dynamic sidelink grants, including some information to be used in the SCI. At this point in time, the contents of SCI itself is unclear. However, DCI will at least carry the following information:

- Time-frequency resource to be used for one/multiple transmissions of a TB (size is FFS but LTE values can be used as a reference).

- HARQ flag, indicating whether the transmitter can request HARQ feedback for the transmission scheduled in the resources signaled by DCI (1 bit).

Clearly, a new DCI format will be required to convey dynamic sidelink grants. In our view, it should have the same size as one of the existing DCI formats, preferably format 0-0. A sidelink-specific RNTI can be used to distinguish the new format from the existing DCI format(s).

A new DCI format is introduced to convey dynamic sidelink grants.

The size of the new DCI format is the same as one of the existing DCI formats. A new RNTI is used for scrambling the new DCI format.

# 4	Configured grant

## 4.1	Configured grant for broadcast, groupcast, and multicast

Like for dynamic grants, we believe that for configured grants, the gNB decides for each grant whether the corresponding PSSCH transmission can be (N)ACKed or not. However, a configured grant may be used for multiple TBs over a long period of time. Some of the TBs may require HARQ feedback while others may not. Thus, for each transmission the UE may decide not to request feedback, even if allowed by the gNB.

Sidelink configured grants (type-1 and type-2) indicate whether the receiver can transmit HARQ feedback in the corresponding PSFCH resource or not.

- The transmitter UE can always decide not to request feedback.

## 4.2	Signalling aspects

In RAN1#AH-0119 it was agreed to support both type-1 and type-2 configured grants for sidelink. Type-1 and Type-2 configured grants were originally introduced for UL transmission. Consequently, the RRC configuration provides many parameters that the UE must use for transmission. For sidelink, we believe that the network should have the possibility of giving freedom to the transmitter UE to select some of the parameters, within some restrictions set by the gNB:

- The gNB provides time-frequency resources (including periodicity and offset) to be used by the UE, except if the UE has no data to transmit.

- The gNB optionally provides ranges of allowed values for other parameters such as MCS, number of layers, etc. The UE autonomously selects the specific value from the allowed range.

For sidelink configured grant type-1 and grant type-2, the gNB provides:

- Time-frequency resources (time and frequency allocation, periodicity, offset, etc.) to be used by the UE for transmission.

  - The UE shall not transmit if it does not have data to transmit or if the grant cannot accommodate the TB.

- Optionally, ranges of values for other parameters (e.g., MCS, number of layers, etc.) from which the UE can autonomously select one for each TB transmission.

One aspect that was left for further study by the agreements from RAN1#96bis is whether different transmissions of a TB can take place across multiple configured grants or not. In our view, from a RAN1 point of view there is no reason to preclude such behavior, neither from a transmitter nor from a receiver point of view. This is, however, an issue with RAN2 implications that may be necessary to discuss.

Support for transmission of a TB across multiple configured grants is up to RAN2.

As captured above, RAN2 has agreed to support multiple configured grants. In our view, the RAN2 agreement refers to simultaneously configured grants. In our view, the RAN1 impact is reduced to introducing the relevant fields in DCI for type-configured grants.

RAN2 has agreed that NR sidelink supports having multiple configured grants for sidelink.

Corresponding identifying fields are introduced in DCI (for type-2 only) and RRC.

In terms of supported periodicities, our view is that both type-1 and type-2 configured should support advanced V2X services as well as basic V2X services. In terms of periodicities, this means:

- 1, 2, 5, 10, 20, 50 ms for advanced V2X.

- 100, 200, 300, …, 1000 ms for basic V2X.

For sidelink configured grant type-1 and grant type-2, the following periodicities are supported: {1, 2, 5, 10, 20, 50, 100, 200, 300, 400, 500, 600, 700, 800, 900, 1000} ms.

In RAN1#96bis, there were discussions on the need to support confirmation of type-2 configured grants. In parallel, RAN2 agreed to support this as captured in the above agreements. The question is then whether L1 or L2 signaling should be used for the purpose and whether RAN1 or RAN2 should take the initiative. In our view, RAN2 should be responsible of specifying this, liaising with RAN1 if necessary for two reasons:

- It was RAN2 who first agreed to support this functionality, leaving details FFS without any mention to RAN1 in their agreements.

- NR already includes support for confirmation messages for uplink grants in the MAC specification. In our view, it is desirable to use the same procedures for SL whenever possible.

RAN2 has agreed that NR sidelink supports confirmation of sidelink configured grant type-2 activation/deactivation.

RAN1 assumes that RAN2 takes the lead in specifying the support for confirmation of sidelink configured grant type-2 activation/deactivation.

### 4.2.1	DCI for type-2 configured grant

For type-1 sidelink configured grant, activation/release is performed using RRC. In contrast, for type-2 sidelink configured grant PDCCH is used for activation/release. Like in UL, we propose to use the DCI format used for dynamic grant to activate the type-2 SL configured grant. To distinguish between dynamic and configured grants, a different RNTI may be used (e.g., C-RNTI vs CS-RNTI). Like in LTE, we propose to use zero-padding to align the sizes of DCI carrying dynamic grants and configured grants type-2.

The DCI format used for providing sidelink dynamic grants is also used for activation of type-2 sidelink configured grants but scrambled with a different RNTI.

- Zero-padding is used to align the size of DCI carrying dynamic grants and configured grants type-2.

Regarding the contents of DCI, some considerations are necessary in order to support multiple simultaneously type-2 configured grants for sidelink. First, it is to identify each of the type-2 configured grants. In our view, we should follow the same approach taken in LTE and agreed for the Uu enhancements, and include the identifier in SCI. Second, it is necessary to discuss the number of type-2 configured grants that can be addressed by a single DCI. This discussion has taken place multiple times in 3GPP in the context of V2X. In our view, the conclusions still hold and, thus, propose to schedule a single such grant. In addition, a quick inspection of the sizes of LTE DCI format 5A and NR DCI format 0-0 reveals that it is unlikely that a single DCI can address more than one configured grant.

A field in DCI identifies the type-2 configured grant that is activated/deactivated.

Each DCI activates/deactivates a single SL type-2 configured grant.

# 5	Cross-RAT management of NR sidelink

## 5.1	LTE Uu controlling NR mode-1 UEs

During RAN1-ah1901 it has been agreed to support scheduling by LTE Uu of NR sidelink mode-1 transmissions based on type 1 configured grant. The support is done based on type 1 configured grant with configurations restricted to time, frequency, and periodicity, with the condition that no additional function/procedure is to be introduced for LTE Uu. For type 1 configured grant, in which the grant is configured and activated directly via RRC, the configuration also conveys transmission parameters such as periodicity, time offset, MCS, precoder matrix and frequency resources etc. DCI based scheduling and type 2 configured grant scheduling are not supported for scheduling NR sidelink mode 1.

RAN2 has already agreed to support configuration for NR mode-2 operation through RRC signaling and through SIB signaling (with some restrictions). In our view, no further work is expected in RAN1.

RAN2 has agreed to support configuration for NR mode-2 operation through RRC signaling and through SIB signaling. No further work is required in RAN1 on the topic of control of NR mode-1 UEs over LTE Uu.

# 6	Conclusion

In the previous sections we made the following observations:

Observation 1	The use of consecutive repetitions (like in PDSCH or PUSCH) leads to severe half duplex issues.

Observation 2	RAN2 has agreed that NR sidelink supports having multiple configured grants for sidelink.

Observation 3	RAN2 has agreed that NR sidelink supports confirmation of sidelink configured grant type-2 activation/deactivation.

Observation 4	RAN2 has agreed to support configuration for NR mode-2 operation through RRC signaling and through SIB signaling. No further work is required in RAN1 on the topic of control of NR mode-1 UEs over LTE Uu.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Sidelink HARQ ACK/NACK report from UE to gNB is supported in Rel-16.

Note: this reverts the agreement that sidelink HARQ ACK/NACK report from UE to gNB is not supported in Rel-16.

Proposal 2	Dynamic sidelink grants are valid for K retransmissions of a TB, equally spaced in time.

	K and the spacing between repetitions are signaled in DCI.

Proposal 3	Sidelink dynamic grants indicate whether the receiver is expected to transmit HARQ feedback in the corresponding PSFCH resource or not.

Proposal 4	A dynamic sidelink grant is not associated with a specific sidelink HARQ process. No additional HARQ information is carried in DCI.

Proposal 5	A new DCI format is introduced to convey dynamic sidelink grants.

Proposal 6	The size of the new DCI format is the same as one of the existing DCI formats. A new RNTI is used for scrambling the new DCI format.

Proposal 7	Sidelink configured grants (type-1 and type-2) indicate whether the receiver can transmit HARQ feedback in the corresponding PSFCH resource or not.

	The transmitter UE can always decide not to request feedback.

Proposal 8	For sidelink configured grant type-1 and grant type-2, the gNB provides:

	Time-frequency resources (time and frequency allocation, periodicity, offset, etc.) to be used by the UE for transmission.

o	The UE shall not transmit if it does not have data to transmit or if the grant cannot accommodate the TB.

	Optionally, ranges of values for other parameters (e.g., MCS, number of layers, etc.) from which the UE can autonomously select one for each TB transmission.

Proposal 9	Support for transmission of a TB across multiple configured grants is up to RAN2.

Proposal 10	Corresponding identifying fields are introduced in DCI (for type-2 only) and RRC.

Proposal 11	For sidelink configured grant type-1 and grant type-2, the following periodicities are supported: {1, 2, 5, 10, 20, 50, 100, 200, 300, 400, 500, 600, 700, 800, 900, 1000} ms.

Proposal 12	RAN1 assumes that RAN2 takes the lead in specifying the support for confirmation of sidelink configured grant type-2 activation/deactivation.

Proposal 13	The DCI format used for providing sidelink dynamic grants is also used for activation of type-2 sidelink configured grants but scrambled with a different RNTI.

	Zero-padding is used to align the size of DCI carrying dynamic grants and configured grants type-2.

Proposal 14	A field in DCI identifies the type-2 configured grant that is activated/deactivated.

Proposal 15	Each DCI activates/deactivates a single SL type-2 configured grant.

# 7	References

Chairman’s notes, RAN1#96bis, April 2019, Xian, China.

Chairman’s notes, RAN2#105bis, April 2019, Xian, China.