3GPP TSG-RAN WG1 Meeting #97	 R1-1907148

Reno, Nevada, U.S., May 13th – 17st 2019

Agenda Item:	7.2.4.8

Source:	Ericsson

Title:	Link level evaluations of NR PSSCH

Document for:	Discussion

# 1	Introduction

In this contribution, we provide link-level evaluation results focusing on several aspects of the PSSCH and discuss their implication to the design as well as to the transmission schemes of the PSSCH. Specifically, the following are evaluated:

- DMRS design for PSSCH and selection of subcarrier spacing.

- Signaling in the AGC settling symbol.

The channels used in our evaluations are the CDL models specified in TR37.885, with a recent correction of AoA provided in [1] and endorsed in email discussion.

# 2	DMRS design for PSSCH

## 2.1	Simulation setup

We consider the baseline physical layer format of an NR SL slot (14 OFDM symbols). We further assume in our simulations that the PSSCH consists of 10 symbols, from the 4th to the 13th symbol in the baseline format, as illustrated in Figure 1.

Figure 1: Simulation setup of DMRS for PSSCH

The DMRS used for our evaluation is placed in the 1st, the 4th, the 7th, and the 10th symbol of the PSSCH, i.e., the 4th, the 7th, the 10th, and the 13th symbol of the baseline slot format. We use comb-2 for the DMRS and the remaining subcarriers are used for data. We focus on the highway scenario and use the corresponding CDL channel models for V2V derived from the general channel model in TR37.885.

The remaining simulation assumptions are captured in Table 2 in the Appendix.

## 2.2	Simulation results

To capture the impact of DMRS on the channel estimation performance, we investigate the BLER vs. SNR at different relative UE speeds and subcarrier spacings. We focus on high-speed scenarios since the channel estimation is more challenging in those scenarios.

Figure 2: BLER vs. SNR for TR 37.885 CDL HIGHWAY_NLOS_V channel, 2Tx and 4Rx antennas. Left: QPSK code rate ½, 15 kHz SCS. Right: 16-QAM code rate 0.8, 30 kHz SCS.

Figure 3: BLER vs. SNR for TR 37.885 CDL HIGHWAY_LOS channel. 2Tx and 4Rx antennas. Left: QPSK code rate ½, 15 kHz SCS. Right: 16-QAM code rate 0.8, 30 kHz SCS.

As can be observed from Figure 2 and Figure 3, a subcarrier spacing of 30 kHz combined with the DMRS structure in Figure 1 provides good performance even at very high relative speeds and high MCS. Note that the subcarrier spacing has a big impact on the performance since the larger the value, the shorter the distance between two consecutive DMRS symbols, which results in more accurate channel estimation. Specifically, at 15 kHz subcarrier spacing the time interval between the two consecutive DMRS symbols is too big for relative speed of 500 km/h, while at 30 kHz spacing the DMRS symbols are close enough to give good performance.

To summarize, our results show that the DMRS structure (as in Figure 1) is a good candidate for DMRS of the PSSCH. We also show that 30 kHz subcarrier spacing performs better in terms of channel estimation.

DMRS structure for PSSCH in Figure 1 provides acceptable reliability for SL V2X.

For carriers below 6 GHz, 30 kHz subcarrier spacing with 4 DMRS provides acceptable reliability in high-speed scenarios.

# 3	Signalling in the AGC settling symbol

In this section we investigate different options of signaling in the first OFDM symbol of a slot. For that purpose, we employ the DMRS pattern described in Section 2, except for one of the configurations. Furthermore, based on our observations in Section 2, we focus on 30kHz SCS in this section.

Slot and receiver configurations shown in Table 1 are considered to evaluate the need for designing the slot to account for Automatic Gain Control (AGC). The second and third OFDM symbols (OSs) are allocated to PSCCH in all the configurations. The final OS is unused and acts as guard period (GP). Two packet sizes, 300 Byte and 1000 Byte, are considered for the evaluation. A fixed modulation order is used, while the LDPC code rate is selected according to the number of available data resource elements (REs). The simulation parameters used for the evaluation are listed in Table 3 in the Appendix.

Table 1: Slot and receiver configurations for AGC

| Data PSCCH DMRS AGC Preamble |  |
| --- | --- |
| Configuration | Description |
| A) Data on OS1 with Genie AGC B) Punctured OS1 | - No provision in made for AGC and OFDM symbol (OS) 1 carries data - A receiver with Genie AGC (Configuration A) uses the OS 1 for demodulation and decoding of data - A receiver with Practical AGC (Configuration B) uses the OS 1 for AGC operation and we consider a worst-case scenario where the OS 1 is not used for decoding the data. This is equivalent to puncturing the coded bits that are carried by OS 1. - Duration allowed for AGC operation = 1 OS duration. Nrof Data REs per PRB = 108 |
| C) AGC preamble on OS1 | - OS 1 carries a dedicated AGC preamble to aid the receiver in AGC operation. Dedicated AGC preamble reduces the number of REs available for mapping data - Duration allowed for AGC operation = 1 OS duration. Nrof Data REs per PRB = 96 |
| D) IFDM of Data on OS1 | - OS 1 carries data on alternating subcarriers (IFDM) and the remaining subcarriers are unused. - The IFDM structure results in an OS with two repeated identical copies. At the receiver, the first copy is used for AGC operations and the second copy is used for demodulation and decoding of data [2]. - The data REs on OS 1 are power boosted by a factor of 3 dB. - Duration allowed for AGC operation = 0.5 x OS duration. Nrof Data REs per PRB = 102 |

Figure 4 BLER vs. SNR for 300 Byte packet

Figure 5 BLER vs SNR for 1000 Byte packet

300 Byte Packet:

Figure 4 shows BLER as a function of SNR for slots carrying 300 Byte packets. The modulation order is fixed to QPSK and the LDPC code rate is varied according to the number of REs available for data. The LDPC code rate is 0.4274, 0.4525, and 0.4808 for Configurations A/B, D, and C, respectively.

The loss in BLER performance when using Config. B is less than 1 dB and 1.5 dB for 0 and 1 retransmissions, respectively, compared with Config. A. The loss in BLER performance is small since the puncturing of the coded bits (carried by OS1) of the low rate code-block results in only a small fraction of the code-block being lost. The Configurations C and D perform better than Config. B since the code-block is not punctured. The IFDM configuration D performs slightly better than Config. C since the IFDM configuration allows more REs for mapping data and therefore has a smaller code rate.

1000 Byte Packet:

Figure 5 shows BLER as a function of SNR for slots carrying 1000 Byte packets. The modulation order is fixed to 16-QAM and the LDPC code rate is varied according to the number of REs available for data. The LDPC code rate is 0.7123, 0.7541, and 0.8013 for Configurations A/B, D, and C, respectively.

In contrast to the case of 300 Byte packet, the BLER with no retransmissions when using Config. B is large and close to 1. The BLER performance of Config. B improves with a single retransmission, however, the performance gap with the other configurations is large. The large degradation in BLER performance of Config. B is due to the higher code rate and that the puncturing of the OS1 results in a significant portion of the first code-block to be lost. With an additional (again punctured) redundancy version, arriving in the retransmission, the code-block becomes decodable. The behavior of the other configurations is similar to the case of 300 Byte packet.

The degradation of BLER for 300 Byte packet due to puncturing of the coded bits carried by OS1 is not significant.

For large packets (e.g. 1000 Bytes) with higher code rates, BLER can be expected to be large in case of Punctured OS1.

The IFDM configuration have better BLER performance compared to Punctured OS1 and AGC preamble due to lower code rate and no puncturing.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	DMRS structure for PSSCH in Figure 1 provides acceptable reliability for SL V2X.

Observation 2	For carriers below 6 GHz, 30 kHz subcarrier spacing with 4 DMRS provides acceptable reliability in high-speed scenarios.

Observation 3	The degradation of BLER for 300 Byte packet due to puncturing of the coded bits carried by OS1 is not significant.

Observation 4	For large packets (e.g. 1000 Bytes) with higher code rates, BLER can be expected to be large in case of Punctured OS1.

Observation 5	The IFDM configuration have better BLER performance compared to Punctured OS1 and AGC preamble due to lower code rate and no puncturing.

# 5	References

R1-1904226, V2X antenna model and pathloss correction, Fraunhofer HHI, Fraunhofer IIS, Huawei, HiSilicon, RAN1 #96bis, Apr. 2019

R1-162825, “L1 Format for V2V Transmissions using Sidelink,” Ericsson, RAN1 #84bis, Apr. 2016.

# 6	Appendix: Simulation assumptions

Table 2: Simulation settings for the evaluation of DMRS configuration

| Parameters | Values |
| --- | --- |
| Carrier frequency | 6 GHz |
| Subcarrier spacing | 15 kHz, 30 kHz |
| Carrier frequency offset | 0 ppm |
| Channel estimation | Non-ideal (with genie-aided Doppler knowledge) |
| Waveform | CP-OFDM |
| Number of layers | 1 |
| Number of Tx antennas | 2 (one dual-polarized) |
| Number of Rx antennas | 4 (two dual-polarized) |
| Receiver type | MRC |
| Channel model | CDL-based V2V channels in TR 37.885 |

Table 3: Simulation settings for the evaluation of AGC configurations

| Parameter | Value |  |
| --- | --- | --- |
| Carrier frequency | 6 GHz |  |
| Sub-carrier spacing | 30 kHz |  |
| System bandwidth | 10 MHz (26 PRBs) |  |
| Channel model | CDL-based V2V channels in TR 37.885: Highway NLoSv |  |
| Vehicle speed | 60 km/h |  |
| Number of Tx antennas | 2 (one dual-polarized) |  |
| Number of Rx antennas | 2 (one dual-polarized) |  |
| Packet sizes | 300 bytes | 1000 bytes |
| Modulation order | QPSK | 16-QAM |
| LDPC code rate | Variable | Variable |
| HARQ retransmissions | 0 or 1 |  |
| Channel estimation | Non-ideal (with genie-aided Doppler knowledge) |  |
| Receiver type | MRC |  |