3GPP TSG RAN WG1 #97			R1-1907167

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.4.5

Source:	AT&T

Title:	Physical layer procedures for sidelink

Document for:	Discussion/Approval

# Introduction

In this paper, we give our view on the further design of NR sidelink.

# Enhance OLLA to reduce CQI report

Typical link adaption includes two schemes, inner loop and outer loop link adaptation:

ILLA (Inner Loop Link Adaption) is a link adaption scheme which can determine the MCS before the data transmission. It relies on a close loop CSI (RI, PMI, CQI) feedback to select the right MCS. While it provides accurate estimation, it also have certain limitation, especially in sidelink context:

- It requires CSI-RS transmission and reception to estimate the channel, and it includes both signal and interference estimation. However, sidelink resource utilization is sensing based, it’s complicated to coordinate some resources for CSI-RS transmission.

- It requires CQI report with small latency (before the channel or interference condition changed). It may require a feedback channel between receiver and transmitter. Again, it’s rather complicated to coordinate resource for such feedback channel.

OLLA (outer loop link adaption) is a link adaption scheme which can determine the MCS based on historical data transmission. It uses the historical HARQ feedback to adjust the selected MCS. It doesn’t require additional feedback HARQ feedback is anyway available at transmitter. However, it also has its limitation that it takes time to converge. When the channel or interference condition changes too fast, OLLA might be too slow to follow it.

Typically, ILLA and OLLA can be enabled simultaneously, however, if it’s too costly to coordinate the resource for RS transmission and CQI feedback in sidelink. There should be a scheme which works with OLLA only. We propose to enhance HARQ ACK/NAK feedback to carry a little bit more information to make OLLA more efficient. Normally, OLLA scheme is designed to be very sensitive to the NAK and less sensitive to ACK according to target BLER. E.g. when BLER target is 10%, the impact of one NAK is approximately the same as 9 ACKs. As a result, it takes much longer time when OLLA tuning the MCS up than when it tunes MCS down. If HARQ ACK/NAK also includes an additional state, i.e. ACK+, which indicates the receiver not only decode the data packet successfully, it also detects that the MCS of the data packet is too conservative comparing to current channel and interference state. At transmitter side, when received an ACK+, it can tune MCS up faster than a regular ACK. That can help OLLA scheme converge to the optimal MCS much faster.

Proposal-1: HARQ ACK/NAK feedback should be able to carry an additional state which indicate that the MCS of current data packet is too conservative to reduce the need of CQI feedback.

# Basic principles about sidelink power control

Power control is a key tool to reduce the total interference in a system. The basic principle is to control the transmission power to make sure it has just enough SINR to be received. Besides, despite all the enhancements, sidelink is still not a well-coordinated system from resource utilization point of view. Thus, power control is even more needed to limit system total interference level. While the access link power control always targets to the gNB, sidelink power control may have multiple destination depending on the traffic type. Thus, the sidelink PC should be per destination power control configurability. For unicast traffic, the PC configurability is for each destination. For group cast, PC configurability is a for a group destination.

Proposal-2: NR sidelink power control scheme should support configurable destination with different power control parameters.

Another important aspect the power sharing. To support simultaneously uplink and sidelink transmission, the total transmission power must be below a certain threshold which is required by SAR. NR sidelink must support a power sharing scheme involving priority of power usage for uplink and sidelink for which channel, etc.

Proposal-3: determine a dynamic power sharing scheme to determine the priority and share Tx power between uplink and sidelink transmission.

# Conclusion

Proposal-1: HARQ ACK/NAK feedback should be able to carry an additional state which indicate that the MCS of current data packet is too conservative to reduce the need of CQI feedback.

Proposal-2: NR sidelink power control scheme should support configurable destination with different power control parameters.

Proposal-3: determine a dynamic power sharing scheme to determine the priority and share Tx power between uplink and sidelink transmission.

# Reference