3GPP TSG-RAN WG1 Meeting #97	 R1-1907182

Reno, USA, May 13th – 17th, 2019

Source:	Ericsson

Title:	Evaluation methodology and metrics for 2-step RACH

Agenda Item:	7.2.1.3

Document for:	Discussion

# Introduction

In this contribution, we discuss further details of the 2-step RACH evaluation methodology and performance metrics beyond those agreed in RAN1#96bis [1]. Drivers for link vs. system evaluation and  issues with multi-UE link level evaluation are considered, and additional system level parameters needed for coverage evaluations are proposed.

# Link-level evaluations

## Link vs. system evaluation

As agreed in email discussions after RAN1#96, link level simulations will be conducted to evaluate the achievable payload of msgA under different conditions as well as to assess different mechanisms for transmitting msgA. Most of the mechanisms discussed do not address interference, and single UE evaluation was agreed as a starting point. However, in RAN1#96bis, it was agreed that evaluations of 2 or more colliding UEs at the link level can be reported, although the power modeling is FFS and companies are to report the modeling until the FFS is resolved.  Large differences in power can occur in cells where uplink power control is either insufficient to overcome pathloss and/or when power control error is present, and so such power differences should be properly considered in multi-UE link level simulations.  (Note that further consideration of models for UE power differences in different cell sizes is given in a companion paper [2]). Using system level simulations has the benefit of avoiding the difficult problem of modeling multiple UE transmissions using link level simulators. The coupling loss of the different UEs to a serving cell, power control, traffic models, and inter-cell interference can all strongly affect performance and are difficult to take into account in typical link level simulators.

System level simulations are primarily targeted at determining system capacity. However, the greatest 2-step RACH capacity benefits for small packet transmission are expected to be from quick completion of the RACH procedure and from getting good uplink CSI for PUSCH transmissions following msgA, rather than optimizing the capacity of msgA itself. Therefore, how capacity should be evaluated in 2-step RACH should be further considered before launching into a system level evaluation of 2-step RACH capacity.

Although msgA may not contribute greatly to net capacity gains from 2-step RACH, it may still be beneficial to support a variety of msgA payload sizes. The SINR in a cell can be quite high over a large part of a cell, but on the other hand the SINR at the cell edge can be rather lower.  For example, IMT 2020 requirements [4] for rural eMBB are 3.3 bps/Hz mean user throughput, but 0.12 bps/Hz for 5%-ile user throughput. Therefore, limiting the msgA payload to only the 5% coverage would preclude UEs in the majority of the cell from being able to transmit substantially larger payload sizes.  Such a feature may be exploited to efficiently carry bursty small packets in RRC_CONNECTED UEs with low latency, as further discussed in [5].

Observations:

- Large differences in power can occur between UEs in a cell when pathloss is large and/or power control error is present.

- The capacity benefit for 2-step RACH used with small data applications is not likely to come from optimizing the capacity of msgA PUSCH

  - Capacity of PUSCH that can be scheduled after msgB can be substantially better than msgA due to better link adaptation, scheduling flexibility, etc.

- On the other hand 2-step RACH seems well suited to efficiently carry bursty small packets in RRC_CONNECTED UEs with low latency

- Also, SNR is likely to be quite high over a substantial part of e.g. a small cell, so some flexibility to support different msgA sizes has potential.

Proposals:

- Multi-UE simulations take into account relevant ‘system’ behaviors such as power and timing differences among UEs.

- Any evaluations of capacity are done with system level simulation

- Study the benefit of variable msgA payload size

## System parameters for coverage evaluation

System simulation parameters are needed to determine the coverage of different msgA payload sizes. Only those parameters needed to calculate coupling loss are required if simple coverage evaluations not considering interference are used. While it has been agreed so far to use various channel models, scenarios, and ISDs, additional parameters are still needed. We propose the following based on recent evaluations done in the NOMA study [3]:

Table 2: System level parameters for coverage evaluations

| Parameters | Values |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid, 57 cells |
| Inter-BS distance | 200m |
| Carrier frequency | 4 GHz |
| Channel model | UMi in TR 38.901 |
| UE Tx power | Max 23 dBm |
| BS antenna configurations | 2 ports: (M, N, P, Mg, Ng) = (10, [1,2,4], 2, 1, 1), +-45 Polarization dH = dV = 0.5λ; |
| BS antenna downtilt | 102 |
| BS antenna height | 25m |
| BS antenna element gain + connector loss | 8 dBi, 0dB cable loss |
| BS receiver noise figure | 5dB |
| UE antenna configuration | 1 |
| UE antenna height | Follow the modelling of TR 38.901 |
| UE antenna gain | 0dBi |
| UE distribution | Follow the modelling of TR 38.901 |
| UE attachment | Refer to 36.873 |

Proposal:

- 2-step RACH coverage evaluations use the parameters and values in Table 2 for the 200m ISD UMi scenario

# Conclusions

In this contribution, we have considered further details of the 2-step RACH evaluation methodology and performance metrics for 2-step RACH. Drivers for link vs. system evaluation were considered, where we found that evaluation of a single radio link is a sufficient starting point for link level studies. A few additional link level parameters were identified that could help to align results at least on minimum payload size from different companies. Missing system level simulation parameters were identified that are needed for coverage evaluations. We made the observations below leading to the following proposals:

Observations:

- Large differences in power can occur between UEs in a cell when pathloss is large and/or power control error is present.

- The capacity benefit for 2-step RACH used with small data applications is not likely to come from optimizing the capacity of msgA PUSCH

  - Capacity of PUSCH that can be scheduled after msgB can be substantially better than msgA due to better link adaptation, scheduling flexibility, etc.

- On the other hand 2-step RACH seems well suited to efficiently carry bursty small packets in RRC_CONNECTED UEs with low latency

- Also, SNR is likely to be quite high over a substantial part of e.g. a small cell, so some flexibility to support different msgA sizes has potential.

Proposals:

- Multi-UE simulations take into account relevant ‘system’ behaviors such as power and timing differences among UEs.

- Any evaluations of capacity are done with system level simulation

- Study the benefit of variable msgA payload size

- 2-step RACH coverage evaluations use the parameters and values in Table 2 for the 200m ISD UMi scenario

# References

“Draft Report of 3GPP TSG RAN WG1 #96b v0.2.0 (Xi'an, China, 8th – 12th April 2019)”, MCC Support, April 26, 2019

R1-1907187, “Modeling UE power differences in link simulations”, Ericsson, 3GPP TSG-RAN WG1 Meeting #97, Reno, USA, May 13-17, 2019

3GPP TR 36.812 v16.0.0, “Study on Non-Orthogonal Multiple Access (NOMA) for NR (Release 16)”, December 2018

Report ITU-R M.2410, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, Nov. 2017

R1-1907179, “Use Cases and Scenarios for 2-Step RACH”, Ericsson, 3GPP TSG-RAN WG1 Meeting #97, Reno, USA, May 13-17, 2019