3GPP TSG-RAN WG1 Meeting #97	R1-1907187

Reno, USA, May 13th – 17th, 2019

Agenda Item:	7.2.1.3

Source:	Ericsson

Title:	Modeling UE power differences in link simulations

Document for:	Discussion

# 1	Introduction

In RAN1#96bis, it was agreed that evaluations of two or more colliding UEs at the link level can be reported, although the power modeling is FFS and companies are to report the modeling until the FFS is resolved [1].  This contribution considers requirements for such models, studying some example scenarios with different inter-site spacing in order roughly quantify the degree of power control differences that may be expected as a function of cell size.

# 2	UE Near-Far Statistics vs. Cell Size

We consider the NOMA Case 1, Case 2, and Case 3 NOMA scenarios, which were for a large (1732m ISD), medium (500m ISD), and a small cell (200m ISD), respectively [2].   While these are somewhat different from those agreed for the 2-step RACH work, the behavior as a function of cell size should be similar and sufficient to study basic differences in modelling requirements in large and small cells.

We use coupling loss results from the NOMA system level simulation calibration exercise after RAN1#94 [3] to determine the average received power at the gNB when open loop power control is used.  The power control formula from 38.213 is applied using a P0=-90 with full path loss compensation (α=1), and transmission in 2 PRBs is assumed. Detailed simulation parameters are in the Appendix. The resulting Rx power CDFs are in Figure 1 below. As can be seen, the UE is in power limit about 85%, 35%, and 20% of the time, for Cases 1, 2, and 3, respectively.  Therefore, UEs have relatively constant power (assuming ideal power control) at gNB for cases 2 and 3, but the UE has difficulty reaching its power control target for Case 1 over the all but the positions in the cell closest to gNB.

Figure 1

We use the receive power CDFs to determine the statistics of the received power when multiple UEs transmit. One statistic of interest is the ratio of the strongest to the weakest UE that transmit simultaneously. This is given for Case1 and Case 3 in Figure 2 below. For Case 1, the median power difference for 2 UEs is 10 dB. This rises to 17 dB for 3 UEs, then to 20 and 24 dB for 4 and 6 UEs, respectively. The 90% CDF points are 23, 27, 29, and 31 dB for 2, 3, 4, and 6 UEs. For Case 3, the median power differences are 0 dB for 2 and 3 UEs, but rise to 1 and 4 dB for 4 and 6 UEs, respectively. The 90% points are 7, 9, 10, and 11 dB for 2, 3, 4, and 6 UEs.

Observations:

- Modeling the power difference between multiple colliding UEs in large cells is essential, but rather less so for small cells with a small number of colliding UEs.

  - Median differences between the strongest and weakest UE received at gNB in a 1732m Case 1 cell vary from 10 to 24 dB for 2 to 6 colliding UEs.

    - 90% differences range from 23 to 31 dB.

  - The median difference between the strongest and weakest UE received at gNB in a 200m Case 3 cell is 0 dB for 2 and 3 UEs, and 1 and 4 dB for 6 UEs.

    - 90% differences range from 7 to 11 dB.

Figure 2: Near-Far UE Power Differences for Case 1 and Case 3

Proposals:

- If link level performance of UE collision in large cells (e.g. 1732m ISD) is to be studied, power differences are carefully modelled

- Link level evaluations of collision use at most a few (say 2 or 3) UEs as a starting point, assuming conditions reflective of small cells (e.g. 200m ISD).

# 3	Conclusions

This contribution considered requirements for models of power difference needed in 2-step RACH work, studying some example scenarios with different inter-site spacing in order roughly quantify the degree of power control differences that may be expected as a function of cell size.  We made the following observations:

Observations:

- Modeling the power difference between multiple colliding UEs in large cells is essential, but rather less so for small cells with a small number of colliding UEs.

  - Median differences between the strongest and weakest UE received at gNB in a 1732m Case 1 cell vary from 10 to 24 dB for 2 to 6 colliding UEs.

    - 90% differences range from 23 to 31 dB.

  - The median difference between the strongest and weakest UE received at gNB in a 200m Case 3 cell is 0 dB for 2 and 3 UEs, and 1 and 4 dB for 6 UEs.

    - 90% differences range from 7 to 11 dB.

We therefore propose:

Proposals:

- If link level performance of UE collision in large cells (e.g. 1732m ISD) is to be studied, power differences are carefully modelled

- Link level evaluations of collision use at most a few (say 2 or 3) UEs as a starting point, assuming conditions reflective of small cells (e.g. 200m ISD).

# 4	References

- “Draft Report of 3GPP TSG RAN WG1 #96b v0.2.0 (Xi'an, China, 8th – 12th April 2019)”, MCC Support, April 26, 2019

- 3GPP TR 36.812 v16.0.0, “Study on Non-Orthogonal Multiple Access (NOMA) for NR (Release 16)”, December 2018

- R1-1810048, “Email discussion on the SLS calibration and template for collecting SLS evaluation results”, RAN1 #94, Gothenburg, Sweden, August 20-24, 2018.

# Appendix

Table 1: System-level assumptions for calibration purpose

| Parameters | Case 1 | Case 2 |  | Case 3 |  |
| --- | --- | --- | --- | --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |  |  |  |  |
| Inter-BS distance | 1732m | 500m |  | 200m |  |
| Carrier frequency | 700MHz | 700MHz |  | 4GHz |  |
| Channel model | UMa in TR 38.901 |  |  |  |  |
| UE Tx power | Max 23 dBm |  |  |  |  |
| BS Tx power | Max 46 dBm |  |  |  |  |
| BS antenna configurations | 2 ports: (M, N, P, Mg, Ng) = (10, 1, 2, 1, 1), +-45 Polarization dH = dV = 0.8λ; |  |  |  |  |
| BS antenna downtilt | 92 |  | 98 |  | 102 |
| BS antenna height | 25m |  |  |  |  |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |  |  |  |  |
| BS receiver noise figure | 5dB |  |  |  |  |
| UE antenna configuration | 1 |  |  |  |  |
| UE antenna height | Follow the modelling of TR 38.901 |  |  |  |  |
| UE antenna gain | 0dBi |  |  |  |  |
| UE distribution | Follow the evaluation assumptions |  |  |  |  |
| UE power control | Open loop PC, P0 = [-90] dBm, alpha = 1. |  |  |  |  |
| HARQ/repetition | 1 |  |  |  |  |
| UE attachment | Refer to 36.873 |  |  |  |  |