3GPP TSG RAN WG1 Meeting #97	R1-1907191

Reno, USA, 13th – 17th May, 2019

Agenda Item:	6.2.1.1

Source: 	Sony

Title:	UE-group wake-up signal for MTC

Document for:	Discussion and Decision

# Introduction

In RAN1 meeting #96bis [1], the following related agreements on UE-group wake up signal for MTC were made:

Agreement

- For evaluation purpose and down-selection on multiplexing schemes in RAN1#97:

- UE assumes the same WUS max duration and same transmit power for Rel-15 WUS and Rel-16 WUS

- Down-select one of the following options until RAN1#97 based on evaluation results including power saving gain, usage of resources, etc

- Up to 2 orthogonal WUS resources may be configured in time domain

- Up to 2 orthogonal WUS resources may be configured in frequency domain

- Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total)

- Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured)

- Determine in RAN1#97 whether legacy WUS resource is counted as one of the configured WUS resource(s).

- Rel-15 WUS and Rel-16 WUS can be configured on the same legacy WUS resource via SI

- FFS explicitly or implicitly

- Same WUS parameters are assumed for both Rel-15 and Rel-16 WUS in case both are on the same legacy WUS resource

- Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation.

- Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap.

- If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, a common WUS for all the group WUS UEs in the same WUS resource can be configured to be legacy WUS or a non-legacy WUS.

In this contribution, we discuss some aspects on the operation of UE-group wake-up signal for MTC (MWUS), particularly related to the above recent agreements.

# Discussion

Subgrouping of UEs within a MWUS allows only a subgroup of UEs to be woken up by the MWUS for a paging occasion (PO). The UE subgrouping operation will reduce the number of UEs having to wake up unnecessarily to detect MPDCCH and decode PDSCH. Thus, it reduces UE power consumption for such UEs.

# Multiplexing of MWUS

UE-group MWUS in Rel-16 should consider coexistence with MWUS operation introduced in Rel-15 (legacy). One of the easiest solutions is to allocate different WUS physical resources for UE-group MWUS and legacy MWUS. Moreover, MWUS occupies 2 PRBs and theoretically an eNB can configure 3 MWUS resources in a narrowband (FDM-ed approach). In this case, up-to three UE-groups can be introduced for MTC UE Rel-16 and MTC UE Rel-15 can be assigned to any of these 3 MWUS resources. Note: MTC Rel-15 UEs are not aware of UE grouping. Single-seq CDM and TDM have also been considered for the WUS multiplexing scheme. The main issue with TDM is a potentially larger delay. The main issue with FDM is the limited number of groups (up to 3). Some combination options for multiplexing of MWUS, such as single-seq. CDM, FDM, and TDM, have been down-selected as shown in the section 1. Further down-selections are still needed in order to define:

- Multiplexing of legacy rel.15 MWUS and rel.16 MWUS

- Multiplexing of n-groups of rel.16 MWUS

We consider the legacy Rel-15 MWUS and Rel-16 MWUS should be in separated MWUS resources. The introduction of common group and single-seq CDM make it is not desirable to have a common resource between legacy Rel-15 MWUS and Rel-16 MWUS. Depending on single-seq CDM design, this could compromise the legacy rel.15 UEs as those UEs are not aware of those aspects. Other multiplexing strategies are FDD and TDD. TDD operation introduces additional time-gap between the MWUS and PO. We have to aim both legacy Rel-15 MWUS and Rel-16 MWUS should have the same time-gap. FDD operation does not encounter this issue.

Proposal 1: Support FDM for multiplexing between legacy Rel-15 MWUS and Rel-16 MWUS.

The maximum number of supported groups of rel.16 MWUS has not been defined yet. Considering the progress of the previous meeting, the following schemes are considered and further down-selection may still be needed:

- Up to 2 orthogonal WUS resources may be configured in time domain

- Up to 2 orthogonal WUS resources may be configured in frequency domain

- Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total)

- Up to 2 orthogonal WUS resources may be configured either in time or frequency domain (only one of the two can be configured)

We consider the aforementioned WUS resources are dedicated for both Rel-16 MWUS allocation. Thus, related to the previous agreement on: “Determine in RAN1#97 whether legacy WUS resource is counted as one of the configured WUS resource(s).”, we propose to have separate MWUS resources.

Proposal 2: Legacy MWUS (rel.15) and group MWUS (rel.16) are not sharing the same MWUS resource (i.e. multiplex in FDD).

In order to maximize the number of supported groups, we propose that up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total).

Proposal 3: Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total)

In a legacy LTE system, the probability of UEs having the same paging frame as a function of the number of paging frames is given in Table 1. While a reduction of the occasions of many UEs being paged at the same time can be achieved by allocating many PFs, this solution might not be the optimal one as more resources are being used to carry paging information.

Table 1: Number of PF versus Probability UEs with same PF

| N | UE_ID mod N | % UEs have same paging frame |
| --- | --- | --- |
| 1 | 0 | 100% |
| 2 | 0,1 | 50% |
| 4 | 0,1,2,3 | 25% |
| 8 | 0,1,2,…,7 | 12.5% |
| 16 | 0,1,2,…,15 | 6.75% |

In scenarios where the number of paging frames is limited or can be configured to be a small number, introducing large number of UE-groups would be clearly beneficial. The advantage is that the network can have more flexibility and the number of UEs being paged unnecessarily can be further reduced. In terms of the maximum supported number of UE-groups, we propose to maximize the benefit of the UE-group mechanism. In order to limit the probability of overhearing to below 15% we propose the maximum number of UE groups supported is at least 6.

Proposal 4: The maximum number of UE groups supported is at least 6 (i.e. to maximize the benefit of UE-group mechanism, such as reducing false-paging alarm rate).

# UE MWUS Detection Operation

The common MWUS is supported in Rel-16. In such case, basically there is a dedicated MWUS to wake up all UEs assigned to the corresponding paging occasion. Common MWUS is in addition to the individual UE group MWUS. The benefit of having common MWUS is that the eNB only needs to transmit the common MWUS when eNB needs to wake up all UEs. However, from the UE point of view, this could increase UE power consumption and complexity as the UE needs to detect both common MWUS and group specific MWUS when listening to the channel for potential WUS. The design of MWUS signal in Rel-16 should consider the UE power consumption and complexity aspects. A design constraint is, for example, the UE does not have to detect twice even when no MWUS exist on the channel.

Proposal 5: On the supporting of both common MWUS and group MWUS, the MWUS sequence design shall consider UE power consumption and complexity.

MWUS grouping may support the operation where the eNB pages only one group, several groups (sub-set groups), or all groups. In RAN1 meeting #96, we concluded to further study the additionally support the feature of waking up a subset of all WUS UE groups. The information on a subset of WUS groups should be carried in group ID indication embedded in MWUS for the case of MWUS with single-seq CDM. MWUS with single-seq CDM also indicate that a MWUS resource can be assigned for multiple groups. The UE needs to identify the group ID information in order to decide whether to continue with paging reception or go to sleep. The WUS-group ID can be included in form of bitmap operation as illustrated in Table 2. Here, an example is given when the supported maximum number (Nmax) of groups is 4. It can be observed that the table size is significantly bigger in proportional to the maximum number of WUS groups (i.e. 2^Nmax – 1).

Table 2: UE-group ID with bitmap operation

| No. | Bitmap | WUS Group ID being paged |
| --- | --- | --- |
| 1 | 1111 | 1,2,3,4 |
| 2 | 1110 | 2,3,4 |
| 3 | 1101 | 1,3,4 |
| 4 | 1100 | 1,2,4 |
| 5 | 1011 | 1,2,3 |
| 6 | 1010 | 3,4 |
| 7 | 1001 | 2,4 |
| 8 | 1000 | 2,3 |
| 9 | 0111 | 1,4 |
| 10 | 0110 | 1,3 |
| 11 | 0101 | 1,2 |
| 12 | 0100 | 4 |
| 13 | 0011 | 3 |
| 14 | 0010 | 2 |
| 15 | 0001 | 1 |

It can also be observed that most of the possible operation/combination is to page subset of all WUS UE groups. The number of operations/combinations will be significantly less in case MWUS with single-seq CDM for some of the groups is not supported. Furthermore, the number of supported groups can be higher if it does not require to waking up a subset of all WUS UE groups. On the other hand, it may have some potential issues if group MWUS is only designed to page a group and all groups. There may be a case where eNB needs to wake up a subset of all WUS UE groups at the same time. This aspect should be studied in at least RAN2 (beyond RAN1).

Observation 1: There is a design trade-off on supporting maximum number of UE-groups and the need of waking up a subset of all WUS UE groups.

Proposal 6: RAN1 to send LS to RAN2 whether the feature of waking up a subset of all WUS UE groups assigned in a MWUS resource is needed.

In the last RAN1 #96bis, configurable weighting factor for UE distribution per time/freq WUS resourse was discussed. The intention was to reduce false-paging alarm rate by allowing dynamic UE distribution per WUS resource. This would be beneficial for reducing the UE power consumption. However, we consider this topic is similar to the discussion on UE grouping criterion (e.g. coverage-based, service-based, etc). Furthermore, RAN2 is already discussing similar issue (e.g. [2], [3]).

Proposal 7: UE grouping criterion and its configurability are beyond RAN1 area and it should be discussed in RAN2.

# 3	Conclusions

In this contribution, the following proposals and observation are made:

Proposal 1: Support FDM for multiplexing between legacy Rel-15 MWUS and Rel-16 MWUS.

Proposal 2: Legacy MWUS (rel.15) and group MWUS (rel.16) are not sharing the same MWUS resource (i.e. multiplex in FDD).

Proposal 3: Up to 2 orthogonal WUS resources may be configured per dimension (up to 4 orthogonal WUS resources in total)

Proposal 4: The maximum number of UE groups supported is at least 6 (i.e. to maximize the benefit of UE-group mechanism, such as reducing false-paging alarm rate).

Proposal 5: On the supporting of both common MWUS and group MWUS, the MWUS sequence design shall consider UE power consumption and complexity.

Observation 1: There is a design trade-off on supporting maximum number of UE-groups and the need of waking up a subset of all WUS UE groups.

Proposal 6: RAN1 to send LS to RAN2 whether the feature of waking up a subset of all WUS UE groups assigned in a MWUS resource is needed.

Proposal 7: UE grouping criterion and its configurability are beyond RAN1 area and it should be discussed in RAN2.

# References

- RAN1 #96bis Chairman notes, Xian, China, April 2019

- R2-1905261 LS on assistance indication for WUS grouping, Huawei, April 2019.

- R2-1907053 On supporting UE-group wake-up signal for MTC, Sony, RAN2#106, Reno, USA, May 2019.