3GPP TSG RAN WG1 Meeting #97 	R1-1907207

Reno, USA, May 13th – 17th, 2019

Source:	Sharp

Title:	Consideration on UE-group wake-up signal for Rel-16 NB-IoT

Agenda Item:	6.2.2.1

Document for:	Discussion and Decision

Introduction

In the RAN1#96bis meeting, the following agreements were made on UE-group wake-up signal for NB-IoT [1].

Agreement Up to 2 time-multiplexed WUS resources, for both legacy WUS and group WUS, may be configured. FFS whether a group WUS resource may be shared with legacy WUS or not. Agreement Group WUS location in relation to legacy WUS may be configured such that: - If one group WUS resource is configured, that group WUS resource may be configured to coincide with the legacy WUS resource or to occur immediately before the legacy WUS resource, and, - If two group WUS resources are configured, the first group WUS resource coincides with the legacy WUS resource and the second group WUS resource occurs immediately before the first group WUS resource. Agreement Group WUS sequence design is down selected between the following in RAN1#97 - Legacy WUS with shifted scrambling codes - Legacy WUS with time-frequency short OCCs - Phase shifted legacy WUS using a suitable subset of the phase increments. - Legacy WUS with Gold codes Companies are encouraged to provide evaluation results Agreement - Per default, all gaps use the same group WUS configuration regarding number of groups and group WUS resource allocation. - Optionally, eDRX gap(s) may be configured individually if separate from the DRX gap

In this contribution, we share our considerations on UE-group wake-up signal for Rel-16 NB-IoT. This contribution is revised from R1-1904865 [2]

Discussions

In the RAN1#95 meeting [3], it was agreed a common WUS is used to wake up all group WUS UEs monitoring the same WUS (time-frequency) resource if more than one UE group occupies the WUS resource and FFS: Whether to support waking up a subset of all WUS UE groups.

For the FFS, if more than one UE group occupies the same WUS resource, considering single-seq. CDM is applied for multiplexing the UE groups in the same WUS resource, a longer time is needed to wake up all the UEs in the different WUS UE groups if not support waking up a subset of all WUS UE groups, and power consumption will also be increased. So it needs to support waking up a subset of all WUS UE groups.

Proposal 1:

- Waking up a subset of all WUS UE groups is supported.

In the RAN1#96bis meeting, it was agreed up to 2 time-multiplexed WUS resources, for both legacy WUS and group WUS, may be configured and FFS whether a group WUS resource may be shared with legacy WUS or not. For the FFS, we think it depends on whether only 1 WUS resource can be configured. If yes, then a group WUS resource should be shared with legacy WUS on the same WUS resource. If 2 time-multiplexed WUS resources are configured, it is better to configure whether a group WUS resource is shared with legacy WUS or not by eNB depending on the number of legacy UEs.

Proposal 2:

- A group WUS resource should be shared with legacy WUS if only 1 WUS resource can be configured.

- Whether a group WUS resource is shared with legacy WUS or not can be configured by eNB if 2 time-multiplexed WUS resources are configured.

In the RAN1#95 meeting, it was agreed the number of UE groups is configurable and broadcasted in SIB and FFS: Further details on the number of UE groups. In Rel-14 [4], a weight of paging carrier was introduced to determine the UE’s paging carrier, i.e., the UEs are not evenly allocated on the paging carriers. The purpose for the introduction of paging carrier weights is to achieve load balancing. The similar consideration can be applied to determine the number of UE groups, i.e., the number of UE groups for a paging carrier is related to the weight of the paging carrier.

Proposal 3:

- The weight of a paging carrier can be applied to determine the number of UE groups for the paging carrier.

Conclusion

In this contribution, we have the following proposals:

Proposal 1:

- Waking up a subset of all WUS UE groups is supported.

Proposal 2:

- A group WUS resource should be shared with legacy WUS if only 1 WUS resource can be configured.

- Whether a group WUS resource is shared with legacy WUS or not can be configured by eNB if 2 time-multiplexed WUS resources are configured.

Proposal 3:

- The weight of a paging carrier can be applied to determine the number of UE groups for the paging carrier.

References

Chairman’s notes in RAN1#96bis, Xi’an, China, April 8 – 12, 2019.

R1-1904865, “Consideration on UE-group wake-up signal for Rel-16 NB-IoT”, Sharp, RAN1#96bis, Xi’an, China, April 8 – 12, 2019.

Chairman’s notes in RAN1#95, Spokane, USA, November 12 – 16, 2018.

3GPP TS 36.304 V15.2.0 (2018-12).