3GPP TSG RAN WG1 Meeting #97	R1-1907218

Reno, USA, May 13th – 17th, 2019

Source:	Sharp

Title:	Resource allocation mode 2 for NR sidelink

Agenda Item:	7.2.4.2.2

Document for:	Discussion and Decision

Introduction

In RAN#83, a Rel-16 work item on NR V2X was approved [1]. One of the objectives of the NR V2X WI is to specify sidelink resource allocation mode 2.

- Resource allocation [RAN1, RAN2] - Mode 1 - NR sidelink scheduling by NR Uu and LTE Uu as per the study outcome - Mode 2 - Sensing and resource selection procedures based on sidelink pre-configuration and configuration by NR Uu and LTE Uu as per the study outcome - Support for simultaneous configuration of Mode 1 and Mode 2 for a UE - Transmitter UE operation in this configuration is to be discussed after the design of mode 1 only and mode 2 only. - Receiver UE can receive the transmissions without knowing the resource allocation mode used by the transmitter UE. - UE relaying resource pool configuration or resource configuration is not supported in this work in Rel-16.

In this document, we share our views on a few aspects of resource allocation mode 2 for NR sidelink.

This document is a revision of R1-1905393 [2].

Discussion

Resource reservation

The following agreements related to resource reservation were made in RAN1#96bis [3].

Agreements: - NR V2X supports an initial transmission of a TB without reservation, based on sensing and resource selection procedure - NR V2X supports reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB, based on sensing and resource selection procedure - This functionality can be enabled/disabled by (pre-)configuration - FFS Standalone PSCCH transmissions for resource reservations are supported in NR V2X

Resource reservation for potential retransmission based on HARQ-ACK

According to RAN1 agreements during SI phase, reservation of sidelink resources for blind retransmission of a TB is supported, while whether reservation of sidelink resources for potential retransmission of a TB is supported is still pending a decision.

Regarding the case of potential retransmission, our understanding is that this is necessary at least in the case of low latency services where it may be risky to assume that resources for the potential retransmission are always available after decoding the HARQ-ACK and deciding that retransmission is necessary.

Proposal 1: NR V2X Mode-2 supports reservation of sidelink resources for potential retransmission of a TB based on HARQ-ACK at least in some scenarios.

Assistance information

In LTE, each UE announces its reserved resources, and a given UE may have to successfully decode all other UEs’ SCI in order to have a full picture of the reserved resources on the sidelink. However, considering the time-varying radio conditions and the UE’s processing capability and different UE’s geo-locations, it may be very difficult for any UE to successfully decode all other UEs’ SCIs over a given time window, leading to different understandings of the overall reserved resources from the UEs operating on the sidelink, and consequently potential clashes in the “reserved resources” announced by different UEs.

The above issue can be alleviated by some means of “assistance information transmission”. For example, in case of unicast and groupcast, a receiver UE can inform the corresponding transmitter UE a set of reserved resources as the result of the receiver UE’s sensing procedure. And for another example, a UE when announcing reserved resources (e.g. via SCI) can simultaneously announce the result of its sensing procedure. In this way, any given UE can quickly learn all reserved resources on the sidelink by successfully decoding only one or a few other UEs’ SCIs containing resource reservation information. In fact, the resource reservation information in SCI from UE A can be a combination of “resources reserved by UE A” and “resources reserved by all other UEs as observed by UE A”, as UEs other than UE A, when reading resource reservation information in the SCI, do not actually care about which part of the combination will be used by which UE.

Proposal 2: Resource reservation information in SCI from a UE contains a combination of resources reserved by the UE and resources reserved by all other UEs as observed by the UE.

Sensing and resource (re-)selection

The following agreements made during the SI phase are related to sensing and resource (re-)selection [4],

RAN1 NR AH 1901 agreements: - ● Mode-2 supports the sensing and resource (re)-selection procedures according to the previously agreed definitions. - ♦ FFS resource granularity for sensing & resource (re)-selection, e.g., PRB(s), slots, resource patterns (when applicable), etc. - ♦ FFS detailed conditions when these procedures can apply

Resource granularity for sensing

Sensing by means of SCI decoding means a UE should attempt to decode all SCIs transmitted on the sidelink. Therefore, UE complexity is a major concern when designing the sensing procedure.

In order to alleviate the issue of UE complexity, the resource granularity for sensing should be carefully dimensioned. For example, similarly to LTE V2X, the frequency resource granularity of a “PSCCH candidate” should be one (or even more) sub-channels, each containing a few RBs. And the time-domain granularity can be one slot. Furthermore, the starting slots and sub-channels of PSCCH candidates should not be arbitrary. Both should be (pre-) configurable.

Proposal 3: The granularity for sensing is one or a few sub-channels in the frequency domain and one slot in the time domain.

Proposal 4: The starting slots and starting sub-channels of PSSCH candidates are (pre-) configured.

Whether to support sensing with granularity of resource patterns is not decided during SI phase. From our perspective, the resource pattern with periodicity is beneficial at least for periodic traffic in NR V2X. When UE selects the resource pattern based on its own sensing procedure, the periodical resource pattern can be easily utilized for e.g. future TB(s). Meanwhile, design of resource patterns may alleviate half-duplex constraint.

Proposal 5: Consider to support sensing with granularity of resource patterns.

Conclusion

In this contribution, we discuss some issues on resource allocation mode 2 in NR sidelink, and make the following proposals:

Proposal 1: NR V2X Mode-2 supports reservation of sidelink resources for potential retransmission of a TB at least in some scenarios.

Proposal 2: Resource reservation information in SCI from a UE contains a combination of resources reserved by the UE and resources reserved by all other UEs as observed by the UE.

Proposal 3: The granularity for sensing is one or a few sub-channels in the frequency domain and one slot in the time domain.

Proposal 4: The starting slots and starting sub-channels of PSSCH candidates are (pre-) configured.

Proposal 5: Consider to support sensing with granularity of resource patterns.

References

RP-190766, “New WID on 5G V2X with NR sidelink”, LG Electronics, Huawei, 3GPP RAN#83.

R1-1905393, “Resource allocation mode 2 for NR sidelink”, Sharp, 3GPP RAN1#96bis.

Chairman’s notes, 3GPP RAN1#96bis.

Chairman’s notes, 3GPP RAN1 AH1901.